# 3GPP TSG-RAN WG3 Meeting #129															R3-255297

# Bengaluru, India, 25 – 29 August 2025

Agenda item:	21.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 37.340) Discussion on the remaining issues for Rel-19 XR

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses the remaining issues, i.e., RLC enhancements, Alternative QoS with PDU Set QoS parameters, Format of MMSID, and Support of exposure of available data rate.

# 2	RLC enhancements

RAN2 agreed following RLC enhancements:

For operation of RLC AM, the following improvements are introduced: - To avoid unnecessary RLC retransmissions: - On the transmitter side, when the RLC entity receives a discard indication for an SDU from PDCP, it immediately stops any further transmission or retransmission of that SDU and its corresponding segment(s) if any; - On the receiver side, a complete PDU detected as missing can be abandoned after a configured duration, and the transmitter is then notified through a status report. Editor’s Note: SR was agreed, but FFS whether it is usual SR or some changes are needed. Might need some updates in the future. Terminology to be aligned with RLC (“abandoned”, “obsolete”…) - To ensure timely RLC retransmissions, when the remaining time before discard of an RLC SDU: - Falls below a configured retransmission threshold, a retransmission may be triggered; and/or - Falls below a configured polling threshold, polling is triggered.

For avoiding unnecessary RLC retransmissions, RAN3 agreed

Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement.

FFS on whether need modifications to TS 38.425

In the RAN2 specifications:

- no change was introduced to the discard indication from PDCP to RLC;

- stopping the transmissions of an RLC SDU based on the discard indication from PDCP is an RLC-internal decision that PDCP will not know about.

Observation 1-1:	Even in the RAN2 specifications, PDCP is not impacted by whether or not RLC will stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU.

Similarly, the CU-UP need not “indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU”, it only needs to indicate the discard as before.

Proposal 1-1:	No RAN3-specification impact from the agreement to reuse the existing PDCP-discard indication in F1-U.

For timely RLC retransmissions,

- For UL, RAN2#130 agreed:

The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

Accordingly, the UE need to be configured with the related PDCP parameter according to RAN2 TS 38.331 Running CR:

[[ remainingTimeBasedReTxThreshold-r19 RLC-AM-RemainingTimeThreshold-r19 OPTIONAL, -- Cond RLC-AM remainingTimeBasedPollingThreshold-r19 RLC-AM-RemainingTimeThreshold-r19 OPTIONAL -- Cond RLC-AM ]]

Observation 1-2:	No enhancement is needed for UL.

- For DL, there are different options for gNB-DU to trigger timer-based RLC retransmission or polling.

- Option 1: the gNB-CU checks the remaining time, and informs gNB-DU “for PDCP PDU #Y, retransmit/poll now”.

-	the gNB-CU-CP determines the thresholds for triggering timely RLC retransmission (including autonomous / timer-based retransmission and enhanced polling). The gNB-CU-CP sends the thresholds to the gNB-CU-UP.

- 	when the remaining time of an PDCP SDU falls below the threshold, the gNB-CU-UP sends a timely retransmission/poll indication for the PDCP SDU to the gNB-DU.

-	the gNB-DU performs timely RLC retransmission or poll.

- Option 2: the gNB-DU checks the remaining time, e.g. gNB-CU informs gNB-DU “retransmit/poll X ms after receiving this PDCP PDU”, the gNB-DU checks the remaining time.

- Option 3: Based on the PDCP thresholds, the gNB-CU configures the gNB-DU with duration timers that are started for a PDCP PDU when received over F1-U and whose expiry triggers the retransmission or polling.

- 	the gNB-CU-CP determines the thresholds for triggering timely RLC retransmission (including autonomous / timer-based retransmission and enhanced polling). Based on the discard timer, the gNB-CU-CP derives the retransmit time value and poll timer value, and provides them to gNB-DU.

- 	when a PDCP SDU is received, the gNB-DU starts the retransmission timer, and/or poll timer.

- 	When the respective timer expires, the gNB-DU performs timely RLC retransmission or poll.

Option 1 requires enhancement to both E1AP and F1-U. Option 1 can result in a lot of additional F1-U signaling for indicating the retransmission/poll (e.g. send a retransmision/poll indication even in case of no DL PDCP SDU to be sent to gNB-DU).

Option 2 results in a F1-U header field whose value should be set just before the actual time of transmission. In certain scenarios that a separate Security gateway is used to protect the F1-U traffic, it may be difficult for the gNB-CU-UP to know the precise time for the delay in lower layers (e.g. delay in lower layer, delay in the transmission to the separate security gateway, etc), so it may be a challenge for the gNB-CU-UP to add an accurate time in the GTP-U header before passing it to lower layer/security gateway.

Option 3, despite not taking into account different remaining times of PDCP PDUs received over F1-U, seems to avoid the drawbacks of the other options due to its simplicity.

In a summary, Option 3 is preferred.

Proposal 1-2:	For timer-based RLC retransmission or polling in the DL by gNB-DU, the gNB-CU configures the gNB-DU with duration timers that are started for a PDCP PDU when received over F1-U and whose expiry triggers the retransmission or polling.

TP for F1AP can be found in (4).

# 3	Alternative QoS with PDU Set QoS parameters

Last RAN3 meeting agreed TS 37.340 TP to clarify that the notification control in MR-DC is updated to cover the PSER/PSDB (TP is copied below)

### 10.13.2	MR-DC with 5GC

The Notification Control Indication procedure may be initiated either by the MN or by the SN and is used to indicate that GFBR/PSER/PSDB for one or several QoS flows cannot be fulfilled any more or can be fulfilled again by the reporting node.

However, current TS 38.423 (coped as below) only support the notification control when GFBR cannot be fulfilled or can be fulfilled, but not for PDB/PER.

### 8.3.10 Notification Control Indication

8.3.10.1 General

The purpose of the Notification Control indication procedure is to provide information that for already established GBR QoS flow(s) for which notification control has been requested, the NG-RAN node involved in Dual Connectivity cannot fulfil the GFBR anymore or that it can fulfil the GFBR again.

…

This procedure is triggered to notify the S-NG-RAN node for SN-terminated bearers, that resources requested from the M-NG-RAN node can either not fulfil the GFBR anymore or that the GFBR can be fulfilled again, as specified in TS 7.340 [8].

It would be very difficult to measure whether PSER/PSDB can be fulfilled or not fulfilled in MR-DC, especially for split bearer. So we propose to align with current Xn Notification Control, and not support the monitoring of PSER/PSDB in MR-DC.

Proposal 2: remove the PSDB/PSER in the notification control for MR-DC.

TP to BL CR for TS 37.340 can be found in Annex A – TP to BL CR for TS 37.340

# 4	Format of MMSID

Current RAN3 BL CRs define MMSID as 1-Octet. But CT3/CT4 spec defines MMSID as a string.

TS 29.514:

MultiModalId 		string 		Contains a multi-modal service identifier.

Although using a 1-Octet may be simple for the implementation, it requires the AMF to perform the “translation” between the RAN and other CN nodes. We would like to discuss whether need to align between RAN3 specifications and CT specifications.

Proposal 3: For MMSID, RAN3 discuss whether using the STRING format defined in CT3 specification also for RAN3 specifications.

# 5	Support of exposure of available data rate

In last RAN3 meeting, RAN3 could not reach agreement on whether the gNB need to report the status of Available Bitrate monitoring. RAN3 agreed a LS ([4]) to SA2, asking SA2 to provide feedback on the introduction of a status information for Available Bitrate monitoring from gNB to SMF. Even we hope SA2 can provide a reply LS during RAN3#129, it is unclear whether SA2 can confirm the necessity of reporting the status. In case no confirmation from SA2, we propose to not introduce it in Rel-19 considering it is the last meeting for the WI.  If needed, it can be treated as TEI-19 in future meetings.

Proposal 4: In case no confirmation from SA2 on gNB report the status of Available Bitrate monitoring, it is proposed to not introduce the report of status.

# 6	Conclusion

In this contribution we have analyzed the remaining issues. Our proposals are:

Proposal 1-1:	No RAN3-specification impact from the agreement to reuse the existing PDCP-discard indication in F1-U.

Proposal 1-2:	For timer-based RLC retransmission or polling in the DL by gNB-DU, the gNB-CU configures the gNB-DU with duration timers that are started for a PDCP PDU when received over F1-U and whose expiry triggers the retransmission or polling.

TP for F1AP can be found in (4).

Proposal 2: remove the PSDB/PSER in the notification control for MR-DC.

TP to BL CR for TS 37.340 can be found in Annex A – TP to BL CR for TS 37.340

Proposal 3: For MMSID, RAN3 discuss whether using the STRING format defined in CT3 specification also for RAN3 specifications.

Proposal 4: In case no confirmation from SA2 on gNB report the status of Available Bitrate monitoring, it is proposed to not introduce the report of status.

# References

- R3-253773, Summary of offline discussion on CB: # R19XR

- R3-255101, (BL CR to 37.340) Support of XR enhancements

- R3-255422, (TP for BL CR for TS 38.473) enhancements to support timely RLC retransmissions

- R3-253929, LS to SA2 on reporting status of Exposure of available bitrate

# Annex A – TP to BL CR for TS 37.340

### 10.13.2	MR-DC with 5GC

The Notification Control Indication procedure may be initiated either by the MN or by the SN and is used to indicate that GFBR for one or several QoS flows cannot be fulfilled any more or can be fulfilled again by the reporting node.

Figure 10.13.2-1: Notification Control Indication procedure

Figure 10.13.2-1 shows an example signalling flow for the Notification Control Indication procedure.

1.	The MN may, for an SN terminated bearer, indicate, that the GFBR requested from the MN cannot be fulfilled anymore.
In case the SN terminated bearer is configured as a split bearer, the SN may decide to increase the share provided by the SN or it may decide to notify the MN that resources requested for the SN terminated bearer cannot fulfill the GFBR any more.

2.	Continuing the example message flow from step 1, the SN informs the MN that the GFBR for an SN terminated bearer cannot be fulfilled any more.

3.	The MN decides to inform the 5GC that NG-RAN cannot fulfill the GFBR for a GBR QoS flow any more.