3GPP TSG-RAN WG3 Meeting #129	R3-255346

Bangaluru, IND, 25th – 29th August, 2025

Agenda item:	16.2

Source: 	Samsung

Title: 	(TP to BL CR for 38.413 and 38.401) Discussion on general issues for AIoT

Document for:	Discussion and decision

Introduction

Last meeting discussed the general aspect, and the following agreements and open issues are captured,

Define the Expected D2R Message Size IE in Inventory Request Transfer IE. FFS on the detail encoding.

Mandatorily include the Expected D2R Message Size in Inventory Request Transfer? Check progress in SA2.

Remove the FFSs on the presence of Cause IE in the INVENTORY FAILURE message and the COMMAND FAILURE message.

FFS on the detail encoding of the A-IoT Correlation Identifier IE? Wait for further inputs from CT4.

FFS on the detail encoding of the AIOTF Identifier IE? Refer to the NfInstanceId IE defined in TS 29.571?

Define the A-IoT Device Identification Requested IE as CHOICE type, with three branches:

- single device inventory:	OCTECT STRING	refer to Device Identifier

- group devices inventory:	FFS on the encoding

- all devices inventory:	NULL

Define the Estimate of Expected D2R Message Size IE in the Command Assistance Information IE as INTEGER (1..128, …) unit: byte. Add the semantic description that this IE refers to NAS PDU size.

Reader Selection refers to SA2 TS 23.369 clause 5.3.3. Further check is needed.

It is FFS to introduce Command Type (write, read, disable) in Command Request Transfer.

It is FFS on the Time Interval as assistance information in the Inventory Request message. Clarify with SA2 on the usage of this information in NG-RAN node.

WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …)

Whether to have A-IoT Support Indicator in the NG SETUP RESPONSE message?

In this contribution, we further discuss these leftover issues.

Discussion

Last meeting continued to discuss the general aspects for AIoT, and a bunch of open issues are captured.

The first open issue is about the encoding of Expected D2R Message Size IE,

Define the Expected D2R Message Size IE in Inventory Request Transfer IE. FFS on the detail encoding.

Mandatorily include the Expected D2R Message Size in Inventory Request Transfer? Check progress in SA2.

More specifically, the FFS in the following tabular in BLCR to 38.413 should be resolved,

| Expected D2R Message Size | M[FFS] |  | FFS | Unit: FFS. This IE indicates the size of the A-IoT NAS PDU(s) carried in Inventory Report. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

It should be noted that the encoding of the Expected D2R Message Size IE in Command Assistance Information has been agreed as follows,

Define the Estimate of Expected D2R Message Size IE in the Command Assistance Information IE as INTEGER (1..128, …) unit: byte. Add the semantic description that this IE refers to NAS PDU size.

As a result, the encoding of the same IE in Inventory Request message can be similar in principle.

Based on RAN2 agreement, the expected D2R message size should be byte aligned, so it is fine to set the Unit as Byte, and the IE type can be an Integer.

In addition, the expected D2R message size for inventory procedure depends on the Device ID length as well as information related to device authentication.

As can be observed from the ID structure defined in 23.003, the maximum length of both single device ID and group device ID does not exceed 256 bits, i.e. 32 bytes in current spec.

In addition, the information related to device authentication has been defined in 33.369, wherein the RAND and RES are also sent within the D2R message for inventory procedure. Normally, the length of both RAND and RES is 128 bits, i.e. 16 bytes for each.

As a result, the maximum length of D2R message for inventory procedure defined for now is 32+16+16=64 bytes. For future proofness, it is enough to define the expected D2R message size in Inventory Request as 128 bytes.

Moreover, as provided in reply LS from SA2 in R3-255021, the following information is provided,

SA2 thanks RAN2 for their LS on D2R message size for inventory response. SA2 has discussed RAN2’s assumption and can confirm that the inventory response message size will be provided as part of assistance information for Inventory procedure. SA2 has agreed pCR S2-2505848 for this purpose. The attached pCR also reflects other changes in assistance information.

It can be observed from the LS that the expected D2R message size for inventory procedure will be provided mandatorily.

Proposal 1: Define the Estimate of Expected D2R Message Size IE in Inventory Request Transfer IE as INTEGER (1..128, …) unit: byte. Confirm that the Estimate of Expected D2R Message Size IE is mandatorily provided.

The next open issue is about the encoding of Correlation Identifier IE,

FFS on the detail encoding of the A-IoT Correlation Identifier IE? Wait for further inputs from CT4.

It seems that neither CT4 nor SA2 has agreed anything on the detailed encoding of the Correlation ID. The best we can do in our NGAP spec for now is to define the IE type of Correlation ID as OCTET STRING without size.

Since the detailed encoding of the Correlation ID is still unclear in CT4/SA2, we are only able to capture some basic semantic description in our NGAP spec for now. And we may add more reference in the semantic description when CT4/SA2 achieve more detailed encoding in future meetings.

Proposal 2: Define Correlation Identifier IE as OCTET STRING. Describe it as ‘The identifier generated by AIOTF corresponding to the service request’.

The next open issue is about the encoding of AIOTF Identifier IE,

FFS on the detail encoding of the AIOTF Identifier IE? Refer to the NfInstanceId IE defined in TS 29.571?

Similar situation as Correlation Identifier, i.e. CT4/SA2 has not provided more detailed encoding of the AIOTF identifier. So we think currently defining it as OCTET STRING, and refer to NfInstanceId IE defined in TS 29.571 is the best we can do for now. And we may update the semantic description when CT4/SA2 provides more detailed encoding in future.

Proposal 3: Define AIOTF Identifier IE as OCTET STRING, and refer to the NfInstanceId IE defined in TS 29.571.

The next open issue is about the encoding of device Identifier IE,

Define the A-IoT Device Identification Requested IE as CHOICE type, with three branches:

- single device inventory:	OCTECT STRING	refer to Device Identifier

- group devices inventory:	FFS on the encoding

- all devices inventory:	NULL

As defined in 23.003, the structure of single device ID is defined as follows,

And the structure of group device ID (i.e. Filtering Information) is defined as follows,

It can be observed from above that neither single device id nor group device id is byte aligned, so it is better to define these Identities as BITSTRING. In addition, in order to be future proof, the size of the BITSTRING can be set as 1024.

Proposal 4: Define both Single Device Identity and Group Device Identity as BITSTRING(SIZE(1024)).

The next open issue is,

It is FFS to introduce Command Type (write, read, disable) in Command Request Transfer.

We think it could be beneficial for the gNB to understand the command type in some circumstances, e.g. the Write Command may take device more time to process so the D2R transfer containing the reply to the Write Command may not happen immediately. So the gNB can perform more efficient resource allocation with the help of such information.

Proposal 5: Introduce Command Type (write, read, disable, …) in Command Request Transfer.

The next open issue is,

It is FFS on the Time Interval as assistance information in the Inventory Request message. Clarify with SA2 on the usage of this information in NG-RAN node.

We observe that current TS 23.369 has clearly defined the usage of this Time Interval IE as follows,

5.9	AIoT Service Operation Result Aggregation

An AIoT service operation may involve many AIoT Devices. The NG-RAN may perform AIoT service operation result aggregation with a specific Correlation ID based on the aggregation assistance information received from the AIOTF for a service operation request as specified in clause 5.4.

The AIOTF determines the aggregation assistance information based on the request from the AF or local configuration, which includes:

-	Time interval: the fixed time interval for which NG-RAN collects multiple AIoT Devices’ operation responses before reporting the aggregated AIoT response to the AIOTF. The reporting based on time interval may potentially happen multiple times until the NG-RAN completes the request operation.

If the AF has provided a time interval, then the AIOTF should signal a time interval to the NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

If the AIOTF does not provide aggregation assistance information, the aggregation process in the NG-RAN may be determined by implementation.

The AIOTF may also aggregate the results of a requested service operation before sending them to the NEF or trusted AF.

We are fine to follow SA2 design, if no severe issue is identified.

Proposal 6: Introduce Time Interval as Inventory Assistance Information.

The next open issue is,

WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …)

Whether to have A-IoT Support Indicator in the NG SETUP RESPONSE message?

Firstly, we think the AIoT Support IE in NG Setup Request defined in current BL CR to 38.413 is clear enough, which will not cause any misunderstanding to AIoT CN, so we suggest to turn the WA into agreement.

Secondly, for the open issue, we think the intention was to add such information for indirect deployment; however, we think the information on whether the AMF has connectivity to AIOTF can be directly known via OAM configuration, and there’s no need to do any enhancement in NG SETUP RESPONSE message.

Proposal 7: No need to have A-IoT Support Indicator in the NG SETUP RESPONSE message.

The last open issue is about the BL CR to 38.401, especially on the following paragraphs in Sub-clause 6.2.1,

For UE-associated signalling, an Application Protocol Identity (AP ID) is allocated when a new UE-associated logical connection is created in either an NG-RAN node or an AMF. An AP ID shall uniquely identify a logical connection associated to a UE or A-IoT device over the NG interface or Xn interface within a node (NG-RAN node or AMF) or over the F1 interface or over the E1 interface or over the W1 interface. Upon receipt of a message that has a new AP ID from the sending node, the receiving node shall store the AP ID of the sending node for the duration of the logical connection. The receiving node shall assign the AP ID to be used to identify the logical connection associated to the UE and include it as well as the previously received new AP ID from the sending node, in the first returned message to the sending node. In all subsequent messages to and from sending node, both AP IDs of sending node and receiving node shall be included.

For MBS-associated logical connections of the E1 interface and the F1 interface the same principles for AP IDs apply as for UE-associated logical connections.

For A-IoT command operation related signalling, per A-IoT session (corresponding to an AIOTF identifier and a Correlation ID) and per A-IoT device, a RAN A-IoT Device NGAP ID to be used via the NG-C interface are introduced, as specified below.

In the first paragraph, the ‘or A-IoT device’ is added to state the fact that an AP ID shall uniquely identify a logical connection associated to an A-IoT device within a NG-RAN node, which is true.

However, after adding ‘For UE-associated signalling,’ in the first paragraph, the whole paragraph seems to describe the case for UE-associated signalling (and the last paragraph is newly added to specifically describe the case for A-IoT command operation related signaling), and actually the RAN A-IoT Device NGAP ID is only used in Non UE-associated signalling. In addition, we do not adopt the concept of logical connection for A-IoT device. So we suggest to remove the ‘or A-IoT device’ in the first paragraph.

Proposal 8: Agree TPs for 38.413 and 38.401 as in Annex.

Conclusion

In this paper, we provide further discussion on general aspects for Ambient IoT. The following proposals are provided,

Proposal 1: Define the Estimate of Expected D2R Message Size IE in Inventory Request Transfer IE as INTEGER (1..128, …) unit: byte. Confirm that the Estimate of Expected D2R Message Size IE is mandatorily provided.

Proposal 2: Define Correlation Identifier IE as OCTET STRING. Describe it as ‘The identifier generated by AIOTF corresponding to the service request’.

Proposal 3: Define AIOTF Identifier IE as OCTET STRING, and refer to the NfInstanceId IE defined in TS 29.571.

Proposal 4: Define both Single Device Identity and Group Device Identity as BITSTRING(SIZE(1024)).

Proposal 5: Introduce Command Type (write, read, disable, …) in Command Request Transfer.

Proposal 6: Introduce Time Interval as Inventory Assistance Information.

Proposal 7: No need to have A-IoT Support Indicator in the NG SETUP RESPONSE message.

Proposal 8: Agree TPs for 38.413 and 38.401 as in Annex.

Annex A: TP to BL CR for TS 38.413

9.3.3.xx1	A-IoT Correlation Identifier

This IE uniquely identifies an A-IoT session within an AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | OCTET STRING | The identifier generated by AIOTF corresponding to service request. |

9.3.3.xx2	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identifier | M |  | OCTET STRING | Coded as the NfInstanceID IE defined in TS 29.571 [35]. |

9.3.3.xx3	Requested Service Area Information

This IE includes the Requested Service Area Information for the A-IoT inventory.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Reader List |  | 0..1 |  |  |
| >Requested Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| Requested A-IoT Area List |  | 0..1 |  |  |
| >Requested A-IoT Area Item |  | 1..<maxnoofAIoTAreas> |  |  |
| >>A-IoT Area ID | M |  | 9.3.1.aa2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers. Value is 65535. |
| maxnoofAIoTAreas | Maximum no. of A-IoT Areas. Value is 256. |

9.3.3.xx4	Inventory Assistance Information

This IE includes the assistance information for the A-IoT Inventory Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Approximate Number of Target A-IoT Devices | O |  | INTEGER (1..2048, …) |  |
| Time Interval | O |  | INTEGER (1..600, …) | Unit: seconds. As defined in TS 23.369 [z]. |

9.3.3.xx5	A-IoT Device Identification Requested

This IE represents the Requested Identity of the A-IoT device(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Device Identification for Paging | M |  |  |  |
| >Single Device |  |  |  |  |
| >>Single Device Identity | M |  | BITSTRING (SIZE(1024)) | Includes the AIoT device Identifier defined in TS 23.369 [z]. |
| >Group Devices |  |  |  |  |
| >>Group Device Identity | M |  | BITSTRING (SIZE(1024)) | Includes the Filtering Information defined in TS 23.369 [z]. |
| >All Devices |  |  | NULL |  |

--<< Skip unchanged>>--

9.3.x	AIOTF Related IEs

9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.xx5 |  | YES | reject |
| Requested Service Area Information | M |  | 9.3.3.xx3 |  | YES | reject |
| Inventory Assistance Information | O |  | 9.3.3.xx4 |  | YES | reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | reject |
| Expected D2R Message Size | M |  | INTEGER (1.. 128, ...) | Unit: Byte. This IE indicates the size of the A-IoT NAS PDU(s) carried in Inventory Report. | YES | reject |

--<< Skip unchanged>>--

9.3.x.5	Command Request Transfer

This IE provides the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  | YES | reject |
| A-IoT NAS PDU | M |  | OCTET STRING |  | YES | reject |
| Command Type | O |  | ENUMERATED (write, read, disable, ...) |  | YES | ignore |
| Command Assistance Information | O |  | 9.3.3.yy3 |  | YES | reject |

Annex B: TP to BL CR for TS 38.401

6.2.1	Principle of handling Application Protocol Identities

For UE-associated signalling, an Application Protocol Identity (AP ID) is allocated when a new UE-associated logical connection is created in either an NG-RAN node or an AMF. An AP ID shall uniquely identify a logical connection associated to a UE over the NG interface or Xn interface within a node (NG-RAN node or AMF) or over the F1 interface or over the E1 interface or over the W1 interface. Upon receipt of a message that has a new AP ID from the sending node, the receiving node shall store the AP ID of the sending node for the duration of the logical connection. The receiving node shall assign the AP ID to be used to identify the logical connection associated to the UE and include it as well as the previously received new AP ID from the sending node, in the first returned message to the sending node. In all subsequent messages to and from sending node, both AP IDs of sending node and receiving node shall be included.

For MBS-associated logical connections of the E1 interface and the F1 interface the same principles for AP IDs apply as for UE-associated logical connections.

For A-IoT command operation related signalling, per A-IoT session (corresponding to an AIOTF identifier and a Correlation ID) and per A-IoT device, a RAN A-IoT Device NGAP ID to be used via the NG-C interface are introduced, as specified below.