3GPP TSG-RAN WG3 #129	R3-255406

Bangaluru, India, 25th – 29th August 2025

Agenda Item:	20.2

Source:	Lenovo

Title: 		Logged Data Handling During Handover

Document for:	Discussion and Approval

# Introduction

In this paper, we discuss the NW side data collection issue raised by RAN2 in LS R3-255016.

# Discussion

R3-255016 Logged Data Handling During Handover Regarding the handling of logged data collected for NW-side AI/ML model training at handover RAN2 has made the following agreements: At RAN2#127bis: UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network. At RAN2#129: - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. At RAN2#130: - (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced. In case of HO, the source sends the 1-bit indication to target cell in HandoverPreparationInformation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data). We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it. Notify RAN3 - The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback. RAN2 would like to clarify that the data collection configuration could originate in the source gNB or from OAM. RAN2 would also like to clarify that the purpose of data availability indication in RRCReconfigurationComplete message sent during handover is for the target gNB to be able to fetch the collected data from the UE. RAN2 respectfully asks RAN3 to take the agreement into account, and provide concern, if any, on RAN2 agreement.

The scenario described by the RAN2 LS can be summarized into the following:

- Source gNB has configured the UE to perform data measurement and logging

- UE has started data measurement and logging accordingly and there is data available in UE’s AS buffer that has not been reported to the source gNB yet.

- A L3 inter-gNB handover is triggered, and in the HandoverPreparationInformation message sent from the source gNB to target gNB, it will be indicated if target gNB shall further retrieve and forward the logged data in UE buffer after the handover.

- Target gNB shall include the same indication in the HandoverCommand for the UE, so UE shall retain the logged data in its buffer accordingly.

- After successful handover, the target gNB will retrieve the logged data in UE’s buffer and further transmit the data to the source gNB via Xn interface.

Considering the 1-bit indication is in HandoverPreparationInformation which is further conveyed in the HANODVER REQUEST Xn message, and the same indication will be further included in the HandoverCommand which is further conveyed in the HANDOVER REQUEST ACKNOWLEDGE message, it seems existing Handover procedure can already support such scenario.

Legacy Handover Preparation procedure can already support the transfer of retaining-logged-data indication.

On the other hand, after the handover, when target gNB needs to transfer the retrieved UE logging data to the source gNB, RAN3 needs to discuss which Xn interface message shall be used, e.g., ACCESS AND MOBILITY INDICATION, or DATA COLLECTION UPDATE.

Besides, if the data collection is initiated by OAM, the target gNB may directly transfer the retrieved UE data to OAM instead of source gNB.

Enhancement to Xn interface message is required for target gNB to transfer the retrieved UE data after handover back to source gNB.

However, given that in Rel19 RAN3 is not assigned with any TU for data collection topic in AI for air interface, RAN3 is suggested to not seeking any conclusion by Rel19 and fix this issue in Rel20 time frame, e.g., TEI.

RAN3 is suggested to resolve the logged UE data transmission from target gNB to source gNB or OAM after handover in Rel20 time frame, e.g., TEI.

RAN3 replies RAN2’s LS and explains the situation.

# Conclusion

Based on the discussion above, we observe:

Observation 1	Legacy Handover Preparation procedure can already support the transfer of retaining-logged-data indication.

Observation 2	Enhancement to Xn interface message is required for target gNB to transfer the retrieved UE data after handover back to source gNB.

Based on the discussion above, we propose:

Proposal 1	RAN3 is suggested to resolve the logged UE data transmission from target gNB to source gNB or OAM after handover in Rel20 time frame, e.g., TEI.

Proposal 2	RAN3 replies RAN2’s LS and explains the situation.