3GPP TSG RAN WG3 Meeting #129	R3-255478

Bengaluru, IN, 25-29 August 2025

Title: 	(TP to 37.320) - Continuous MDT collection targeting the same UE across RRC states

Source: 	Ericsson, ZTE Corporation, BT, Deutsche Telekom, InterDigital, Jio Platforms

Agenda item:	11.4

Document Type:	Discussion and agreement

# 1. Introduction

During the last RAN3#128 meeting progress has been achieved and the following agreements were taken:

Information from OAM to participating nodes in the NG-RAN allowing for the identification of a continuous MDT process are provided in the form of specific MDT TR(s).

In the most obvious solution, this implies that specific TRs are allocated for Continuous MDT and the RAN knows that these TRs are specific for the continuous MDT process.

Solution for continuous MDT shall not have an impact on the 5GC in R19 from RAN3 perspective.

This is an important statement that removes solutions involving the AMF.

Agree to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem.

This agreement can derive in different solutions, but one of them covers the possibility to reuse the TRSR across gNBs and RRC states.

Furthermore, an LS to SA5 was agreed in R3-253958, capturing the agreements above and answering some questions SA5 asked RAN3 specifically.

We are aware that RAN3 is waiting for an answer from SA5 to the LS sent at the last RAN3 meeting. However, we also note that RAN3-129 is the last meeting for Rel19. 
We would like to recall that the Rel19 WI on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN in RP-251245 captures the following objective:

The detailed objectives of the WI are listed as follows: […] - Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: […] - Continuous MDT collection targeting the same UE across RRC states

We would therefore like to present TPs to TS37.320 (below) and to TS38.423 (in R3-255479) as a trigger to discussions aimed at taking the needed agreements in RAN3 to fulfil the objective above.

The TPs for Continuous MDT collection targeting the same UE across RRC states  are based on the agreements during the last RAN3#128 meeting and, anticipating the LS reply from SA5, they introduce a description of the overall solution at stage 2 level (TP to 37.320) as well as the relevant Continuous MDT flag to be signalled as part of the handover signalling, as discussed extensively in earlier meetings.

We propose that the TPs are agreed with the understanding that amendments can be made at RAN3-129bis, should there be a need for it.

Proposal: Agree to TPs to TS37.320 and TS38.423 for support of Continuous MDT with the understanding that amendments to the introduced specification can be made at RAN3-129bis

2. References

- R3-253958, Reply LS on Continuous MDT, RAN3

# 3. TP to TS37.320

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 5.1.x	Continuous MDT procedures

The Continuous data collection for management-based MDT is described as follows: a UE in the NG-RAN is configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network needs to select again the same UE for continuous MDT for subsequent MDT data collection. Such solution is based on the encoding of the Trace Reference ID signalled from the OAM to the RAN and reported by the UE in the relevant MDT logs. When the UE reports the logs, the UE reports the TR, which implicitly flag that the UE is configured for Continuous MDT, and its RAN assigned TRSR, which should be re-selected again for the continuous MDT process for the UE.

The Continuous MDT operation is provided by the following figure and steps:

Figure 5.1.x-1: Example of “continuous MDT” signalling support

- RAN receives two Management Based MDT configurations: an Immediate MDT configuration and a Logged MDT configuration which are associated to a range of TRs that identify Continuous MDT configurations. TR 1 and TR2 are assigned respectively to the Management Based Immediate MDT job and to the Management Based Logged MDT job.

- The RAN may configure the UE with Immediate MDT and logged, The Immediate MDT configuration corresponds to TR1, TRSR A and the logged MDT configuration corresponds to TR2 and TRSR A. The same TRSR is assigned to the UE during the course of the continuous MDT session.

- The UE reports Immediate MDT measurements collected while in RRC_CONNECTED and according to the Immediate MDT configuration received.

- NG-RAN1 reports the measurements corresponding to the Management Based Immediate MDT configuration for TR1 and TRSR A to the TCE.

- The UE moves to RRC_IDLE/INACTIVE. The UE will collect logged MDT measurements and store them as per received Logged MDT configuration. Logs include TR2 and TRSR A assigned by the source gNB.

- When the UE moves from RRC_IDLE/INACTIVE to RRC_CONNECTED, it reports Logged MDT logs, including TR2 and  + TRSR A. NG-RAN 2 realizes that the UE shall be configured with continuous MDT by means of reading the TR (TR2).

- NG-RAN 2 reports to the TCE the Trace Records including the logged MDT measurements. These include TR 2 and TRSR 2. By checking the TRSR the TCE can correlate the previous immediate MDT measurements (for TR 1 and TRSR A) with the logged MDT measurements (for TR 2 and TRSR A).

- Step 2, 3 and 4 are repeated at NG-RAN 2. The UE is configured with Management Based Immediate MDT with TR 1 and TRSR A and immediate MDT measurements are signalled to the TCE.  By checking the TRSR the TCE can correlate the previous logged MDT measurements (for TR 2 and TRSR A) with the immediate MDT measurements (for TR 1 and TRSR A)

- During connected mode mobility NG-RAN 2 icludes in the Handover Preparation signalling towards NG-RAN 1 TR1 and TRSR A assigned to the management based Immediate MDT process for the UE. NG-RAN 1 is able to deduce that the UE has to be configured with continuous MDT (by checking the TR) and that the same TRSR received from NG-RAN 2 shall be applied to the UE.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>