3GPP TSG-RAN WG3 Meeting #129	R3-255518

Bengaluru, India, 25 – 29 August 2025

Agenda item:	31.1

Source:	Nokia

Title:	PNI NPN with limited CAG validity

WID/SID:	TEI19

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#125 the study item of NR Femto was successfully completed and the outcome is visible in TR 38.799 which was agreed in R3-244849.

At the following RAN Plenary#105, the corresponding work item was agreed in [1] including the following objective:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

At the last RAN3#128, text proposals for stage 2 and stage 3 were agreed and the baseline CR for NR Femtos were endorsed for TS 38.300 and TS 38.413 which are almost complete.

One remaining open issue which was discussed for NR Femtos at RAN3#128 concerns temporary CAG ID validity. Because this is generic for PNI NPN it was decided at last RAN3#128 to continue the discussion under TEI19.

Also in SA2 all features related to CAG in general are referred to PNI NPN clause.

This paper explains again this issue and proposes a way forward.

# 2	Discussion

In PNI NPN, when CAG ID expires the N2 connection is currently abruptly removed if it is in use according to TS 23.501:

-	The AMF shall update the Allowed CAG list in the Mobility Restrictions towards NG-RAN if the evaluation of the time validity information of an entry in the Allowed CAG list changes between true and false, unless the AMF releases the NAS signalling connection to the UE based on operator's policy if the evaluation of the validity condition changes from true to false.

This design was made at a time when Time Validity for CAG ID had not been introduced.

With the introduction of limited duration of CAG ID validity a graceful approach is more suitable than releasing N2. For example, the gNB could be notified in order to enable performing a handover on time and avoid the complete release.

Even though applicable to all CAG ID usage and PNI NPN, this is especially true with NR femtos where SA2 TS 23.501 has specified the provisioning of CAG ID by the femto owner which can be for temporary usage. This means that temporary allowed CAG ID and occurrence of CAG ID expiry becomes “normal” case and no more exceptional. This temporary CAG ID case requires some special handling.

Proposal 1: enhance TS 38.413 for PNI NPN to inform gNB of CAG time validity before it expires in order to enable handovers before CAG expiry.

# 3	Conclusion

This paper has explored the handling of expiry of CAG ID with limited validity time and made the following proposal:

Proposal 1: enhance TS 38.413 for PNI NPN to inform gNB of CAG time validity before it expires in order to enable handovers before CAG expiry.

The above proposal is implemented in the CR in annex A.

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

# 5	Annex A: CR on CAG Validity Time for TS 38.413

3GPP TSG-RAN WG3 Meeting # 129	R3-25xxxx

# Bengaluru, India,  25 – 29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | xxxx | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CAG Validity Time [CAG_Validity] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In PNI NPN, when CAG ID expires the N2 connection is currently abruptly removed. With the introduction of limited validity duration of CAG ID in SA2 specification, a graceful approach is more appropriate than releasing N2. For example, by making gNB aware of this CAG validity time the gNB can perform a handover of the UE on time and avoid the complete release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CAG validity Time IE is sent by AMF to the gNB. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR has an impact on the PNI NPN function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Abrupt disconnection of N2 connection at expiry of CAG validity time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 9.3.3.45, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 8.3.1.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

The NG-RAN node shall consider that roaming or access to CAG cells is only allowed if the Allowed PNI-NPN List IE is contained in the INITIAL CONTEXT SETUP REQUEST message, as described in TS 23.501 [9]. If the CAG ID of the served cell was included in the Allowed PNI NPN List IE with a time validity, the NG-RAN node may consider this time validity and trigger handover before its expiry.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.45	Allowed PNI-NPN List

This IE contains information on allowed UE mobility in PNI-NPN including allowed PNI-NPNs and whether the UE is allowed to access non-CAG cells for each PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Allowed PNI-NPN Item |  | 1..<maxnoofEPLMNs+1> |  |  |
| >PLMN Identity | M |  | 9.3.3.5 |  |
| >PNI-NPN Restricted | M |  | ENUMERATED (restricted, not-restricted, …) | If set to “restricted”, indicates that the UE is not allowed to access non-CAG cells for this PLMN. |
| >Allowed CAG List per PLMN |  | 1..<maxnoofAllowedCAGsperPLMN> |  |  |
| >>CAG ID | M |  | 9.3.3.43 |  |
| >Allowed Temporary CAG List per PLMN |  | 0..<maxnoofAllowedCAGsperPLMN> |  |  |
| >>CAG ID | M |  | 9.3.3.43 |  |
| >>CAG Time Validity | M |  | INTEGER (0..65535, …) | Unit: seconds. |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs+1 | Maximum no. of equivalent PLMNs plus one serving PLMN. Value is 16. |
| maxnoofAllowedCAGsperPLMN | Maximum number of CAGs per PLMN in UE’s Allowed PNI-NPN list. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-Allowed-Temporary-CAG-List-per-PLMN,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Allowed-CAG-List-per-PLMN ::= SEQUENCE (SIZE(1..maxnoofAllowedCAGsperPLMN)) OF CAG-ID

Allowed-Temporary-CAG-List-per-PLMN ::= SEQUENCE (SIZE(1..maxnoofAllowedCAGsperPLMN)) OF CAG-ID-Temporary-Item

AllowedNSSAI ::= SEQUENCE (SIZE(1..maxnoofAllowedS-NSSAIs)) OF AllowedNSSAI-Item

AllowedNSSAI-Item ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {AllowedNSSAI-Item-ExtIEs} } OPTIONAL,

...

}

AllowedNSSAI-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Allowed-PNI-NPN-List ::= SEQUENCE (SIZE(1..maxnoofEPLMNsPlusOne)) OF Allowed-PNI-NPN-Item

Allowed-PNI-NPN-Item ::= SEQUENCE {

pLMNIdentity				PLMNIdentity,

pNI-NPN-restricted			ENUMERATED {restricted, not-restricted, ...},

allowed-CAG-List-per-PLMN	Allowed-CAG-List-per-PLMN,

iE-Extensions			ProtocolExtensionContainer { {Allowed-PNI-NPN-Item-ExtIEs} } OPTIONAL,

...

}

Allowed-PNI-NPN-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-Allowed-Temporary-CAG-List-per-PLMN	CRITICALITY ignore	EXTENSION Allowed-Temporary-CAG-List-per-PLMN		PRESENCE optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

CAG-TimeValidity	::= INTEGER (0..65535, ...)

CAG-ID-Temporary-Item ::= SEQUENCE {

cAG-ID					CAG-ID,

cAG-TimeValidity		CAG-TimeValidity,

iE-Extensions		ProtocolExtensionContainer { {CAG-ID-Temporary-Item-ExtIEs} } OPTIONAL,

...

}

CAG-ID-Temporary-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-TAIMBSSupportList									ProtocolIE-ID ::= 441

id-ExtendedBackupAMFName    							ProtocolIE-ID ::= 442

id-ExtendedOldAMF			    						ProtocolIE-ID ::= 443

id-Allowed-Temporary-CAG-List-per-PLMN					ProtocolIE-ID ::= xxx

END

-- ASN1STOP