3GPP TSG-RAN WG3 Meeting #129	R3-255593

# Bangaluru, India, 25 – 29 August 2025

Agenda Item:	10.2

Source:	Ericsson, Huawei, Jio Platforms

Title:	(TP for BL CR to 38.401 for SON) Signaling of MRO for LTM between target DU and source DU

Document for:	Agreement

# TP for BL CR to 38.401 for SON

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt).The target gNB-DU performs initial analysis and may send the recovery beam information to the source gNB-DU via the gNB-CU using MRO for LTM information contains in DU-CU ACCESS AND MOBILITY INDICATION and ACCESS AND MOBILITY INDICATION messages.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell but to a beam different from the beam used during the failed LTM cell switch execution. The target gNB-DU may send the re-established or recovery beam information to the source gNB-DU via the gNB-CU using MRO for LTM information contains in DU-CU ACCESS AND MOBILITY INDICATION and ACCESS AND MOBILITY INDICATION messages. The source gNB-DU performs root cause analysis to detect RACH-less LTM failure caused by wrong beam selection.

The target gNB-DU identifies that the UE successfully performed a RACH-based access while re-establishing or recovering to the same target cell and same target beam but with TA value different from the one which was included in CU-DU CELL SWITCH NOTIFICATION message received from gNB-CU at LTM Cell Switch execution. The target gNB-DU sends the TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU using MRO for LTM information contains in DU-CU ACCESS AND MOBILITY INDICATION and ACCESS AND MOBILITY INDICATION messages.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>