3GPP TSG-RAN WG3 Meeting #129	R3-255604

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	13.2

Source:	CATT

Title:	Discussion for general issues in Inter-CU LTM

Document for:	Decision

- Introduction

In this paper, we will analyse the potential issues in inter-CU LTM.

The SCG reference configuration is provided by an implicit way in the CG-Config RRC container in the SN Change Required message. FFS on the SN Addition Request Acknowledge message from the source SN/candidate SN to the MN.

FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2:

Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure.

FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and UE Context Modification Request message.

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM.

Enhance XN-U ADDRESS INDICATION message and define IE to cover Inter-CU MCG LTM case. FFS on Inter-SN LTM case.

MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

FFS on whether LTM modification/cancel related procedures are needed.

- Discussion

  - CSI-RS for RSRP

Granularity for CSI-RS activation

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

RAN3 had reached the following agreements for CSI-RS activation/de-activation, and wait for RAN2 further conclusion. In the incoming LS R3-255011, RAN2 confirmed that LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE, instead of candidate cell id or SP CSI-RS resource set id.

LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations.

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18         LTM-SSB-ResourceSet-r18,

...,

[[

ltm-NZP-CSI-RS-ResourceSet-r19      LTM-NZP-CSI-RS-ResourceSet-r19                     OPTIONAL, -- Need R

ltm-CSI-IM-ResourceSet-r19          LTM-CSI-IM-ResourceSet-r19                         OPTIONAL, -- Need R

resourceType-r19                    ENUMERATED {periodic, semiPersistent}              OPTIONAL  -- Cond CSI-RS

]]

}

LTM-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-NZP-CSI-RS-ResourceSet-r19 ::=     SEQUENCE {

ltm-CSI-RS-ResourceList-r19         SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF NZP-CSI-RS-ResourceId,

ltm-CandidateIdList-r19             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

Last meeting, RAN3 have raise solution concern two granularities for CSI-RS activation

Option 1: per LTM-CSI-ResourceConfigId-r18 (CSI-RS-ResourceSet)

Option 2: per NZP-CSI-RS-ResourceId

Option 1 is a straight forward way, which is aligning with RAN2. Option 2 divides the Resource Set to a finer granularity, which may reduce the signaling overhead in Xn/F1 interface. However, option 2 is a risky solution, firstly RAN3 need to check whether this division is workable. What’s more, once RAN2 change their designation, RAN3 may need to do according modification, which is too detail and hard to tracking.

Based on our understating, we prefer option 1 as it straight forward and safety, e.g.,. Otherwise, we need to inform RAN2 about our solution for double check and future coordination.

Proposal 1: For the granularity of SP CSI-RS activation/de-activation, adopt the same granularity with RAN 2, e.g., per LTM-CSI-ResourceConfigId-r18.

Proposal 2: If we adopt per NZP-CSI-RS-ResourceId granularity, e.g., option 2, RAN3 shall inform RAN2 for double check the feasibility and enable future coordination.

TCI state information

As agreed by RAN2, SP CSI-RS activation/deactivation MAC CE also contains the TCI state ID.

The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

As defined in RAN2, SP CSI-RS resource is contains in LTM-CSI-ResourceConfig-r18→

LTM-NZP-CSI-RS-ResourceSet-r19→NZP-CSI-RS-Resource. After check RAN2 spec., only periodic CSI-RS, candidate cell will pre-configure qcl-InfoPeriodicCSI-RS. For SP CSI-RS, no associateion TCI information in candidate cell preparation phase.

As RAN2 include TCI state ID in MAC CE, source node need to transfer the TCI state ID to the candidate cells.

NZP-CSI-RS-Resource

The IE NZP-CSI-RS-Resource is used to configure Non-Zero-Power (NZP) CSI-RS transmitted in the cell where the IE is included, which the UE may be configured to measure on (see TS 38.214 [19], clause 5.2.2.3.1). A change of configuration between periodic, semi-persistent or aperiodic for an NZP-CSI-RS-Resource is not supported without a release and add.

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]]

}

-- TAG-NZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

Observation 1: Only periodic CSI-RS will be configured with an associated TCI State during candidate cell preparation phase.

Proposal 3: Include the TCI state ID in CSI-RS COORDINATION REQUEST message from source to candidate node.

Report type

Another left issue is whether periodic or semi-persistent is needed from candidate node to source node, explicitly. Based on our understanding, the Report type information is already contained in the LTM-CSI-ResourceConfig information, and can be interrupt by source node as source shall use this resource to trigger early sync.

LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations.

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18         LTM-SSB-ResourceSet-r18,

...,

[[

ltm-NZP-CSI-RS-ResourceSet-r19      LTM-NZP-CSI-RS-ResourceSet-r19                     OPTIONAL, -- Need R

ltm-CSI-IM-ResourceSet-r19          LTM-CSI-IM-ResourceSet-r19                         OPTIONAL, -- Need R

resourceType-r19                    ENUMERATED {periodic, semiPersistent}              OPTIONAL  -- Cond CSI-RS

Proposal 4: No need to transfer Report Type of each CSI-Resource from candidate node to source node explicitly.

2.2 CSI-RS for CSI acquisition

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

- Regard of CSI-RS for CSI acquisition request/response

In the case of CSI-RS resources for candidate cell L1 RSRP measurement, the source node determines whether the candidate gNB needs to provide CSI-RS resources. A similar logic can be applied for CSI-RS resources used for candidate cell CSI acquisition.

Considering the CSI acquisition can be based on periodic CSI-RS resource or semi-persistent CSI-RS resource, the candidate gNB can decided which kind of CSI-RS resource will be used.

What’s more, in the LTM Information Response IE in HANDOVER REQUEST ACKNOWLEDGA message, the candidate cell shall provide CSI-RS resource for CSI acquisition. To support providing CSI-RS for RSPR and CSI-RS for CSI acquisition at the same time, we suggest to introducing a new IE for contain CSI-RS resource for CSI acquisition.

Proposal 5: Add a new indicator in HANDOVER REQUEST message to request the candidate cell providing CSI-RS resource for CSI acquisition.

Proposal 6: Add a new CSI-RS Resource Configuration for CSI acquisition IE in LTM Information Response IE in HANDOVER REQUEST ACKNOWLEDGE.

- Regard of SP CSI-RS for CSI acquisition activation:

Once the SP CSI-RS resource for CSI acquisition is configured, the activation and deactivation procedure should be considered. We suggest to use the same procedure as of SP CSI-RS for RSRP. The source node indicates to the candidate node which SP CSI-RS resources should be activated or deactivated, and the candidate gNB provides a response message to the source node.

Proposal 7: For the CSI-RS for CSI acquisition activation/deactivation, it is proposed to use the same approach as for SP CSI-RS activation/deactivation for RSRP. The granularity is pending on the discussion on CSI-RS for RSRP.

2.3 Handling of multiple UE associations

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

To enable UE context identification between two candidate gNBs, the initial gNB should transmit the target UE’s XnAP ID to the other candidate cells. One feasible approach is to extend the Configuration ID Mapping List IE in the following manner. The source node can inform each candidate cell the assigned target XnAP of other candidate cells at the first round LTM CONFIGURATION UPDATE message. And the candidate cells can used it in future UE association handing.

2.1.xx5	LTM Configuration ID Mapping List

This IE indicates the list of LTM candidate cells associated with its configuration IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Configuration ID Mapping Item IEs |  | 1..< maxnoofLTMCells> |  |  |
| >LTM Cell ID | M |  | NR CGI 9.2.2.7 |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [10]. |
| Assigned target UE XnAP ID | O | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |

Proposal 8: The initial source gNB may send the assigned target UE XnAP ID mapping list to all candidate cells via the LTM CONFIGURATION UPDATE message.

Another option is inform the list of target UE XnAP ID assigned by last serving cell via CELL SWITCH NOTIFICATION procedure from source node to candidate node, in case of no candidate cell left in the initial source and XnAP assigned for the initial source is released.

We think we can support the two options to keep flexibility. When the initial source is not release, UE association can be achieved by the Assigned target XnAP ID assigned by the initial source, then the other candidate can understand the assigned target UE XnAP ID mapping list at the first round LTM CONFIGURATION UPDATE procedure. If initial source release or other accidents, source can update the assigned target UE XnAP ID mapping list via CELL SWITCH MODIFICATION procedure.

Proposal 9: The new source gNB can update the assigned target UE XnAP ID mapping list to target cell via CELL SWITCH NOTIFICATION procedure.

9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node ‘UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Assigned target UE XnAP ID | O | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES |  |  |

Another option is use the Assigned XnAP ID between the candidate cell and last serving cell. However, by this way, we notice may need two round LTM configuration update message between new serving cell and other candidate cells.

First round: new source cell send LTM configuration update message to the candidate cell

Proposal 10: When establishing a new UE association between two candidate gNBs, the request node should include the old assigned target UE XnAP ID in the LTM CONFIGURATION UPDATE message, the old assigned target UE XnAP may form the initial source or the last serving node.

2.4	Security key handling

FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2:

- Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

- Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

To meet the flexible requirement, two options left on the security key handling. Technically, two options both workable; differences are on the signalling procedure.

For option 1, the Rel-19 security ID assignment can be done at step 5. And if subsequent LTM is supported, source gNB can further sync security information to other candidate gNB by step 6.7. For option 2, the Rel-19 security ID assignment will be done after LTM configuration update message once the ID collision occurs, whatever subsequent LTM is configure or not. Without any assistance information like option 1, the ID collision will happen in a high possibility.

Based on current specific, step 6 and step7 are optional and can be triggered after step 14, or after step 17 by implementation. If we go for option 2, the presence and signalling order of step 6 and step 7 need to be further considered.

For reduce the complexity, we suggest to go for option 1. Compared to the suggested Rel-19 Set ID or Rel-19 set ID range, we slightly prefer to use suggested Rel-19 Set ID, as each time in HO REQUEST will only contain one candidate cell.

Observation 3: If we adopt option 2, the presence and signalling order of step 6 and step 7 need to be further considered.

Proposal 11: For reduce the complexity, source gNB sends the suggested Rel-19 Set ID to the candidate gNB in the HANDOVER REQUEST message(per candidate cell), then candidate gNB apply Rel-19 set ID to its own candidate cell based on security requirements. The candidate gNB feedback via HANDOVER REQUEST ACKNOWLEDGE message.

2.5 TA relative

2.5.1 TA information transfer

MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

In additional, RAN2 has concluded to support MIMO 2TA in inter-CU LTM. In the TA information transfer message, the Tag ID Pointer IE should also be contained.

Proposal 12: Introduce Tag ID Pointer IE in TA information transfer message for indicating the TAG corresponding to the TA value

2.5.2 TA validity

TA validity is discussed in Rel-18 and decided to leave it to source DU implementation. In Rel.19, companies re-raise concern about TA validity due to longer transmit latency among Xn interface.

Back to the legacy TA mechanism, TA validity is maintained by TAT, which is configured by network and inform to UE according to SIB1.

38.321, 5.2 RRC configures the following parameters for the maintenance of UL time alignment: - timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG; 38.331, 5.3.3.2 1> else: 2> apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1; 2> apply the default MAC Cell Group configuration as specified in 9.2.2; 2> apply the CCCH configuration as specified in 9.1.1.2; 2> apply the timeAlignmentTimerCommon included in SIB1;

To avoid unnecessary early synchronization, in Rel-18, we enable the source to transfer the obtained TA (Timing Advance) values in the Cell Switch Notification message. In the intra-DU scenario, the movement range of the UE is limited, so the TA remains valid in the target cell with a high probability. However, extending this to the inter-CU case, where the UE's movement range is larger, it becomes difficult to assert that the TA obtained at the source is still valid at the target side, regardless of whether it is at the source or target. Using an unsuitable TA may lead to handover failure, and the drawbacks of transferring TA among different CUs outweigh the potential advantages.

Proposal 13: Source DU/gNB is in charge of checking the TA validity by implementation.

Proposal 14: Given that TA is relative to UE location and mobility, transferring TA values among different gNBs is not support in inter-CU LTM.

2.6 Mirror correction for XnAP

A. Align the Request IE and response IE not in the same level

9.1.1.1	HANDOVER REQUEST

| LTM Handover Information Request | O |  |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Information Request | O |  | 9.2.3.xx1 |  | – |  |
| >Request for CSI-RS Resource Configuration | O |  | ENUMERATED (true, …) |  | – |  |
| Early Sync Information Request | O |  | 9.2.3.xx3 |  | YES | ignore |

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

| LTM Handover Information Response | O |  |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >SSB Information | M |  | 9.2.3.x10 |  | – |  |
| >LTM Information Response | O |  | 9.2.3.xx2 |  | – |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [10]. | – |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [10]. This IE applies for SUL carrier. | – |  |
| Early Sync Information Response | O |  | 9.2.3.xx4 |  | YES | ignore |

B. Remove the Request for CSI-RS Resource Configuration IE in LTM CONFIGURATION UPDATE message.

C. Based on our agreements, only contain NCC form new source to other candidate cells

9.1.x.x3	LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| LTM Information Request | O |  | 9.2.3.xx1 |  | YES | ignore |
| LTM Updates to Candidate Cell Information List | 0..1 |  |  |  | YES | ignore |
| >LTM Updates to Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>Request for CSI-RS Resource Configuration (FFS) | O |  | ENUMERATED (true, …) | Indicates the request to the candidate cell to provide CSI-RS resource configuration. | - |  |
| >>Early Sync Information Request | O |  | 9.2.3.xx3 | Indicates the request to the candidate cell for early synchronization information. | - |  |
| >>Early Sync Information | O |  | 9.2.3.xx6 | Indicates the consolidated early synchronization information to be used for subsequent LTM. | - |  |
| >>Early Sync Information for SUL | O |  | 9.2.3.xx6 | Indicates the consolidated early synchronization information for SUL to be used for subsequent LTM. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate cell identified by the Candidate Cell ID IE. | - |  |
| Key NG-RAN Star | O |  | BIT STRING (256) | KNG-RAN* defined in TS 33.501 [28]. | - |  |
| UE Security Information for LTM | O |  | 9.2.3.x12 |  | YES | ignore |

D correct the unclear procedure text

9.1.x.x7 	CSI-RS COORDINATION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to coordinate the activation and deactivation of CSI-RS transmission at NG-RAN node2 for a UE.

Direction: NG-RAN node1  NG-RAN node2.

Proposal 15: Check the above mirror correct in XnAP.

4. Conclusion

Proposal 1: For the granularity of SP CSI-RS activation/de-activation, adopt the same granularity with RAN 2, e.g., per LTM-CSI-ResourceConfigId-r18.

Proposal 2: If we adopt per NZP-CSI-RS-ResourceId granularity, e.g., option 2, RAN3 shall inform RAN2 for double check the feasibility and enable future coordination.

Observation 1: Only periodic CSI-RS will be configured with an associated TCI State during candidate cell preparation phase.

Proposal 3: Include the TCI state ID in CSI-RS COORDINATION REQUEST message from source to candidate node.

Proposal 4: No need to transfer Report Type of each CSI-Resource from candidate node to source node explicitly.

Proposal 5: Add a new indicator in HANDOVER REQUEST message to request the candidate cell providing CSI-RS resource for CSI acquisition.

Proposal 6: Add a new CSI-RS Resource Configuration for CSI acquisition IE in LTM Information Response IE in HANDOVER REQUEST ACKNOWLEDGE.

Proposal 7: For the CSI-RS for CSI acquisition activation/deactivation, it is proposed to use the same approach as for SP CSI-RS activation/deactivation for RSRP. The granularity is pending on the discussion on CSI-RS for RSRP.

Proposal 8: The initial source gNB may send the assigned target UE XnAP ID mapping list to all candidate cells via the LTM CONFIGURATION UPDATE message.

Proposal 9: The new source gNB can update the assigned target UE XnAP ID mapping list to target cell via CELL SWITCH NOTIFICATION procedure.

Proposal 10: When establishing a new UE association between two candidate gNBs, the request node should include the old assigned target UE XnAP ID in the LTM CONFIGURATION UPDATE message, the old assigned target UE XnAP may form the initial source or the last serving node.

Observation 3: If we adopt option 2, the presence and signalling order of step 6 and step 7 need to be further considered.

Proposal 11: For reduce the complexity, source gNB sends the suggested Rel-19 Set ID to the candidate gNB in the HANDOVER REQUEST message(per candidate cell), then candidate gNB apply Rel-19 set ID to its own candidate cell based on security requirements. The candidate gNB feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Proposal 12: Introduce Tag ID Pointer IE in TA information transfer message for indicating the TAG corresponding to the TA value

Proposal 13: Source DU/gNB is in charge of checking the TA validity by implementation.

Proposal 14: Given that TA is relative to UE location and mobility, transferring TA values among different gNBs is not support in inter-CU LTM.

Proposal 15: Check the above mirror correct in XnAP.

4. Reference

[1]	R3-255011 LS on RAN2 agreements for SP CSI-RS activation/deactivation