3GPP TSG-RAN WG3 Meeting #129	R3-255676

Bengaluru, India, 25th ~29th Aug, 2025

Title:	Discussion on MBS Communication Service Type

Agenda Item:	8.1

Source:	Huawei

Document for:	Discussion and Approval

1	Introduction

This discussion paper mainly discussed the potential RAN3 impacts, on the received LS from SA4 in S4-251096 [1].

2	Discussion

In the received LS, SA4 answered RAN3’s question and indicated to RAN3 that SA4 had updated their spec accordingly, to summarize:

- the “Use” of @communicationServiceType is updated from OD to O, the absence of this IE means that QoE collection is requested for all communication services, regardless of where the content comes from.

- A new service type, namely unicast is added, which refers to the common unicast communication type carrying media streaming services.

With the above updates, there is a need to check whch whether there are any RAN3 updates needed or not. The current RAN3 spec regarding service type goes like this:

| MBS Communication Service Type | O |  | ENUMERATED (broadcast, multicast, ...) | This IE indicates the type of MBS communication service for which the QoE measurement collection should be performed. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

As we could see that RAN3 already defined broadcast and multicast as two MSB communication service types, here the remaining question is:

-	whether there is a need to add the new service type “unicast” as what SA4 did?

In our understanding, there is no need to to add a specific service type of “unicast”, since in current 38.413 spec, we already defined a mandatory IE which by default points to unicast.

| Service Type | M |  | ENUMERATED (QMC for DASH streaming, QMC for MTSI, QMC for VR, ...) | This IE indicates the service type of QoE measurements. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

Observation 1: The current RAN3 spec already defined a mandatory IE representing service type of unicast.

Another trivial point is, it is stated in the incoming LS that absence of this IE “@communicationServiceType” means that QoE collection is requested for all communication services, while from RAN3 perspective, this case was already supported with the IE QoE reference which allows upper layer to configure more than one QMC task with a unique QoE reference towards the same UE, i.e. the upper layer could use different QoE reference for different service type towards the same UE to achieve the purpose of performing QMC for all the service types towards the same UE, needless to say that QoE report anyway is service specific.

| QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [45]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3-byte Octet String. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

Observation 2: The current RAN3 spec already support to perform QMC task for all the service types towards the same UE

With the two observations above, we think the current RAN3 spec already supported what SA4 wanted to achieve as seen in their incoming LS, maybe the only thing is just to update the RAN3 spec with the addition of reference to SA4, as follows:

Table-1: Example of adding reference to SA4

| MBS Communication Service Type | O |  | ENUMERATED (multicast, broadcast, …) | This IE indicates the type of MBS communication service for which the QoE measurement collection should be performed, as defined in clause 10.5 in [47]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Proposal: RAN3 to agree the correction as exempled in Table-1.

According to the proposal, 38.423 and 38.413 should be updated, drafts could be seen in [2] and [3].

3	Conclusion

This document discussed the potential impacts to RAN3 spec about the incoming LS from SA4, and made the following observations and proposal.

Observation 1: The current RAN3 spec already defined a mandatory IE representing service type of unicast.

Observation 2: The current RAN3 spec already support to perform QMC task for all the service types towards the same UE

Proposal: RAN3 to agree the correction to 38.413 as exempled in Table-1.

4	Reference

[1]	S4-251096, Draft LS to RAN3 on MBS Communication Service Type, Nokia

[2]	R3-25xxxx, Clarification on MBS communication service type for QMC configuration, Huawei

[3]	R3-25xxxx, Clarification on MBS communication service type for QMC configuration, Huawei