3GPP TSG-RAN WG3 Meeting #129          	          R3-255711

Bengaluru, India, Aug 25th- Aug 29th, 2025

Agenda item:	16.2

Source:	CMCC

Title:	(TP to BLCR for TS 38.300) Introduction of Ambient IoT

Document for:	Approval

# 1	Introduction

This contribution provides the TP to BLCR for TS 38.300 for all Ambient IoT related aspects based on the discussion paper.

# Text Proposal to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 16.xx	Support of Ambient IoT

### 16.xx.x1	Introduction

### 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or multiple readers.

-	A gNB is aware of the mapping relations between A-IoT Area and readers by means of OAM configuration.

-	An A-IoT Area may include readers belonging to the same or different gNB.

-	One reader only belongs to one gNB.

-	One reader can be mapped to one or multiple A-IoT Areas.

-	As specified in TS 23.369 [xx], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF).

NOTE 1:	A deployment with both direct connectivity and indirect connectivity is not expected.

-	In this version of the specification, A-IoT specific information is transported between the gNB and the A-IoT  CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of readers. The A-IoT Area is represented by an A-IoT Area ID.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the readers supported by the gNB.

The AIOTF may also be aware of the reader’s location by means of OAM configuration.

The AIOTF is aware of the mapping relationship among gNBs, readers and A-IoT areas by means of OAM configuration, as needed.

Upon receiving the requested service area in inventory request from the A-IoT CN, the gNB selects the readers with the following principles:

-	If only the area in Inventory Request, the gNB selects readers within the indicated area.

-	If only the reader list in Inventory Request, the gNB takes the reader list into account and selects readers within list.

-	If neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the Expected D2R message size and may include Inventory Assistance Information e.g. Approximate number of Target A-IoT devices, and time interval for A-IoT device response aggregation.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

7.	Upon completion of the inventory procedure the gNB sends an Inventory Complete Indication in the last Inventory Report message.

### 16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices or in parallel to other command procedures targeting other A-IoT devices within the same session.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB in order to send a command to a single A-IoT device. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU corresponding to the targeted A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size, command type.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response message including the AIOTF Identifier  the Correlation ID and the RAN A-IoT device NGAP ID received in the Command Request, and the A-IoT NAS PDU received from the device.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

### 16.xx.x5	A-IoT CN initiated A-IoT Session Release procedure

Figure 16.xx.x5-1 depicts the A-IoT CN initiated A-IoT session release procedure between the gNB and the A-IoT CN node.

Figure 16.xx.x5-1: A-IoT CN initiated A-IoT Session Release procedure

1.	The A-IoT CN node initiates the A-IoT Session Release procedure over NG-C by sending the A-IoT Session Release Command message to the gNB. The A-IoT Session Release Command message includes the Correlation ID and the AIOTF Identifier to identify the session to be released.

2.	The gNB releases the session context, the associated NG resources and the A-IoT radio resources.

3.	The gNB confirms the release to the A-IoT CN node by replying with the A-IoT Session Release Complete message. The A-IoT Session Release Complete message includes the AIOTF Identifier and the Correlation ID received in the A-IoT Session Release Command message.

### 16.xx.x6	gNB initiated A-IoT Session Release procedure

Figure 16.xx.x5-1 depicts the gNB initiated A-IoT session release procedure from the gNB to the A-IoT CN node.

Figure 16.xx.x6-1: gNB initiated A-IoT Session Release procedure

1.	The gNB initiates the A-IoT Session Release Request procedure over NG-C by sending the A-IoT Session Release Request message to the A-IoT CN node in order to request the release of an A-IoT session. The A-IoT Session Release Request message includes the Correlation ID and the AIOTF Identifier to identify the session requested to be released.

2.	The A-IoT CN node accepts the request and triggers the CN initiated A-IoT Session Release procedure.

3.	The CN initiated A-IoT Session Release procedure is performed as described in section 16.xx.x5.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>