3GPP TSG RAN3 meeting #130														R3-258002

Dallas, US, 17 - 21 November 2025

Agenda Item:			4

Source:				ETSI MCC

Title:					RAN3 #129-bis Meeting Report

Prague, Czech, 13/10/2025 to 17/10/2025

Document for:		Approval

Contents:

1	Opening of the meeting	4

2	Reminders	4

2.1	IPR Declaration	4

2.2	Statement of Antitrust Compliance	4

2.3	Consensus Principles	4

2.4	Responsible IT Behavior	4

2.5	Additional reminders	5

3	Approval of the Agenda	6

4	Approval of the minutes from previous meetings	6

5	Documents for immediate consideration	6

6	Organizational topics	6

7	General, protocol principles and issues	6

8	Incoming LSs	6

8.1	New Incoming LSs	6

8.2	LSin received during the meeting	22

8.3	Left over LSs / pending actions	22

9	Corrections to Rel-19 or earlier releases	22

9.1	LTE	22

9.1.1	R19 IoT NTN enhancements	22

9.1.2	R19 LTE-based 5G Broadcast	24

9.1.3	Other	25

9.2	NR	26

9.2.1	R19 SON/MDT	26

9.2.2	R19 AI/ML for NG-RAN	30

9.2.3	R19 WAB	40

9.2.4	R19 NR Femto	44

9.2.5	R19 Mobility enhancements	47

9.2.6	R19 NR NTN enhancements	58

9.2.7	R19 Ambient IoT	61

9.2.8	R19 Network Energy Saving	66

9.2.9	R19 Low Power WUS/WUR	70

9.2.10	R19 Evolution of Duplex Operation	74

9.2.11	R19 AI/ML for air interface	77

9.2.12	R19 NR XR Enhancements	81

9.2.13	R19 NR Sidelink Multi-hop Relay	84

9.2.14	Other	86

9.3	R19 Rapporteur Corrections	102

10	Study on 6G Radio [FS_6G_Radio]: RP-252912 (target: RAN #115) [TU: 4 (4, 4, 5, 6, 6, 6, 6, 6, 6)]	103

10.1	General	103

10.2	Overall RAN architecture	105

10.2.1	General principles and requirements	105

10.2.2	RAN functions and logical architecture	111

10.3	RAN-CN interface	112

10.3.1	General principles and requirements	112

10.3.2	RAN-CN interface options	117

10.4	RAN internal functional split and interfaces	119

10.5	AI/ML for RAN	123

10.5.1	AI/ML use cases	123

10.5.2	AI/ML framework	127

10.6	Mobility for 6GR	127

10.7	Interworking between 6GR and NR	127

11	Data Collection for SON/MDT in NR Phase 5 (RAN3-led) [NR_SON_MDT_Ph5-Core]: RP-252560 (target: RAN#115) [TU: 1 (1, 1, 1, 1, 1, 1, 1, 1, 1)]	127

11.1	General	127

11.2	MRO Enhancements	128

11.2.1	Inter-CU LTM	128

11.2.2	Intra-CU conditional LTM	130

12	Study on AI/ML for NG-RAN Phase 3 (RAN3-led) [FS_NR_AIML_NGRAN_Ph3]: RP-252867 (target: RAN #111) [TU: 1 (1, 1, 1)]	132

12.1	General	132

12.2	AI/ML-based mobility	133

12.2.1	Multi-hop UE trajectory across gNBs	133

12.2.2	Intra-CU LTM	136

12.2.3	Handover enhancements	139

13	Study on Integrated Sensing And Communication (ISAC) for NR [FS_Sensing_NR_bis]: RP-252819 (target: RAN #112) [TU: 1 (1, 1, 1, 1, 1)]	141

13.1	General	141

13.2	Network architecture	142

13.3	RAN-CN procedures and signaling	146

14	Solutions for Ambient IoT in NR Phase 2 [Ambient_IoT_Solutions_Ph2-Core]: RP-252894 (target: RAN #115) [TU: 0.5 (0.5, 0.5, 1, 1, 1, 1, 1, 1, 1)]	149

14.1	General	149

14.2	Topology 2	149

14.3	Topology 1	152

15	NR mobility enhancements Phase 5 [NR_Mob_Ph5-Core]: RP-252113 (target: RAN #115) [TU: 0 (0, 0, 0, 1, 1, 1, 1, 1, 1)]	152

15.1	General	152

15.2	LTM SCell activation enhancements	152

20	XR for NR Phase 4 [NR_XR_Ph4-Core]: RP-252755 (target: RAN #115) [TU Basket: 0 (0, 0, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5)]	152

20.1	General	153

20.2	Coordination between gNB and CN on N3 delay measurement	153

21	AI/ML for NR air interface Phase 2 [NR_AIML_air_Ph2-Core]: RP-252445 (target: RAN #115) [TU Basket: 0 (0, 0, 0, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5)]	153

21.1	General	153

21.2	Two-sided AI/ML model	153

31	Corrections and Enhancements to Rel-20	153

31.1	Corrections	153

31.2	Enhancements	153

32	Any other business	153

33	Closing of the meeting	153

## 1	Opening of the meeting

3GPP TSG RAN3 Chair Mr. Sean Kelley (Nokia) opened the f2f meeting RAN3 #129 in Prague, Czech on Monday October 13th, 2025 at 9am.

## 2	Reminders

### 2.1	IPR Declaration

I draw your attention to your obligations under the 3GPP Partner Organizations’ IPR policies. Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

Delegates are asked to take note that they are thereby invited:

to investigate whether their organization or any other organization owns IPRs which were, or were likely to become, essential in respect of the work of 3GPP.

to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (see: http://ipr.etsi.org/).

### 2.2	Statement of Antitrust Compliance

I also draw your attention to the fact that 3GPP activities are subject to all applicable antitrust and competition laws and that compliance with said laws is therefore required of any participant of this TSG/WG/SWG meeting including the Chair and Vice Chairs. In case of question I recommend that you contact your legal counsel.

The leadership shall conduct the present meeting with impartiality and in the interests of 3GPP.

Furthermore, I would like to remind you that timely submission of work items in advance of TSG/WG/SWG meetings is important to allow for full and fair consideration of such matters.

### 2.3	Consensus Principles

The attention of the delegates to the meeting is drawn to the fact that 3GPP endeavours to reach consensus on all decisions and therefore depends on a cooperative spirit of the Individual Members. In particular, Individual Members are encouraged to seek a consensus-based solution and only to sustain objections as a very last resort, and where absolutely necessary and well justified. The leadership will conduct the present meeting in a manner whereby informal methods of reaching consensus are encouraged, whilst ensuring that well justified concerns are taken into account.

### 2.4	Responsible IT Behavior

We all share meeting IT resources with one another. Delegates should restrict their IT usage to things which are essential for the meeting, and they:

- shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws.

- shall not engage in non-work-related activities that consume excessive bandwidth or cause significant network performance degradation.

And most importantly:

1. DON’T place your WiFi device in ad-hoc mode;

2. DON’T set up a personal hotspot in the meeting room;

3. DO try 802.11a if your device supports it;

4. DON’T manually allocate an IP address;

5. DON’T stream video, play online games, or download huge files;

6. DON’T use packet probing software (e.g., packet sniffers or port scanners) which clogs the local network.

### 2.5	Additional reminders

This is an ordinary face-to-face meeting with 1-way remote access.

1. All agreed TDocs must be provided during the meeting week, i.e., BEFORE the end of the meeting. In order to continue with the principle of “agreed unseen”, please ensure that all such TDocs are uploaded in time and reflect exactly the agreed changes.

2. During physical meetings, prefer face-to-face offline discussion to e-mail discussion.

3. When a CB is set up, e.g.:

CB: # 1_Name

- topics of the offline discussion

(Company Owner - moderator)

Rev in R3-xxxxxx

Summary of offline disc R3-xxxxxy

- Create a folder in “Inbox/Drafts/1_Name” with the assigned CB number (1) and name;

- Upload all drafts, corrections, revisions, etc. in the same folder “Inbox/Drafts/1_Name”;

- Avoid sending drafts via e-mail or on the reflector!

- When sending e-mails, do not attach any documents, and please minimize e-mail discussion (e.g. it is enough to announce the start of discussion, availability of drafts on server, support for a document, discussion conclusion).

- It is highly beneficial if the summary of offline discussion contains proposals for “official” group conclusions, e.g. “propose to agree R3-xxxxxx”, “propose to agree that….”, “no agreement”, “to be continued”, etc.

4. To encourage the use of pCRs, if there are discussion papers and pCRs from the same company on the same topic, only the pCRs will be treated.

5. Papers submitted to the wrong AI will not be treated.

6. When subsections are available, please do not submit papers to the “top level” AI. If you think none of the available subsections fits your contribution, then it should go to the “Others” subsection. Any papers submitted to the “top level” AIs should not be expected to be treated.

7. To save time, incoming LSs which have no action for RAN3 will not be treated unless they are flagged to the Chair before the start of the meeting.

8. QUOTAS: When a quota is indicated for an Agenda Item, each company may submit up to that number of contributions in total across all its sub-Agenda Items. Please refer to the example at the end of this document. Quota rules are described in R3-255141 and continue to be the basis for working with quotas in RAN3.

9. RAN3 breakout room: Only a CB moderator or SI/WI rapporteur may request the RAN3 breakout room (via MCC), and should follow these guidelines to ensure fair access:

- The offline session time slot should be announced via the RAN3 email reflector.

- The maximum total duration of all CBs for a given SI/WI is 2 hours.

- 1-way remote access may be provided via GoToWebinar (GTW), on a best-effort basis.

Some suggestions for better RAN3 meetings can also be found here.

## 3	Approval of the Agenda

R3-256501	RAN3#129-bis Meeting Agenda

Type: agenda		For: Approval
					Source: RAN3 Chair

Decision: 		The document was approved.

## 4	Approval of the minutes from previous meetings

R3-256502	RAN3#129 Meeting Report

Type: report		For: Approval
					Source: ETSI-MCC

Decision: 		The document was approved.

## 5	Documents for immediate consideration

## 6	Organizational topics

## 7	General, protocol principles and issues

R3-256503	TR 30.531 v1.60.0 Work Plan and Working Procedures - RAN WG3

Type: draft TR		For: Endorsement
					30.531 v1.59.0
					Source: ETSI-MCC

Decision: 		The document was endorsed.

R3-256532	LS on Study on Modernization of Specification Format and Procedures for 6G

Type: LS in		For: Discussion
					Original outgoing LS: SP-251228, to 3GPP WSs, cc TSG RAN, TSG CT
					Source: TSG SA(Nokia)

Decision: 		The document was noted.

## 8	Incoming LSs

### 8.1	New Incoming LSs

[Average window for alternative QoS]

R3-256515	LS on Average Window for Alternative QoS

Type: LS in		For: Discussion
					Original outgoing LS: S2-2508118, to RAN3, cc -
					Source: SA2(CICT mobile)

Decision: 		The document was noted.

R3-256645	Discussion on LS on Average Window for Alternative QoS

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-256646	Correction on Average Window for Alternative QoS for NGAP

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1323  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was not treated.

R3-256647	Correction on Average Window for Alternative QoS for XnAP

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1523  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was not treated.

R3-256648	Correction on Average Window for Alternative QoS for F1AP

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1588  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was not treated.

R3-256649	Correction on Average Window for Alternative QoS for E1AP

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0177  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was not treated.

R3-256701	Discussion on Average Window for Alternative QoS

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was noted.

R3-256702	[draft] Reply to R3-256515 on Average Window for Alternative QoS

Type: LS out		For: Approval
					to SA2
					Source: Huawei

Decision: 		The document was not treated.

R3-256849	Discussion on Average Window for Alternative QoS

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was noted.

R3-256848	[Draft] Reply LS to SA2 on Average Window for Alternative QoS

Type: LS out		For: Approval
					to SA2
					Source: CMCC

Decision: 		The document was not treated.

R3-257002	Discussion on Average Window for Alternative QoS

Type: discussion		For: Discussion
					Source: Ericsson

Decision: 		The document was noted.

R3-257003	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1355  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R3-257204.

R3-257204	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1355  rev 1 Cat: F (Rel-18)

					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257003)

Decision: 		The document was withdrawn.

R3-257004	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1356  Cat: A (Rel-19)

					Source: Ericsson

Decision: 		The document was revised to R3-257205.

R3-257205	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1356  rev 1 Cat: A (Rel-19)

					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257004)

Decision: 		The document was withdrawn.

R3-257005	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1571  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R3-257206.

R3-257206	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1571  rev 1 Cat: F (Rel-18)

					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257005)

Decision: 		The document was withdrawn.

R3-257006	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1572  Cat: A (Rel-19)

					Source: Ericsson

Decision: 		The document was revised to R3-257207.

R3-257207	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1572  rev 1 Cat: A (Rel-19)

					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257006)

Decision: 		The document was withdrawn.

R3-257007	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1618  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R3-257208.

R3-257208	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1618  rev 1 Cat: F (Rel-18)

					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257007)

Decision: 		The document was withdrawn.

R3-257008	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1619  Cat: A (Rel-19)

					Source: Ericsson

Decision: 		The document was revised to R3-257209.

R3-257209	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1619  rev 1 Cat: A (Rel-19)

					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257008)

Decision: 		The document was withdrawn.

R3-257009	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0181  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R3-257210.

R3-257210	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0181  rev 1 Cat: F (Rel-18)

					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257009)

Decision: 		The document was withdrawn.

R3-257010	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					37.483 v19.0.0	  CR-0182  Cat: A (Rel-19)

					Source: Ericsson

Decision: 		The document was revised to R3-257211.

R3-257211	Correction on Alternative QoS parameter

Type: CR		For: Agreement
					37.483 v19.0.0	  CR-0182  rev 1 Cat: A (Rel-19)

					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257010)

Decision: 		The document was withdrawn.

R3-257011	Correction on Alternative QoS parameter

Type: draftCR		For: Endorsement
					38.300 v18.7.0
					Source: Ericsson

Decision: 		The document was revised to R3-257212.

R3-257212	Correction on Alternative QoS parameter

Type: draftCR		For: Endorsement
					38.300 v18.7.0
					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257011)

Decision: 		The document was withdrawn.

R3-257012	Correction on Alternative QoS parameter

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson

Decision: 		The document was revised to R3-257213.

R3-257213	Correction on Alternative QoS parameter

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson, CATT, Qualcomm Inc., Jio Platforms, Lenovo

(Replaces R3-257012)

Decision: 		The document was withdrawn.

R3-257083	Handling of Average window for alternative QoS

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was noted.

R3-257084	Correction of Average QoS Window for AQP

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1362  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was not treated.

R3-257085	Correction of Average QoS Window for AQP

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1363  Cat: A (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R3-257086	Response LS on Average window for alternative QoS

Type: LS out		For: Approval
					to SA2
					Source: Nokia

Decision: 		The document was not treated.

R3-257168	Discussion on  average window for alternative QoS

Type: discussion		For: Approval
					Source: ZTE Corporation

Decision: 		The document was noted.

Understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

Understanding 2: The Alternative QoS Parameter sets may contain different averaging window configurations. It means that the NG-RAN will use different durations to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. This represents a new requirement to NG-RAN.

CATT: RAN3 agree to use the averaging window of current activated QoS Profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

HUAWEI: Understanding 1

CMCC: Understanding 1, but understanding 2 could be adopted if simplified implementation guidelines are defined

Ericsson: Understanding 1 (with CRs)

Nokia: Understanding 1

ZTE: Understanding 1

QUALCOMM: Understanding 1 is simple, but CATT proposal has benefits

Samsung: prefer Understanding 1

[Agreement]  RAN3 agrees on understanding 1 as described in R3-256515

CB # 1_AverageWindow

- Check whether any CRs are needed for understanding 1, and attempt convergence

- Reply LS to SA2?

(Ericsson - moderator)

[Per-UE UE performance metrics R18]

R3-256518	Reply LS on per-UE UE performance metrics

Type: LS in		For: Discussion
					Original outgoing LS: S5-253854, to RAN3, cc RAN2
					Source: SA5(HuaWei)

Abstract:

Att: S5-253900, S5-253899

Decision: 		The document was noted.

R3-256973	Further discussion on per-UE UE performance in AI/ML for NG-RAN

Type: discussion		For: Agreement
					Source: Huawei, FiberCop, BT, Jio Platforms

Decision: 		The document was noted.

R3-256974	Correction on Average Packet Delay and Average Packet Loss DL - Option 1

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1562  Cat: F (Rel-18)

					Source: Huawei, FiberCop, BT, Jio Platforms

Decision: 		The document was revised to R3-257241.

R3-257241	Correction on UE Performance metrics

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1562  rev 1 Cat: F (Rel-18)

					Source: Huawei, FiberCop, BT, Jio Platforms

(Replaces R3-256974)

Abstract:

NBC

Discussion:

CR is NBC

-	Add CMCC as co-source

Decision: 		The document was revised to R3-257302.

R3-257302	Correction on UE Performance metrics

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1562  rev 2 Cat: F (Rel-18)

					Source: Huawei, FiberCop, BT, Jio Platforms, NEC, ZTE, Samsung, CATT, Ericsson, Nokia, CMCC

(Replaces R3-257241)

Abstract:

NBC

Decision: 		The document was endorsed.

R3-256975	Correction on Average Packet Delay and Average Packet Loss DL - Option 2

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1563  Cat: F (Rel-18)

					Source: Huawei, FiberCop, BT, Jio Platforms

Decision: 		The document was not treated.

R3-256976	Correction on Average Packet Delay and Average Packet Loss DL - Option 1

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1564  Cat: A (Rel-19)

					Source: Huawei, FiberCop, BT, Jio Platforms

Decision: 		The document was revised to R3-257242.

R3-257242	Correction on UE Performance metrics

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1564  rev 1 Cat: A (Rel-19)

					Source: Huawei, FiberCop, BT, Jio Platforms

(Replaces R3-256976)

Abstract:

NBC

Discussion:

CR is NBC

-	Add CMCC as co-source

Decision: 		The document was revised to R3-257303.

R3-257303	Correction on UE Performance metrics

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1564  rev 2 Cat: A (Rel-19)

					Source: Huawei, FiberCop, BT, Jio Platforms, NEC, ZTE, Samsung, CATT, Ericsson, Nokia, CMCC

(Replaces R3-257242)

Abstract:

NBC

Decision: 		The document was endorsed.

R3-256977	Correction on Average Packet Delay and Average Packet Loss DL - Option 2

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1565  Cat: A (Rel-19)

					Source: Huawei, FiberCop, BT, Jio Platforms

Abstract:

NBC

Decision: 		The document was not treated.

R3-256979	Addition of the Average Packet Loss UL measurement as UE Performance

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1567  Cat: C (Rel-18)

					Source: Huawei, FiberCop, BT

Decision: 		The document was not treated.

R3-256980	Addition of the Average Packet Loss UL measurement as UE Performance

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1568  Cat: A (Rel-19)

					Source: Huawei, FiberCop, BT

Decision: 		The document was not treated.

R3-257112	[DRAFT] Reply LS on per-UE UE performance metrics

Type: LS out		For: Approval
					to SA5, cc RAN2
					Source: Huawei

Decision: 		The document was not treated.

R3-256560	Discussion on per-UE UE performance metrics

Type: discussion		For: (not specified)
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256561	Correction to per-UE UE performance metrics

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1511  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256562	Correction to per-UE UE performance metrics

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1512  Cat: A (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256799	Correction on UE performance for AI/ML

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1538  Cat: F (Rel-18)

					Source: NEC

Decision: 		The document was not treated.

R3-256837	Discussion on supporting UL Packet Loss in AIML UE Performance feedback

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-256838	Correction on R18 UE performance feedback for UL packet loss

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1542  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was not treated.

R3-256839	Correction on R19 UE performance feedback for UL packet loss

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1543  Cat: A (Rel-19)

					Source: CMCC

Decision: 		The document was not treated.

R3-256868	Discussion on UE level performance

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-256869	Correction to XnAP on UL Packet Loss Rate metrics

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1547  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was not treated.

R3-256870	Correction to XnAP on UL Packet Loss Rate metrics

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1548  Cat: A (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R3-256931	On LS discussions with SA5 on Packet loss measurements in context of Release-19

Type: discussion		For: Approval
					Source: Ericsson, InterDigital, Jio Platforms, Verizon

Decision: 		The document was not treated.

R3-256932	Corrections to Release-18 Packet Loss measurements in UE Performance

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1556  Cat: F (Rel-18)

					Source: Ericsson, InterDigital, Jio Platforms, Verizon

Decision: 		The document was not treated.

R3-256933	Corrections to Release-19 Packet Loss measurements in UE Performance

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1557  Cat: F (Rel-19)

					Source: Ericsson, InterDigital, Jio Platforms, Verizon

Decision: 		The document was not treated.

R3-256943	Reply to R3-256518 on per-UE UE performance metrics

Type: LS out		For: Approval
					to SA5
					Source: Ericsson, InterDigital, Jio Platforms

Decision: 		The document was not treated.

R3-257093	UE Performance Metrics in Rel-18

Type: discussion		For: (not specified)
					38.423 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-257094	Removal of Average Packet Loss DL IE from Rel-18 specifications

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1580  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was not treated.

R3-257095	[Draft] Reply LS on per-UE UE performance metrics

Type: LS out		For: Approval
					to SA5, cc RAN2
					Source: Nokia

Decision: 		The document was not treated.

[Agreement] RAN3 to agree to add a reference to clause 6.3.1.1 of TS 28.558 (Packet delay) for the Average Packet Delay IE.

CATT: SA5 did not clarify the aggregation method, we either need to define in RAN3 or ask SA5 to define

Nokia: Same view as CATT, important to specify the aggregation method

ZTE: RAN3 already asked SA5 how to aggregate, it should be summation per-DRB (not RAN3 scope)

NEC: Same view as ZTE

Ericsson: Agree with ZTE, SA5 has explained how their spec calculates per UE metrics

Huawei: Share the same view as Ericsson

CATT, Nokia: Even if per-DRB is assumed, there is insufficient details in SA5 specs

Samsung: Same view as ZTE & Ericsson

Qualcomm: Also believe SA5 specs are not fully clear.

CATT: SA5 may not be aware of RAN3 need to have well-defined aggregation for inter-operability

CMCC: No need for reply LS

[Agreement]

RAN3 to agree to change Average Packet Loss DL IE to Average Packet Drop DL IE and add a reference to clause 6.3.1.6.1.1 of TS 28.558 (DL PDCP SDU Drop Rate in gNB).

Average Packet Loss DL will be discussed in R20.

RAN3 to agree to introduce the Average Packet Loss UL IE including a reference to clause 6.3.1.7.1 of TS 28.558 (UL PDCP SDU Loss Rate) in the corresponding semantics description.

Ericsson: SA5 has defined UL/DL Average Packet Loss, we are fine with the proposal

Nokia: SA5 has defined a Packet Drop Rate, not Packet Loss Rate

CB # 2_UEPerformanceMetrics

- Work on XnAP CRs capturing above agreements

- Reply LS related to aggregation methods?

(Huawei - moderator)

[Temporary suspension of trace production]

R3-256519	LS on temporary suspension of trace production

Type: LS in		For: Discussion
					Original outgoing LS: S5-253909, to RAN3, RAN2, cc -
					Source: SA5(Ericsson)

Decision: 		The document was noted.

R3-257070	Discussion on Trace Suspension feasibility

Type: discussion		For: Approval
					Source: Ericsson

Discussion:

Response in R3-257195

Decision: 		The document was noted.

R3-257195	Response to R3-256747, R3-257070

Type: response		For: Discussion
					Source: Nokia

Decision: 		The document was noted.

R3-256724	Discussion on Average Window for Alternative QoS

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was withdrawn.

R3-256725	[draft] Reply LS on Average Window for Alternative QoS

Type: LS out		For: Approval
					to SA2
					Source: Huawei

Decision: 		The document was withdrawn.

R3-257071	Reply to R3-256519 on temporary suspension of trace production

Type: LS out		For: Approval
					to SA5
					Source: Ericsson

Decision: 		The document was revised to R3-257198.

R3-257198	Reply to R3-256519 on temporary suspension of trace production

Type: LS out		For: Approval
					to SA5, cc RAN2
					Source: Ericsson

(Replaces R3-257071)

Decision: 		The document was revised to R3-257308.

R3-257308	Reply to R3-256519 on temporary suspension of trace production

Type: LS out		For: Approval
					to SA5, cc RAN2
					Source: Ericsson

(Replaces R3-257198)

Discussion:

-	Correct TDoc number on cover page

-	Remove “to R3-256519” from title

Decision: 		The document was revised to R3-257327.

R3-257327	Reply on temporary suspension of trace production

Type: LS out		For: Approval
					to SA5, cc RAN2
					Source: Ericsson

(Replaces R3-257308)

Decision: 		The document was approved.

R3-256747	Temporary suspension of trace production

Type: discussion		For: (not specified)
					Source: ZTE Corporation

Discussion:

Response in R3-257195

Decision: 		The document was noted.

R3-256748	Reply LS on temporary suspension of trace production

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

ERICSSON: SA5 is only asking for feasibility

HUAWEI: Even if technically feasible, further discussion would be needed

Nokia: We don’t believe there is any RAN3 impact

CB # 3_TempSuspendTrace

- Applicable to Logged MDT?

- Work on Reply LS based on 7071, technically feasible but RAN3 will not discuss solutions in Rel-19

(Ericsson - moderator)

[Other]

R3-256505	Reply LS on non-RedCap UE UL SRS frequency hopping for positioning

Type: LS in		For: Discussion
					Original outgoing LS: R1-2506531, to RAN3, cc RAN2
					Source: RAN1(ZTE)

Decision: 		The document was noted.

R3-256506	LS on Rel-19 higher layers parameters list Post RAN1#122

Type: LS in		For: Discussion
					Original outgoing LS: R1-2506626, to RAN2, RAN3, cc RAN4
					Source: RAN1(Ericsson)

Abstract:

Att: R1-2506622

Decision: 		The document was noted.

R3-256521	Reply to RAN2 LS on Number of UEs in RRC_INACTIVE state with data transmission

Type: LS in		For: Discussion
					Original outgoing LS: S5-254084, to RAN2, cc RAN3
					Source: SA5(Chinatelecom)

Abstract:

Att: S5-253898

Decision: 		The document was noted.

R3-256524	LS on UE data collection and data transfer

Type: LS in		For: Discussion
					Original outgoing LS: S2-2508119, to RAN2, cc RAN1, RAN3
					Source: SA2(Nokia)

Decision: 		The document was noted.

R3-256526	LS on “IETF Network Slice Application in 3GPP 5G End-to-End Network Slice”

Type: LS in		For: Discussion
					Original outgoing LS: liaison-teas-on-draft-ietf-teas-5g-network-slice-application, to SA2, SA3, SA5, RAN3, cc -
					Source: IETF teas(HuaWei)

Decision: 		The document was noted.

R3-257184	Discussion on the IETF Network Slice Application

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was not treated.

R3-257185	[draft] Reply LS on “IETF Network Slice Application in 3GPP 5G End-to-End Network Slice”

Type: LS out		For: Approval
					to IETF TEAS, cc SA2, SA3, SA5
					Source: Huawei

Decision: 		The document was revised to R3-257231.

R3-257231	Reply LS on “IETF Network Slice Application in 3GPP 5G End-to-End Network Slice”

Type: LS out		For: Approval
					to IETF TEAS, cc SA2, SA3, SA5
					Source: RAN3(Huawei)

(Replaces R3-257185)

Discussion:

-	Remove spaces before and after quotes

Decision: 		The document was revised to R3-257304.

R3-257304	Reply LS on “IETF Network Slice Application in 3GPP 5G End-to-End Network Slice”

Type: LS out		For: Approval
					to IETF TEAS, cc SA2, SA3, SA5
					Source: RAN3(Huawei)

(Replaces R3-257231)

Decision: 		The document was approved.

R3-256749	IETF Network Slice

Type: discussion		For: (not specified)
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-256750	Reply LS on IETF Network Slice

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256917	IETF Network Slice Application and 3GPP Slicing

Type: discussion		For: Discussion
					Source: Ericsson

Decision: 		The document was noted.

R3-256918	[DRAFT] Reply LS on IETF Network Slice Application in 3GPP 5G End-to-End Network Slice

Type: LS out		For: Approval
					to IETF TEAS, cc SA2, SA3, SA5
					Source: Ericsson

Decision: 		The document was not treated.

RAN3 to reply that the points mentioned in R3-244785 (e.g., referring to the 3GPP specifications on 5G NG-RAN architecture) are worthwhile to be considered when necessary.

RAN3 to reply that the FH, and MH are not 3GPP defined terminologies. And for the IETF network slices used for FH, it is up to IETF to consider the methods for mapping between 3GPP E2E Network Slice and these IETF network slices.

RAN3 to reply to IETF TEAS that in Figure 20 and Figure 24, the DU/CU-UP as well as the F1-U interface are defined in 3GPP specification, as indicated in the TS 38.401.

In Sec. 3.3, what is referred to as CU and DU, connected by the F1 interface, seems inconsistent with the definitions given by RAN3 (especially the DU).

In Sec. 3.4 there is a reference to Figure 6, but the text seems related to Figure 5.

In Sec. 7.1, Figure 20 mentions that the association between DU and CU-UP (F1-U) is according to O-RAN, but gNB-DU, gNB-CU-UP and F1-U are defined by 3GPP RAN3.

CB # 4_ReplyLSIETF

- Work on a Reply LS, taking into account the feedback in 7184, 6749, and 6917

(Huawei - moderator)

### 8.2	LSin received during the meeting

R3-257296	LS on Security parameter in A-IoT paging

Type: LS in		For: discussion
					Original outgoing LS: R2-2507920, to SA3, cc CT1, RAN3
					Source: RAN2(CMCC)

Decision: 		The document was noted.

### 8.3	Left over LSs / pending actions

## 9	Corrections to Rel-19 or earlier releases

### 9.1	LTE

#### 9.1.1	R19 IoT NTN enhancements

R3-256683	Clarification on S1 Removal for IoT NTN

Type: draftCR		For: Endorsement
					36.300 v19.0.0
					Source: ZTE Corporation, LG Electronics, Ericsson, CMCC, Nokia, Nokia Shanghai Bell, CATT, Samsung

Discussion:

-	Add Huawei, Deutsche Telekom, Jio Platforms as co-sources

Decision: 		The document was revised to R3-257199.

R3-257199	Clarification on S1 Removal for IoT NTN

Type: draftCR		For: Endorsement
					36.300 v19.0.0
					Source: ZTE Corporation, LG Electronics, Ericsson, CMCC, Nokia, Nokia Shanghai Bell, CATT, Samsung, Huawei, Deutsche Telekom, Jio Platforms

(Replaces R3-256683)

Decision: 		The document was endorsed.

R3-256737	Correction of TNL address change and S1 removal

Type: draftCR		For: Endorsement
					36.300 v19.0.0
					Source: Huawei, Deutsche Telekom, Jio Platforms

Discussion:

1st change

ERICSSON, Nokia, CATT: correct but seems obvious for transparent case

Note in 23.21.5.3 of TS 36.300 applies to the case of regenerative NTN payloads. Same applies for NR NTN.

Decision: 		The document was noted.

R3-256733	Correction on the Presence of the MME Name IE in the S1 REMOVAL RESPONSE message

Type: CR		For: Agreement
					36.413 v19.0.0	  CR-1975  Cat: F (Rel-19)

					Source: Nokia, Nokia Shanghai Bell, Xiaomi, Qualcomm, Ericsson, CATT, Airbus, Jio Platforms, ZTE, LG Electronics

Discussion:

-	Change criticality for MME Name IE to ignore

-	Add Huawei as co-source

Decision: 		The document was revised to R3-257200.

R3-257200	Correction on the MME Name IE in the S1 REMOVAL RESPONSE message

Type: CR		For: Agreement
					36.413 v19.0.0	  CR-1975  rev 1 Cat: F (Rel-19)

					Source: Nokia, Nokia Shanghai Bell, Xiaomi, Qualcomm, Ericsson, CATT, Airbus, Jio Platforms, ZTE, LG Electronics, Huawei

(Replaces R3-256733)

Decision: 		The document was endorsed.

R3-256740	Correction of MME name for S1 Removal

Type: CR		For: Agreement
					36.413 v19.0.0	  CR-1976  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was noted.

R3-256738	Provision of S&F Mode Indication Information

Type: draftCR		For: Endorsement
					36.300 v19.0.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, Ericsson

Discussion:

-	Change “Store and forward” to “Store and Forward”

-	Change “serving cell and neighbour cells” to “serving eNB and neighbour eNBs”

-	Add Nokia, Nokia Shanghai Bell, Xiaomi, CATT, ZTE, CMCC, NEC as co-sources

Decision: 		The document was revised to R3-257201.

R3-257201	Provision of S&F Mode Indication Information

Type: draftCR		For: Endorsement
					36.300 v19.0.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, Ericsson, Nokia, Nokia Shanghai Bell, Xiaomi, CATT, ZTE, CMCC, NEC

(Replaces R3-256738)

Decision: 		The document was endorsed.

R3-256739	Clarify MME unchange for store and forward mode change

Type: draftCR		For: Endorsement
					36.300 v19.0.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, Ericsson

Discussion:

At the time instance of the store and forward mode transition, the MME is not necessarily changed for the NTN regenerative payload served UEs.

Decision: 		The document was noted.

R3-256843	Discussion on introducing S1 suspend resume during hard FLSO

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-256614	Suspend/resume S1 during hard FLSO

Type: draftCR		For: Endorsement
					36.300 v19.0.0
					Source: Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, ZTE, CATT, China Telecom, CMCC, LG Electronics, Xiaomi, Samsung

Decision: 		The document was not treated.

R3-256732	Suspend/resume S1 during hard FLSO

Type: CR		For: Agreement
					36.413 v19.0.0	  CR-1974  Cat: F (Rel-19)

					Source: Nokia, Nokia Shanghai Bell, ZTE Corporation, CATT, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Decision: 		The document was not treated.

#### 9.1.2	R19 LTE-based 5G Broadcast

R3-256751	Correction of Time Interleaving parameters

Type: CR		For: Agreement
					36.443 v19.0.0	  CR-0135  Cat: F (Rel-19)

					Source: ZTE Corporation,Pengcheng Laboratory,China Unicom,China Telecom

Discussion:

-	Change WI code to LTE_terr_bcast_Ph2-Core

-	Update revision history

Decision: 		The document was revised to R3-257202.

R3-257202	Correction of Time Interleaving parameters

Type: CR		For: Agreement
					36.443 v19.0.0	  CR-0135  rev 1 Cat: F (Rel-19)

					Source: ZTE Corporation, Pengcheng Laboratory, China Unicom, China Telecom

(Replaces R3-256751)

Decision: 		The document was endorsed.

#### 9.1.3	Other

R3-256741	Correction of time-based handover for IoT NTN

Type: draftCR		For: Endorsement
					36.300 v18.6.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson

Discussion:

Response in R3-257197

QUALCOMM: Prefer 7197 proposal for normative text

CATT: Nothing is needed in stage 3, agree to remove “single” in stage 2 with Note as clarification

-	Replace “to a single target eNB via the Source eNB to Target eNB Transparent Container” with “to a target enB”

-	No additional note needed

-	Add Nokia, Nokia Shanghai Bell, LG Electronics as co-sources

Decision: 		The document was revised to R3-257214.

R3-257214	Correction of time-based handover for IoT NTN

Type: draftCR		For: Endorsement
					36.300 v18.6.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson, Nokia, Nokia Shanghai Bell, LG Electronics

(Replaces R3-256741)

Decision: 		The document was endorsed.

R3-257197	Response to R3-256741, R3-256742, R3-256745, R3-256746

Type: response		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256742	Correction of time-based handover for IoT NTN

Type: draftCR		For: Endorsement
					36.300 v19.0.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson

Discussion:

Response in R3-257197

-	Replace “to a single target eNB via the Source eNB to Target eNB Transparent Container” with “to a target enB”

-	No additional note needed

-	Add Nokia, Nokia Shanghai Bell, LG Electronics as co-sources

Decision: 		The document was revised to R3-257215.

R3-257215	Correction of time-based handover for IoT NTN

Type: draftCR		For: Endorsement
					36.300 v19.0.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson, Nokia, Nokia Shanghai Bell, LG Electronics

(Replaces R3-256742)

Decision: 		The document was endorsed.

### 9.2	NR

#### 9.2.1	R19 SON/MDT

R3-256628	Correction of the description of the geographical area scope

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1318  Cat: F (Rel-19)

					Source: Nokia, ZTE, Huawei, CATT, Samsung, Ericsson

Decision: 		The document was endorsed.

R3-256658	Correction on the description of the geographical area scope

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1526  Cat: F (Rel-19)

					Source: ZTE Corporation, Nokia, Huawei, Samsung, CATT, Ericsson

Decision: 		The document was endorsed.

R3-256674	Correction of the description of the geographical area scope

Type: draftCR		For: Endorsement
					37.320 v19.0.0
					Source: Huawei, Nokia, Samsung, CATT, ZTE, Ericsson

Decision: 		The document was endorsed.

R3-256959	Correcting SCG activation time after CHO

Type: discussion		For: (not specified)
					Source: Huawei, CMCC, Deutsche Telekom

Discussion:

Stage 2, or Stage 3, or do nothing

ERICSSON: Prefer not to have stage 3 solution, problem is acknowledged so we can work on stage 2 text

ZTE: Prefer Stage 2

Nokia: Same view as ERICSSON

Samsung: Strong concern on stage 2 solution prefer do nothing

CATT: Prefer stage 2, but details need further discussion

Lenovo: Same view as CATT

QUALCOMM: No strong view between stage 2 and stage 3

CB # 5_SCGactTime

-  Work on a stage 2 CR

(Huawei - moderator)

Decision: 		The document was noted.

R3-256960	Correcting SCG activation time after CHO

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1561  Cat: F (Rel-19)

					Source: Huawei, CMCC, Deutsche Telekom, Qualcomm

Decision: 		The document was noted.

R3-256961	Correcting SCG activation time after CHO

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, CMCC, Deutsche Telekom

Decision: 		The document was revised to R3-257216.

R3-257216	Correcting SCG activation time after CHO

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, CMCC, Deutsche Telekom

(Replaces R3-256961)

Discussion:

-	capitalize “handover preparation”

Decision: 		The document was revised to R3-257305.

R3-257305	Correcting SCG activation time after CHO

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, CMCC, Deutsche Telekom

(Replaces R3-257216)

Decision: 		The document was endorsed.

R3-256599	Correction in the Target Cell CGI IE of the HO Report for too late CHO with Candidate SCG(s)

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1520  Cat: F (Rel-19)

					Source: Nokia

Discussion:

ZTE: prefer to revise the semantics of the Target Cell CGI IE since it’s a mandatory IE

HUAWEI: Should say that Target Cell CGI is ignored

ERICSSON: What we have works, even though not perfect

Decision: 		The document was noted.

R3-256629	Problem with location-based selection of UEs for immediate MDT

Type: discussion		For: Discussion
					Source: Nokia

Discussion:

ZTE: can be up to implementation, but then note does not seem needed

HUAWEI: no issue

Telecom Italia: no need for CR

Can UE location be obtained by gNB on request?

Decision: 		The document was noted.

R3-256650	Correction on MRO for CHO with candidate SCG for 38.423

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1524  Cat: F (Rel-19)

					Source: CATT

Discussion:

SAMSUNG: UE ID is needed since it is UE-associated signaling

Nokia, ERICSSON: Same view as Samsung

Decision: 		The document was noted.

R3-256651	Correction on MRO for CHO with candidate SCG and S-CPAC for 37.340

Type: draftCR		For: Endorsement
					37.340 v19.0.0
					Source: CATT

Discussion:

Samsung, Nokia: 2nd and 3rd change not needed

For 1st change, only remove the sentence that begins with “In case the intra-SN PSCell change has been triggered in the last serving SN”?

Decision: 		The document was noted.

R3-256659	Correction on beam information

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0482  Cat: F (Rel-19)

					Source: Samsung

Discussion:

HUAWEI: It’s not necessary for Stage 2 to be the same as Stage 3

Decision: 		The document was noted.

R3-256660	Correction on beam information

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1590  Cat: F (Rel-19)

					Source: Samsung

Discussion:

ERICSSON: alternative in 7114

Lenovo: support this CR

Decision: 		The document was noted.

R3-256856	Correction of missing elementary procedure table

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1544  Cat: F (Rel-19)

					Source: ZTE Corporation

Discussion:

Merged in XnAP rapporteur CR

Decision: 		The document was merged.

R3-257113	Clarifying the MRO for LTM use-case

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1630  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was noted.

R3-257114	BFR in CSI-RS beam

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1631  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was noted.

[Reporting without RLF report]

R3-256962	Failure report without RLF report

Type: discussion		For: (not specified)
					Source: Huawei, CMCC, Deutsche Telekom

Decision: 		The document was noted.

R3-256963	Failure report without RLF report

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0495  Cat: F (Rel-19)

					Source: Huawei, CMCC, Deutsche Telekom

Decision: 		The document was not treated.

R3-256964	Failure report without RLF report

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1614  Cat: F (Rel-19)

					Source: Huawei, CMCC, Deutsche Telekom

Decision: 		The document was not treated.

R3-256781	Correction on MRO for LTM failure without RLF report

Type: discussion		For: Agreement
					38.473 v..
					Source: NEC

Discussion:

HUAWEI: Agree that Source Cell CGI IE is needed, and should be mandatory. But some other changes are not needed.

ERICSSON: CR needed but details need further discussion

SAMSUNG: The two new IEs are needed

CB # 6_LTMfailureWithoutRLFreport

- Work on CR details

(NEC - moderator)

Decision: 		The document was noted.

R3-257235	Correction for LTM failure without RLF-report

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1641  Cat: F (Rel-19)

					Source: NEC, Huawei, Ericsson, Nokia, Google, CATT, ZTE, Samsung

Decision: 		The document was endorsed.

R3-256752	Correction of missing stage 2 description of LTM MRO without RLF Report

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0488  Cat: F (Rel-19)

					Source: ZTE Corporation,Google,Pengcheng Laboratory,China Unicom,China Telecom

Decision: 		The document was noted.

Introduce a new reporting of re-establishment cell ID for candidate cells (neither target nor source) in the reporting from the target DU to source DU.

Extend the reporting of SSB information to also cover the candidate cells (neither target nor source).

Extend the reporting of TA information to also cover the candidate cells (neither target nor source).

gNB-CU include the re-establishment cell, the last serving cell and the C-RNTI in the failure reporting without RLF report.

Discuss whether failure type is required. If required, it should be mandatory and not optional.

Send the failure report information without RLF report only when the UE re-establishes in a cell different from source and candidate cells, or when the UE re-connects after a failure.

ZTE, Nokia: enhancement, not a correction

SAMSUNG: it’s a new scenario, not a correction

CATT: same view as ZTE and Samsung

#### 9.2.2	R19 AI/ML for NG-RAN

R3-256523	Reply to LS on Continuous MDT

Type: LS in		For: Discussion
					Original outgoing LS: S5-254110, to RAN3, cc RAN2
					Source: SA5(Ericsson)

Abstract:

Att: S5-254099, S5-254111, S5-253907, S5-253908, S5-254070

Decision: 		The document was noted.

R3-256603	Introduction of predicted PSCell for Mobility Optimization in NR-DC

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1521  Cat: F (Rel-19)

					Source: ZTE Corporation, Qualcomm, Samsung, CATT, CMCC, China Telecom, China Unicom, Ofinno

Discussion:

Nokia: Don’t believe Predicted PSCell ID can be predicted by the MN

HUAWEI: Same view as Nokia

ERICSSON: Already discussed without consensus during WI

Decision: 		The document was noted.

R3-256923	Abnormal conditions for gNB-CU Configuration Update

Type: discussion		For: Approval
					Source: Ericsson, InterDigital, Jio Platforms, FiberCop

Decision: 		The document was noted.

R3-256924	Abnormal conditions for gNB-CU Configuration Update

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1610  Cat: F (Rel-19)

					Source: Ericsson, InterDigital, Jio Platforms, FiberCop

Decision: 		The document was noted.

R3-257079	Corrections of AI/ML-based CCO

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1627  Cat: F (Rel-19)

					Source: Nokia, Ofinno, Jio Platforms

Decision: 		The document was not treated.

R3-256828	Miscellaneous correction to XnAP - AI/ML for NG-RAN

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1541  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was revised to R3-257243.

R3-257243	Miscellaneous corrections on AI/ML for NG-RAN

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1541  rev 1 Cat: F (Rel-19)

					Source: ZTE Corporation, Huawei, NEC, Ericsson, Nokia

(Replaces R3-256828)

Decision: 		The document was endorsed.

R3-256829	Miscellaneous correction to F1AP - AI/ML for NG-RAN

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1601  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was revised to R3-257244.

R3-257244	Miscellaneous correction to F1AP - AI/ML for NG-RAN

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1601  rev 1 Cat: F (Rel-19)

					Source: ZTE Corporation

(Replaces R3-256829)

Discussion:

-	add Huawei as co-source

Decision: 		The document was revised to R3-257328.

R3-257328	Miscellaneous corrections on AI/ML for NG-RAN

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1601  rev 2 Cat: F (Rel-19)

					Source: ZTE Corporation, CMCC, NEC, Ericsson, Nokia, Huawei

(Replaces R3-257244)

Decision: 		The document was endorsed.

R3-256840	Correction on R19 AIML F1-U stage2

Type: CR		For: Agreement
					38.470 v19.0.0	  CR-0168  Cat: B (Rel-19)

					Source: CMCC

Decision: 		The document was noted.

R3-256866	Correction on AI/ML for NG-RAN

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1546  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was noted.

R3-256867	Correction to E1AP on AI/ML for NG-RAN

Type: CR		For: Agreement
					37.483 v19.0.0	  CR-0179  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was revised to R3-257245.

R3-257245	Correction to E1AP on AI/ML for NG-RAN

Type: CR		For: Agreement
					37.483 v19.0.0	  CR-0179  rev 1 Cat: F (Rel-19)

					Source: CATT, Huawei, Nokia, ZTE, CMCC

(Replaces R3-256867)

Decision: 		The document was endorsed.

R3-256925	Correction for gNB-DU Configuration Update

Type: discussion		For: Approval
					Source: Ericsson, InterDigital, Jio Platforms, FiberCop

Decision: 		The document was not treated.

R3-256926	Correction for gNB-DU Configuration Update

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1611  Cat: F (Rel-19)

					Source: Ericsson, InterDigital, Jio Platforms, FiberCop

Decision: 		The document was not treated.

R3-256927	Stage 2 corrections for AI/ML based CCO

Type: discussion		For: Approval
					Source: Ericsson, InterDigital, Jio Platforms, FiberCop

Decision: 		The document was not treated.

R3-256928	Stage 2 corrections for AI/ML based CCO

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0492  Cat: F (Rel-19)

					Source: Ericsson, InterDigital, Jio Platforms, FiberCop

Decision: 		The document was not treated.

R3-256929	Corrections to Performance Delay Monitoring IEs

Type: discussion		For: Approval
					Source: Ericsson, InterDigital, Jio Platforms, Verizon

Decision: 		The document was not treated.

R3-256930	Semantic description of Performance Delay Monitoring IEs

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1612  Cat: F (Rel-19)

					Source: Ericsson, InterDigital, Jio Platforms, Verizon

Decision: 		The document was not treated.

R3-256986	Correction on AI/ML-based CCO use case

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1570  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was noted.

R3-256987	Correction on AI/ML-based CCO use case

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1617  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was not treated.

R3-257078	(CR to TS 38.423, TS 37.483, TS 38.300, TS 38.401) Corrections of UE performance and CCO

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was noted.

R3-257096	Alignment on UE Performance Terminology

Type: discussion		For: (not specified)
					Source: Nokia, Deutsche Telekom, Verizon Wireless

Decision: 		The document was not treated.

R3-257097	UE Performance Delay Monitoring naming alignment

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1629  Cat: F (Rel-19)

					Source: Nokia, Deutsche Telekom, Verizon Wireless

Decision: 		The document was not treated.

CB# 7_R19AIMLRAN

- XnAP misc corrections: check 6828; merge agreeable corrections (if any) from 6866, 6986, 7078

- F1AP misc corrections: check 6829; merge agreeable corrections (if any) from 6926, 6930, 6987, 7097

- E1AP misc corrections: check 6867, merge agreeable corrections (if any) from 7078

- F1-U check 6840 if time allows

- How to handle error cases, e.g., by abnormal conditions?

(ZTE - moderator)

R3-257248	Summary of Offline Discussion for CB: # 7_R19AIMLRAN

Type: discussion		For: discussion
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-257246	Correction for Slice UE performance metrics

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1601  Cat: F (Rel-19)

					Source: Nokia, ZTE, Huawei, CMCC, NEC, Ericsson

Decision: 		The document was endorsed.

R3-257247	Correction to E1AP for UE performance metrics

Type: CR		For: discussion
					37.483 v19.0.0	  CR-0185  Cat: F (Rel-19)

					Source: CMCC, ZTE, Huawei, Ericsson, Nokia, CATT

Decision: 		The document was endorsed.

[Continuous MDT]

R3-256935	Support for Continuous MDT in RAN3 specifications

Type: discussion		For: Approval
					Source: Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange, Verizon

Decision: 		The document was noted.

R3-256936	Continuous MDT support

Type: draftCR		For: Endorsement
					37.320 v19.0.0
					Source: Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange, Verizon

Decision: 		The document was not treated.

R3-256937	Continuous MDT support

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1559  Cat: F (Rel-19)

					Source: Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange, Verizon

Decision: 		The document was revised to R3-257285.

R3-257285	Continuous MDT support

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1559  rev 1 Cat: F (Rel-19)

					Source: Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange, Verizon

(Replaces R3-256937)

Discussion:

-	New IE also needed in RETRIEVE UE CONTEXT RESPONSE

-	Cover page: uncheck “other specs”, remove TEI19, etc.

-	Add ZTE as co-source

Decision: 		The document was revised to R3-257309.

R3-257309	Continuous MDT support

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1559  rev 2 Cat: F (Rel-19)

					Source: Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange, Verizon

(Replaces R3-257285)

Discussion:

Objection from Nokia

Decision: 		The document was noted.

R3-256938	Continuous MDT support

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1352  Cat: F (Rel-19)

					Source: Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange, Verizon

Decision: 		The document was not treated.

R3-256944	Reply to R3-256523 on Continuous MDT

Type: LS out		For: Approval
					to SA5
					Source: Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange

Abstract:

Att: R3-256936, R3-256938

Decision: 		The document was revised to R3-257310.

R3-257310	Reply to R3-256523 on Continuous MDT

Type: LS out		For: Approval
					to SA5
					Source: Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange

(Replaces R3-256944)

Decision: 		The document was not treated.

R3-256865	Completion of Continuous management-based MDT in RAN3

Type: discussion		For: Decision
					Source: CATT, ZTE, Samsung,China Telecom, CMCC

Decision: 		The document was noted.

R3-256864	Support for continuous management-based MDT

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1545  Cat: F (Rel-19)

					Source: CATT, ZTE, Samsung,China Telecom, CMCC

Decision: 		The document was not treated.

R3-256606	CR to 38.300 to support Continuous MDT collection

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: ZTE Corporation, CATT, Samsung, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256607	CR to 37.320 to support Continuous MDT collection

Type: draftCR		For: Endorsement
					37.320 v19.0.0
					Source: ZTE Corporation, CATT, Samsung, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256690	Continuous MDT for split architecture

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0484  Cat: F (Rel-19)

					Source: Samsung, CATT, ZTE, CMCC, China Telecom

Decision: 		The document was not treated.

R3-257189	Discussion on the Reply LS from SA5 on Continuous MDT (C-MDT)

Type: discussion		For: Agreement
					Source: Huawei, Jio Platforms, Ofinno

Decision: 		The document was noted.

R3-256982	Discussion on the Reply LS from SA5 on Continuous MDT (C-MDT)

Type: discussion		For: Agreement
					Source: Huawei, Jio Platforms, Ofinno

Decision: 		The document was withdrawn.

R3-256983	Introducing support for Continuous management-based MDT (C-MDT)

Type: draftCR		For: Endorsement
					37.320 v19.0.0
					Source: Huawei, Jio Platforms, Ofinno

Decision: 		The document was withdrawn.

R3-256985	Introducing support for Continuous management-based MDT (C-MDT)

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1569  Cat: B (Rel-19)

					Source: Huawei, Jio Platforms, Ofinno

Decision: 		The document was withdrawn.

R3-256984	Correction on support for Continuous management-based MDT (C-MDT)

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, Jio Platforms, Ofinno

Decision: 		The document was not treated.

R3-257187	Support for Continuous management-based MDT (C-MDT)

Type: draftCR		For: Endorsement
					37.320 v19.0.0
					Source: Huawei, Jio Platforms, Ofinno

Decision: 		The document was not treated.

R3-257188	Support for Continuous management-based MDT (C-MDT)

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1599  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, Ofinno

Decision: 		The document was not treated.

R3-256800	Correction on continuous MDT

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1539  Cat: F (Rel-19)

					Source: NEC

Decision: 		The document was not treated.

R3-257098	Continuous MDT Discussion

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was noted.

Option 1:

Add the TR and TRSR assigned by a source gNB to a UE configured for Continuous management-based MDT in the Xn: Handover Request and Retrieve UE Context Response.

Option 2:

Re-use the existing NG-RAN Trace ID IE included in the Trace Activation IE of the HANDOVER REQUEST message to allow the target gNB to identify that the handed over UE was previously selected by the source gNB for a C-MDT session.

Add a new codepoint “Immediate MDT and Logged MDT” needs to be introduced in the MDT Activation IE included in the Trace Activation IE of the HANDOVER REQUEST message to allow the target gNB to re-configure the UE with both Immediate and Logged MDT.

Only the TR is an indication to the RAN for continuous MDT and not the TRSR Prefix Configuration parameter?

Area over which Continuous Management based MDT is configured?

Security aspects?

ERICSSON: this is management-based MDT, and SA5 has discussed security for this

HUAWEI: SA5 has already discussed the security

ZTE, NEC, CATT: Same view as ERICSSON and HW

Nokia: Fine that SA5 discussed but they are not responsible for security

MEASUREMENTS:

Measurements to be configured in a C-MDT session should be the ones which are common to be collected in both Immediate MDT and Logged MDT.

Include at least RSRP, RSRQ and SINR measurements as the list of measurements which can be configured in C-MDT. Whether additional measurements are to be considered needs further discussion.

R3-257284	Summary of Offline Discussion for CB: # 8_ContinuousMDT

Type: discussion		For: discussion
					Source: Ericsson(moderator)

Decision: 		The document was noted.

[Agreement] It is agreed to add a new IE in the Xn Handover Request and Xn: Retrieve UE context Response message to indicate that the UE is configured with Management Based continuous MDT. The IE references the existing NG RAN Trace ID IE

It is discussed whether Signalling based MDT can be always prioritised with respect to Continous MDT. It should be checked whether it is possible to deconfigure a UE from Management Based Immediate and Logged MDT to give priority to the Signaling Based MDT configuration.

How can a UE be deconfigured from Logged Continuous MDT.

[Per-UE UE performance metrics R19]

R3-256604	CR to 38.423 to Slice UE performance metrics

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1522  Cat: F (Rel-19)

					Source: ZTE Corporation, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256605	CR to 37.483 to UE performance per DRB

Type: CR		For: Agreement
					37.483 v19.0.0	  CR-0176  Cat: F (Rel-19)

					Source: ZTE Corporation, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256688	Correction on the reference of Slice Average Packet Loss

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1529  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R3-256689	Correction on the reference of Average Packet Loss

Type: CR		For: Agreement
					37.483 v19.0.0	  CR-0178  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R3-256798	Correction on UE performance for AI/ML

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1537  Cat: F (Rel-19)

					Source: NEC

Decision: 		The document was not treated.

R3-256934	Corrections to Release-19 Packet Loss measurements in UE Performance for Slicing

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1558  Cat: F (Rel-19)

					Source: Ericsson, InterDigital, Jio Platforms, Verizon

Decision: 		The document was not treated.

R3-256978	Correction on Slice UE Performance

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1566  Cat: F (Rel-19)

					Source: Huawei, FiberCop, BT, Jio Platforms

Decision: 		The document was not treated.

R3-256981	Correction on UE Performance

Type: CR		For: Agreement
					37.483 v19.0.0	  CR-0180  Cat: F (Rel-19)

					Source: Huawei, FiberCop, BT, Jio Platforms

Decision: 		The document was not treated.

#### 9.2.3	R19 WAB

R3-256806	Correction on WAB NG management

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1340  Cat: F (Rel-19)

					Source: Huawei, CANON Research Centre France, Lenovo

Discussion:

ERICSSON: terminology does not seem correct

“consider this transmitting NG-RAN node has WAB-gNB functionality.”?

Decision: 		The document was revised to R3-257258.

R3-257258	Correction on the NG Setup related to WAB

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1340  rev 1 Cat: F (Rel-19)

					Source: Huawei, CANON Research Centre France, Lenovo, Samsung, Nokia, Nokia Shanghai Bell

(Replaces R3-256806)

Decision: 		The document was endorsed.

R3-256890	Correction on handover a WAB-MT to a target WAB-gNB

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1348  Cat: F (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

SA2 spec (23.501) and XnAP first needs checking

Decision: 		The document was revised to R3-257259.

R3-257259	Correction on handover a WAB-MT to a target WAB-gNB

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1348  rev 1 Cat: F (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R3-256890)

Decision: 		The document was noted.

R3-256728	Correction on WAB Xn management

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1533  Cat: F (Rel-19)

					Source: Huawei

Discussion:

1. Add Neighbour NG-RAN Node TNL Address List IE

ERICSSON, QUALCOMM: Not a correction, its new functionality that has been discussed with no consensus

2. Remove WAB-MT Identifier from NG-RAN NODE CONFIGURATION UPDATE

QUALCOMM: No need, scenario has not been precluded

Nokia: Nothing is broken

Decision: 		The document was noted.

R3-256761	Correction on the description of WAB-MT Identifier

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1534  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was endorsed.

R3-256951	Corrections for WAB

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1560  Cat: F (Rel-19)

					Source: ZTE Corporation

Discussion:

QUALCOMM: Change 1 is already captured in Stage 2, wording is too strong.

Nokia: “may” is typically not used in reject cases

Decision: 		The document was revised to R3-257260.

R3-257260	Corrections of Xn setup failure for WAB-gNB

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1560  rev 1 Cat: F (Rel-19)

					Source: ZTE Corporation, Ericsson, Nokia, Nokia Shanghai Bell, CATT, Qualcomm

(Replaces R3-256951)

Decision: 		The document was revised to R3-257306.

R3-257306	Corrections of Xn-related issues for WAB-gNB

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1560  rev 2 Cat: F (Rel-19)

					Source: ZTE Corporation, Ericsson, Nokia, Nokia Shanghai Bell, CATT, Qualcomm, Canon, Huawei

(Replaces R3-257260)

Decision: 		The document was endorsed.

R3-257191	Corrections of WAB

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1600  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was merged.

R3-257190	(CR TS 38.423): Correction of WAB

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0504  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was withdrawn.

R3-256715	Correction of WAB

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson, Jio Platforms, Qualcomm, CATT

Decision: 		The document was endorsed.

R3-256714	Corrections of WAB

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0486  Cat: F (Rel-19)

					Source: Ericsson, Jio Platforms

Decision: 		The document was merged.

R3-256727	Correction on WAB

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0487  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was revised to R3-257263.

R3-257263	Correction of WAB

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0487  rev 1 Cat: F (Rel-19)

					Source: Huawei

(Replaces R3-256727)

Decision: 		The document was endorsed.

R3-256760	Corrections to WAB stage-2

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0489  Cat: F (Rel-19)

					Source: CATT, Ericsson

Decision: 		The document was revised to R3-257262.

R3-257262	Corrections to WAB stage-2

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0489  rev 1 Cat: F (Rel-19)

					Source: CATT, Ericsson

(Replaces R3-256760)

Decision: 		The document was endorsed.

R3-256889	Correction on AULI

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0491  Cat: F (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was merged.

R3-256950	Corrections for WAB

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0493  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was revised to R3-257261.

R3-257261	Corrections for WAB

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0493  rev 1 Cat: F (Rel-19)

					Source: ZTE Corporation, Ericsson, Jio Platforms, Nokia, Nokia Shanghai Bell, Samsung, Qualcomm

(Replaces R3-256950)

Decision: 		The document was revised to R3-257307.

R3-257307	Corrections for WAB

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0493  rev 2 Cat: F (Rel-19)

					Source: ZTE Corporation, Ericsson, Jio Platforms, Nokia, Nokia Shanghai Bell, Samsung, Qualcomm, Huawei

(Replaces R3-257261)

Decision: 		The document was endorsed.

R3-257138	Correction to TS 38.401 for WAB

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0501  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was merged.

CB# 9_R19WAB

- NGAP corrections: check 6806 and 6890

- XnAP corrections: check 6951 and 7191

- 38.401 corrections: check CRs

(ZTE -moderator)

R3-257286	Summary of Offline Discussion for CB: # 9_R19WAB

Type: discussion		For: discussion
					Source: ZTE Corporation

Decision: 		The document was noted.

#### 9.2.4	R19 NR Femto

R3-256516	LS on security verification related to NR Femto nodes

Type: LS in		For: Discussion
					Original outgoing LS: S3-252930, to RAN3, cc SA2
					Source: SA3(HuaWei)

Abstract:

Att: S3-252929

Decision: 		The document was noted.

R3-256729	Correction on NR femto procedures

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1328  Cat: F (Rel-19)

					Source: Huawei

Discussion:

-	Fix missing deleted text

Decision: 		The document was revised to R3-257217.

R3-257217	Correction on NR femto procedures

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1328  rev 1 Cat: F (Rel-19)

					Source: Huawei, Nokia

(Replaces R3-256729)

Decision: 		The document was endorsed.

R3-256763	Correction on GW Context Release Indication

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1333  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was noted.

R3-256879	Correction of Paging NR Femtos

Type: discussion		For: (not specified)
					Source: Nokia, NTT Docomo, BT

Discussion:

QUALCOMM: This has been discussed during WI with no consensus.

Decision: 		The document was noted.

R3-256881	Correction of Paging NR Femtos

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1345  Cat: F (Rel-19)

					Source: Nokia, NTT Docomo, BT

Decision: 		The document was noted.

R3-256953	Corrections for Femto

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1353  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was revised to R3-257225.

R3-257225	Corrections for Femto

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1353  rev 1 Cat: F (Rel-19)

					Source: ZTE Corporation, Nokia, LG Electronics

(Replaces R3-256953)

Decision: 		The document was endorsed.

R3-257139	Correction to TS 38.413 for NR Femto

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1367  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was noted.

R3-256762	Corrections on NR Femto Architecture

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: CATT

Decision: 		The document was merged.

R3-256807	Correction on NR Femto functions

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei

Decision: 		The document was merged.

R3-256952	Corrections for Femto

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: ZTE Corporation

Decision: 		The document was merged.

R3-257092	Correction of NR Femtos functions

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Nokia

Decision: 		The document was revised to R3-257218.

R3-257218	Correction of NR Femtos functions

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Nokia

(Replaces R3-257092)

Discussion:

-	change “CONFIGU²RATION” to “CONFIGURATION”

-	revert last change in 4.10.2.3

Decision: 		The document was revised to R3-257312.

R3-257312	Correction of NR Femtos functions

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Nokia, CATT, Huawei, ZTE, LG Electronics

(Replaces R3-257218)

Decision: 		The document was endorsed.

R3-257105	Correction of NR Femtos Security

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Nokia

Decision: 		The document was not treated.

CB# 10_R19Femto

- NGAP misc corrections: check 6953

- 38.300 misc corrections: check CRs

(Nokia - moderator)

#### 9.2.5	R19 Mobility enhancements

R3-257127	CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM

Type: discussion		For: (not specified)
					Source: Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Nokia

Decision: 		The document was noted.

R3-257128	Correction on CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1583  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Nokia

Discussion:

-	Add Ofinno, Qualcomm as co-sources

Decision: 		The document was revised to R3-257223.

R3-257223	Correction on CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1583  rev 1 Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Nokia

(Replaces R3-257128)

Decision: 		The document was revised to R3-257313.

R3-257313	Correction on CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1583  rev 2 Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Nokia, ZTE, LG Electronics, NEC, Google, Samsung, Ericsson, Offinno, Qualcomm

(Replaces R3-257223)

Decision: 		The document was endorsed.

R3-257129	Correction on CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1633  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Nokia

Decision: 		The document was revised to R3-257224.

R3-257224	Correction on CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1633  rev 1 Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Nokia

(Replaces R3-257129)

Discussion:

-	Add Ofinno, Qualcomm as co-sources

-	Change “TS 38.331 [10]” to “TS 38.331 [8]”

Decision: 		The document was revised to R3-257314.

R3-257314	Correction on CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1633  rev 2 Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Nokia, ZTE, LG Electronics, NEC, Google, Samsung, Ericsson, Offinno, Qualcomm

(Replaces R3-257224)

Decision: 		The document was endorsed.

R3-256530	Discussion on remaining LTM Issues

Type: discussion		For: Agreement
					Source: Nokia

Decision: 		The document was noted.

R3-256825	Semi-Persistent CSI-RS activation/deactivation with TCI state

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1540  Cat: F (Rel-19)

					Source: Nokia, CATT, China Telecom,

Decision: 		The document was revised to R3-257295.

R3-257295	Semi-Persistent CSI-RS activation/deactivation with TCI state

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1540  rev 1 Cat: F (Rel-19)

					Source: Nokia, CATT, China Telecom,

(Replaces R3-256825)

Discussion:

-	IEs with range field should be in bold

-	Add criticality

-	Add semantics description, “Indicates the TCI states where the semi persistent CSI-RS resource is transmitted. The mapping between the CSI-RS Resource indicated by the LTM CSI Resource Configuration ID IE and the TCI state is defined in TS 38.321 [x].”

-	Add Ofinno as co-source

Decision: 		The document was revised to R3-257315.

R3-257315	Semi-Persistent CSI-RS activation with TCI state

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1540  rev 2 Cat: F (Rel-19)

					Source: Nokia, CATT, China Telecom, Ericsson, Huawei, NEC, ZTE, Google, Samsung, LG Electronics, Qualcomm, Ofinno, Lenovo, Jio Platforms

(Replaces R3-257295)

Decision: 		The document was endorsed.

R3-256872	Semi-Persistent CSI-RS activation/deactivation with TCI state

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1606  Cat: F (Rel-19)

					Source: CATT, Nokia, ZTE, China Telecom

Decision: 		The document was revised to R3-257300.

R3-257300	Semi-Persistent CSI-RS activation/deactivation with TCI state

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1606  rev 1 Cat: F (Rel-19)

					Source: CATT, Nokia, ZTE, China Telecom, Ericsson, Huawei, NEC, ZTE, Google, LG Electronics, Qualcomm

(Replaces R3-256872)

Discussion:

-	In tabular, change “transmits” to “is transmitted”

-	8.3.12.2: change “gNB-DU shall” to “gNB-CU shall”

-	removed unchanged sections

-	Add Lenovo as co-source

Decision: 		The document was revised to R3-257316.

R3-257316	Semi-Persistent CSI-RS activation with TCI state

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1606  rev 2 Cat: F (Rel-19)

					Source: CATT, Nokia, ZTE, China Telecom, Ericsson, LG Electronics, Samsung, Huawei, NEC, ZTE, Google, LG Electronics, Qualcomm, Offinno,Lenovo

(Replaces R3-257300)

Decision: 		The document was endorsed.

R3-256531	Remaining Conditional LTM Issues

Type: discussion		For: Agreement
					Source: Nokia

Decision: 		The document was not treated.

R3-256699	Discussion on shared CFRA resource for inter-CU LTM

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R3-256705	LTM CFRA Resource Information transfer

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1531  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256706	LTM UE Association Information Transfer

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1532  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256802	Early Data Forwarding for subsequent inter-CU LTM

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: NEC, Huawei, Google, Jio Platforms, Qualcomm Inc, Nokia, Samsung, Lenovo, CATT, LG Electronics, Ericsson

Discussion:

-	Add ZTE, Ofinno as co-sources

Decision: 		The document was revised to R3-257219.

R3-257219	Early Data Forwarding for subsequent inter-CU LTM

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: NEC, Huawei, Google, Jio Platforms, Qualcomm Inc, Nokia, Samsung, Lenovo, CATT, LG Electronics, Ericsson, ZTE, Ofinno

(Replaces R3-256802)

Decision: 		The document was endorsed.

R3-256803	Early Data Forwarding for subsequent MCG LTM

Type: draftCR		For: Endorsement
					37.340 v19.0.0
					Source: NEC, Huawei, Google, Jio Platforms, Qualcomm Inc, Nokia, Samsung, Lenovo, CATT, LG Electronics, Ericsson

Discussion:

-	Add ZTE, Ofinno as co-sources

Decision: 		The document was revised to R3-257220.

R3-257220	Early Data Forwarding for subsequent MCG LTM

Type: draftCR		For: Endorsement
					37.340 v19.0.0
					Source: NEC, Huawei, Google, Jio Platforms, Qualcomm Inc, Nokia, Samsung, Lenovo, CATT, LG Electronics, Ericsson, ZTE, Ofinno

(Replaces R3-256803)

Decision: 		The document was endorsed.

R3-256830	[Draft TP for TS 38.473, TS 38.423] Remaining issues in Inter-CU LTM

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-256831	[Draft TP for TS 38.473, TS 38.401] Remaining issues in Intra-CU Conditional LTM

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-256857	Essential Corrections for Inter-CU (SCG) LTM

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-256858	(CR to 38.473) Essential correction for Inter-CU SCG LTM

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1602  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R3-256871	Essential correction for inter-CU LTM on F1AP

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1605  Cat: F (Rel-19)

					Source: CATT,Huawei, China Telecom

Decision: 		The document was not treated.

R3-256873	Essential correction on inter-CU LTM in DC

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1549  Cat: F (Rel-19)

					Source: CATT,Huawei, China Telecom

Decision: 		The document was not treated.

R3-256874	Discussion on LTM candidate PSCell cancel procedure in DC

Type: discussion		For: Decision
					Source: CATT,Huawei

Decision: 		The document was not treated.

R3-256875	Correction on LTM candidate PSCell cancel procedure in DC

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1550  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R3-257033	Correction on SN initiated inter-SN SCG LTM procedure

Type: draftCR		For: Endorsement
					37.340 v19.0.0
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-257042	PRACH resources for RACH-less LTM

Type: discussion		For: (not specified)
					Source: Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO

Decision: 		The document was not treated.

R3-257043	PRACH resources for Inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1574  Cat: F (Rel-19)

					Source: Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO

Decision: 		The document was not treated.

R3-257044	PRACH resources for RACH-less LTM

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1625  Cat: F (Rel-19)

					Source: Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO

Decision: 		The document was not treated.

R3-257045	PRACH resource for RACH-less LTM

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0499  Cat: F (Rel-19)

					Source: Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO

Decision: 		The document was not treated.

R3-257046	PRACH resources for RACH-less LTM

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO

Decision: 		The document was not treated.

R3-257047	Fast LTM recovery for Inter-CU LTM

Type: discussion		For: (not specified)
					Source: Ericsson, Jio Platforms, Lenovo

Decision: 		The document was not treated.

R3-257048	Fast LTM Recovery for Inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1575  Cat: F (Rel-19)

					Source: Ericsson, Jio Platforms, Lenovo

Decision: 		The document was not treated.

R3-257050	CSI-RS Coordination for inter-gNB LTM

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson

Decision: 		The document was not treated.

R3-257058	Corrections for LTM in NR-DC

Type: draftCR		For: Endorsement
					37.340 v19.0.0
					Source: Ericsson

Decision: 		The document was not treated.

R3-257125	Correction on LTM Cell Switch Notification

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1582  Cat: F (Rel-19)

					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-257126	Correction related to LTM Security Information

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1632  Cat: F (Rel-19)

					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-257130	Correction on CSI Report Configuration for CSI Acquisition for inter-CU LTM

Type: discussion		For: (not specified)
					Source: Huawei, Jio Platforms, CMCC, China Telecom, NEC, Lenovo

Decision: 		The document was not treated.

R3-257131	Correction on CSI report configuration for CSI acquisition for inter-CU LTM

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1634  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CMCC, China Telecom, Lenovo

Decision: 		The document was not treated.

R3-257132	Correction on CSI report configuration for CSI acquisition for inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1584  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CMCC, China Telecom, Lenovo

Decision: 		The document was not treated.

R3-257146	Essential corrections to XnAP for inter-CU LTM

Type: discussion		For: (not specified)
					Source: Huawei, Jio Platforms, CMCC, Lenovo

Decision: 		The document was not treated.

R3-257147	Essential corrections to XnAP for inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1590  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CMCC, Lenovo

Decision: 		The document was not treated.

R3-257148	Exchange of early RACH Resources Requester ID for inter-CU LTM

Type: discussion		For: (not specified)
					Source: Huawei, Jio Platforms, CATT, CMCC, China Telecom, Lenovo

Decision: 		The document was not treated.

R3-257149	Exchange of early RACH Resources Requester ID for inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1591  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CATT, CMCC, China Telecom, Lenovo

Decision: 		The document was not treated.

R3-257150	Correction on the Rel-19 set ID assignment in inter-CU LTM

Type: discussion		For: (not specified)
					Source: Huawei, Jio Platforms, CATT, China Telecom

Decision: 		The document was not treated.

R3-257151	Correction on the Rel-19 set ID assignment in inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1592  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CATT, China Telecom

Decision: 		The document was not treated.

R3-257152	Correction on inter-CU LTM and conditional intra-CU LTM

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0502  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, CATT, China Telecom, Lenovo

Decision: 		The document was not treated.

R3-257153	Correction on inter-CU LTM

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, Jio Platforms, CATT, China Telecom, Lenovo

Decision: 		The document was not treated.

R3-257154	Corrections on inter-CU MCG LTM without SN change

Type: discussion		For: (not specified)
					Source: Huawei, Jio Platforms, Lenovo, Google

Decision: 		The document was not treated.

R3-257155	Correction on security key update for inter-CU MCG LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1593  Cat: F (Rel-19)

					Source: Huawei, Jio Platforms, China Telecom, Lenovo, ,Google

Decision: 		The document was not treated.

R3-257156	Stage 2 corrections to inter-CU MCG LTM

Type: draftCR		For: Endorsement
					37.340 v19.0.0
					Source: Huawei, Jio Platforms, Lenovo, Google

Decision: 		The document was not treated.

R3-257157	Discussion on supporting inter-CU SCG LTM and the L2 reset function in inter-CU LTM

Type: discussion		For: Decision
					Source: Google

Decision: 		The document was not treated.

R3-257158	Correction to L2 reset in inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1594  Cat: F (Rel-19)

					Source: Google

Decision: 		The document was not treated.

R3-257172	Remaining issues for CSI-RS coordination

Type: discussion		For: (not specified)
					Source: Ericsson, Jio Platforms, Verizon Wireless, LG Electronics

Decision: 		The document was not treated.

R3-257173	Corrections on CSI-RS coordination for LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1596  Cat: F (Rel-19)

					Source: Ericsson, Jio Platforms, Verizon Wireless, LG Electronics

Decision: 		The document was noted.

R3-257174	Corrections on CSI-RS coordination for LTM

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1638  Cat: F (Rel-19)

					Source: Ericsson, Jio Platforms, Verizon Wireless, LG Electronics

Decision: 		The document was not treated.

R3-257175	Corrections on Inter-CU LTM

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1597  Cat: F (Rel-19)

					Source: Ericsson, Lenovo, LG Electronics

Decision: 		The document was not treated.

To introduce the three IEs, ltm-NZP-CSI-RS-ResourceSet ltm-CSI-IM-Resource, and ltm-CSI-IM-ResourceSet into LTM preparation related procedures in F1AP and XnAP.

[Agreement] The candidate gNB decides whether to provide the CSI-IM resource to the source gNB by implementation

ERICSSON: Option 1 is not valid, support Option 2 plus some deltas

QUALCOMM: Support Option 2

ZTE: Support Option 2

CB # 11_R19Mob_CSI-IMRscConfig

- Work on XnAP and F1AP CRs based on agreement above

- Both Periodic and Semi-persistent

(Huawei - moderator)

RAN3 to discuss the following two options on the LTM modification for inter-CU LTM

LTM modification is not supported for inter-CU LTM in Rel-19?

QUALCOMM: Prefer Option 1, avoids RAN2 impact of Option 2

ERICSSON: No change needed

HUAWEI: Same view as Qualcomm

ZTE: prefer Option 2 in LTM preparation phase

Nokia: prefer Option 1, Option 2 is a new feature

Lenovo: Same view as Nokia and Qualcomm

CATT: Option 1 is enough

Samsung: Prefer Option 1

CB # 12_R19Mob_CSI-RSRscConfigRelease

- Discuss whether LTM modification is supported for inter-CU LTM in Rel-19?

- No new procedure

(Ericsson - moderator)

R3-257280	Summary of offline discussion on modification of LTM configurations

Type: discussion		For: discussion
					Source: Ericsson

Decision: 		The document was noted.

R3-257281	LS on candidate-initiated modification of LTM configurations

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: RAN3(Ericsson)

Decision: 		The document was noted.

Whether the modification of LTM configurations is allowed over network signalings?

If yes, which configuration needs to be modified?

When the LTM modification can be triggered? For example, during or after LTM preparation? Is it possible to modify during LTM execution or in subsequent LTM?

If there is any configuration that can be modified, which way is preferred?

a. Existing LTM Cancel procedure, and the source re-initiate the configuration procedure

b. New procedure

Observation: For each periodic CSI-RS, the QCL-info is shared (via qcl-InfoPeriodicCSI-RS) with an associated TCI state ID. However, for SP CSI-RSs this is not feasible, since they are dynamically activated and their TCI state is determined dynamically and signalled to the UE in the same MAC CE that triggers activation.

The F1AP DU-CU CSI-RS COORDINATION REQUEST and CU-DU CSI-RS COORDINATION REQUEST messages include a TCI State/QCL-info List which includes the TCI State Index for each SP CSI-RS being activated.

The XnAP CSI-RS COORDINATION REQUEST message includes a TCI State/QCL-info List which includes the TCI State Index for each SP CSI-RS being activated.

QUALCOMM: support dynamic signaling but should be provided from candidate to source (source cannot make decision)

Lenovo: should be in Response message, not Request

ERICSSON: Not convinced of the benefit, may need to check with RAN2

Huawei, CATT, NEC, ZTE, Google, SAMSUNG: Support the proposals

CB # 13_R19Mob_TCI-StateInfo

- Discussion whether source initiated or target initiated

- Potential stage 3 impacts and/or LS to RAN2

(Nokia - moderator)

R3-256552	LTM CFRA Resource Information transfer

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1509  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was withdrawn.

R3-256553	LTM UE Association Information Transfer

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1510  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was withdrawn.

R3-257038	Correction on ETA resource release in DU

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1623  Cat: A (Rel-19)

					Source: Ofinno, LLC

Decision: 		The document was withdrawn.

#### 9.2.6	R19 NR NTN enhancements

R3-256919	Adding Extended AMF Name to NG REMOVAL RESPONSE message

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1351  Cat: F (Rel-19)

					Source: Ericsson, Thales, LG Electronics, Xiaomi, CATT, ZTE, Nokia, Nokia Shanghai Bell, Eutelsat Group, Deutsche Telekom, Qualcomm Incorporated, Airbus, Jio Platforms

Discussion:

-	remove TEI19

-	Add Huawei, NEC, CMCC, Samsung as co-source

Decision: 		The document was revised to R3-257221.

R3-257221	Adding Extended AMF Name to NG REMOVAL RESPONSE message

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1351  rev 1 Cat: F (Rel-19)

					Source: Ericsson, Thales, LG Electronics, Xiaomi, CATT, ZTE, Nokia, Nokia Shanghai Bell, Eutelsat Group, Deutsche Telekom, Qualcomm Incorporated, Airbus, Jio Platforms, Huawei, NEC, CMCC, Samsung

(Replaces R3-256919)

Decision: 		The document was endorsed.

R3-256743	Correction of TNL address change and O&M requirements

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, Deutsche Telekom, Jio Platforms

Discussion:

-	keep only change in 16.14.7, remove “(s)”

-	replace “serving neighbour cell(s)” with “of neighbour cell(s)”

-	Add ZTE as co-source

Decision: 		The document was revised to R3-257222.

R3-257222	Correction of O&M requirements for NTN

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, ZTE

(Replaces R3-256743)

Decision: 		The document was endorsed.

R3-256744	Correction of NG Removal Response and Broadcast Session Modification messages

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1329  Cat: F (Rel-19)

					Source: Huawei, Deutsche Telekom, Jio Platforms

Decision: 		The document was noted.

[NG suspend/resume]

R3-256631	Further discussion on NG Suspend/Resume

Type: discussion		For: Decision
					Source: CATT

Discussion:

Response in R3-257196

CATT: Time T3 may not be fully deterministic

In some NTN deployments, there is an interruption time by design

Decision: 		The document was noted.

R3-257196	Response to R3-256631, R3-256734, R3-256842, R3-256614

Type: response		For: Discussion
					Source: Ericsson, Thales, Deutsche Telekom, ESA, Airbus

Decision: 		The document was noted.

R3-256632	Support of NG Suspend and Resume

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: CATT, Nokia, Nokia Shanghai Bell, ZTE Corporation, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Decision: 		The document was not treated.

R3-256633	[DRAFT] LS on Support of NG/S1 Suspend and Resume

Type: LS out		For: Approval
					to SA2
					Source: CATT

Decision: 		The document was not treated.

R3-256675	NG Suspend and Resume during Hard FLSO

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1325  Cat: F (Rel-19)

					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, CATT, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Decision: 		The document was not treated.

R3-256920	Short Feeder Link Interruption and UE Retention

Type: discussion		For: Discussion
					Source: Ericsson, Jio Platforms, Thales, Huawei, ESA, Deutsche Telekom

Discussion:

[MCC minutes] R3-256920 is also co-signed by Huawei, ESA, and Deutsche Telekom

Decision: 		The document was not treated.

R3-256921	UE Context Retention at SCTP Recovery

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson, Jio Platforms, Thales

Decision: 		The document was not treated.

R3-256734	Discussion on UE Retention at hard FLSO

Type: discussion		For: (not specified)
					Source: Nokia, Nokia Shanghai Bell, ZTE Corporation, CATT, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Discussion:

Response in R3-257196

Decision: 		The document was not treated.

R3-256787	NG interface suspend and resume indication for hard feeder link switchover

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: NEC

Decision: 		The document was not treated.

R3-256788	NG interface suspend and resume indication

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1337  Cat: F (Rel-19)

					Source: NEC

Decision: 		The document was not treated.

R3-256842	Discussion on introducing NG suspend resume during hard FLSO

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

Response in R3-257196

Decision: 		The document was not treated.

#### 9.2.7	R19 Ambient IoT

R3-256512	Reply LS to Reply LS on the removal of service type information

Type: LS in		For: Discussion
					Original outgoing LS: S2-2507689, to RAN3, RAN2, cc RAN1
					Source: SA2(LGE)

Abstract:

Att: TS 23.369 CR#0001

Decision: 		The document was noted.

R3-256522	Reply LS on Ambient IoT progress of RAN3 on OAM requirements

Type: LS in		For: Discussion
					Original outgoing LS: S5-254085, to RAN3, SA2, cc -
					Source: SA5(HuaWei)

Abstract:

Att: S5-254105

Decision: 		The document was noted.

R3-256525	LS on Structure updates of AIoT Identifiers

Type: LS in		For: Discussion
					Original outgoing LS: C4-253575, to SA2, RAN3, cc SA3, RAN2
					Source: CT4(CICT mobile)

Abstract:

Att: C4-253570

Decision: 		The document was noted.

R3-256504	LS on the maximum supported AIoT NAS container length

Type: LS in		For: Discussion
					Original outgoing LS: C1-255679, to RAN2, cc RAN1, RAN3
					Source: CT1(Lenovo)

Decision: 		The document was noted.

R3-256517	Reply LS on paging ID length

Type: LS in		For: Discussion
					Original outgoing LS: S3-252933, to SA2, RAN2, CT4, cc RAN3
					Source: SA3(CATT)

Decision: 		The document was noted.

R3-256634	Corrections to A-IoT context definition

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0481  Cat: F (Rel-19)

					Source: CATT

Discussion:

HUAWEI, Nokia: question whether change is correct

NEC: supports CR

Decision: 		The document was noted.

R3-256641	Consideration on A-IoT leftovers on IE details

Type: discussion		For: Agreement
					Source: Huawei, CMCC, China Unicom, China Telecom

Decision: 		The document was noted.

R3-256642	Correction on A-IoT leftovers on IE details

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1321  Cat: F (Rel-19)

					Source: Huawei, CMCC, China Unicom, China Telecom

Decision: 		The document was revised to R3-257251.

R3-257251	Correction on A-IoT leftovers on IE details

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1321  rev 1 Cat: F (Rel-19)

					Source: Huawei, CMCC, China Unicom, China Telecom, Nokia, LG Electronics, CATT, NEC

(Replaces R3-256642)

Decision: 		The document was endorsed.

R3-256643	Consideration on Interface Management procedures for A-IoT

Type: discussion		For: Agreement
					Source: Huawei, CMCC, Lenovo, China Unicom, China Telecom

Decision: 		The document was noted.

R3-256644	Correction on Interface Management procedures for A-IoT

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1322  Cat: F (Rel-19)

					Source: Huawei, CMCC, Lenovo, China Unicom, China Telecom

Decision: 		The document was revised to R3-257252.

R3-257252	Correction on Interface Management procedures for A-IoT

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1322  rev 1 Cat: F (Rel-19)

					Source: Huawei, CMCC, Lenovo, China Unicom, China Telecom, Nokia, LG Electronics, CATT, NEC

(Replaces R3-256644)

Decision: 		The document was endorsed.

R3-256661	Correction on AIoT IE encoding

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1324  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was merged.

R3-256708	Correction on AIoT

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1327  Cat: F (Rel-19)

					Source: ZTE Corporation, China Telecom

Decision: 		The document was revised to R3-257276.

R3-257276	Correction on AIoT

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1327  rev 1 Cat: F (Rel-19)

					Source: ZTE Corporation, China Telecom

(Replaces R3-256708)

Decision: 		The document was noted.

R3-256882	Open issues of Ambient IOT Release 19

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was noted.

R3-256883	Correction of Ambient IOT

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1346  Cat: F (Rel-19)

					Source: Nokia

Decision: 		The document was merged.

R3-257061	On further open issue for Rel-19 AIoT

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was merged.

R3-256554	Clarification on AIoT

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1316  Cat: F (Rel-19)

					Source: ZTE Corporation, China Telecom

Decision: 		The document was withdrawn.

R3-256639	Consideration on A-IoT dedicated Cause values

Type: discussion		For: Agreement
					Source: Huawei, CMCC, Futurewei, Lenovo, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256640	Correction on A-IoT dedicated Cause values

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1320  Cat: F (Rel-19)

					Source: Huawei, CMCC, Futurewei, Lenovo, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256555	Correction on AIoT

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1317  Cat: F (Rel-19)

					Source: ZTE Corporation, China Telecom

Decision: 		The document was withdrawn.

R3-256635	Corrections to A-IoT procedure texts

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1319  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was revised to R3-257253.

R3-257253	Corrections to A-IoT procedure texts

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1319  rev 1 Cat: F (Rel-19)

					Source: CATT, Huawei, Nokia, LG Electronics, NEC, ZTE, Samsung

(Replaces R3-256635)

Decision: 		The document was endorsed.

R3-256707	Clarification on AIoT

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1326  Cat: F (Rel-19)

					Source: ZTE Corporation, China Telecom

Decision: 		The document was noted.

Update the encoding of A-IoT Correlation Identifier IE from OCTET STRING to INTEGER (0..65535, ...).

Update the encoding of AIOTF Identifier IE from OCTET STRING to OCTET STRING (SIZE(16)).

Update the presence of Device Report List IE in the Inventory Report Transfer IE from mandatory to optional.

Keep the current encoding of the Time Interval IE.

Keep the current encoding of the A-IoT Device Identification Requested IE.

[Agreement] Include the A-IoT Support IE in the RAN CONFIGURATION UPDATE message

CATT, Nokia, QUALCOMM: agreeable

NEC: Not needed, NG interface can be reset

Proposal 2: Introduce an A-IoT Supported PLMN List IE in the NG SETUP REQUEST and RAN CONFIGURATION UPDATE messages.

CATT, Nokia, ERICSSON: does not seem needed, can be done by configuration

QUALCOMM: Not sure what action is for AMF

Explicit indication of command type

Nokia, Xiaomi, NEC, QUALCOMM: not needed, can be understood from the expected data size

HUAWEI, CATT: support

ERICSSON: RAN2 discussing whether device can respond in time, so expected data size may not be enough, at least “write” codepoint could be useful

Lenovo: Agree with ERICSSON

CMCC: there may be some cases where it’s useful

Add “maximum time to respond” as assistance information in the Inventory Request.

ZTE: We already have Time Interval

CATT: Don’t see the need

HUAWEI: Similar purpose as Command Type

Lenovo: Expected Number of Devices can help serve the purpose

CB # 14_R19AmbientIOT

- NGAP encoding details: check 6642, taking into account 6661, 6883, 7061

- NGAP CR implementing above agreement

- whether Command Type is needed?

- NGAP misc corrections: check 6635, 6707

- Discuss security-related corrections if time allows

(Huawei - moderator)

R3-257254	Summary of Offline Discussion for CB # 14_R19AmbientiIOT

Type: discussion		For: discussion
					Source: Huawei

Decision: 		The document was noted.

FFS on the NGAP impact on the A-IoT security aspects.

FFS on the A-IoT dedicated cause values.

[Security-related]

R3-256758	Correction on sercurity support in A-IoT

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Xiaomi

Decision: 		The document was not treated.

R3-256759	Correction on sercurity support in A-IoT

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1332  Cat: F (Rel-19)

					Source: Xiaomi

Decision: 		The document was not treated.

R3-257060	On security related open topics for Rel-19 AIoT

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was not treated.

R3-256789	Cyphering indication for A-IoT NAS Command Request PDU

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1338  Cat: F (Rel-19)

					Source: NEC

Decision: 		The document was not treated.

R3-256790	A-IoT device identifier privacy protection impact on RAN3

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1339  Cat: F (Rel-19)

					Source: NEC

Decision: 		The document was not treated.

R3-256844	Correction on A-IoT security aspects

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: CMCC, Huawei

Decision: 		The document was not treated.

R3-256845	Correction on A-IoT security aspects

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1344  Cat: B (Rel-19)

					Source: CMCC, Huawei

Decision: 		The document was not treated.

#### 9.2.8	R19 Network Energy Saving

R3-256513	Reply LS on energy saving indication from CN to RAN

Type: LS in		For: Discussion
					Original outgoing LS: S2-2507784, to RAN3, cc RAN2
					Source: SA2(LGE)

Abstract:

Att: TS 23.501 CR#6329, TS 23.502 CR#5494

Decision: 		The document was noted.

R3-256703	Correction on OD-SIB1

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1530  Cat: F (Rel-19)

					Source: ZTE Corporation

Discussion:

ERICSSON, NEC: We don’t normally specify “if not included”

Nokia: Stage 2 covers this already?

Decision: 		The document was noted.

R3-256704	Correction on OD-SIB1

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1599  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was merged.

R3-256791	Correction on allowed cell list for OD-SSB

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1600  Cat: F (Rel-19)

					Source: NEC

Discussion:

HUAWEI: support the CR to allow CU some control

ZTE: support

QUALCOMM,, Rakuten: Was discussed during the WI but no consensus

CATT: Not needed

ERICSSON: Nothing broken

Decision: 		The document was noted.

R3-256792	Correction on inter-gNB OD-SIB1 capability transfer

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1536  Cat: F (Rel-19)

					Source: NEC

Discussion:

Nokia: Support the new “Cell A Indicator”

ZTE: New indicator is not needed, there are other ways

SAMSUNG: Not needed, prefer OAM solution

CATT: Not needed, capabilities are static, OAM enough

Rakuten: Already possible using Provision Request

Decision: 		The document was noted.

R3-257088	Corrections to Xn support for on-demand SIB1 coordination

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1578  Cat: F (Rel-19)

					Source: Ericsson, Qualcomm, Jio Platforms

Decision: 		The document was revised to R3-257255.

R3-257255	Corrections to Xn support for on-demand SIB1 coordination

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1578  rev 1 Cat: F (Rel-19)

					Source: Ericsson, Qualcomm, Jio Platforms , ZTE, Deutsche Telekom, Huawei, Samsung, NEC. Rakuten, Nokia, CATT

(Replaces R3-257088)

Discussion:

-	Cover page: add 3.2 to clauses affected, replace period with comma after NEC

-	Add Telecom Italia to co-sources

Decision: 		The document was revised to R3-257317.

R3-257317	Corrections to Xn support for on-demand SIB1 coordination

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1578  rev 2 Cat: F (Rel-19)

					Source: Ericsson, Qualcomm, Jio Platforms , ZTE, Deutsche Telekom, Huawei, Samsung, NEC, Rakuten, Nokia, CATT, Telecom Italia

(Replaces R3-257255)

Decision: 		The document was endorsed.

R3-257089	Corrections to F1 support of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1628  Cat: F (Rel-19)

					Source: Ericsson, Qualcomm, Jio Platforms

Decision: 		The document was not treated.

R3-256764	Corrections on OD-SIB1 Configuration Provision

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1535  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was merged.

R3-256899	Correction of OD-SIB1 procedure for network energy saving

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1554  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was not treated.

R3-256900	Correction of OD-SIB1 procedure for network energy saving

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1609  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was revised to R3-257256.

R3-257256	Correction of OD-SIB1 procedure for network energy saving

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1609  rev 1 Cat: F (Rel-19)

					Source: Huawei, Ericsson, Qualcomm, Jio Platforms, ZTE, Deutsche Telekom,  Samsung, NEC. Rakuten, Nokia

(Replaces R3-256900)

Decision: 		The document was revised to R3-257318.

R3-257318	Correction of OD-SIB1 procedure for network energy saving

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1609  rev 2 Cat: F (Rel-19)

					Source: Huawei, Ericsson, Qualcomm, Jio Platforms, ZTE, Deutsche Telekom, Samsung, NEC, Rakuten, Nokia, CATT, Telecom Italia

(Replaces R3-257256)

Decision: 		The document was endorsed.

R3-257080	Discussion on ANR for OD-SIB1

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-257081	Correction on ANR for OD-SIB1

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1577  Cat: F (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R3-257091	Corrections to OD-SIB1 Configuration Provision Status Update

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1579  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was not treated.

R3-257140	Correction to TS 38.423 for NES

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1589  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was merged.

R3-257141	Correction to TS 38.473 for NES

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1635  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R3-257087	Corrections to stage-2 specification for support for on-demand SIB1

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0500  Cat: F (Rel-19)

					Source: Ericsson, Qualcomm, Jio Platforms

Decision: 		The document was not treated.

R3-256765	Corrections on OD-SIB1 Stage-2

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0490  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R3-257090	Corrections to TS 38.420 on support of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.420 v19.0.0	  CR-0052  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was not treated.

CB# 15_R19NES

- XnAP misc corrections: check 7088, 6764, 6899, 7091, 7140

- F1AP misc corrections: check 6704, 7089, 6900

- Stage 2: check 38.401, 38.420 CRs if time allows

(Ericsson - moderator)

R3-257257	Summary of offline discussions: Rel-19 Network Energy Saving FALSE)

Type: discussion		For: discussion
					Source: Ericsson

Decision: 		The document was noted.

#### 9.2.9	R19 Low Power WUS/WUR

R3-256507	LS on enabling/disabling LP-WUS per UE with NAS signalling

Type: LS in		For: Discussion
					Original outgoing LS: R2-2506261, to SA2, CT1, RAN3, cc -
					Source: RAN2(HuaWei)

Decision: 		The document was noted.

R3-256766	On introducing LP-WUS disabling indication

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: CATT, Nokia, Huawei, Ericsson, ZTE, Qualcomm

Discussion:

-	remove changes on changes

-	update cover page, correct styles

Decision: 		The document was revised to R3-257227.

R3-257227	On introducing LP-WUS disabling indication

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: CATT, Nokia, Huawei, Ericsson, ZTE, Qualcomm

(Replaces R3-256766)

Decision: 		The document was endorsed.

R3-256956	Introducing of LP-WUS disabling

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1354  Cat: F (Rel-19)

					Source: ZTE Corporation, Ericsson, Nokia, Huawei, Qualcomm, CATT

Decision: 		The document was endorsed.

R3-257019	Introducing LP-WUS disabling indication

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1573  Cat: F (Rel-19)

					Source: Ericsson, ZTE, Huawei, CATT, Nokia, Qualcomm Inc.

Decision: 		The document was endorsed.

R3-256901	[draft] Reply LS on enabling/disabling LP-WUS per UE with NAS signalling

Type: LS out		For: Approval
					to RAN2, SA2, cc CT1
					Source: Huawei

Abstract:

Att: R3-256956, R3-257019

Discussion:

-	do not attached CRs, update LS text accordingly

-	Fix title and source

Decision: 		The document was revised to R3-257228.

R3-257228	Reply LS on enabling/disabling LP-WUS per UE with NAS signalling

Type: LS out		For: Approval
					to RAN2, SA2, cc CT1
					Source: RAN3(Huawei)

(Replaces R3-256901)

Decision: 		The document was approved.

R3-256697	Discussion on RAN2 LS on per-UE enable disable LP-WUS by NAS

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R3-256698	(TP for 38.413 38.423 38.473) Support per-UE enable disable LP-WUS by NAS

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R3-256884	Correction of LP-WUS Assistance Information

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Nokia

Decision: 		The document was revised to R3-257230.

R3-257230	Correction of LP-WUS Assistance Information

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Nokia, Ericsson, Huawei, NTT Docomo Inc.

(Replaces R3-256884)

Discussion:

-	Remove first change (overlaps with 7227)

Decision: 		The document was revised to R3-257319.

R3-257319	Correction of LP-WUS Assistance Information

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Nokia, Ericsson, Huawei, NTT Docomo Inc.

(Replaces R3-257230)

Decision: 		The document was endorsed.

R3-256885	Correction of LP-WUS Assistance Information

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1347  Cat: F (Rel-19)

					Source: Nokia

Discussion:

ERICSSON: Maybe stage 2 correction is sufficient

Decision: 		The document was noted.

R3-256902	Correction of Further Extended UE Identity Index Value

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1350  Cat: F (Rel-19)

					Source: Huawei

Discussion:

ZTE, CATT, QUALCOMM: Conditional presence does not seem correct for all cases

Decision: 		The document was noted.

R3-256903	Correction of RAN Paging for low-power wake-up signal and receiver

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1555  Cat: F (Rel-19)

					Source: Huawei

Discussion:

-	Add Nokia, Ericsson, Qualcomm, ZTE, CATT co-sources

-	Check affected clauses

Decision: 		The document was revised to R3-257229.

R3-257229	Correction of RAN Paging for low-power wake-up signal and receiver

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1555  rev 1 Cat: F (Rel-19)

					Source: Huawei, Nokia, Ericsson, Qualcomm, ZTE, CATT

(Replaces R3-256903)

Decision: 		The document was endorsed.

R3-257020	LP-WUS correction

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson

Discussion:

DCM, ZTE, QUALCOMM,, Nokia: Does not align with RAN3 agreements

HUAWEI: RAN2 scope

Vodafone: support the CR

Decision: 		The document was noted.

R3-257021	LS on LP-WUS usage over Xn

Type: LS out		For: Approval
					to RAN2
					Source: Ericsson

Decision: 		The document was noted.

CB# 16_R19LP-WUS

-  Check 6884, find agreeable wording?

(Nokia - moderator)

#### 9.2.10	R19 Evolution of Duplex Operation

R3-256508	Reply LS on simultaneous configuration of SBFD and DC

Type: LS in		For: Discussion
					Original outgoing LS: R2-2506492, to RAN1, RAN4, cc RAN3
					Source: RAN2(ZTE)

Decision: 		The document was noted.

R3-256653	Correction on SBFD RACH configuration for XnAP

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1525  Cat: F (Rel-19)

					Source: CATT, Qualcomm, CMCC, Charter

Discussion:

ERICSSON, Nokia: Why use OCTET STRING, should use explicit IEs

QUALCOMM: It is only for specification simplicity

Decision: 		The document was noted.

R3-256654	Correction on SBFD RACH configuration for F1AP

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1589  Cat: F (Rel-19)

					Source: CATT, Qualcomm, CMCC, Charter

Decision: 		The document was not treated.

R3-256692	Correction on the indication of L1 UE-to-UE CLI

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1598  Cat: F (Rel-19)

					Source: Samsung

Discussion:

Nokia, QUALCOMM: Support

ZTE: Already stage 2 text in 38.401, which may be sufficient

HUAWEI: Not critical

CATT: Same view as ZTE

ERICSSON: Stage 3 not needed

Decision: 		The document was noted.

R3-257068	Corrections of semantic descriptions for SBFD IEs

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1576  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was noted.

R3-257069	Corrections to the procedures supporting for SBFD

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1626  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was not treated.

R3-257162	Correction to XnAP on CLI Indication

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1595  Cat: F (Rel-19)

					Source: ZTE Corporation, China Telecom, Pengcheng Laboratory, Huawei

Decision: 		The document was noted.

R3-257163	Correction to F1AP on CLI Indication

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1636  Cat: F (Rel-19)

					Source: ZTE Corporation, China Telecom, Pengcheng Laboratory, Huawei

Decision: 		The document was not treated.

R3-257164	Correction to F1AP on SBFD RACH configuration

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1637  Cat: F (Rel-19)

					Source: ZTE Corporation

Discussion:

ERICSSON: Why is list needed

Decision: 		The document was noted.

R3-257178	Corrections to SBFD operation

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1639  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was revised to R3-257250.

R3-257250	Corrections to SBFD operation

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1639  rev 1 Cat: F (Rel-19)

					Source: Huawei

(Replaces R3-257178)

Discussion:

-	9.2.18.1: revert change “, to request CLI mitigation”

Decision: 		The document was revised to R3-257320.

R3-257320	Corrections to SBFD operation

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1639  rev 2 Cat: F (Rel-19)

					Source: Huawei, Nokia, Samsung

(Replaces R3-257250)

Decision: 		The document was endorsed.

R3-257176	Clarification to UE-to-UE CLI mitigation in SBFD operation

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, CATT, China Unicom

Decision: 		The document was not treated.

R3-256691	Correction on the introduction of SBFD

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0485  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R3-257177	Clarification to CLI mitigation in SBFD operation

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0503  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was not treated.

R3-256652	Correction on Evolution of NR duplex operation for 38.470

Type: CR		For: Agreement
					38.470 v19.0.0	  CR-0167  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

CB# 17_R19Duplex

-  discuss SBFD RACH configuration exchange over Xn, 6653/6654 and 7164 can be used as starting point, reuse explicit IEs if feasible

- discuss indication of L1 UE-to-UE CLI based on 6692

- XnAP misc corrections: check 7068, 7162 (and merge if possible?)

- F1AP misc corrections: check 7069, 7163, 7178 (and merge if possible?)

- Stage 2: check 38.300, 38.401, 38.470 if time allows

(Samsung - moderator)

R3-257249	Summary of Discussion on CB: # 17_R19Duplex

Type: discussion		For: discussion
					Source: Samsung

Decision: 		The document was noted.

[Agreement] Reuse existing AP level explicit IEs for SBFD RACH configuration.

Further discuss which IEs shall be added for the SBFD RACH configuration.

Check the problem with the possibility of transmitting a whole empty message, by changing the presence of CLI Measurement Result IE to optional over Xn/F1. (Regarding 7162 and 7163)

Clarify how the gNB-CU can be aware about whether DU has configured L1 UE-to-UE CLI measurement configuration in CellGroupConfig.

CellGroupConfig is generated and compiled by which node, i.e., CU or DU?

#### 9.2.11	R19 AI/ML for air interface

R3-256511	LS to RAN1 and RAN3 on NW side data collection

Type: LS in		For: Discussion
					Original outgoing LS: R2-2506545, to RAN1, RAN3, cc -
					Source: RAN2(ZTE)

Decision: 		The document was noted.

R3-256509	LS on User consent for Data collection at the UE for NW-side model training

Type: LS in		For: Discussion
					Original outgoing LS: R2-2506541, to SA3, cc RAN3, SA5
					Source: RAN2(NTT DoCoMo)

Decision: 		The document was noted.

R3-256510	Reply LS on OAM-centric solution for NW-side data collection

Type: LS in		For: Discussion
					Original outgoing LS: R2-2506543, to SA5, cc RAN3
					Source: RAN2(HuaWei)

Decision: 		The document was noted.

R3-256514	Reply LS on signalling feasibility of dataset and parameter sharing

Type: LS in		For: Discussion
					Original outgoing LS: S2-2508104, to RAN2, cc RAN, RAN1, RAN3, SA, SA3, SA5
					Source: SA2(Samsung)

Decision: 		The document was noted.

R3-256520	Reply LS on signalling feasibility of dataset and parameter sharing

Type: LS in		For: Discussion
					Original outgoing LS: S5-254083, to RAN2, cc RAN, SA, RAN1, RAN3, SA2, SA3
					Source: SA5(HuaWei)

Decision: 		The document was noted.

R3-256693	Correction on the ASN.1 of Positioning Data Information for data collection

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0199  Cat: F (Rel-19)

					Source: Samsung

Discussion:

-	Editorial corrections needed to cover page including clauses affected

-	Add Ericsson, Nokia, CATT, ZTE, Huawei, Xiaomi, NEC, CMCC as co-sources

Decision: 		The document was revised to R3-257232.

R3-257232	Correction on the ASN.1 of Positioning Data Information for data collection

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0199  rev 1 Cat: F (Rel-19)

					Source: Samsung, Ericsson, Nokia, CATT, ZTE, Huawei, Xiaomi, NEC, CMCC

(Replaces R3-256693)

Decision: 		The document was endorsed.

R3-256757	Correction on positioning data collection for case 3a

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0200  Cat: F (Rel-19)

					Source: Xiaomi

Discussion:

HUAWEI: Is something broken, or is this an optimization?

ZTE: Enhancement, not a critical issue

ERICSSON: Support (beneficial), alternative encoding in 7022

Nokia: Same view as HUAWEI, ZTE

QUALCOMM: This is an essential correction, otherwise LMF is overloaded to calculate data for all TRPs

CATT: Agree with Qualcomm

Samsung: Support, gNB should have say

Decision: 		The document was noted.

R3-256780	Remaining Issue on AI/ML based Positioning Accuracy Enhancement

Type: discussion		For: Decision
					38.455 v..
					Source: NEC

Decision: 		The document was not treated.

R3-256989	(NRPPa CR & draftCR to TS 38.305) Miscellaneous corrections for supporting AI/ML-based positioning

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

R3-257022	Correction on Positioning Data Collection Needed

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0203  Cat: F (Rel-19)

					Source: Ericsson

Discussion:

Time Stamp IE

Xiaomi: Acknowledge but need further discussion

Nokia: Not yet agreed in RAN1

QUALCOMM: Not clear how this works

CATT, ZTE, HUAWEI: Useful

NEC: motivation not clear

Decision: 		The document was revised to R3-257237.

R3-257237	Correction on Positioning Data Collection Needed

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0203  rev 1 Cat: F (Rel-19)

					Source: Ericsson, Xiaomi, ZTE, Huawei, Samsung, CATT, Nokia, CEWIT, NEC, Qualcomm Inc.

(Replaces R3-257022)

Decision: 		The document was endorsed.

R3-257099	Discussion on terminology related to AI/ML for PHY

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-257100	Corrections on support of AI/ML assisted positioning with gNB-side AI/ML model

Type: draftCR		For: Endorsement
					38.305 v19.0.0
					Source: Nokia

Decision: 		The document was not treated.

R3-257171	Correction to NRPPa on AI/ML assisted positioning

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0205  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

CB# 18_R19AIML_air

- Discuss per-TRP “data collection needed”, based on 6757 and 7022, NRPPa CR (if agreeable)

- NRPPa misc corrections: check 6989, 7171

(Xiaomi - moderator)

R3-257238	Miscellaneous corrections for supporting AI/ML-based positioning

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0206  Cat: F (Rel-19)

					Source: Huawei, CATT, Nokia, NEC, Ericsson, CEWiT, ZTE, Samsung, Xiaomi

Discussion:

-	Remove “if available” in 8.6.1.2

Decision: 		The document was revised to R3-257321.

R3-257321	Miscellaneous corrections for supporting for AI/ML-based positioning

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0206  rev 1 Cat: F (Rel-19)

					Source: Huawei, CATT, Nokia, NEC, Ericsson, CEWiT, ZTE, Samsung, Xiaomi

(Replaces R3-257238)

Decision: 		The document was endorsed.

[NW side data collection]

R3-257170	Discussion on reply LS on NW side data collection

Type: discussion		For: (not specified)
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-256851	Discussion on NW side data collection

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was noted.

R3-256850	[Draft] Reply LS on NW side data collection

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: CMCC

Decision: 		The document was not treated.

R3-256988	(Draft Reply LS to RAN2) Discussion on the Reply LS from RAN2 on NW side data collection

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was noted.

R3-257075	Discussion on incoming LSes to RAN3 on AIML for PHY

Type: discussion		For: Approval
					Source: Ericsson

Decision: 		The document was noted.

R3-256735	[Draft LS] Reply LS to RAN2 about NW-side data colleciton

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: ZTE Corporation

Decision: 		The document was noted.

#### 9.2.12	R19 NR XR Enhancements

R3-256655	Correction on PDU Set based QoS handling

Type: draftCR		For: Endorsement
					37.340 v19.0.0
					Source: CATT, Nokia, Nokia Shanghai Bell, Huawei, Offinno, ZTE, Ericsson

Discussion:

-	“Ofinno”

-	Update to latest spec

Decision: 		The document was revised to R3-257264.

R3-257264	Correction on PDU Set based QoS handling

Type: draftCR		For: Endorsement
					37.340 v19.0.0
					Source: CATT, Nokia, Nokia Shanghai Bell, Huawei, Ofinno, ZTE, Ericsson, China Telecom

(Replaces R3-256655)

Decision: 		The document was endorsed.

R3-256808	Correction on PDU Set handling during Handover

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, Nokia, Nokia Shanghai Bell, Ericsson, CATT, Offinno, ZTE

Discussion:

-	“Ofinno”

Decision: 		The document was revised to R3-257265.

R3-257265	Correction on PDU set handling during handover

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, Nokia, Nokia Shanghai Bell, Ericsson, CATT, Ofinno, ZTE

(Replaces R3-256808)

Decision: 		The document was endorsed.

R3-256891	Add the missing behavior text for DL PDU Set Information Marking Support Indication

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1551  Cat: F (Rel-19)

					Source: Nokia, Nokia Shanghai Bell, CATT, Qualcomm, ZTE, Ericsson, Ofinno

Discussion:

-	Add Huawei as co-source

Decision: 		The document was revised to R3-257266.

R3-257266	Add the missing behavior text for DL PDU Set Information Marking Support Indication

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1551  rev 1 Cat: F (Rel-19)

					Source: Nokia, Nokia Shanghai Bell, CATT, Qualcomm, ZTE, Ericsson, Ofinno, Huawei, China Telecom

(Replaces R3-256891)

Decision: 		The document was endorsed.

R3-257166	Correction on stage 3 description on Indication of Bitrate Adaptation IE in NGAP

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1370  Cat: F (Rel-19)

					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell,CATT, Qualcomm, Huawei, Ericsson,

Decision: 		The document was merged.

R3-257034	Add procedure texts on XR uplink rate control in NGAP

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1358  Cat: F (Rel-19)

					Source: Ofinno

Discussion:

-	Add ZTE Corporation, Nokia, Nokia Shanghai Bell,CATT, Qualcomm, Huawei, Ericsson, Samsung, China Telecom as co-sources

Decision: 		The document was revised to R3-257267.

R3-257267	Add procedure texts on XR uplink rate control in NGAP

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1358  rev 1 Cat: F (Rel-19)

					Source: Ofinno, ZTE Corporation, Nokia, Nokia Shanghai Bell, CATT, Qualcomm, Huawei, Ericsson, Samsung, China Telecom

(Replaces R3-257034)

Decision: 		The document was endorsed.

R3-257143	Correction to TS 38.413 for XR

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1368  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was merged.

R3-256730	Discussion on RLC enhancement for XR

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was noted.

R3-256731	Correction on RLC enhancement for XR

Type: CR		For: Agreement
					38.425 v19.0.0	  CR-0158  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was revised to R3-257290.

R3-257290	Correction on RLC enhancement for XR

Type: CR		For: Agreement
					38.425 v19.0.0	  CR-0158  rev 1 Cat: F (Rel-19)

					Source: Huawei, Nokia, Nokia Shanghai Bell, CATT

(Replaces R3-256731)

Decision: 		The document was endorsed.

R3-257023	RAN status indication of Available Bitrate reporting

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1357  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was noted.

R3-257142	Correction to TS 37.483 for XR

Type: CR		For: Agreement
					37.483 v19.0.0	  CR-0184  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was endorsed.

R3-257165	Correction on semantic description for Notification Cause IE in NGAP

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1369  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was revised to R3-257291.

R3-257291	Correction on the Notification Cause IE in NGAP

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1369  rev 1 Cat: F (Rel-19)

					Source: ZTE Corporation

(Replaces R3-257165)

Decision: 		The document was endorsed.

R3-257167	Discussion on  UL Rate Control on XR with the correction and draft LS

Type: discussion		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

The gNB-CU is able to indicate the discarded PDCP PDUs properly when PSI based discard is enabled. The F1-U procedure text can be simplified as “The corresponding node may stop transmission or retransmission of the NR PDCP PDUs indicated to be discarded”.

ERICSSON: Not necessary to unify behavior

Nokia, Lenovo: Original text is preferred

CATT: Support

[Agreement] RAN3 to use one-bit F1-U indication to indicate the condition for remaining-time-based polling is met.

QUALCOMM: Existing functionality seems sufficient, nothing more needed

ERICSSON, Nokia, Lenovo, ZTE, CATT: Support

CB # 24_R19XR

- Revise 6731 to capture above agreement

- Check 7165, what (if anything) needs to be changed?

(Nokia - moderator)

#### 9.2.13	R19 NR Sidelink Multi-hop Relay

R3-257186	Corrections on Multi-hop relay

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1640  Cat: F (Rel-19)

					Source: LG Electronics Inc.

Decision: 		The document was revised to R3-257282.

R3-257282	Corrections on Multi-hop relay

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1640  rev 1 Cat: F (Rel-19)

					Source: LG Electronics Inc., Nokia, Nokia Shanghai Bell, CATT, ZTE Corporation, Huawei

(Replaces R3-257186)

Discussion:

-	Add Ericsson as co-source

Decision: 		The document was revised to R3-257322.

R3-257322	Corrections on Multi-hop relay

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1640  rev 2 Cat: F (Rel-19)

					Source: LG Electronics Inc., Nokia, Nokia Shanghai Bell, CATT, ZTE Corporation, Huawei, Ericsson

(Replaces R3-257282)

Decision: 		The document was endorsed.

R3-256955	Corrections for Multi-hop relay

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1613  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was merged.

R3-256968	Correction for Multihop relay

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was not treated.

R3-256969	Correction for Multihop relay

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1616  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was merged.

R3-257030	Corrections on SL Multi-hop

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson

Decision: 		The document was not treated.

R3-256954	Corrections for multi-hop relay

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0494  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was revised to R3-257283.

R3-257283	Corrections for Multi-hop relay

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0494  rev 1 Cat: F (Rel-19)

					Source: ZTE Corporation, Samsung, Ericsson, LG Electronics

(Replaces R3-256954)

Decision: 		The document was endorsed.

R3-256662	Correction on multihop initial access

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0483  Cat: F (Rel-19)

					Source: Samsung

Decision: 		The document was merged.

R3-257031	Corrections on SL Multi-hop

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0498  Cat: F (Rel-19)

					Source: Ericsson

Decision: 		The document was merged.

CB# 25_R19SL-MultiHop

- F1AP misc corrections, check 7186, 6955, 6969

- 38.401 misc corrections, check 6954, 6662, 7031

(LGE - moderator)

#### 9.2.14	Other

[Positioning activation and deactivation]

R3-256663	Discussion on positioning activation and deactivation procedure

Type: discussion		For: Agreement
					Source: Samsung, CATT, ZTE, Huawei

Decision: 		The document was noted.

R3-256636	Correction to Positioning activation and deactivation procedure

Type: draftCR		For: Endorsement
					38.305 v18.6.0
					Source: CATT, Samsung, ZTE, Huawei

Discussion:

-	Add China Telecom, Xiaomi as co-source

Decision: 		The document was revised to R3-257268.

R3-257268	Correction to Positioning activation and deactivation procedure

Type: draftCR		For: Endorsement
					38.305 v18.6.0
					Source: CATT, Samsung, ZTE, Huawei, China Telecom, Xiaomi

(Replaces R3-256636)

Decision: 		The document was endorsed.

R3-256637	Correction to Positioning activation and deactivation procedure

Type: draftCR		For: Endorsement
					38.305 v19.0.0
					Source: CATT, Samsung, ZTE, Huawei

Discussion:

-	Add China Telecom, Xiaomi as co-source

Decision: 		The document was revised to R3-257269.

R3-257269	Correction to Positioning activation and deactivation procedure

Type: draftCR		For: Endorsement
					38.305 v19.0.0
					Source: CATT, Samsung, ZTE, Huawei, China Telecom, Xiaomi

(Replaces R3-256637)

Decision: 		The document was endorsed.

R3-256664	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1591  Cat: F (Rel-18)

					Source: Samsung, CATT, ZTE Corporation, Huawei

Discussion:

-	Add China Telecom, Xiaomi as co-source

Decision: 		The document was revised to R3-257270.

R3-257270	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1591  rev 1 Cat: F (Rel-18)

					Source: Samsung, CATT, ZTE Corporation, Huawei, China Telecom, Xiaomi

(Replaces R3-256664)

Decision: 		The document was endorsed.

R3-256665	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1592  Cat: A (Rel-19)

					Source: Samsung, CATT, ZTE Corporation, Huawei

Discussion:

-	Add China Telecom, Xiaomi as co-source

Decision: 		The document was revised to R3-257271.

R3-257271	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1592  rev 1 Cat: A (Rel-19)

					Source: Samsung, CATT, ZTE Corporation, Huawei, China Telecom, Xiaomi

(Replaces R3-256665)

Decision: 		The document was endorsed.

R3-256678	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.455 v18.6.0	  CR-0197  Cat: F (Rel-18)

					Source: Huawei, Samsung, CATT, ZTE

Discussion:

-	Add China Telecom, Xiaomi as co-source

Decision: 		The document was revised to R3-257272.

R3-257272	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.455 v18.6.0	  CR-0197  rev 1 Cat: F (Rel-18)

					Source: Huawei, Samsung, CATT, ZTE, China Telecom, Xiaomi

(Replaces R3-256678)

Decision: 		The document was endorsed.

R3-256679	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0198  Cat: A (Rel-19)

					Source: Huawei, Samsung, CATT, ZTE

Discussion:

-	Add China Telecom, Xiaomi as co-source

Decision: 		The document was revised to R3-257273.

R3-257273	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0198  rev 1 Cat: A (Rel-19)

					Source: Huawei, Samsung, CATT, ZTE, China Telecom, Xiaomi

(Replaces R3-256679)

Decision: 		The document was endorsed.

R3-256680	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1527  Cat: F (Rel-18)

					Source: ZTE Corporation, Samsung, CATT, Huawei

Discussion:

-	Add China Telecom, Xiaomi as co-source

Decision: 		The document was revised to R3-257274.

R3-257274	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1527  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation, Samsung, CATT, Huawei, China Telecom, Xiaomi

(Replaces R3-256680)

Decision: 		The document was endorsed.

R3-256681	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1528  Cat: A (Rel-19)

					Source: ZTE Corporation, Samsung, CATT, Huawei

Discussion:

-	Add China Telecom, Xiaomi as co-source

Decision: 		The document was revised to R3-257275.

R3-257275	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1528  rev 1 Cat: A (Rel-19)

					Source: ZTE Corporation, Samsung, CATT, Huawei, China Telecom, Xiaomi

(Replaces R3-256681)

Decision: 		The document was endorsed.

Activation Procedure

LMF makes the activation decision for area-specific SP SRS.

The LMF sends the NRPPa POSITIONING ACTIVATION REQUEST message to the Last Serving gNB, after the reception of the NRPPa POSITIONING INFORMATION UPDATE message at Step 3.

Introduce SP SRS activation related information (e.g. Activated SRS Resource Set ID, spatial relation, etc.) in XnAP RETRIEVE UE CONTEXT RESPONSE/FAILURE message to support SP SRS Activation procedure.

RAN3 is kindly asked to discuss whether SFN and Slot Number IE may be included in the NRPPa POSITIONING ACTIVATION RESPONSE message for area-specific SP SRS activation.

Deactivation Procedure

LMF makes the deactivation decision for area-specific SP SRS.

Introduce SP SRS deactivation related information (e.g. Deactivated SRS Resource Set ID) in XnAP RETRIEVE UE CONTEXT RESPONSE/FAILURE message to support SP SRS Deactivation procedure.

F1AP Impact

Introduce SP SRS activation/deactivation related information (e.g. SRS Resource Set ID, spatial relation, etc.) in F1AP DL RRC MESSAGE TRANSFER message.

ERICSSON: OK for Rel-19, but not essential for Rel-18

QUALCOMM,, CATT, ZTE, SS, HUAWEI, China Telecom: Support as Rel-18 correction, decisions from other WGs came late

[Paging capability loss issue]

R3-256815	Further discussion on paging capability loss issue

Type: discussion		For: (not specified)
					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was noted.

R3-256816	Avoiding paging capability loss for CN paging

Type: CR		For: Agreement
					38.413 v17.13.0	  CR-1341  Cat: F (Rel-17)

					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256817	Avoiding paging capability loss for CN paging

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1342  Cat: A (Rel-18)

					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256818	Avoiding paging capability loss for CN paging

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1343  Cat: A (Rel-19)

					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256819	Avoiding paging capability loss for CN paging

Type: draftCR		For: Endorsement
					38.300 v17.14.0
					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256820	Avoiding paging capability loss for CN paging

Type: draftCR		For: Endorsement
					38.300 v18.7.0
					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256821	Avoiding paging capability loss for CN paging

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-257015	Correction on Paging Loss issue

Type: draftCR		For: Endorsement
					38.300 v17.14.0
					Source: Ericsson, Nokia, Jio Platforms

Decision: 		The document was revised to R3-257287.

R3-257287	Correction on Paging Loss issue

Type: draftCR		For: Endorsement
					38.300 v17.14.0
					Source: Ericsson, Nokia, Jio Platforms

(Replaces R3-257015)

Decision: 		The document was noted.

R3-257016	Correction on Paging Loss issue

Type: draftCR		For: Endorsement
					38.300 v18.7.0
					Source: Ericsson, Nokia, Jio Platforms

Decision: 		The document was revised to R3-257288.

R3-257288	Correction on Paging Loss issue

Type: draftCR		For: Endorsement
					38.300 v18.7.0
					Source: Ericsson, Nokia, Jio Platforms

(Replaces R3-257016)

Decision: 		The document was not treated.

R3-257017	Correction on Paging Loss issue

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson, Nokia, Jio Platforms

Decision: 		The document was revised to R3-257289.

R3-257289	Correction on Paging Loss issue

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Ericsson, Nokia, Jio Platforms

(Replaces R3-257017)

Decision: 		The document was not treated.

R3-257018	[DRAFT] Reply LS on Response to Reply LS on paging capability loss issue

Type: LS out		For: Approval
					to SA2, cc RAN2, CT1, RAN
					Source: Ericsson

Decision: 		The document was not treated.

R3-257106	Correction of UE Radio Capability for Paging

Type: CR		For: Agreement
					38.413 v17.13.0	  CR-1364  Cat: F (Rel-17)

					Source: Nokia, Ericsson, Jio Platforms

Decision: 		The document was revised to R3-257277.

R3-257277	Correction of UE Radio Capability for Paging

Type: CR		For: Agreement
					38.413 v17.13.0	  CR-1364  rev 1 Cat: F (Rel-17)

					Source: Nokia, Ericsson, Jio Platforms, Huawei, ZTE, CATT, NTT Docomo

(Replaces R3-257106)

Decision: 		The document was endorsed.

R3-257107	Correction of UE Radio Capability for Paging

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1365  Cat: A (Rel-18)

					Source: Nokia, Ericsson, Jio Platforms

Decision: 		The document was revised to R3-257278.

R3-257278	Correction of UE Radio Capability for Paging

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1365  rev 1 Cat: A (Rel-18)

					Source: Nokia, Ericsson, Jio Platforms, Huawei, ZTE, CATT, NTT Docomo

(Replaces R3-257107)

Decision: 		The document was endorsed.

R3-257108	Correction of UE Radio Capability for Paging

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1366  Cat: A (Rel-19)

					Source: Nokia, Ericsson, Jio Platforms

Decision: 		The document was revised to R3-257279.

R3-257279	Correction of UE Radio Capability for Paging

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1366  rev 1 Cat: A (Rel-19)

					Source: Nokia, Ericsson, Jio Platforms, Huawei, ZTE, CATT, NTT Docomo

(Replaces R3-257108)

Decision: 		The document was endorsed.

R3-257110	Response LS on Paging Capability Loss Issue

Type: LS out		For: Approval
					to SA2
					Source: Nokia

Abstract:

Att: R3-257106

Decision: 		The document was not treated.

R3-257053	Missing UE radio paging capabilities

Type: discussion		For: Discussion
					Source: Vodafone GmbH

Decision: 		The document was noted.

R3-257054	Paging capability loss in RAN

Type: CR		For: Agreement
					38.413 v17.13.0	  CR-1359  Cat: F (Rel-17)

					Source: Vodafone

Decision: 		The document was not treated.

R3-257055	Paging capability loss in RAN

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1360  Cat: A (Rel-18)

					Source: Vodafone

Decision: 		The document was not treated.

R3-257056	Paging capability loss in RAN

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1361  Cat: A (Rel-19)

					Source: Vodafone GmbH

Decision: 		The document was not treated.

R3-256767	Discussion on Paging capability missing issue

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-256822	Avoiding UE paging loss

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: CATT

Decision: 		The document was not treated.

R3-256823	Avoiding UE paging loss

Type: draftCR		For: Endorsement
					38.300 v18.7.0
					Source: CATT

Decision: 		The document was not treated.

R3-256824	Avoiding UE paging loss

Type: draftCR		For: Endorsement
					38.300 v17.14.0
					Source: CATT

Decision: 		The document was not treated.

R3-256957	Remaining issues for paging capability loss

Type: discussion		For: Decision
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-256958	[draft] Reply LS on paging capability loss issue

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

Context Setup is common denominator

RAN3 to agree to add interactions with the UE Radio Capability Info Indication procedure, when the received URCP does not contain all the paging related features for the following procedures:

•	Handover Resource Allocation (for the URCP contained in the CNAI)

•	Path Switch Request (for the URCP contained in the CNAI)

DCM, QUALCOMM,, CATT: Extending to mobility is straightforward, no reason not to include it to support all scenarios, SA2 mentioned initial context setup only as an example

ZTE, ERICSSON, Nokia, Vodafone: Missing RAT checking is not supported in handover case (only during initial context setup)

Add a new field (UE Radio Capability for Paging check status) to the UE Radio Capability for Paging.

CB # 26_R17PagCapLoss

- Initial Context Setup

- Check NGAP and Stage 2 CRs in 7106 and 7015

- new field to the UE Radio Capability for Paging?

- Reply LS?

(Nokia - moderator)

[Other]

R3-256527	Clarification for propagation of MDT Configuration in stage2

Type: other		For: Agreement
					Source: ZTE Corporation,China Unicom,China Telecom,CMCC,Huawei

(Replaces R3-255737)

Decision: 		The document was not treated.

R3-256598	Clarification for propagation of roaming and access restrictions

Type: draftCR		For: Endorsement
					38.300 v18.7.0
					Source: Samsung

Decision: 		The document was not treated.

R3-256682	Specifying procedure texts for the Core Network Assistance Information for RRC INACTIVE IE

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1288  rev 1 Cat: F (Rel-19)

					Source: Huawei, Ericsson, Nokia

(Replaces R3-255232)

Decision: 		The document was not treated.

R3-256995	Extensions for enumerated type definitions over several Releases - and what can go wrong

Type: discussion		For: Discussion
					Source: Ericsson, Jio Platforms, China Telecom, CMCC, Nokia

(Replaces R3-255560)

Decision: 		The document was not treated.

R3-256996	Correcting the extension of FR1-bandwidth type definition

Type: CR		For: Agreement
					38.473 v16.21.0	  CR-1582  rev 1 Cat: F (Rel-16)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC, Nokia

(Replaces R3-255561)

Decision: 		The document was not treated.

R3-256997	Correcting the extension of FR1-bandwidth type definition

Type: CR		For: Agreement
					38.473 v17.13.0	  CR-1583  rev 1 Cat: A (Rel-17)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC, Nokia

(Replaces R3-255562)

Decision: 		The document was not treated.

R3-256998	Correcting the IE extension definition for bandwidth SRS

Type: CR		For: Agreement
					38.455 v16.15.0	  CR-0194  rev 1 Cat: F (Rel-16)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC, Nokia

(Replaces R3-255563)

Decision: 		The document was not treated.

R3-256999	Correcting the IE extension definition for bandwidth SRS

Type: CR		For: Agreement
					38.455 v17.10.0	  CR-0195  rev 1 Cat: A (Rel-17)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC, Nokia

(Replaces R3-255564)

Decision: 		The document was not treated.

R3-257000	Correcting the IE extension definition for SRS Pos SIB Type

Type: CR		For: Agreement
					38.455 v17.10.0	  CR-0196  rev 1 Cat: F (Rel-17)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC, Nokia

(Replaces R3-255565)

Decision: 		The document was not treated.

R3-257037	Addition of PDCP discard timer for scheduling with DSR

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1584  rev 1 Cat: F (Rel-18)

					Source: Ericsson, Qualcomm Inc., Jio Platforms

(Replaces R3-255566)

Decision: 		The document was not treated.

R3-257039	Addition of PDCP discard timer for scheduling with DSR

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1624  Cat: A (Rel-19)

					Source: Ericsson, Qualcomm Inc., Jio Platforms

Decision: 		The document was not treated.

R3-256563	Additional procedure text for Data Collection Update

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1513  Cat: F (Rel-18)

					Source: ZTE Corporation, Ercisson, Samsung, NEC

Decision: 		The document was not treated.

R3-256564	Correction on the description of UE Context Information - Retrieve UE Context Response

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1514  Cat: F (Rel-19)

					Source: ZTE Corporation, Nokia, Huawei, CATT, Samsung

Decision: 		The document was not treated.

R3-256566	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.423 v15.20.0	  CR-1515  Cat: F (Rel-15)

					Source: Nokia, Huawei, Qualcomm

Decision: 		The document was not treated.

R3-256567	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.423 v16.19.0	  CR-1516  Cat: A (Rel-16)

					Source: Nokia, Huawei, Qualcomm

Decision: 		The document was not treated.

R3-256568	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.423 v17.13.0	  CR-1517  Cat: A (Rel-17)

					Source: Nokia, Huawei, Qualcomm

Decision: 		The document was not treated.

R3-256569	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1518  Cat: A (Rel-18)

					Source: Nokia, Huawei, Qualcomm

Decision: 		The document was not treated.

R3-256570	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1519  Cat: A (Rel-19)

					Source: Nokia, Huawei, Qualcomm

Decision: 		The document was not treated.

R3-256669	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.473 v15.18.0	  CR-1593  Cat: F (Rel-15)

					Source: Huawei, Nokia, Qualcomm

Decision: 		The document was not treated.

R3-256670	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.473 v16.21.0	  CR-1594  Cat: A (Rel-16)

					Source: Huawei, Nokia, Qualcomm

Decision: 		The document was not treated.

R3-256671	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.473 v17.13.0	  CR-1595  Cat: A (Rel-17)

					Source: Huawei, Nokia, Qualcomm

Decision: 		The document was not treated.

R3-256672	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1596  Cat: A (Rel-18)

					Source: Huawei, Nokia, Qualcomm

Decision: 		The document was not treated.

R3-256673	Correction of Transmission Bandwidth Asymmetric

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1597  Cat: A (Rel-19)

					Source: Huawei, Nokia, Qualcomm

Decision: 		The document was not treated.

R3-256621	Discussion on UL data forwarding

Type: discussion		For: Decision
					Source: CATT,Huawei

Decision: 		The document was not treated.

R3-256745	Correction of time-based handover for NR NTN

Type: draftCR		For: Endorsement
					38.300 v18.7.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson

Discussion:

Response in R3-257197

Decision: 		The document was not treated.

R3-256746	Correction of time-based handover for NR NTN

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson

Discussion:

Response in R3-257197

Decision: 		The document was not treated.

R3-256753	Correction on UHI

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1330  Cat: F (Rel-18)

					Source: ZTE Corporation,China Telecom, Pengcheng Laboratory

Decision: 		The document was not treated.

R3-256754	Correction on UHI

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1331  Cat: A (Rel-19)

					Source: ZTE Corporation,China Telecom, Pengcheng Laboratory

Decision: 		The document was not treated.

R3-256804	Correction on QNC

Type: draftCR		For: Endorsement
					38.300 v18.7.0
					Source: Huawei, Nokia, Nokia Shanghai Bell, China Telecom, CMCC

Decision: 		The document was not treated.

R3-256805	Correction on QNC

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: Huawei, Nokia, Nokia Shanghai Bell, China Telecom, CMCC

Decision: 		The document was not treated.

R3-256859	R18 correction on the CU-DU Mobility Initiation Request message

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1603  Cat: F (Rel-18)

					Source: Samsung, Nokia, China Telecom, Jio Platforms, LG Electronics, Ericsson, Qualcomm, Huawei, Lenovo, CATT, ZTE Corporation

Decision: 		The document was not treated.

R3-256860	R19 correction on the CU-DU Mobility Initiation Request message

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1604  Cat: A (Rel-19)

					Source: Samsung, Nokia, China Telecom, Jio Platforms, LG Electronics, Ericsson, Qualcomm, Huawei, Lenovo, CATT, ZTE Corporation

Decision: 		The document was not treated.

R3-256892	Correction on Notification Control Indication

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1552  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell, Huawei, CMCC, China Telecom

Decision: 		The document was not treated.

R3-256893	Correction on Notification Control Indication

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1553  Cat: A (Rel-19)

					Source: Nokia, Nokia Shanghai Bell, Huawei, CMCC, China Telecom

Decision: 		The document was not treated.

R3-256898	Correction on Configured TAC

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1349  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was not treated.

R3-257001	LTM Resource consumption for the target cells

Type: discussion		For: (not specified)
					Source: SONY, Ericsson, vivo, Jio Platforms, LG Electronics

Decision: 		The document was not treated.

R3-257013	Addition of 2Rx XR UE Capability

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1620  Cat: F (Rel-18)

					Source: Ericsson, Qualcomm inc. Nokia, Nokia Shanghai Bell, ZTE

Decision: 		The document was not treated.

R3-257014	Addition of 2Rx XR UE Capability

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1621  Cat: A (Rel-19)

					Source: Ericsson, Qualcomm Inc., Nokia, Nokia Shanghai Bell, ZTE

Decision: 		The document was not treated.

R3-257026	Correction for Cell Switch execution

Type: draftCR		For: Endorsement
					38.300 v19.0.0
					Source: SONY, Ericsson, vivo, Jio Platforms, LG Electronics

Decision: 		The document was not treated.

R3-257028	Correction for Cell Switch execution

Type: CR		For: Agreement
					38.401 v18.7.0	  CR-0496  Cat: F (Rel-18)

					Source: Ericsson, Sony, Jio Platforms, LG Electronics, vivo

Decision: 		The document was not treated.

R3-257029	Correction for Cell Switch execution

Type: CR		For: Agreement
					38.401 v19.0.0	  CR-0497  Cat: A (Rel-19)

					Source: Ericsson, Sony, Jio Platforms, LG Electronics, vivo

Decision: 		The document was not treated.

R3-257036	Correction on ETA resource release in DU

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1622  Cat: F (Rel-18)

					Source: Ofinno

Decision: 		The document was not treated.

R3-257117	Correction on ETA resource release in DU

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1623  rev 1 Cat: A (Rel-19)

					Source: Ofinno

(Replaces R3-257038)

Decision: 		The document was not treated.

R3-257101	Alignment of DATA COLLECTION UPDATE messages between Rel-18 and Rel-19

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1581  Cat: F (Rel-18)

					Source: Nokia, Deutsche Telekom, FiberCop, Orange

Decision: 		The document was not treated.

R3-257133	Discussion on TNLA handling during Xn handover

Type: discussion		For: Discussion
					Source: DOCOMO Communications Lab.

Decision: 		The document was not treated.

R3-257134	Clarification on TNLA handling during Xn handover

Type: CR		For: Agreement
					38.423 v16.19.0	  CR-1585  Cat: F (Rel-16)

					Source: NTT DOCOMO

Decision: 		The document was not treated.

R3-257135	Clarification on TNLA handling during Xn handover

Type: CR		For: Agreement
					38.423 v17.13.0	  CR-1586  Cat: A (Rel-17)

					Source: NTT DOCOMO

Decision: 		The document was not treated.

R3-257136	Clarification on TNLA handling during Xn handover

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1587  Cat: A (Rel-18)

					Source: NTT DOCOMO

Decision: 		The document was not treated.

R3-257137	Clarification on TNLA handling during Xn handover

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1588  Cat: A (Rel-19)

					Source: NTT DOCOMO

Decision: 		The document was not treated.

R3-257182	Connection of Secondary RAT Data Usage Report for NG based handover

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1371  Cat: F (Rel-19)

					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-257183	Connection of Secondary RAT Data Usage Report for Xn based handover

Type: CR		For: Agreement
					38.423 v19.0.0	  CR-1598  Cat: F (Rel-19)

					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-256768	withdraw

Type: CR		For: Agreement
					38.413 v17.13.0	  CR-1334  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was withdrawn.

R3-256769	withdraw

Type: CR		For: Agreement
					38.413 v18.7.0	  CR-1335  Cat: A (Rel-18)

					Source: CATT

Decision: 		The document was withdrawn.

R3-256770	withdraw

Type: CR		For: Agreement
					38.413 v19.0.0	  CR-1336  Cat: A (Rel-19)

					Source: CATT

Decision: 		The document was withdrawn.

R3-256876	Correction on Reference Signal for Positioning

Type: CR		For: Agreement
					38.455 v18.6.0	  CR-0201  Cat: F (Rel-18)

					Source: China Telecom, China Unicom, CATT

Decision: 		The document was not treated.

R3-256877	Correction on Reference Signal for Positioning

Type: CR		For: Agreement
					38.473 v18.7.0	  CR-1607  Cat: F (Rel-18)

					Source: China Telecom, China Unicom, CATT

Decision: 		The document was not treated.

R3-256878	Correction on Reference Signal for Positioning

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0202  Cat: A (Rel-19)

					Source: China Telecom, China Unicom, CATT

Decision: 		The document was not treated.

R3-256880	Correction on Reference Signal for Positioning

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1608  Cat: A (Rel-19)

					Source: China Telecom, China Unicom, CATT

Decision: 		The document was not treated.

### 9.3	R19 Rapporteur Corrections

R3-256894	Correction on the reference to TS 36.331 IEs

Type: CR		For: Agreement
					36.443 v19.0.0	  CR-0136  Cat: D (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to R3-257226.

R3-257226	Correction on the reference to TS 36.331 IEs

Type: CR		For: Agreement
					36.443 v19.0.0	  CR-0136  rev 1 Cat: D (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R3-256894)

Decision: 		The document was not treated.

R3-256965	Rapporteur Corrections

Type: CR		For: Agreement
					38.473 v19.0.0	  CR-1615  Cat: D (Rel-19)

					Source: Huawei

Decision: 		The document was not treated.

R3-257024	NRPPa Rapporteur Corrections

Type: CR		For: Agreement
					38.455 v19.0.0	  CR-0204  Cat: D (Rel-19)

					Source: Ericsson, Nokia

Decision: 		The document was not treated.

R3-257062	[Draft] Rapporteur Rel-19 Corrections

Type: draftCR		For: Agreement
					38.423 v19.0.0
					Source: Ericsson

Decision: 		The document was not treated.

R3-257120	Rapporteur's editorial corrections for E1AP

Type: CR		For: Agreement
					37.483 v19.0.0	  CR-0183  Cat: D (Rel-19)

					Source: Ericsson

Decision: 		The document was not treated.

## 10	Study on 6G Radio [FS_6G_Radio]: RP-252912 (target: RAN #115) [TU: 4 (4, 4, 5, 6, 6, 6, 6, 6, 6)]

### 10.1	General

R3-256533	LS on Guidance on 6G data related work tasks

Type: LS in		For: Discussion
					Original outgoing LS: SP-251261, to SA2, SA5, cc SA3, SA6, TSG RAN, RAN2, RAN3
					Source: TSG SA(China mobile)

Decision: 		The document was noted.

R3-256534	Reply to LS on Early Alignment on Access Stratum security aspects

Type: LS in		For: Discussion
					Original outgoing LS: SP-251268, to TSG RAN, SA3, RAN2, RAN3, cc SA2
					Source: TSG SA(Vodafone)

Decision: 		The document was noted.

R3-256535	LS on Early Alignment on Access Stratum security aspects

Type: LS in		For: Discussion
					Original outgoing LS: RP-252891, to TSG SA, cc RAN2, RAN3, SA3, SA2
					Source: TSG RAN(Vodafone)

Decision: 		The document was noted.

R3-256536	LS on 6G fronthaul

Type: LS in		For: Discussion
					Original outgoing LS: RP-252967, to TSG SA, cc RAN3
					Source: TSG RAN(Nokia)

Discussion:

RAN3 will follow the Way Forward on 6G Fronthaul in R3-256536 during normative phase.

Decision: 		The document was noted.

R3-256939	Workplan for Rel-20 Study of 6GR

Type: Work Plan		For: Decision
					Source: Vodafone GmbH

Discussion:

-	Remove “Mobility between 5G and 6G” from each meeting

-	Remove “Initiation of discussions on a standardized AI/ML framework based on identified use cases.”

Decision: 		The document was revised to R3-257233.

R3-257233	Workplan for Rel-20 Study of 6GR

Type: Work Plan		For: Decision
					Source: Vodafone GmbH

(Replaces R3-256939)

Decision: 		The document was noted.

R3-257035	Skeleton for 38.760-3

Type: draft TR		For: Endorsement
					38.760-3 v0.0.0
					Source: Vodafone GmbH

Discussion:

-	Remove red text from section 7.2

Decision: 		The document was revised to R3-257234.

R3-257234	Skeleton for 38.760-3

Type: draft TR		For: Endorsement
					38.760-3 v0.0.1
					Source: Vodafone GmbH

(Replaces R3-257035)

Decision: 		The document was revised to R3-257330.

R3-257330	Skeleton for 38.760-3

Type: draft TR		For: Endorsement
					38.760-3 v0.0.2
					Source: Vodafone GmbH

(Replaces R3-257234)

Abstract:

Post Meeting: allocated for merging pCRs

Decision: 		The document was endorsed.

### 10.2	Overall RAN architecture

#### 10.2.1	General principles and requirements

R3-256571	6G RAN general principles and requirements

Type: discussion		For: Agreement
					Source: Qualcomm Inc, Charter Communications, T-Mobile USA, Verizon Wireless, KT Corp , Tejas Networks, Fujitsu, Rakuten, NTT DOCOMO, JIO Platforms, Reliance JIO, FiberCop, CEWiT, Telstra

Decision: 		The document was noted.

R3-256615	Discussion on general principles and requirements for 6G RAN

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-256716	6G Service Aware RAN Architecture Option

Type: discussion		For: (not specified)
					Source: T-Mobile USA Inc.

Discussion:

Support RAN service characteristics and awareness (service-aware RAN) for 6G that adapts to application/service requirements

6G RAN shall support real-time service characteristic performance observability

6G RAN shall be designed to provide service specific performance adaptation based on dynamic service performance observability, as close to real-time as possible

Vodafone: No need to restrict to real-time services

Decision: 		The document was noted.

R3-256718	E911 Voice Support for 6G

Type: discussion		For: (not specified)
					Source: T-Mobile USA Inc.

Discussion:

RAN3 shall study interface and function requirements to support native E911 voice and messaging for 6G Day 1 deployment, ensuring that emergency service capabilities are not deferred to later release

Support E911 (emergency call and location services) without falling back to previous technology when 6G coverage is available

Support E911 location services per regulatory compliance

Support highest priority handling, with guaranteed preemption capabilities for emergency services

BT: We should be aware also of mission critical services.

Decision: 		The document was noted.

R3-256846	Considerations on RAN data collection function for 6G RAN

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

A unified data collection framework shall be designed to support multiple 6G service requirements, ensuring data is collected within the appropriate domain and exposed across domains only when necessary.

HUAWEI: What is meant by “unified”?

Vodafone: We need to look service by service.

ZTE: We should discuss how to capture RAN3 requirements.

FiberCop: We agree with the principles.

Orange: Do we expect different functional split between RAN and Core?

Decision: 		The document was noted.

R3-256904	General Principles and Requirements of 6G RAN Architecture

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-256945	(TP to 6G TR) General Principles and Requirements for the 6G RAN architecture

Type: other		For: Agreement
					Source: Ericsson

Decision: 		The document was not treated.

R3-256970	Considerations on 6G General principles and requirements

Type: discussion		For: Approval
					Source: China Unicom

Decision: 		The document was not treated.

R3-257041	6G Data Management Framework Considerations

Type: discussion		For: Decision
					Source: AT&T

Decision: 		The document was not treated.

R3-257057	6G RAN Sharing for Multiple Operator Core Networks

Type: discussion		For: Decision
					Source: Boost Mobile Network

Decision: 		The document was not treated.

R3-257159	Requirements and deployment scenarios toward 6G RAN

Type: discussion		For: Discussion
					Source: DOCOMO Communications Lab.

Decision: 		The document was not treated.

R3-256539	RAN architecture general principles and requirements

Type: discussion		For: (not specified)
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256544	Discussion on Feasibility of RAN Service Based Architecture

Type: discussion		For: Decision
					Source: FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra

Decision: 		The document was not treated.

R3-256556	Initial discussion on 6G study in RAN3 and 6G RAN general principles and requirements

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-256592	Discussion on RAN architecture principle and requirement

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-256622	General principles and requirements on RAN architecture

Type: discussion		For: Agreement
					Source: Xiaomi

Decision: 		The document was not treated.

R3-256638	Discussion on the general principles and requirements of 6G NTN

Type: discussion		For: Decision
					Source: CSCN

Decision: 		The document was not treated.

R3-256687	High-level principles and requirements for 6G Architecture

Type: discussion		For: Decision
					Source: Tejas Network Limited

Decision: 		The document was not treated.

R3-256777	6G overall RAN architecture principle and requirements

Type: discussion		For: Agreement
					Source: NEC

Decision: 		The document was not treated.

R3-257040	RAN3 naming

Type: discussion		For: Decision
					Source: InterDigital, Inc.

Decision: 		The document was not treated.

R3-257067	Discussion on 6G General Principles and Requirements

Type: discussion		For: (not specified)
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-257121	Design principles for 6G RAN architecture

Type: discussion		For: Decision
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-257179	Consideration on General requirement and priniciples for RAN architecture

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

R3-257192	6G RAN overall architecture for NTN

Type: discussion		For: Decision
					Source: THALES, Echostar

Decision: 		The document was not treated.

From R3-256571:

6G RAN Architecture shall support both macro and small cell deployment scenarios to enable heterogeneous deployments for both indoor and outdoor deployment scenarios.

6G RAN shall support RAN sharing mechanisms same as in 5G RAN Sharing (i.e., MOCN and RAN Sharing with Multiple Cell Id)

6G RAN network functions and interfaces shall allow network function virtualization in cloud native deployments to enable RAN, Core, OAM to be deployed in (multi)-cloud environment.

6G RAN architecture shall allow flexible deployment that enables introduction of new 6G services in flexible manner during life cycle of 6G RAN with minimum or not impacting already deployed interface functions/services.

6G RAN shall support inter-vendor inter operable interfaces between RAN nodes and between RAN-CN.

6G RAN shall allow plug and play mechanism between different RAN network functions.

6G RAN protocol stack shall allow support for evolution of each protocol layer over the 6G lifespan.

6G RAN framework shall allow RAN awareness of various services to enable real time service observability and service performance optimization.

6G RAN shall make use of native AI/ML framework for various functions to improve RAN functionality and performance.

6G RAN design shall allow self-organization and performance optimization of various features

ERICSSON: We should try to capture problem statements that we need to fix

NEC: We should ensure there is no requirement to operate with AI/ML

TIM: Agree with ERICSSON, need to check the words, but need to also identify our priorities

CATT: Are we focusing only one Day 1?

ZTE: We need to separate proposals between principles and deployment scenarios. RANP has already provided some guidance on principles.

HUAWEI: How to handle parallel discussion in RANP.

Jio Platforms: We should capture problem states related to use cases or deployments

From R3-256615:

RAN3 confirms the RAN architecture requirements in TR 38.914 and capture them in our own TR to guide further study.

Capture the following architecture requirements in 5G into 6G RAN3 TR.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable low latency services, etc.

-	The RAN architecture shall allow deployments using Network Function Virtualization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

Capture the architecture requirements on the services supported in 6G into the RAN3 TR.

-	The design of the RAN architecture shall allow the support of existing services (e.g. Mobile Broadband, Immersive Communication, massive IoT, Voice).

-	The design of the RAN architecture shall support the deployment of the new services (e.g. AI/ML, sensing) rapidly and efficiently.

- Support efficient signalling exchange: Enable direct signalling exchange between network entities while guarantee elastic scalability as well

-	Support Efficient Data Transfer: Enable direct data transfer for new services (e.g., AI/ML, sensing) across all network entities (UE, RAN node, CN)

During the initial phase of architecture design, it should be prioritized to consider separating network function to meet specific design targets.

-	The design of 6G RAN architecture should support separating user planes and control plane functions.

-	To support the flexible deployment of various service types, RAN3 should be conducted from the perspective of separating real-time functions and near-real-time functions.

-	To support new service scenarios (e.g., AI, Sensing, etc.), separating legacy service functions and new service functions should be considered.

Vodafone: Deployment scenarios should look at RAN3 impacts

Samsung: Avoid duplicating discussion at plenary, we should focus on those that impact our architecture and interface design. Some requirements are problems we observed in 5G, and some are also good features from 5G that we should continue.

DEUTSCHE TELEKOM: We should see what new uses demand, and what problems we need to outcome.

CMCC: Agree that we need to consider problems and new services.

CB # 19_6GRANarch

- Work on TP for section 5.1 (General Principles) and section 5.2 (Deployment Scenarios) of the RAN3 TR

- Requirements already agreed by RAN do not need to be rediscussed, RAN3 requirements should not conflict

- Requirements that will help drive our RAN architecture work

- Open issues via FFSes, Editor’s Notes, new TR sub-sections, etc.

(Qualcomm - moderator)

R3-257292	SoD CB # 19 6G RANArch

Type: discussion		For: discussion
					Source: Qualcomm (moderator)

Decision: 		The document was noted.

R3-257293	[TP for draft TR 38.760] General principles and requirements

Type: pCR		For: Agreement
					38.760-3 v0.0.1
					Source: Qualcomm

Discussion:

-	Remove “[TP for draft TR 38.760-3]” from TDoc title

-	Add following to section 5.1 and 5.2

Decision: 		The document was revised to R3-257323.

R3-257323	General principles and requirements

Type: pCR		For: Agreement
					38.760-3 v0.0.1
					Source: Qualcomm

(Replaces R3-257293)

Decision: 		The document was agreed.

Section 5.1:

All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles.

The 6G RAN architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS]

Section 5.2:

The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914.

-  FFS on the implications of this requirement on 6G RAN architecture.

-  FFS whether all deployment scenarios of TR 38.914 can be supported.

Study whether the protocol stack of RAN3-defined interfaces can evolve over the lifespan of 6G.

Study the impact of enhanced RAN-based service awareness on 6G RAN architecture.

#### 10.2.2	RAN functions and logical architecture

R3-256847	Initial consideration on RAN functions and logical architecture

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-256543	Discussion on RAN logical architecture and functions

Type: discussion		For: Decision
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256623	Initial views on RAN logical architecture

Type: other		For: Agreement
					Source: Xiaomi

Decision: 		The document was not treated.

R3-256545	Discussion on possible RAN internal architecture evolution towards Service Based Architecture

Type: discussion		For: Decision
					Source: FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra

Decision: 		The document was not treated.

R3-256557	Resilient 6G RAN Architecture

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-256574	Discussion on 6G RAN logical architecture

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-256593	Discussion on 6G RAN logical architecture and 6G RAN function

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-256616	Discussion on 6G RAN Logical Architecture

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-256709	Discussion on 6G overall RAN architecture

Type: discussion		For: Decision
					Source: OPPO

Decision: 		The document was not treated.

R3-256778	6G RAN function and logical architecture

Type: discussion		For: Agreement
					Source: NEC

Decision: 		The document was not treated.

R3-256812	RAN Architecture

Type: other		For: Agreement
					Source: InterDigital, Inc.

Decision: 		The document was not treated.

R3-256905	6G RAN Logical Architecture

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-256946	(TP to 6G TR) Functions and Architecture for the 6G RAN

Type: other		For: Agreement
					Source: Ericsson

Decision: 		The document was not treated.

R3-257066	Discussion on 6G RAN Functions and Architecture

Type: discussion		For: (not specified)
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-257180	Consideration on RAN logical architecture

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

### 10.3	RAN-CN interface

R3-256994	Discussion on RAN-CN interface

Type: discussion		For: (not specified)
					Source: Rakuten Mobile, Inc

Decision: 		The document was withdrawn.

#### 10.3.1	General principles and requirements

R3-256590	[TP to BL pCR] General principles and functions on RAN-CN interface

Type: other		For: Agreement
					Source: Huawei, Jio Platforms

Decision: 		The document was noted.

R3-256779	On RAN-CN interface principle and requirement

Type: discussion		For: Agreement
					Source: NEC

Decision: 		The document was noted.

R3-256811	RAN-CN General Principles

Type: other		For: Agreement
					Source: InterDigital, Inc.

Decision: 		The document was noted.

R3-256906	General Principles and Requirements of 6G RAN-CN Functional Split and Interface

Type: discussion		For: (not specified)
					Source: Lenovo

Discussion:

Vodafone: We should avoid “6GC” terminology

Decision: 		The document was noted.

R3-257051	General principles and requirements for 6G RAN-CN interface

Type: other		For: Agreement
					Source: LG Electronics

Decision: 		The document was noted.

R3-257063	On General Principles and Requirements for the 6G RAN-CN interface

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was noted.

R3-256624	General principles and requirements on RAN-CN interface

Type: other		For: Agreement
					Source: Xiaomi

Discussion:

6G RAN-CN interface between RAN node and network functions for new services (e.g., Sensing, AI) shall support low latency and less complexity transmission and avoid singalling storms for irrelevant NF (e.g., 6G AMF).

6G RAN-CN interface shall support efficient transmission of large-volume data for both UE-associated and non-UE-associated cases, with the ability to handle different latency requirements relevant to AI/ML and sensing.

Oppo: direction connection is already being considered in SA2, so 5GA could be considered a starting point for 6G study

HUAWEI: Whether there will be centralized AMF is up to SA2

ZTE, CATT: Direct interface between RAN & CN is beneficial

CATT: Data plane should wait for SA2 progress

ERICSSON: Scalability in AMF is related to signaling that contains data, no issue for pure control messaging

Nokia: Nicely separates control plane and data plane

QUALCOMM: Seems too solution driven

Samsung: Requirements are too detailed

Lenovo: New services may have more RAN-CN signaling

Decision: 		The document was noted.

R3-256540	Discussion on Principles of RAN-CN Interface

Type: other		For: Agreement
					Source: ZTE Corporation

Discussion:

RAN3 needs to collaborate with SA2 to define the functional split for new 6G services, ensuring the requirements as analyzed in 2.2.1. The following principles are proposed for the new 6G RAN–CN interface design:

1.	A new interface between 6G RAN and 6G CN is considered to support new 6G services and functionalities;

2.	For these new services, which are not directly associated with UE dedicated signalling and traffic, both signalling and the corresponding service related data transmission are carried over this new interface;

Decision: 		The document was noted.

R3-256575	6G RAN-CN interface: General principles and requirements

Type: discussion		For: Discussion
					Source: Qualcomm Inc, NTT Docomo Inc, Fibercop, Tejas Network Limited, KT Corp, Charter Communications Inc

Decision: 		The document was not treated.

R3-256594	Discussion on general principles of RAN-CN interface

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-256617	Discussion on general principles and requirements on RAN-CN interface

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-256717	AI Native Architecture for 6G

Type: discussion		For: (not specified)
					Source: T-Mobile USA Inc.

Decision: 		The document was not treated.

R3-256852	Discussion on general principles and requirements for RAN-CN interface

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-256886	6G RAN-CN Interface Requirements and Principles

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-256940	SBI and Point2Point Definitions

Type: discussion		For: Decision
					Source: Vodafone GmbH

Decision: 		The document was not treated.

R3-256949	General Principles and Requirements for RAN-CN interface

Type: discussion		For: (not specified)
					Source: China Telecom

Decision: 		The document was not treated.

R3-257064	[potentialTP for TR 38.xxx] On Principles and Requirements for the 6G RAN-CN interface

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was not treated.

R3-257109	General Principles and Requirements for RAN-CN interface

Type: discussion		For: (not specified)
					Source: CEWiT, Tejas Networks

Decision: 		The document was not treated.

R3-257160	Requirements toward 6G RAN-CN functional split

Type: discussion		For: Discussion
					Source: DOCOMO Communications Lab.

Decision: 		The document was not treated.

From R3-256590:

Reuse the general principles for the NG interface for 6G RAN-CN interface as much as possible, in particlar:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer;

-	the 6G RAN-CN interface shall be future proof to fulfil different new requirements and support of new services and new functions;

-	the 6G RAN-CN interface is decoupled with the possible NG-RAN deployment variants;

-	the 6G RAN-CN interface, from locial standpoint, is be a point-to-point interface between an 6G RAN node and a CN node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the 6G RAN node and CN node.

The 6G RAN-CN interface, supports reliable signalling transmission, large scale of RAN nodes deployment, and clear boundaries between RAN and CN.

RAN-CN interface supports the following basic functions:

-	Interface management: The functionality to manage the RAN-CN Control Plane interface;

-	UE context management: The functionality to manage the UE context between the RAN and CN;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE;

-	Paging: The functionality to send paging requests to the RAN nodes involved in the paging area;

-	PDU Session Management: The functionality to establish, manage and remove PDU sessions and respective RAN resources.

-	Configuration Transfer: The functionality to transfer the RAN configuration information (e.g., transport layer addresses for establishment of Xn* interface) between two RAN nodes via the CN.

Qualcomm: Needs further discussion, e.g. some already imply a solution.

FiberCop: Should be solution agnostic

Ericsson: For legacy functions, it is expected that same functions should be in 6G.

CATT, ZTE, Samsung, Nokia: Similar view as Ericsson

Lenovo: OK to take legacy for basic functions, but some of the above solutions are not fully aligned with 5G

Vodafone: 5G functions can be a starting point

From R3-257063:

Proposal 1: The 6G RAN-CN interface design shall enable realisation of RAN node and CN node modular functions in a cloud environment while neither affecting inter-vendor operability nor being visible to end users.

Proposal 2: How 6G RAN-CN interface traffic is distributed to node-internal modular functionalities shall be transparent to the peer nodes.

Proposal 3: Agree on the assumption that for 5G functionalities also supported in 6G, 6G RAN-CN application protocol functions assume the same RAN-CN functional split as for 5G and are terminated at a single logical node in the 6G RAN and the 6G CN.

Proposal 4: 6G RAN-CN (and RAN-RAN) application protocol design shall consider expected 6G logical node realisations applying modularisation by defining dedicated procedures for different protocol functions, minimising interactions between the node-internal functions and avoiding serving multiple functionalities with a single procedure as much as possible.

Proposal 5: 6G RAN-CN (and RAN-RAN) application protocol design shall be decoupled from the signalling transport, "signalling TNL agnostic", i.e. it shall not expect any other function from the signalling transport than delivery of SDUs.

Proposal 6: Introduce protocol means to enable efficient routing of AP messages supporting 6G logical node realisations in a modularised cloud environment.

QUALCOMM: Some proposals may be agreeable, but some need further discussion/clarification

Nokia: Proposal 5/6 need further discussion. Proposal 4 seems to imply very careful exercise

Lenovo: Support proposal 1, others need to be further checked

ZTE: Proposal 3 should clarify that legacy functionalities should be supported in 6G

FiberCop: Does this imply protocol-based interface?

NEC: In proposal 5, do we really not expect any other function from the signalling transport than delivery of SDUs?

Nokia: Acknowledge the issue behind proposal 5, but there may be other solutions

CB # 20_6GRAN-CNinf

- TP for section 6.1.1 & 6.1.2 (RAN-CN interface general principles and functions)

- Capture open issues for next meeting

- Introduce new sections in the TR, if agreeable

(Huawei - moderator)

R3-257240	(TP to TR 38.760-3) RAN-CN interface principles and functions

Type: other		For: Agreement
					38.760-3 v0.0.1
					Source: Huawei

Discussion:

-	Change “the clear functional split” to “a clear functional split”

-	Change “shall support future enhancements” to “supports future enhancements”

-	Change “RA N-CN” to RAN-CN

-	Remove “(TP to TR 38.760-3)” and change tdoc type to pCR

Decision: 		The document was revised to R3-257324.

R3-257324	RAN-CN interface principles and functions

Type: pCR		For: Agreement
					38.760-3 v0.0.1
					Source: Huawei

(Replaces R3-257240)

Decision: 		The document was agreed.

#### 10.3.2	RAN-CN interface options

R3-257052	Consideration on RAN-CN interface options

Type: other		For: Agreement
					Source: LG Electronics

Decision: 		The document was not treated.

R3-256542	Consideration on 6G RAN-CN interface options

Type: discussion		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256576	Discussion on 6G RAN-CN Interface options

Type: discussion		For: Discussion
					Source: Qualcomm Inc, NTT Docomo Inc, Fibercop, Tejas Network Limited, KT Corp, Charter Communications Inc

Decision: 		The document was not treated.

R3-256581	RAN-CN interface considerations

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-256591	[TP to BL pCR] Consideration on RAN-CN interface options

Type: other		For: Agreement
					Source: Huawei, Jio Platforms

Decision: 		The document was not treated.

R3-256595	Discussion on RAN-CN interface design

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-256618	Considerations on 6G RAN-CN interface options

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-256625	RAN-CN interface and protocol option

Type: other		For: Agreement
					Source: Xiaomi

Decision: 		The document was not treated.

R3-256710	Discussion on 6G RAN-CN interfaces

Type: discussion		For: Decision
					Source: OPPO

Decision: 		The document was not treated.

R3-256719	Discussion on possible RAN-CN interface evolution towards Service Based Architecture

Type: discussion		For: Decision
					Source: FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra

Decision: 		The document was not treated.

R3-256796	Considerations on RAN-CN interface

Type: discussion		For: Decision
					Source: NEC

Decision: 		The document was not treated.

R3-256809	Service Based Core Network

Type: other		For: Agreement
					Source: InterDigital, Inc.

Decision: 		The document was not treated.

R3-256853	Discussion on RAN-CN interface options

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-256887	6G RAN-CN Enhanced P2P Solution

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-256907	6G RAN-CN Interface Options

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-257059	Discussion on RAN-CN interface Principal Interface Options

Type: discussion		For: (not specified)
					Source: Jio Platforms, FiberCop

Decision: 		The document was not treated.

R3-257161	Candidate options for 6G RAN-CN interface

Type: discussion		For: Discussion
					Source: DOCOMO Communications Lab.

Decision: 		The document was not treated.

R3-257181	Discussion on RAN-CN interface

Type: discussion		For: (not specified)
					Source: Rakuten Mobile, Inc

Decision: 		The document was not treated.

### 10.4	RAN internal functional split and interfaces

R3-256832	Views on 6G RAN internal functionality split and interfaces

Type: discussion		For: (not specified)
					Source: NTT DOCOMO INC..

Discussion:

CU-DU split architecture should be supported in 6G

Decision: 		The document was noted.

R3-256596	Discussion on RAN internal architecture

Type: discussion		For: Agreement
					Source: Samsung, Verizon, NTT DoCoMo, 	Rakuten

Discussion:

Support CU-DU split and CP-UP separation for 6G RAN

Decision: 		The document was noted.

R3-256630	6G RAN internal functional split and interfaces

Type: discussion		For: Discussion
					Source: Nokia

Discussion:

RAN3 to wait with the study on the CP-UP split (i.e. E1 interface) until some progress is made for CU-DU split (i.e. F1 interface).

RAN3 to study whether and how higher layer split is supported for 6G.

Decision: 		The document was noted.

R3-256711	Discussion on 6G RAN internal interfaces

Type: discussion		For: Decision
					Source: OPPO

Discussion:

RAN3 discuss whether to support CU-DU split in 6G RAN or not, before studying different CU-DU split options.

Decision: 		The document was noted.

R3-257082	About 6G RAN-internal architecture topics

Type: discussion		For: Agreement
					Source: Ericsson

Decision: 		The document was noted.

R3-256541	Discussion on internal functional split and interfaces

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256577	6G RAN internal functional split and interfaces

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-256583	Views on 6G RAN internal functional split and interfaces

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-256620	Consideration on internal split of 6G RAN node

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-256626	RAN internal functional split and interfaces

Type: other		For: Agreement
					Source: Xiaomi

Decision: 		The document was not treated.

R3-256700	Consideration on RAN internal function split and interfaces

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

R3-256736	Initial views on 6G RAN internal functions split

Type: discussion		For: (not specified)
					Source: Fujitsu Limited

Decision: 		The document was not treated.

R3-256801	Discussion on RAN internal functional split and interfaces

Type: discussion		For: Decision
					Source: NEC

Decision: 		The document was not treated.

R3-256810	6G Xn General Principles

Type: other		For: Agreement
					Source: InterDigital, Inc.

Decision: 		The document was not treated.

R3-256897	Discussion on RAN internal functional split and interfaces

Type: discussion		For: (not specified)
					Source: Rakuten Mobile, Samsung

Decision: 		The document was not treated.

R3-256908	6G RAN Internal Functional Split and Interfaces

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-256971	Considerations on 6G RAN internal functional split and interfaces

Type: discussion		For: Approval
					Source: China Unicom

Decision: 		The document was not treated.

R3-257049	On the Discussion of 6G RAN Internal Functional Spit

Type: discussion		For: (not specified)
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-257122	Considerations on 6G RAN internal functional split

Type: discussion		For: Decision
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-257169	Discussion on supporting a F1-like interface in 6G RAN

Type: discussion		For: Discussion
					Source: Google

Decision: 		The document was not treated.

R3-257193	6G RAN internal functional split and interfaces for NTN

Type: discussion		For: Decision
					Source: THALES, Echostar

Decision: 		The document was not treated.

ZTE: CU-DU split in 5G required tremendous standardization effort in RAN3, but limited deployments. Also different companies have different views regarding UE and performance impacts. Study should capture all of these objectively.

VERIZON: Verizon has deployed significant number of split-gNBs, and we have experienced benefits including pooling gains and flexibility. Support for HLS should not be left out of 6G despite pain points leveraging what we’ve done in 5G rather than from scratch.

FiberCop: We have observed benefits in trials, maybe we need to rethink how to manage it better.

Jio: CU-DU split can solve problems seen in our networks.

Vodafone: Utilization of this split is low, and multivendor even lower. One reason is that 4G networks are still being run alongside 5G. Should consider how to support split deployments but with low effort. We should do only one split.

CATT: Known fact that CU-DU split is already deployed, so we should consider how to continue support in 6G while fixing any known issues.

Huawei: We should learn lessons from 5G, and understand better why it was not widely deployed. Also, if there is any change in motivation for 6G.

Xiaomi: Agree that CP-UP can wait (e.g., study later).

DEUTSCHE TELEKOM: Should investigate reasons why HLS was not deployed, and address those first. We spend too much effort standardizing things that aren’t deployed.

Orange: Not deployed in our network, but would like to see in detail the motivation to potentially deploy in 6G.

NEC: Should not judge based on current market, but support studying enhancements.

Google: Similar view as Vodafone, take 5G as baseline and improve it.

Lenovo: Support standardized interface and enhancements.

QUALCOMM: CU-DU has been successfully deployed, there are many reasons why operators have not (yet) deployed it.

Tejas: Similar view as QC and Lenovo

Charter: We have not deployed, but interesting to understand better the issues

BT: It was perhaps considered risky to deploy in 5G

Boost: Agree with QUALCOMM,, we have deployed CU-DU split. Many 5G deployments are still quite new. CU-DU split can be improved, but support should not be removed in 6G.

CMCC: Not deployed in our network, but fine to study.

Rakuten: Successfully deployed in Japan, vendor lock in may be preventing F1 deployments

T-Mobile USA: Open for CU-DU although unsure about intervendor.

Fujitsu: real benefits in deployments

Thales: benefits may exist with NTN scenarios

Ericsson: Acknowledge that HLS was standardized to exploit virtualization 10 years ago, now in 2025 its possible to virtualize any prt of the protocol stack. Motivation for HLS has changed, we should investigate how to virtualize different parts of the stack in a flexible way. Static split is an impediment to flexibility.

Nokia: Agree we should not repeat 5G study from scratch, but study “what went wrong” with HLS in 5G and whether it can be addressed by standardization.

DCM: Fine to follow Nokia suggestion. This is a basic deployment strategy for operators, particularly those who deployed in 5G.

Capture a way to split functionalities that can be flexibility distributed in a centralized or distributed way.

Take 5G HLS as baseline, capture pain points from 5G HLS

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

### 10.5	AI/ML for RAN

#### 10.5.1	AI/ML use cases

R3-256797	AI/ML use cases for 6G

Type: discussion		For: Decision
					Source: NEC

Decision: 		The document was noted.

R3-256835	Discussion on 6G AIML use cases

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was noted.

R3-257076	Discussion on AI/ML use cases to be considered in 6GR

Type: discussion		For: (not specified)
					Source: LG Electronics Inc.

Decision: 		The document was noted.

R3-256990	(TP for 6G TR) Considerations on AI/ML use cases for 6G RAN

Type: other		For: Agreement
					Source: Huawei

Discussion:

ERICSSON: Digital twin is a new service, not a use case. Should be firstly decided whether DT will be supported in 3GPP.

HUAWEI: this use case should be discussed in RAN3.

Decision: 		The document was noted.

R3-256627	AI/ML for Network in 6G – Use Cases

Type: discussion		For: Agreement
					Source: Xiaomi

Decision: 		The document was noted.

R3-256538	Initial discussion on AIRAN for 6G Network

Type: other		For: Agreement
					Source: ZTE Corporation

Discussion:

Nokia: no question about the feasibility of network slicing. For AI agent, is AI model also out of scope of 3GPP?

ZTE: detail AL/ML algorithm and models is out of the scope of 3GPP

CATT: For task level Qos, whether RAN3 could discuss this? It should be triggered by SA2.

ZTE: we could coordinate with SA2 if needed.

Ericsson: propose to start from the general principles.

Decision: 		The document was noted.

R3-256580	Discussion on high priority use cases for 6G

Type: other		For: Agreement
					Source: China Telecom

Decision: 		The document was noted.

R3-256597	Discussion on AI use cases for 6G

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was noted.

R3-256613	Discussion on 6G AI/ML Use Cases

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

R3-256619	Consideration on AI/ML use cases

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-256713	6G Data Collection

Type: discussion		For: Agreement
					Source: Ericsson, Jio Platforms

Decision: 		The document was not treated.

R3-256813	6G AI/ML Use Cases

Type: other		For: Agreement
					Source: InterDigital, Inc.

Decision: 		The document was noted.

R3-256909	Consideration on 6G AI  use cases

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was noted.

R3-256948	Initial Considerations for AI-native Radio Access Network

Type: discussion		For: (not specified)
					Source: Hanbat National University

Decision: 		The document was noted.

R3-256972	Considerations on Use Cases of 6G AI/ML for RAN

Type: discussion		For: Approval
					Source: China Unicom

Decision: 		The document was noted.

R3-257102	AI/ML Use Cases for 6G

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was noted.

R3-257194	Use cases of AI/ML for NG-RAN in 6G

Type: discussion		For: Discussion
					Source: DOCOMO Communications Lab.

Decision: 		The document was noted.

Should we start the discussion from use cases or principles for AI/ML design?

Companies are fine to discuss the principles firstly.

High-level principles for AI/ML RAN in 6G:

[Agreement] The design of AI/ML algorithms and models for RAN3 led use cases are implementation specific and out of RAN3 scope.

CATT: how about model transfer, whether it is out of scope

ZTE: At the internal RAN node, it is out of scope. Model transfer could be discussed in standard.

TELECOM ITALIA: fully agree with this principle.

User data privacy should be respected during data collection, transmission and AI/ML operation.

ERICSSON, Samsung, CATT: This may not impact RAN3. It is SA3 work.

CMCC: Need more time to think whether it impact data transfer

For all 6G AI/ML use cases, a unified LCM framework is needed to support data collection and model management.

ERICSSON support the principle.

Telecom Italia/Nokia/QUALCOMM: we think this principle really make some risky now. We don’t know yet the status in SA2/SA5.

VODAFONE: after the use cases are clear, we can know whether a unified framework is possible.

ZTE: This has no relation with SA2/SA5 discussion. All WGs should have the same framework.

CATT: are we discussing LCM framework or data collection framework?

When needed, for RAN3 led AI use cases, exchange of AI related data among multiple network entities should be enabled.

QUALCOMM: support the principle

Lenovo/CATT: very unclear whether the collaboration is model training or something else

ZTE: For example, between CN and RAN or UE and RAN collaboration

Ericsson: this means that data should be exchanged between network nodes?

Nokia/Huawei/Xiaomi/CMCC: this principle is confusing as it is. It depends on the use cases. We should not change the principles agreed in RAN3.

ZTE: network entities at least including RAN node.

FiberCorp/Nokia/Lenovo: propose not to capture it.

The study of 6G AI/ML use cases takes some existing 5G use cases as starting point?

-	Energy Saving

-	Mobility Optimization

-	Network slicing

-	Load Balancing

-	Capacity and Coverage Optimization

AI/ML based mobility optimization and network energy saving propose to be supported with high priority in 6G Day 1, since the network operation and user experience guarantee are two critical aspects for operators.

LV: mobility should be discussed together with RAN2.

CMCC: We firstly identify the use cases, this does not mean to preclude other use cases.

Samsung: Support the proposal. For LV comment, RAN3 discuss the use case from different angles.

CATT: agree with this proposal. Maybe LB should be prioritized as well.

ZTE: agree with the two use cases. For ES, which aspects should be studied?

NEC: agree with the two use cases. Network slicing should have higher priority as well.

Ericsson: The two use cases could be a good starting point.

QUALCOMM: We would like to study network ES from scratch and not consider 5GA cases of ES

FiberCorp: ES and mobility are really important use cases.

Telecom Italia/Nokia/NTT Docomo/Jio: support the two use cases.

[Agreement]

The following use cases will be studied in RAN3:

- AI/ML based Network energy saving

- AI/ML based mobility optimization

Other use cases can be discussed in future meetings based on contributions.

In 6G AI/ML study, the following new use cases can be considered:

-	AI/ML assisted QoE

-	Cross-domain collaboration for AI/ML

-	Agentic AI

- AI computing resource management

- AI/ML-enabled RAN Digital Twin for leveraging environmental data

- AI/ML-assisted resilience

- Network for AI

#### 10.5.2	AI/ML framework

R3-256602	Federated Learning Framework for AI/ML for RAN

Type: discussion		For: Decision
					Source: Tejas Network Limited

Decision: 		The document was not treated.

### 10.6	Mobility for 6GR

### 10.7	Interworking between 6GR and NR

## 11	Data Collection for SON/MDT in NR Phase 5 (RAN3-led) [NR_SON_MDT_Ph5-Core]: RP-252560 (target: RAN#115) [TU: 1 (1, 1, 1, 1, 1, 1, 1, 1, 1)]

### 11.1	General

R3-256774	Workplan for Rel-20 SON_MDT Enhancement

Type: Work Plan		For: Decision
					Source: China Unicom

Decision: 		The document was revised to R3-257203.

R3-257203	Workplan for Rel-20 SON_MDT Enhancement

Type: Work Plan		For: Decision
					Source: China Unicom

(Replaces R3-256774)

Discussion:

TS/TR No. BL CR Rapporteur

38.300	China Unicom

37.320	Nokia

38.401	ZTE

38.410	CMCC

38.413	Ericsson

38.420	CATT

38.423	Huawei

37.480	NEC

37.483	Lenovo

38.470	Qualcomm

38.473	Samsung

Decision: 		The document was noted.

### 11.2	MRO Enhancements

#### 11.2.1	Inter-CU LTM

R3-256666	Discussion on MRO enhancement on inter-CU LTM

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was noted.

R3-257115	MRO for inter-gNB LTM

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was noted.

R3-256656	Discussion on SON enhancement for inter-CU LTM

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-256601	MRO Enhancements for Inter-CU LTM

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was noted.

R3-256608	MRO enhancements for Inter-CU LTM

Type: discussion		For: (not specified)
					Source: Qualcomm Korea

Abstract:

Discussion paper on MRO enhancements for Inter-CU LTM

Decision: 		The document was noted.

R3-256755	Initial consideration on MRO for inter CU LTM

Type: discussion		For: (not specified)
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256775	Discussion on MRO Enhancements for inter-CU LTM

Type: discussion		For: (not specified)
					Source: China Unicom

Decision: 		The document was not treated.

R3-256782	MRO for inter-CU LTM

Type: discussion		For: Agreement
					Source: NEC

Decision: 		The document was not treated.

R3-256910	Discussion on MRO for inter-CU LTM

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-256966	(TP for BLCR for 38.413) Inter-CU LTM

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

1.	Use cases

[Agreement]

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.:

BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA;

LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell;

Near failure case: SHR successful HO

Successful case: ping/pong

Potential new cases will not be precluded.

2.	Solution options

[Agreement]

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as base line.

For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

For the connection failure cases:

-	After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis.

L3 based inter-CU LTM is within the scope of the WID

#### 11.2.2	Intra-CU conditional LTM

R3-256657	Discussion on SON enhancement for conditional intra-CU LTM

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-256783	MRO for Intra-CU conditional LTM

Type: discussion		For: Agreement
					Source: NEC

Decision: 		The document was noted.

R3-256600	(TP for SON BL CR for TS 38.300) MRO Enhancements for Conditional Intra-CU LTM

Type: other		For: Agreement
					Source: Nokia

Decision: 		The document was noted.

R3-256611	MRO enhancements for Intra-CU conditional LTM

Type: discussion		For: (not specified)
					Source: Qualcomm Korea

Abstract:

MRO enhancements for conditional LTM

Decision: 		The document was noted.

R3-256911	(TP for TS38.300) Discussion on MRO for intra-CU CLTM

Type: other		For: Agreement
					Source: Lenovo

Decision: 		The document was noted.

R3-256756	Initial consideration on MRO for CLTM

Type: discussion		For: (not specified)
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-256667	(TP for SON BLCR to TS 38.401 and TS 38.300) Discussion on MRO enhancement on intra-CU conditional LTM

Type: other		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-256776	Discussion on MRO Enhancements for C-LTM

Type: discussion		For: (not specified)
					Source: China Unicom

Decision: 		The document was not treated.

R3-256967	Intra-CU conditional LTM

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was not treated.

R3-257116	MRO for Conditional LTM

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was not treated.

1.	Use cases

[Agreement] At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU conditional LTM, i.e.:

BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA;

FFS whether both L1 and L3 apply to beam related selection optimization.

[Agreement]

LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell;

Near failure case: SHR successful HO

Successful case: ping/pong

L3 based intra-CU conditional LTM is witin the scope of the WID

Lenovo: second bullet is more UE side behavior, how network could optimize?

Nokia: for network, it is important to identify a L1 C-LTM.

QUALCOMM: L3 C-LTM should also be included.

ZTE: not sure if L1 and L3 will impact the beam selection.

Potential new cases will not be precluded.

Wrong selection of candidate cells

Failure due to C-LTM execution condition

RACH-less failure due to TAT, or due to TA timer not sent to UE.

Successful fallback from RACH-less to RACH-based C-LTM

Samsung/HUAWEI: these bullets are more like causes to failures.

ERICSSON: need clarification on fallback

QUALCOMM: different cases for RACH less failure. Even TA timer is not received, UE may perform HO.

HUAWEI: let’s just add the new case

2.	Solution options

[Agreement]

[Agreement]

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO for intra-CU C-LTM, if applicable.

For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as base line.

For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

For the connection failure cases, the last serving gNB-CU performs initial analysis.

Additional information required from UE

QUALCOMM: the definition should point to CHO

Lenovo: regarding the wording, we could check when we have TP.

HUAWEI: we need one more round discussion on UE information, which is important, and influence RAN2.

## 12	Study on AI/ML for NG-RAN Phase 3 (RAN3-led) [FS_NR_AIML_NGRAN_Ph3]: RP-252867 (target: RAN #111) [TU: 1 (1, 1, 1)]

### 12.1	General

R3-256546	Skeleton for TR38.745 v0.0.0

Type: draft TR		For: Endorsement
					38.745 v0.0.0
					Source: ZTE Corporation, NEC

Discussion:

The present document provides the description and investigation of new AI/ML based use cases, i.e., Network Slicing and Coverage and Capacity Optimization, and its corresponding solutions, and initial analysis of Rel-18 leftovers.

The scope should be corrected by replacing the use cases in the SID.

Decision: 		The document was revised to R3-257331.

R3-257331	Skeleton for TR38.745 v0.0.0

Type: draft TR		For: Endorsement
					38.745 v0.0.1
					Source: ZTE Corporation, NEC

(Replaces R3-256546)

Abstract:

Post Meeting: Allocated for merging a pCR

Decision: 		The document was endorsed.

R3-256547	Work plan for Study on AI/ML for NG-RAN Phase 3

Type: Work Plan		For: Decision
					Source: ZTE Corporation, NEC

Decision: 		The document was noted.

### 12.2	AI/ML-based mobility

#### 12.2.1	Multi-hop UE trajectory across gNBs

R3-256548	(TP to TR 38.745) Consideration on Mulltiple-hop UE trajectory

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was revised to R3-257311.

R3-257311	(TP to TR 38.745) Consideration on Mulltiple-hop UE trajectory

Type: other		For: Agreement
					Source: ZTE Corporation

(Replaces R3-256548)

Discussion:

-	Remove “one or more” from “Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.”

-	Add FiberCop, Huawei, Ericsson, CATT, NEC, CMCC to co-sources

Decision: 		The document was revised to R3-257329.

R3-257329	(TP to TR 38.745) Consideration on Mulltiple-hop UE trajectory

Type: pCR		For: Agreement
					38.745 v0.0.0
					Source: ZTE Corporation, LG Electronics Inc, Samsung, Qualcomm, Nokia, FiberCop, Huawei, Ericsson, CATT, NEC, CMCC

(Replaces R3-257311)

Decision: 		The document was agreed.

R3-256565	Discussion on Multiple-hop UE trajectory

Type: discussion		For: Decision
					Source: Lekha Wireless Solutions

Decision: 		The document was noted.

R3-256991	(TP for TR 38.745) Considerations on multi-hop UE trajectory in AI/ML-based mobility

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was noted.

R3-257103	(TP to TR 38.745) Multi-hop UE trajectory across gNBs

Type: other		For: Agreement
					Source: Nokia

Decision: 		The document was noted.

R3-257072	(TP to TR 38.745) Support for multi-hop UE trajectory

Type: other		For: Agreement
					Source: Ericsson

Decision: 		The document was noted.

R3-256579	Discussion on multi-hop UE trajectory prediction

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was noted.

R3-256609	Discussion on Multi-hop UE Trajectory Prediction and Feedback

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

R3-256694	(TP to TR 38.745) Multi-hop UE trajectory across gNBs

Type: other		For: Agreement
					Source: Samsung

Decision: 		The document was noted.

R3-256720	Study on Multi-hop UE Trajectory Across gNBs

Type: discussion		For: Decision
					Source: Xiaomi Technology

Decision: 		The document was noted.

R3-256793	(pCR for TR38.745) Multi-hop UE trajectory across gNBs

Type: other		For: Agreement
					38.745 v0.0.0
					Source: NEC

Decision: 		The document was noted.

R3-256814	Multi-Hop UE Trajectory Options

Type: other		For: Agreement
					Source: InterDigital, Inc.

Decision: 		The document was noted.

R3-256833	(TP to TR 38.745) Discussion on AI/ML based Multi-hop UE trajectory across gNBs

Type: other		For: Agreement
					Source: CMCC

Decision: 		The document was noted.

R3-256861	Support of multi-hop UE trajectory prediction/feedback

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-256912	(TP to TR38.745) Discussion on multi-hop UE trajectory

Type: other		For: Agreement
					Source: Lenovo

Decision: 		The document was noted.

Definition of multi-hop UE trajectory:

[Agreement] Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

Nokia: also consider the information from the UE in RRC idle/inactive.

ZTE: for single hop, only connected mode UE is considered. Follow the R19 principle.

CATT: fine with ZTE proposal

Ericsson: Keep RRC connected state.

Deployment of AI/ML model training/inference:

[Agreement]

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Focus on cell-based granularity for measured and predicted multi-hop UE trajectory.

Study multi-hop UE trajectory for L3 HO.

- L3

- Inter-CU LTM

Huawei/Samsung/ZTE/CMCC/Lenovo/Nokia/LG/Xiaome/NEC/CATT/Interdigital: only on L3 HO

Ericsson/QUALCOMM: should try to have a common solution for both

QUALCOMM: multi-hop UE trajectory for Inter-CU LTM can be discussed in 12.3.

Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message?

Nokia: how the predicated multi-hop UE trajectory can be useful?

ZTE: Data collection request is non-UE associated message. break the principle of R18/R19.

Configuration of measurement feedback:

- Data collection initiate procedure

How to transfer measurement feedback:

•	Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

•	Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

•	Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

RAN3 to discuss whether either the whole UE trajectory prediction as received from the source gNB(-CU) is transferred from the 1st hop target gNB to the subsequent (intermediate) gNBs, or that each intermediate gNB provides just a subset of the cells predicted to be visited (i.e., it removes its own cells from the list or predicted cells).

CB # AIMLMultiHop

-  TP to capture the agreement in appropriate way

-  transfer the multi-hop UE trajectory prediction in UE associated message?

-  discuss the above open issues

(ZTE - moderator)

R3-257236	CB:#21_AIMLMultiHop

Type: discussion		For: discussion
					Source: ZTE Corporation

Decision: 		The document was noted.

[Agreement]

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

#### 12.2.2	Intra-CU LTM

R3-256610	Discussion on Intra-CU LTM

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

R3-256578	Discussion on support of AIML based intra-CU LTM

Type: other		For: Agreement
					Source: China Telecom

Decision: 		The document was noted.

R3-256841	Discussion on AIML based intra CU LTM use cases

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was noted.

R3-256537	Discussion on AI/ML for Intra-CU LTM

Type: discussion		For: Decision
					Source: Nokia

Decision: 		The document was noted.

R3-256549	(TP to TR 38.745) Consideration on AIML assisted intra-CU LTM

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-256684	AI/ML Enhancements for Intra-CU LTM

Type: discussion		For: Decision
					Source: Tejas Network Limited

Decision: 		The document was noted.

R3-256695	Discussion on AI/ML-based intra-CU LTM

Type: other		For: Agreement
					Source: Samsung

Decision: 		The document was noted.

R3-256721	Study on Intra-CU AI/ML-assisted LTM Mobility Support

Type: discussion		For: Decision
					Source: Xiaomi Technology

Decision: 		The document was noted.

R3-256794	(pCR for TR38.745) AI/ML based intra-CU LTM

Type: other		For: Agreement
					38.745 v0.0.0
					Source: NEC

Decision: 		The document was noted.

R3-256862	Discussion on support of AI/ML enabled intra-CU LTM

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-256913	AI/ML for Intra-CU LTM

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was noted.

R3-256992	(TP to TR 38.745) Discussion for AI/ML assisted intra-CU LTM

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was noted.

R3-257073	(TP to TR 38.745) AI/ML based mobility – Intra-CU LTM

Type: other		For: Agreement
					Source: Ericsson

Decision: 		The document was noted.

R3-257077	Discussion on considerations in intra-CU LTM use case

Type: discussion		For: (not specified)
					Source: LG Electronics Inc.

Decision: 		The document was noted.

prioritize normal intra-CU LTM scenarios?

Conditional Intra-CU LTM?

ZTE: focus on normal intra-CU LTM scenarios. Consider conditional LTM after intra-CU LTM.

NEC/Huawei/Lenovo/Nokia/Ericsson/Sam: Conditional Intra-CU LTM is not in the scope.

CATT/QUALCOMM: should consider conditional LTM

[Agreement] Prioritize normal intra-CU LTM scenarios.

Cover both L1 and L3 Measurement Report based Intra-CU LTM?

Common understanding: CU-DU split architecture will be covered in the study.

Deployment of AI/ML model for intra-CU LTM:

[Agreement]

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

RAN3 to study the following

[Agreement] - AI/ML assisted candidate cell selection for LTM Handover Preparation

- TA value and validity predication for Intra-CU LTM in spatial and/or temporal domain

AI/ML assisted potential candidate cell selection among the prepared candidate cells to trigger Early Synchronization

AI/ML assisted potential candidate cell selection among the prepared candidate cells to trigger Early Data Forwarding

AI/ML assisted Target cell selection among the prepared candidate cells for LTM Handover execution

beam prediction should also be considered?

CB # AIMLintraCULTM

- TP to capture the agreement in appropriate way

- discuss the above open issues

(QC - Moderator)

R3-257239	CB:#22_AIMLintraCULTM

Type: discussion		For: discussion
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

R3-257301	(TP to TR 38.745) AIML based Intra-CU LTM

Type: pCR		For: Agreement
					38.745 v0.0.0
					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM.

[Agreement] Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

Further discuss whether to study addition of beam prediction in UE Trajectory prediction.

Further discuss predicting TA value in the next meeting.

Further discuss prediction of validity time of a measured TA value.

#### 12.2.3	Handover enhancements

R3-256863	Discussion on support of AI/ML enabled inter-CU LTM

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-256795	(pCR for TR38.745) Other AI/ML based handover enhancements

Type: other		For: Agreement
					38.745 v0.0.0
					Source: NEC

Decision: 		The document was noted.

R3-256550	(TP to TR 38.745) Consideration on Mulltiple-hop UE trajectory

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-256612	Discussion on Inter-CU LTM

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

R3-256696	Discussion on other handover enhancements

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was noted.

R3-256722	Study on Inter-CU AI/ML-assisted LTM in NG-RAN

Type: discussion		For: Decision
					Source: Xiaomi Technology

Decision: 		The document was noted.

R3-256834	(TP to TR 38.745) Discussion on AI/ML based handover enhancement

Type: other		For: Agreement
					Source: CMCC

Decision: 		The document was noted.

R3-256993	(TP for TR 38.745) Discussion for Handover Enhancements

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was noted.

R3-257074	(TP to TR 38.745) Considerations for Handover Enhancements in Release-20

Type: other		For: Agreement
					Source: Ericsson

Decision: 		The document was noted.

R3-257104	(TP to TR 38.745) Handover Enhancements

Type: other		For: Agreement
					Source: Nokia

Decision: 		The document was noted.

Study inter-CU LTM in R20 SI?

Ericsson: should be discussed together.

HUAWEI/ZTE/LG/Nokia/LV: firstly discuss intra-CU LTM.

QUALCOMM: operators have supported it in the plenary discussion.

Samsung/CATT/NEC: support to have this in the scope.

CATT: F1 is the common part for intra and inter-CU LTM. Latency issue should be considered for inter-CU LTM.

VODAFONE: Agree with Qualcomm. It should be discussed.

Study AI/ML based NG handover in Rel-20 SI?

## 13	Study on Integrated Sensing And Communication (ISAC) for NR [FS_Sensing_NR_bis]: RP-252819 (target: RAN #112) [TU: 1 (1, 1, 1, 1, 1)]

### 13.1	General

R3-256588	Work plan for study on Integrated Sensing And Communication (ISAC) for NR

Type: Work Plan		For: Decision
					Source: China Telecom, Xiaomi

Decision: 		The document was noted.

R3-256589	TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR

Type: discussion		For: Approval
					38.765 v..
					Source: China Telecom, Xiaomi

Discussion:

RAN3 is responsible for sections 7 and 8

Decision: 		The document was revised to R3-257332.

R3-257332	TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR

Type: pCR		For: Approval
					38.765 v..
					Source: China Telecom, Xiaomi

(Replaces R3-256589)

Abstract:

Post Meeting: Allocated for merging TPs

Decision: 		The document was endorsed.

R3-256584	Work plan for study on Integrated Sensing And Communication (ISAC) for NR

Type: Work Plan		For: Decision
					Source: China Telecom

Decision: 		The document was withdrawn.

R3-256585	TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR

Type: discussion		For: Approval
					38.765 v..
					Source: China Telecom

Decision: 		The document was withdrawn.

### 13.2	Network architecture

R3-256528	Discussion on ISAC Architecture and General Aspects

Type: other		For: Agreement
					Source: Xiaomi

Decision: 		The document was noted.

R3-256558	Discussion on Network Architecture for ISAC

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was revised to R3-257294.

R3-257294	TP to TR 38.765 for Network Architecture

Type: other		For: Agreement
					38.765 v..
					Source: ZTE Corporation

(Replaces R3-256558)

Discussion:

-	Replace “attempts to identify and describe” with “identifies and describes”

-	Add LGE, CATT, InterDigital as co-sources

-	Title should include (TP to BL pCR to 38.765)

Decision: 		The document was revised to R3-257325.

R3-257325	(TP to BL pCR to 38.765) Network Architecture

Type: other		For: Agreement
					38.765 v..
					Source: ZTE Corporation, Huawei, Xiaomi, Nokia, Nokia Shanghai Bell, NEC, Ericsson, China Telecom, Samsung, LG Electronics, CATT, InterDigital

(Replaces R3-257294)

Decision: 		The document was agreed.

R3-256586	(TP for 38.765) Network architecture solution for gNB-based mono-static sensing

Type: other		For: Agreement
					Source: China Telecom

Decision: 		The document was noted.

R3-256676	(TP for ISAC TR) Network architecture for ISAC

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was noted.

R3-256685	Discussion on Network Architecture for NR ISAC

Type: discussion		For: Decision
					Source: Tejas Network Limited

Decision: 		The document was noted.

R3-256771	Discussion on network architecture for ISAC

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-256784	The discussion on ISAC network architecture

Type: discussion		For: Agreement
					Source: NEC

Decision: 		The document was not treated.

R3-256826	Network Architecture and Protocol Aspects for NR Sensing Support

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-256854	Discussion on Network Architecture for Sensing

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-256895	Discussion on architecture for ISAC

Type: discussion		For: (not specified)
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R3-256914	Discussion on ISAC network architecture

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-256922	Discussion on Network Architecture for ISAC

Type: discussion		For: Agreement
					Source: Ericsson, Jio Platforms

Decision: 		The document was not treated.

R3-256941	Discussions on the framework of ISAC

Type: discussion		For: (not specified)
					Source: China Unicom

Decision: 		The document was not treated.

R3-256947	Network architecture enhancements for NR ISAC

Type: discussion		For: (not specified)
					Source: Hanbat National University

Decision: 		The document was not treated.

R3-257111	Discussion paper on ISAC network architecuture

Type: discussion		For: (not specified)
					Source: CEWiT

Decision: 		The document was not treated.

R3-257118	NR ISAC Network Architecture

Type: discussion		For: Agreement
					Source: InterDigital, Inc.

Decision: 		The document was not treated.

R3-257123	Discussions for ISAC network architecture in RAN3

Type: discussion		For: Decision
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-257144	Discussion on network architecture for ISAC

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

Architecture

RAN3 starts the study on both direct and indirect architectures in the first meeting.

RAN3 starts the architecture study by treating the SF as a unified entity, and further discuss the need of supporting interface between RAN-SF for SF function split based on the progress in SA2 and RAN1.

RAN3 should consider split architecture as a low priority or not support it in Release 20.

ERICSSON: We should start with requirements

CMCC: SID indicates to coordinate with SA2 as needed, does this mean RAN3 can come up with new options?

Xiaomi: Sensing function is in CN

HUAWEI: SA2 is expected to reach conclusion in November, RAN3 should not analyze in parallel

OPPO: Same view as HUAWEI, SA2 may send LS to RAN3 requesting feedback

ZTE: Not big issue to support both direct and indirect architecture

QUALCOMM: RAN3 should not need to study architecture, can focus on the protocol. Also dependencies on RAN1. Only thing left in RAN3 scope right now is split architecture.

Nokia: At first meeting, don’t need to worry about which CN Node the RAN connects to

Samsung: Suggestions to start with indirect architecture

CATT: No reason why RAN3 shouldn’t also be able to study architecture

NEC: Architecture should first be decided by SA2, but RAN3 can discuss protocol stack

China Telecom: We don’t need to concern about which CN node is connected to by RAN

Lenovo: Disaggregated is not precluded. RAN3 focus on protocol.

QUALCOMM: Sensing is not so fundamentally new from architecture perspective

ERICSSON: RAN3 should discuss RAN architecture

ZTE: For sure RAN3 can discuss RAN architecture as stated in the SID.

ERICSSON: Should we be going for something that is “5G like” or something that fits 6G vision.

[Agreement]

Capture logical architecture for ISAC

RAN3 focus on sensing protocol in coordination with SA2

Capture the logical system architecture for sensing in TR.

Option 1: New interface solution

Option 2: Positioning like solution

Option 3: Ambient-IoT like solution

The above candidate options need to be studied and evaluated in RAN3 for 5GA sensing. Option3 has less standard impact compared with other two options.

Interface and protocol options

RAN3 agree to study both direct and indirect interface between RAN node and SF, and study the potential protocol including new protocol and enhanced NG protocol.

In case of direct connectivity, RAN3 discuss how to manage the interface connection between RAN node and SF based on different protocol options.

General aspects to support sensing

RAN3 agrees the following definitions and functionalities of a sensing RAN node

- A sensing RAN node serves one or multiple sensing units (i.e., sensing TRP).

- A sensing TRP supports transmitting and/or receiving the sensing RS.

Sensing gNB: Indicating a gNB capable of serving as a sensing transmitter and/or as a sensing receiver.

Sensing Entity: The Sensing Entity referring to a Sensing Transmitter and/or to a Sensing Receiver.

Sensing Function: Indicating the logical function which is involved to support Sensing Service in 5GC.

To facilitate RAN3 discussion on the applicable interface, and protocol stack, RAN3 can define and use the Sensing CN during the SI, which may include the AMF and Sensing function, or Sensing Function only, up to SA2 decision.

CB # 21_ISAC

- TP for section 7 of TR, capturing logical architecture for ISAC e.g. 6558 but with two boxes

- Terminology in section 3?

- Capture general requirements, if time allows and agreeable

- Introduce basic/general call flow using 6529 as baseline, with Editor’s Notes, FFSes, refinements, etc.

- Capture open issues

(China Telecom - moderator)

To continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.

R3-257299	Summary of Offline Discussion for CB: # 21_ISAC

Type: discussion		For: discussion
					Source: China Telecom

Decision: 		The document was noted.

R3-257298	TP to TR38.765 for Terms, Abbreviations and References

Type: other		For: Agreement
					38.765 v..
					Source: Ericsson

Decision: 		The document was noted.

### 13.3	RAN-CN procedures and signaling

R3-256529	Discussion on Sensing procedures and singalling

Type: other		For: Agreement
					Source: Xiaomi

Decision: 		The document was revised to R3-257297.

R3-257297	TP to TR38.765 for basic sensing call flow

Type: other		For: Agreement
					38.765 v..
					Source: Xiaomi

(Replaces R3-256529)

Discussion:

-	Replace Editor’s Note text with “The following may need further refinement”

-	Title should include (TP to BL pCR to 38.765)

-	Add LG Electronics, NEC, OPPO, InterDigital, Telecom Italia, CATT, CMCC as co-sources

Decision: 		The document was revised to R3-257326.

R3-257326	(TP to BL pCR to 38.765) for basic sensing call flow

Type: other		For: Agreement
					38.765 v..
					Source: Xiaomi, Huawei, Samsung, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, China Telecom, LG Electronics, NEC, OPPO, InterDigital, Telecom Italia, CATT, CMCC

(Replaces R3-257297)

Decision: 		The document was agreed.

R3-256559	Discussion on RAN-CN Procedures and Signalling for ISAC

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-256587	On RAN-CN procedures and signalling for supporting sensing

Type: discussion		For: Decision
					Source: China Telecom, BUPT

Decision: 		The document was noted.

R3-256677	(TP for ISAC TR) RAN-CN procedures and signaling for ISAC

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

R3-256686	Discussion on RAN-CN procedures and signaling for ISAC

Type: discussion		For: Decision
					Source: Tejas Network Limited

Decision: 		The document was not treated.

R3-256712	Discussion on RAN3 impact of gNB-based mono-static sensing

Type: discussion		For: Decision
					Source: OPPO

Decision: 		The document was not treated.

R3-256772	RAN-CN procedures and signaling on ISAC

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-256785	The discussion on ISAC RAN-CN procedure and signaling

Type: discussion		For: Agreement
					Source: NEC

Decision: 		The document was not treated.

R3-256827	Signalling and Procedures for NR Sensing Support

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-256855	Discussion on RAN-CN Procedures Supporting ISAC

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-256896	Discussion on RAN-CN procedures and signaling for ISAC

Type: discussion		For: (not specified)
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

R3-256915	Discussion on general procedures for gNB-based sensing

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-256942	RAN-CN procedures and signaling of ISAC

Type: discussion		For: (not specified)
					Source: China Unicom

Decision: 		The document was not treated.

R3-257025	Discussion on RAN-CN procedures and signaling to support ISAC

Type: discussion		For: Discussion
					Source: Ericsson, Jio Platforms

Decision: 		The document was not treated.

R3-257119	NR ISAC RAN-CN Procedures and Signaling

Type: discussion		For: Agreement
					Source: InterDigital, Inc.

Decision: 		The document was not treated.

R3-257124	Discussions on ISAC RAN-CN procedures and signalling

Type: discussion		For: Decision
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-257145	Discussion on RAN-CN procedures and signaling

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

Procedures to support sensing:

- a class 1 sensing procedure including sensing request/response/failure message

- a class 2 sensing report procedure including sensing report message.

- gNB/CN initiated sensing termination procedure(s), FFS on the details.

Lenovo: Not yet clear whether Sensing Report is user plane or control plane

Xiaomi: Figure is intended to be general, editor’s note could be added

CMCC: call flow is too detailed

HUAWEI: Call flow needs further discussion

ERICSSON: Call flow seems nicely simple

Sensing Service Session Setup

Sensing Service Session Modify

Sensing Service Session Modify Indication

Sensing Service Session Release

ERICSSON: Why is “sensing session” needed?

Nokia: We should keep things general at this meeting, e.g. start/stop/modify

Samsung: We prefer to introduce “sensing session” to support different configurations for different sensing tasks

Following functionalities for RAN-CN interface.

• Start Sensing

• Modify Sensing

• SF-initiated Stop Sensing

• gNB-initiated Stop Sensing

• Sensing Data Report.

To continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC.

## 14	Solutions for Ambient IoT in NR Phase 2 [Ambient_IoT_Solutions_Ph2-Core]: RP-252894 (target: RAN #115) [TU: 0.5 (0.5, 0.5, 1, 1, 1, 1, 1, 1, 1)]

### 14.1	General

R3-256572	Work Plan for Solutions for Ambient IoT (Internet of Things) in NR Phase 2

Type: Work Plan		For: Decision
					Source: Huawei, T-Mobile USA

Decision: 		The document was noted.

### 14.2	Topology 2

R3-256573	[TPs to BL CRs] Consideration on Ambient IoT Topology 2

Type: other		For: Agreement
					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was noted.

R3-256551	Discussion on R20 A-IoT for Topology 2

Type: discussion		For: (not specified)
					Source: ZTE Corporation, China Telecom

Discussion:

Nokia: why F1 involved in authorization?

Huawei: resource handled by DU, so DU needs to be involved.

Lenovo: just one indicator

ZTE: intention is to refer to V2X

ERICSSON: details could be left to be open for further discussion.

Decision: 		The document was noted.

R3-256726	Architecture, functionalities and signaling to support Topology 2

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

Ofinno: device info considered during mobility?

QUALCOMM: what stored in gNB but not sent to AMF.

Decision: 		The document was noted.

R3-256773	Discussion on Topology-2 for Ambient IoT

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-257065	Topology 2 - Starting points for discussions

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was noted.

R3-256888	Inventory Procedure for AIoT Topology 2 UEs

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was noted.

R3-256916	On A-IOT Topology 2

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was noted.

R3-256582	Support of Ambient IoT Topology 2

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was noted.

R3-256668	Discussion on Topology 2 for AIoT

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was noted.

R3-256723	Initial Considerations on Solutions for Ambient IoT in NR

Type: discussion		For: Decision
					Source: Xiaomi Technology

Decision: 		The document was noted.

R3-256786	Discussion on Topology 2 of Ambient IoT

Type: discussion		For: Agreement
					Source: NEC

Decision: 		The document was noted.

R3-256836	Discussion on Topology 2 for A-IoT

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was noted.

R3-257027	Discussion on support of Topology 2

Type: discussion		For: (not specified)
					Source: LG Electronics

Decision: 		The document was noted.

R3-257032	Consideration on the Impacts of A-IoT Topology 2

Type: discussion		For: (not specified)
					Source: Ofinno, LLC

Decision: 		The document was noted.

1.	General Arch

[Agreement] For Rel-20 T2 discussion, based on the RRC-based solution, and RAN3 focus is indirect connectivity is applied.

QUALCOMM,, ERICSSON: why only?

Huawei: this SA2 decision, RAN3 needs to follow. And, AMF serving device could be another AMF

Nokia: we should respect SA2 decision.

ZTE: For T2, UE reader is a normal UE, indirect is useful

[Agreement] NGAP procedures specified for Topology 1 are base line for Topology 2.

QUALCOMM: better focus on signaling part.

ZTE, NEC: for architecture, just refer to TR.

Huawei: how about to add “RAN”

QUALCOMM,, CMCC: even procedures could be extended or enhanced.

2.	NGAP signaling procedures (Inventory and Command)

[Agreement] The AIOT procedures (Topology 2, RRC based solution) in TR38.769 can be as base line.

ERICSSON: why refer to TR?

Huawei: in the TR, there is call flow for RRC based solution, we need to capture E2E call flow

Nokia: support Huawei

Lenovo: we need to start with open issues.

QUALCOMM: call flows anyway need to be captured, but may need RAN2 decision.

3.	UE Reader Authorization

[Agreement]

UE Reader Authorization is performed at 5GC

The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB.

FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

Lenovo: what about revocation?

ERICSSON: revocation is stage 3, no RAN3 concern for now.

ERICSSON: for Xn, not needed. Resource authorized in old gNB is not applicable to new gNB.

QUALCOMM: it could be conveyed over Xn.

CATT: necessary to be conveyed over Xn, since the status may not be changed, if changed AMF will handle.

CMCC: should be allowed.

Huawei: CN can do authorization again during HO, but convey on Xn will enable target to be aware earlier.

Ofinno: speed up the procedure

NEC: support.

4.	UE Reader Selection

In case the core network provides an empty requested service area info, the gNB selects among all the served connected mode UE readers.

### 14.3	Topology 1

## 15	NR mobility enhancements Phase 5 [NR_Mob_Ph5-Core]: RP-252113 (target: RAN #115) [TU: 0 (0, 0, 0, 1, 1, 1, 1, 1, 1)]

### 15.1	General

### 15.2	LTM SCell activation enhancements

## 20	XR for NR Phase 4 [NR_XR_Ph4-Core]: RP-252755 (target: RAN #115) [TU Basket: 0 (0, 0, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5)]

### 20.1	General

### 20.2	Coordination between gNB and CN on N3 delay measurement

## 21	AI/ML for NR air interface Phase 2 [NR_AIML_air_Ph2-Core]: RP-252445 (target: RAN #115) [TU Basket: 0 (0, 0, 0, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5)]

### 21.1	General

### 21.2	Two-sided AI/ML model

## 31	Corrections and Enhancements to Rel-20

### 31.1	Corrections

### 31.2	Enhancements

## 32	Any other business

## 33	Closing of the meeting

3GPP TSG RAN WG3 Chair Mr. Sean Kelley (Nokia) closed the f2f meeting RAN WG3 #129-bis on Friday October 17, 2025 at 14:00 thanking the participants for the contributing to this meeting and the host EF for organizing the meeting.

- The reasons for Tdoc numbers allocated after the meeting are in 'Abstract'

| TDoc | revision of | Title | Source | Contact | Type | Abstract |
| --- | --- | --- | --- | --- | --- | --- |
| R3-257331 | R3-256546 | Skeleton for TR38.745 v0.0.0 | ZTE Corporation, NEC | Jiajun Chen | draft TR | Post Meeting: Allocated for merging a pCR |
| R3-257330 | R3-257234 | Skeleton for 38.760-3 | Vodafone GmbH | Alexey Kulakov | draft TR | Post Meeting: allocated for merging pCRs |
| R3-257332 | R3-256589 | TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR | China Telecom, Xiaomi | Zheng Jiang | other | Post Meeting: Allocated for merging TPs |