3GPP TSG-RAN WG3 Meeting #130	R3-258096

Dallas, USA, 17-21 Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1517 | rev | 1 | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Transmission Bandwidth Asymmetric |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei, Qualcomm, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the TS 38.101, the following note is captured for 60 MHz UL channel bandwidth for asymmetric channel bandwidth combination set 0 for TDD band n50 (see the agreed R15 CR in R4-2511805). NOTE 4: The maximum transmission bandwidth configuration NRB for 60 MHz UL channel BW is reduced from the value defined in table 5.3.2-1 to 161 for 30kHz SCS for Asymmetric channel bandwidth combination set 0 for n50. This means that for this TDD asymmetric case, instead of NRB 162 (as defined in table 5.3.2-1), the NRB 161 is the UL transmission bandwidth for 60 MHz UL channel BW. Currently the transmission bandwidth as per cell information in RAN3 specs only refer to TS 38.104, but not to TS 38.101. Also, NRB 161 is not included as one enumerated value. Therefore, to clearly indicate the Asymmetric channel bandwidth for n50 for 30kHz SCS, a similar note should also be captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For transmission bandwidth of NR TDD band n50, introduce a note to indicate that the uplink transmission bandwidth for 60 MHz UL channel BW is considered as nrb161 upon receiving nrb162 for 30kHz SCS for asymmetric channel bandwidth combination set 0. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only impacts the transmission bandwidth for n50 for Asymmetric channel bandwidth combination set 0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UL transmission bandwidth of NR TDD band n50 using 30KHz SCS is incorrectly considered as nRB 162 for asymmetric channel bandwidth combination set 0. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.473 CR 1595 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: R3-256568 Rev 1: Resubmission to RAN3#130 |  |  |  |  |  |  |  |  |

CHANGES START

#### 9.2.2.20	NR Transmission Bandwidth

The NR Transmission Bandwidth IE is used to indicate either the UL or the DL transmission bandwidth.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …, scs480, scs960) | The values scs15, scs30, scs60 and scs120 corresponds to the sub carrier spacing in TS 38.104 [24]. |
| NR NRB | M |  | ENUMERATED (nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ..., nrb33, nrb62, nrb124, nrb148, nrb248, nrb44, nrb58, nrb92, nrb119, nrb188, nrb242) | This IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104 [24]). The values nrb11, nrb18, etc. correspond to the number of resource blocks "NRB" 11, 18, etc. Note: For NR band n50 indicated by the NR Frequency Band IE contained in the NR Frequency Info IE, the uplink transmission bandwidth is considered as nrb161 upon receiving nrb162 indicated by the UL Transmission Bandwidth IE for 30kHz NR SCS indicated by the NR SCS IE for TDD asymmetric channel bandwidth combination set 0 as specified in TS 38.101 [52]. |

CHANGES END