3GPP TSG-RAN3 Meeting #130	R3-258114

Dallas, US, 17 - 21 November, 2025

Agenda item:	13.2

Source:	Xiaomi

Title:	(TP to pCR 38.765) ISAC general aspects and protocol stacks

Document for:	Other

1	Introduction

In RAN3#129bis meeting, RAN3 agreed the following logical architecture for ISAC in [1] for RAN3 further discussion. Currently, there is no conclusion in SA2 whether AMF is involved in the logical architecture, while RAN3 can focus on the protocol design according to the agreements and open issue made in the last RAN3 meeting.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

This contribution discussed the general aspects and presents protocol stack options aligned with the agreed logical architecture, following the guidance provided during the most recent RAN3 meeting and progresses achieved in RAN1/SA2.

Capture logical architecture for ISAC RAN3 focus on sensing protocol in coordination with SA2 To continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.

2	Discussion

2.1 	General aspects

### Architecture related progress in SA2

Based on SA2’s agreed principle 1 for KI#1: System Architecture to Support Sensing [2], the SF may comprise two components - Sensing Control Function (SCF) and Sensing Processing Function (SPF). However, since Release-20 of 5G-A does not define a standardized interface between SCF and SPF, RAN3 considers SF as a single entity, with a single interface expected between the gNB and SF, and from the gNB’s perspective, it does not need to be aware of the internal separation between SCF and SPF.

7.1.1 Agreed Principles for KI#1: System Architecture to Support Sensing To support KI#1 System Architecture to Support Sensing, the following principles are concluded: Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary.

Observation 1, there’s no standardized interface for the SF’s internal separation between SCF and SPF.

Proposal 1, RAN3 agrees that gNB is not aware of the internal function split inside SF.

On the other hand, SA2 has no progress on the direct or indirect options between gNB and SF. For the gNB monostatic use case, a direct architecture appears more straightforward, as the UE is not involved and thus AMF participation is unnecessary. However, the final decision is subject to SA2, with the potential down-selection expected in the SA2 November meeting. From the RAN3 study perspective, RAN3 can still discus the protocol options no matter which option is adopted by SA2.

- Observation 2, direct architecture is more straightforward to support gNB monostaic sensing.

- Proposal 2,  RAN3 discuss the protocol options based on the logical architecture agreed in RAN3#129bis meeting.

### Definitions and terminologies

- SA2 has agreed on the following principle regarding the gNB: it serves as the sensing entity. While the term "sensing entity" (SE) is used in the SA2 TR, RAN3 refers to it as a gNB. Since SA2 has explicitly stated that SE corresponds to the gNB, it is acceptable for different working groups to use distinct terminology, provided that the mapping between them is clearly defined, and it would also be beneficial that RAN3 also has such mapping description.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

- Similar to positioning, as defined in TS 38.305 [3], supporting sensing may involve the gNB controlling multiple Transmission-Reception Points (TRPs), each TRP may possess sensing capabilities. However, this aspect has not been discussed by SA2 and should be addressed within RAN3 scope.

An NG-RAN node may control several TRPs/TPs, such as remote radio heads, or DL-PRS-only TPs for support of PRS-based TBS.

- Observation 3, SA2 defines a Sensing Entity which corresponding to the gNB, it would be beneficial to clarify this in RAN3 TR.

- Observation 4, similar to positioning, a gNB may control several TRPs for sensing, which falls within the scope of RAN3.

According to the above analysis, we provide the following definition to RAN3 pCR.

- Proposal 3, RAN3 capture the following definitions in the logical architecture section to make it clear:

- - gNB: corresponds to the Sensing Entity (SE) defined in TR 23.700-14 , a gNB may control several Transmission-Reception Point (TRP)s.

- - SF: represents the Sensing Function defined in TR 23.700-14.

### gNB selection and TRP selection

- Key Issue #3 discussed in SA2 regarding sensing entity discovery may have impacts for RAN3, as outlined below. The Sensing Function selects one or more gNBs based on the sensing service request and relevant information about the sensing entities, such as their supported sensing areas. In our view, the information required for gNB selection can either be pre-configured in the SF via OAM or dynamically obtained via signaling. If the signaling approach is agreed upon, further discussion can take place under agenda item 10.3.

7.1.3 Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection: - The Sensing Function selection is based on NEF's local configuration or NRF query. - The Sensing Function registers itself to the NRF, with its NF profile. - Sensing Function is selected based on at least its supported sensing area. - Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request. - Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

- As previously noted, a gNB may control multiple TRPs. If the target sensing area in the service request is smaller than the gNB’s overall supported sensing area, TRP-level selection becomes necessary to avoid activating all TRPs for sensing. In positioning scenarios, TRP selection is performed by the LMF, with the gNB providing TRP information to the LMF in advance. However, for sensing, we believe TRP selection can be handled directly by the gNB, which would result in minimal specification impact.

- Observation 5, SA2 agrees that SF select gNB(s) based on the sensing service request and the information about the Sensing Entities, e.g. its supported sensing area.

- Observation 6, when the target sensing area is smaller than the overall supported sensing area of a gNB, TRP-level selection becomes necessary to avoid activating all TRPs for sensing, and this selection can be done by gNB .

- Proposal 4,The sensing function selects gNB(s) based on the sensing service request and the information about the gNB, which can be obtained by SF via OAM or signalling.

- Proposal 5, The selected gNB may further selects the served TRP(s) for sensing operation according to the sensing request.

### Split Architecture related

As split architecture is not widely deployed in commercial networks, we think the support of split architecture can be de-prioritized or not supported in R20.

Proposal 6: RAN3 should consider split architecture as a low priority or not support it in Release 20.

2.2 	Protocol stack options

### Transport protocol dependency on the Measurement types

Before discussing protocol options, we would like to provide an update on the measurement types currently being considered in RAN1, which impacts RAN3’s transport protocol design discussions. RAN1 is evaluating sensing measurements, with potential measurement levels outlined in their offline discussion [4]. Although the final selection of measurement levels remains uncertain and there are different views on the data size associated with each level, it is evident that lower-level measurements could involve large data volumes - comparable to those measurements similar to positioning. If such volumes for the lower level of measurements are confirmed, the existing control plane based on SCTP transport may not be suitable.

[FL5][H] Proposal 7.1-1-rev3 - The following measurements are identified in the study of NR ISAC. - Level 1: raw data, which is the time-domain samples or subcarrier-domain samples of the channel response from each antenna port, or anything equivalent - Level 2: (partial raw data) - Amplitude and phase profile of delay, and/or Doppler, and/or angle - E.g., Amplitude and phase profile of delay-angle - Level 3 per path (subset of Level 2): - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles - A path can correspond to one or multiple detected points - A path can correspond to one or multiple detected objects - Level 4 per point: - Option 1: Range, velocity, angle, and power per detected point for a given time stamp - Definition: A point is defined by one range/delay, one Doppler and one angle - Option 2: Position, velocity, and power per detected point for a given time stamp - Definition: A point is defined by one location and one velocity - Level 5: e.g., {position, velocity, no target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported - Level 6: e.g., {position, velocity, target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported - SINR, RSRP from perspective of sensing - Time stamp - Study the following levels of measurements for potential down-selection within Rel-20 SI - Level 2 - Level 3 - Level 4 - Level 5 - Level 6 - SINR, RSRP from perspective of sensing

Observation 7, The design of the transport protocol for sensing data transfer depending on the RAN1’s ongoing discussion on measurement types. Currently, six candidate measurement levels are being considered, with lower levels potentially requiring large data volumes. In such cases, the SCTP-based control plane may not be suitable.

Given that RAN1 has not yet made progress on this topic, RAN3 can proceed to study transport protocol options for sensing control messages and sensing data separately.

It is also worth noting that, even if different transport protocols are used for sensing control messages and sensing data, a unified application protocol is preferred for defining both sensing control signaling and sensing measurement results.

Observation 8, Regardless of whether the same or different transport protocols are chosen for sensing control signaling and sensing measurement results, a unified application protocol is preferred to define both the control signaling and measurement results.

Proposal 7, A unified application protocol is specified by RAN3 for both sensing control signalling and measurement results, to avoid specification complexity.

Proposal 8, RAN3 discuss the transport options separately for sensing control signalling and measurement results, the final down-selection should take RAN1 conclusion on the measurement types into account.

### Protocols for sensing control signalling and sensing measurement data

From a discussion standpoint, transport protocols for sensing control signaling and sensing measurement results are addressed separately.

Following the same principle used in defining the logical architecture, we use the term “Nx-AP”,  which refers to either a new application protocol or NGAP. To prevent complexity in discussions arising from the currently unclear architectural options, these alternatives are under the Nx-AP, and a simplified protocol stack figures - similar to the approach taken in TS 38.300 are provided below, along with appropriate descriptions to the areas of uncertainty.

|  | Protocol stack for sensing control singalling |
| --- | --- |
| Protocol stack |  |
| Remarks | Note 1, NxAP could be a new application protocol or existing NGAP Note 2, In case of indirect connectivity, the NxAP is either terminated at AMF (reusing NGAP) or SF (new protocol) |

Protocol options for sensing measurement results (i.e., sensing data) are shown as follows:

|  | Protocol stack for sensing measurement results/data |  |
| --- | --- | --- |
|  | Option 1 SCTP-based | Option 2 GTP-U based |
| Protocol stack |  |  |
| Remarks | Suitable for measurement levels with involving low data volumes, similar to positioning measurement results. | Suitable for measurement levels involving large data volumes. It is important to note a key distinction between this protocol stack and NG-U: in NG-U, the gNB is not aware of the user plane PDU, whereas in sensing, the gNB generates the sensing data PDU. This sensing data PDU requires specification by RAN3. As previously discussed, a unified application protocol is preferred for both sensing control signaling and sensing data, and thus the Nx-AP PDU is used, the name of this PDU can be further refined based on ongoing discussions. |

Proposal 9, RAN3 agree to study the following transport options for sensing.

- protocol stack for sensing control signalling: NxAP/SCTP/TCP/IP

- protocol stack for sensing measurement results:

-- option 1, NxAP/SCTP/TCP/IP

-- option 2, NxAP PDU/GTP-U/UDP/IP

Regarding how to capture the candidate options in the TP, the following options can be considered:

- Option A: List the protocols from transport purpose perspective. As we have used above, the transport protocols are discussed separately for sensing control signaling and sensing measurement results, as captured in Annex A.

- Option B: List the protocols from solution perspective. From this viewpoint, Solution 1 involves using SCTP-based transport for both sensing signaling and sensing measurement results. Solution 2 involves using an SCTP-based transport protocol for sensing control signaling and a GTP-U based transport protocol for sensing measurement results, as captured in Annex B.

Proposal 10, RAN3 agree the TP to pCR 38.765 in Annex A or Annex B.

3	Conclusion

In this contribution, we discussed the ISAC general aspects and candidate protocol stacks, and have the following observations and proposals.

General aspects

### Architecture related progress in SA2

Observation 1, there’s no standardized interface for the SF’s internal separation between SCF and SPF.

Proposal 1, RAN3 agrees that gNB is not aware of the internal function split inside SF.

- Observation 2, direct architecture is more straightforward to support gNB monostaic sensing.

- Proposal 2,  RAN3 discuss the protocol options based on the logical architecture agreed in RAN3#129bis meeting.

### Definitions and terminologies

- Observation 3, SA2 defines a Sensing Entity which corresponding to the gNB, it would be beneficial to clarify this in RAN3 TR.

- Observation 4, similar to positioning, a gNB may control several TRPs for sensing, which falls within the scope of RAN3.

- Proposal 3, RAN3 capture the following definitions in the logical architecture section to make it clear:

- - gNB: corresponds to the Sensing Entity (SE) defined in TR 23.700-14 , a gNB may control several Transmission-Reception Point (TRP)s.

- - SF: represents the Sensing Function defined in TR 23.700-14.

### gNB selection and TRP selection

- Observation 5, SA2 agrees that SF select gNB(s) based on the sensing service request and the information about the Sensing Entities, e.g. its supported sensing area.

- Observation 6, when the target sensing area is smaller than the overall supported sensing area of a gNB, TRP-level selection becomes necessary to avoid activating all TRPs for sensing, and this selection can be done by gNB .

- Proposal 4,The sensing function selects gNB(s) based on the sensing service request and the information about the gNB, which can be obtained by SF via OAM or signalling.

- Proposal 5, The selected gNB may further selects the served TRP(s) for sensing operation according to the sensing request.

### Split Architecture related

Proposal 6: RAN3 should consider split architecture as a low priority or not support it in Release 20.

### Transport protocol dependency on the Measurement types

Observation 7, The design of the transport protocol for sensing data transfer depending on the RAN1’s ongoing discussion on measurement types. Currently, six candidate measurement levels are being considered, with lower levels potentially requiring large data volumes. In such cases, the SCTP-based control plane may not be suitable.

Observation 8, Regardless of whether the same or different transport protocols are chosen for sensing control signaling and sensing measurement results, a unified application protocol is preferred to define both the control signaling and measurement results.

Proposal 7, A unified application protocol is specified by RAN3 for both sensing control signalling and measurement results, to avoid specification complexity.

Proposal 8, RAN3 discuss the transport options separately for sensing control signalling and measurement results, the final down-selection should take RAN1 conclusion on the measurement types into account.

### Protocols for sensing control signalling and sensing measurement data

Proposal 9, RAN3 agree to study the following transport options for sensing.

- protocol stack for sensing control signalling: NxAP/SCTP/TCP/IP

- protocol stack for sensing measurement results:

-- option 1, NxAP/SCTP/TCP/IP

-- option 2, NxAP PDU/GTP-U/UDP/IP

Proposal 10, RAN3 agree the TP in Annex.

4	Reference

[1] R3-257332, “(pCR to TR 38.765) TR skeleton for Study on Integrated Sensing And Communication (ISAC) for NR”,  China Telecom, Xiaomi

[2]3GPP TR 23.700-14 V1.1.0, “Study on Integrated Sensing and Communication”, Oct. 2025.

[3]3GPP TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN, Release 19

[4]R1-2507427, “Summary #5 on evaluations for NR ISAC”, Moderator (Xiaomi)

5	Annex  A (TP to pCR 38.765) list the protocols from transport purpose

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-252819: "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[x1]	3GPP 3GPP TR 23.700-14: “Study on Integrated Sensing and Communication; Stage 2”

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

gNB: corresponds to the Sensing Entity (SE)  defined in TR 23.700-14 [x1], a gNB may control several Transmission-Reception Points (TRPs).

SF: represents the Sensing Function defined in TR 23.700-14 [x1].

- 7.x1	Protocol stack for sensing control signalling

Figure 7.x1-1 shows the Protocol stack for sensing control signalling transmission between gNB and SF:

Figure 7.x1-1. Protocol Stack for sensing control signalling

- 7.x2	Protocol stack for sensing measurement result

Figure 7.x2-1 shows one candidate Protocol stack for sensing measurement result transmission between gNB and SF:

Figure 7.x2-1. Protocol Stack for sensing sensing measurement result

Figure 7.x2-2 shows another candidate Protocol stack for sensing measurement result transmission between gNB and SF:

Figure 7.x2-2. Protocol Stack for sensing measurement result

Editor’s Note x1, The above Nx-AP can be  new protocol application layer or NG-AP.

Editor’s Note x2, in case of direct connectivity, the Nx-AP is terminated between gNB and SF. In case of indirect connectivity, the NxAP is terminated at either AMF (reusing NG-AP) or SF (new application protocol).

Editor’s Note x3, FFS on the name of “Nx-AP PDU”

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

5	Annex  B (TP to pCR 38.765) list the protocols from solution perspective

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

gNB: corresponds to the Sensing Entity (SE)  defined in TR 23.700-14 [x1], a gNB may control several Transmission-Reception Point (TRP)s.

SF: represents the Sensing Function defined in TR 23.700-14 [x1].

- 7.x1	Candidate Protocol stack - Unified Transport

One potential protocol stack solution is to use a unified transport protocol for both sensing control signaling and measurement results.

Figure 7.x1-1 shows one candidate protocol stack for both sensing control signalling and measurement results between gNB and SF.

Figure 7.x1-1. Protocol Stack for a Unified Transport.

- 7.x2	Candidate Protocol stack - Seperate Transports

One potential protocol stack solution is to use seperate transport protocols for sensing control signaling and measurement results.

Figure 7.x2-1 shows the Protocol stack for sensing control signalling transmission between gNB and SF:

Figure 7.x2-1. Protocol Stack for Sensing Control Signalling

Figure 7.x2-2 shows another candidate Protocol stack for sensing measurement results transmission between gNB and SF:

Figure 7.x2-2. Protocol Stack for Sensing Measurement Results

Editor’s Note x1, The above Nx-AP can be  new protocol application layer or NG-AP.

Editor’s Note x2, in case of direct connectivity, the Nx-AP is terminated between gNB and SF. In case of indirect connectivity, the NxAP is terminated at either AMF (reusing NG-AP) or SF (new application protocol).

Editor’s Note x3, FFS on the name of “Nx-AP PDU”

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>