3GPP TSG-RAN WG3 #130	R3-258137

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	13.2

Source:	InterDigital, Inc.

Title:	NR ISAC Network Architecture

Document for:	Discussion and Decision

# Introduction

The SID on Integrated Sensing and Communication (ISAC) for NR was approved in RAN#108 [1] and revised at RAN#109 (FS_Sensing_NR_bis) [2]. The RAN3-led objectives of the study are as follows:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3]

Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3]

- Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts.

- No inter-gNB coordination will be studied.

- Coordination with SA2 as necessary.

At RAN3#129b [3], the study on both direct and indirect architectures started, where SF is treated as a unified entity. It was also decided to treat split architecture as lower priority or not support it in Rel-20. Three options for the logical system architecture were discussed, and common terminology was agreed. This contribution presents our views for NR ISAC Network Architecture study.

# Architecture baseline: monolithic gNB

The NR ISAC study focuses on gNB-based mono-static sensing targeting UAVs, with FR1 prioritised, no UE impacts, and no inter-gNB coordination. The applicability to gNB bi-static sensing may be considered only where it does not introduce additional architecture impacts.

The underlying principle for the study should be that the sensing is executed within a monolithic gNB. The same architectural entity schedules sensing occasions, transmits and receives sensing signals, and derives sensing measurements at appropriate levels. The architecture work is therefore constrained to the NG boundary toward the core-side Sensing Function (SF), with no changes to F1/E1, and no new NG-RAN network functions (NFs) introduced.

Observation 1: Hosting sensing within a monolithic gNB aligns with current NG-RAN deployments.

Proposal 1: Network-based sensing in NR should be realized within a monolithic gNB without introducing new NFs in the RAN and without changes to F1/E1 interfaces.

# NG-RAN↔SF interface

The architecture exposes a single service interface from the monolithic gNB to the SF. Its behavior, e.g., sensing task setup, modify, release, reporting, and stop/failure, can be studied with respect to three NG-RAN↔SF interface options.

The first is an NGAP-native approach, carried over N2 between the gNB and the AMF, where sensing control and reporting use dedicated NGAP information elements. The second is an indirect realization via the AMF that reuses NGAP-aligned mechanisms in an LPP-style flow between the NG-RAN and the CN to the sensing function, also over N2. The third is a new direct RAN↔SF control-plane reference point (not via AMF).

Following RAN3#129b discussions, the NG-RAN↔SF realizations can be grouped as follows. Option 1 (New interface solution) is the direct RAN↔SF reference point. Option 2 (Positioning-like solution) is the NG-anchored path on N2, which may be realized via NGAP-native procedures (if introduced for sensing) and/or an NGAP-carried (LPP-style) sub-protocol. Option 3 (Ambient-IoT-like solution) is a low-impact NG-anchored variant that reuses NGAP patterns/containers with minimal change and primarily CP reporting; data-like reporting remains in study (no new path defined here).

Note that in all options, ISAC control requires reliable, in-order delivery and therefore run on the control plane over SCTP/IP (NGAP on N2 when NG-anchored; NG-like CP for the direct option).

Observation 2: NG-RAN↔SF interface Options 1–3 differ in AMF load, latency, discovery, exposure of NG-RAN internals, migration path, termination, and specification impact.

Proposal 2: Study NG-RAN↔SF interface Options 1–3 with relevant comparison—AMF load, latency, discovery, exposure of NG-RAN internals, migration path, termination, and specification impact—towards selecting working baselines.

# Sensing exposure

When sensing measurement exposure is required, the monolithic gNB can provide these toward core network functions and application functions (AFs) via the core network. The exposed sensing measurements can be provided at different levels and may be accompanied by quality indicators such as timestamps, uncertainty, and validity.

Reporting of sensing results may use CP or a user-plane-like (GTP-U-like) path depending on payload/latency/priority. Payload packaging can follow normal NG conventions for CP and GTP-U-like conventions for user-plane-like; mapping is independent of monolithic gNB implementation.

Exposure carries the usual policy and security hooks: authorization context, scope and retention hints, and optional redaction/anonymization. Admission control and rate limits apply across concurrent sensing tasks. If reports are thinned or delayed, the included quality indicators reflect any resulting loss of confidence so downstream functions can react appropriately.

Observation 3: ISAC control uses CP over SCTP/IP. Reporting may use CP or, when payloads are large or latency-sensitive, a user-plane-like (GTP-U-like) path. Selection may depend on payload/rate, latency, reliability, and priority; results may include quality indicators.

Proposal 3: Study reporting carriage over CP or a user-plane-like (GTP-U-like) path, including selection criteria such as payload/rate, latency, reliability, priority, and IE/container mapping with quality-indicator handling.

# Conclusions

This paper made the following observations and proposals regarding RAN architecture for enabling NR ISAC:

Proposal 1: Network-based sensing in NR should be realized within a monolithic gNB without introducing new NFs in the RAN and without changes to F1/E1 interfaces.

Proposal 2: Study NG-RAN↔SF interface Options 1–3 with relevant comparison—AMF load, latency, discovery, exposure of NG-RAN internals, migration path, termination, and specification impact—towards selecting working baselines.

Proposal 3: Study reporting carriage over CP or a user-plane-like (GTP-U-like) path, including selection criteria such as payload/rate, latency, reliability, priority, and IE/container mapping with quality-indicator handling.

# References

- RP-251861, “New SID: Study on Integrated Sensing And Communication (ISAC) for NR,” RAN#108, Prague, Czech Republic, June 2025.

- RP‑252819, “New SID: Study on Integrated Sensing And Communication (ISAC) for NR,” RAN#109, Beijing, China, September 2025.

- R3‑25xxxxx, “Draft report of RAN3#129b meeting,” RAN3#129b, Prague, Czech Republic, October 2025.