3GPP TSG-RAN WG3 #130	R3-258138

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	13.3

Source:	InterDigital, Inc.

Title:	NR ISAC RAN-CN Procedures and Signaling

Document for:	Discussion and Decision

# Introduction

The SID on Integrated Sensing and Communication (ISAC) for NR was approved in RAN#108 [1] and revised at RAN#109 (FS_Sensing_NR_bis) [2]. The RAN3-led objectives of the study are as follows:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3]

Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3]

- Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts.

- No inter-gNB coordination will be studied.

- Coordination with SA2 as necessary.

Per the RAN3#129b agreements [3], the Sensing Function (SF) is treated as a single unified logical entity, and procedures for both NG-anchored (indirect) and direct RAN–SF realizations are in scope for study, while split architectures are de-prioritized or not supported for Rel-20. Three baseline procedure classes were also captured: Class-1 (request/response/failure), Class-2 (report), and termination. This contribution presents our views for NR ISAC RAN-CN procedures and signaling study.

# Procedures and signaling baseline

The architecture for NR ISAC is based on monolithic gNB whereby sensing is executed inside the gNB; architecture impacts limited to the NG boundary; no new RAN NFs and no F1/E1 changes. The NG-RAN↔SF boundary carries task control and exposure of sensing measurements at different levels with associated quality indicators. Plane selection (N2 and/or N3) and control-plane realization (NGAP-native, NGAP-container, or direct) are subjects for study.

Observation 1: Procedure and signaling should be studied aligned with the monolithic gNB baseline architecture.

Proposal 1: Study procedures and signaling on the monolithic gNB baseline with no NG-RAN NF additions and no F1/E1 interface changes, and across different CP/UP options.

A key premise is resource control, with sensing occasions, beams, timing, and bandwidth configured and scheduled by the gNB. This approach is consistent with existing gNB responsibilities and scheduler ownership. The Core Sensing Function (SF) can provide task intent, constraints, policy, and authorization, and receive results, but the Core does not select the radio resources.

Observation 2: To preserve existing NR architecture and ease-of-deployment, sensing should be done with gNB in control of the resources.

Proposal 2: Study RAN–CN procedures in which the CN/SF requests sensing, the gNB processes requests into sensing occasions under gNB-controlled radio resource selection, and the gNB returns processed results with associated quality indicators.

To keep control and reporting coherent without over-specifying NG details, a minimal task lifecycle can be defined. E.g., consider that a sensing task is created once and modified as needed; all messages can then reference to a TaskID.

Observation 3: A task lifecycle improves change management without constraining NG mapping.

Proposal 3: Study Setup, Modify, Modify-Indication, and Release primitives with a TaskID carried in requests and reports.

# Capability, discovery and authorization

Sensing capability can be discovered and authorized primarily through OAM and core policy, which avoids unnecessary AMF impacts. Where operators desire runtime visibility, the gNB may include an optional capability summary during setup to describe supported measurement/result types, applicable cells or bands, and configured limits on reporting rate or latency. Authorization and consent remain core responsibilities while the RAN enforces the admission of tasks.

Observation 4: OAM-centric discovery provides sufficient visibility with low control-plane burden.

Proposal 4: Study a runtime capability exchange, with an optional NG indication to convey brief capability summary.

# Sensing control

With on-demand sensing control, the CN (via the SF) requests sensing at the monolithic gNB and receives results. For study, the request conveys information categories rather than a fixed IE set: (i) operational scope (e.g., cells/areas/beams), (ii) measurement level (consistent with RAN1), (iii) report configuration (one-shot / periodic / event-triggered), and (iv) quality indicators; retention/privacy is handled by CN policy. The exact IEs are FFS and to be aligned with SA2. The gNB returns the requested output or a failure cause.

A publish/subscribe exchange establishes an ongoing flow of results under parameters agreed at setup. The subscription indicates periodic or event-driven reporting, the cadence or trigger conditions, output level(s), quality indicators, report rate, operational scope (cells/TRPs), etc. After acceptance, the gNB issues reports until expiry, suspension, modification, or termination. Suspension pauses reporting without state loss; termination deletes the context. The procedures are realization-agnostic and work with either an indirect AMF/LMF path or a direct NG-RAN↔SF path, using NGAP-native or NGAP-container mapping. Carriage of sensing control and results uses NG plane (N2/N3).

The procedures should work with either a control-plane realization—(i) NGAP-native over N2, (ii) indirect via AMF over N2 (LPP-style), or (iii) a direct NG-RAN↔SF control-plane reference point—or, if NG-u is selected, a user-plane realization using standard N3 user-plane. For the AMF-based control-plane options, either NGAP-native or NGAP-container mapping can be used.

Observation 5: There are different sensing control models such as on-demand and subscription-based.

Proposal 5: Study on-demand and subscription-based sensing control, including triggers, reporting, and termination.

Observation 6: The definition of the specific IE set for ISAC control is in SA2 scope. RAN3 should avoid pre-empting or down-selecting IEs.

Proposal 6: Study in RAN3 the information categories and procedure implications for ISAC control—e.g., operational scope, measurement level (aligned with RAN1), report configuration (one-shot / periodic / event-triggered), and quality—while keeping IE details FFS for alignment with SA2.

Event-driven subscriptions deliver results only when configured conditions are met, complementing periodic reporting and reducing control-plane load. The baseline conditions cover appearance or disappearance within a defined scope, track-state change beyond thresholds (position/velocity), quality indicator dropping below a minimum threshold, aging when no update occurs within a set interval, and changes in object count. Conditions may be combined and evaluated in the gNB. Each event report can carry subscribed outputs (measurement-level and/or object/track-level), event identifier, sensing/decision timestamps, quality indicator, etc.

Observation 7: Event-based triggers enable timely notifications with bounded control-plane traffic.

Proposal 7: Study on-demand, periodic, and event-triggered reporting with corresponding conditions.

To further advance the NR ISAC study, a minimal set of management primitives for sensing signaling can be studied. This keeps the study procedures‑focused (start/modify/stop/terminate) while remaining fully compatible with established NG principles—without pre‑judging NGAP‑native vs NGAP‑container mapping or direct vs indirect interface realization.

Proposed primitive names (CP‑agnostic):

- Sensing Service Setup/Start

- Sensing Service Modify

- Sensing Service Stop/Release

If the group prefers not to adopt “session”, the same four primitive names can be applied to the TaskID lifecycle without the Sensing Service Session (SSS) wrapper.

These primitives are agnostic to whether a “session” abstraction is used: they either bind multiple TaskIDs under shared parameters (if an operator uses an SSS), or they operate directly per‑TaskID with identical semantics, preserving gNB resource control and NG CP/UP separation.

Observation 8: Management primitives for sensing signaling must be compatible with established NG principles - open interface, modular procedures, and CP/UP separation.

Proposal 8: Study a minimal set of management primitives for sensing signaling.

# NG mapping

In the direct RAN–CN connection, NG-RAN exchanges sensing control and reports with a core-hosted SF over a new NG control-plane reference point that does not involve the AMF. This reduces AMF involvement and can simplify budgeting for periodic reporting. In the indirect realization, sensing control is carried via AMF/LMF over N2 using NGAP/LCS-aligned mechanisms, maximizing reuse of control-plane principles such as logging and charging. The application-level procedures are identical in both cases; only the NG mapping and packaging differ.

Two NG mappings may be envisaged. An NGAP-native mapping introduces dedicated procedures and IEs for sensing control and reporting, while an NGAP-container mapping reuses NGAP by encapsulating requests, publish/subscribe, and report payloads in a sensing container. Control remains on NG: for the AMF-based realization this is over N2, and for the direct path it is the new NG control-plane reference point. Carriage follows a single NG plane consistent with the architecture: either N2 (NG-c) or N3 (NG-u).

Observation 9: NG mapping can be based on direct RAN-CN (via SF) or indirect AMF realization with NGAP-native or NGAP-container mapping.

Proposal 9: Study both direct NG-RAN↔SF and indirect AMF realizations with NGAP-native or NGAP-container mapping, with carriage on NG plane (NG-c/NG-u).

# Conclusions

This paper made the following proposals regarding RAN-CN procedures and signalling enabling NR ISAC:

Proposal 1: Study procedures and signaling on the monolithic gNB baseline with no NG-RAN NF additions and no F1/E1 interface changes, and across different CP/UP options.

Proposal 2: Study RAN–CN procedures in which the CN/SF requests sensing, the gNB processes requests into sensing occasions under gNB-controlled radio resource selection, and the gNB returns processed results with associated quality indicators.

Proposal 3: Study Setup, Modify, Modify-Indication, and Release primitives with a TaskID carried in requests and reports.

Proposal 4: Study a runtime capability exchange, with an optional NG indication to convey brief capability summary.

Proposal 5: Study on-demand and subscription-based sensing control, including triggers, reporting, and termination.

Proposal 6: Study in RAN3 the information categories and procedure implications for ISAC control—e.g., operational scope, measurement level (aligned with RAN1), report configuration (one-shot / periodic / event-triggered), and quality—while keeping IE details FFS for alignment with SA2.

Proposal 7: Study on-demand, periodic, and event-triggered reporting with corresponding conditions.

Proposal 8: Study a minimal set of management primitives for sensing signaling.

Proposal 9: Study both direct NG-RAN↔SF and indirect AMF realizations with NGAP-native or NGAP-container mapping, with carriage on NG plane (NG-c/NG-u).

# References

- RP-251861, “New SID: Study on Integrated Sensing And Communication (ISAC) for NR,” RAN#108, Prague, Czech Republic, June 2025.

- RP‑252819, “New SID: Study on Integrated Sensing And Communication (ISAC) for NR,” RAN#109, Beijing, China, September 2025.

- R3‑25xxxxx, “Draft report of RAN3#129b meeting,” RAN3#129b, Prague, Czech Republic, October 2025.