3GPP TSG-RAN3 Meeting #130	R3-258158

Dallas, USA, 17 - 21 Nov 2025

Agenda Item:	11.2.1

Source: 	Qualcomm Incorporated

Title: 	MRO enhancements for Inter-CU LTM

Document for:	Discussion

# 1 Introduction

[Agreement]

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.:

BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA;

LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell;

Near failure case: SHR successful HO

Successful case: ping/pong

[Agreement]

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as base line.

For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

For the connection failure cases:

-	After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis.

RAN3#129bis made the below agreements for Inter-CU LTM.

In this contribution, we discuss the solution details to support these baseline Inter-CU LTM MRO scenarios and also propose new MRO enhancements use cases for Inter-CU LTM.

# 2 Discussion

## 2.1 Solutions for Baseline MRO scenarios

### 2.1.1 Connection failure detection mechanism

In Intra-CU LTM scenarios, both connection failure reporting and failure detection occur within the same NG-RAN node because the source and target cells belong to the same CU. However, in Inter-CU LTM, these functions are split across different NG-RAN nodes since the recovery cell resides on a different node than the source cell. Intra-system handover failure detection mechanism defined in 3GPP TS 38.300 already supports cases where failure detection and reporting are performed at separate nodes. Hence, the existing intra-system handover failure detection mechanism procedures, Failure Indication for communicating RLF report and Handover Report to source CU for MRO, can be extended to Inter-CU LTM cell switch scenarios.

Observation 1: In Intra-CU LTM, both connection failure reporting and failure detection are performed at the same NG-RAN node, as the source and target cells belong to the same node.

Observation 2: In Inter-CU LTM, connection failure detection and failure reporting occur at different NG-RAN nodes, since the recovery cell belongs to a different NG-RAN node than the source cell.

Observation 3: The intra-system connection failure detection mechanism, as defined in 3GPP TS 38.300, already supports scenarios where failure detection and reporting are performed at different NG-RAN nodes.

Proposal 1: Leverage the existing intra-system connection failure detection mechanism, including the Failure Indication and Handover Report procedures described in TS 38.300, to support connection failure detection for Inter-CU LTM cell switch scenarios.

To differentiate if the connection failure is due to Inter-CU LTM cell switch or Handover, the XnAP Handover Report message must be enhanced to include the LTM cell switch failure indication (i.e. too early LTM cell switch and LTM cell switch to Wrong Cell).

Figure 1: Inter-CU LTM Cell Switch to Wrong Cell

Observation 4: Handover Report Type IE in XnAP Handover Report message only indicates HO failure types (HO too early, HO to wrong cell, Inter-system ping-pong, …, Too Late CHO with candidate SCG).

Proposal 2: To distinguish from L3 HO failure types, enhance XnAP Handover Report message to indicate LTM cell switch failure types (e.g., Too early LTM switch, LTM cell switch to wrong cell).

### 2.1.2 Beam Failure Recovery (BFR) shortly after a successful Inter-CU LTM cell switch

In Intra-CU LTM Beam Failure Recovery (BFR) scenarios, the target DU identifies that a BFR has occurred shortly after a successful cell switch and indicates the source DU via the CU using F1AP DU-CU ACCESS AND MOBILITY INDICATION and F1AP ACCESS AND MOBILITY INDICATION Messages. The BFR SSB Index IE within the MRO for LTM Information IE carries the recovery beam information.

The same solution can be extended to Inter-CU LTM. However, in Inter-CU LTM, the target DU and source DU belong to different NG-RAN nodes. Therefore, when a BFR occurs, upon receiving the MRO for LTM Information IE containing the recovery beam information from the target DU, the target CU can communicate this information to the source CU using the XnAP ACCESS AND MOBILITY INDICATION Message, along with the Source CU UE XnAP ID as shown in the figure.

Figure 2: Beam Failure Recovery (BFR) after successful inter-CU LTM Cell Switch

Proposal 3: The target DU shall identify that a Beam Failure Recovery (BFR) has happened in the UE “shortly” after a successful Inter-CU LTM cell switch and send “MRO information” to the source DU via target CU.

Proposal 4: The target CU shall indicate the recovery beam info and Source CU UE XnAP ID to source CU via XnAP: ACCESS AND MOBILITY INDICATION, which the source CU forwards to source DU via F1AP.

### 2.1.3 RACH-less LTM cell switch failure followed by successful re-establish/recovery

In Intra-CU LTM, if the UE re-establishes or recovers in a different beam within the same target cell following a RACH-less LTM cell switch failure, the target DU uses the Target SSB Index after Cell Switch Failure IE within the MRO for LTM Information IE to indicate the re-established/recovery beam information to the source CU.If the UE re-establishes or recovers in the same or a different beam within the same target cell but with a different Timing Advance (TA), the target DU uses TA Information IE within the MRO for LTM Information IE to communicate the TA information to the source DU.

This mechanism can be extended to inter-CU LTM. Upon receiving the MRO for LTM Information IE comprising of re-established/recovery beam information, TA information from the target DU, the target CU forwards it to the source CU using the XnAP ACCESS AND MOBILITY INDICATION message along with the Source CU UE XnAP ID. The source CU can further communicate this to source DU for further analysis.

Proposal 5: The target DU shall identify that the UE successfully performs a Inter-CU LTM RACH-based access via re-establishment/recovery either to the same target cell but in a beam different from the target beam, or, to the same target cell and same target beam with different TA value, and send MRO information to the source DU via target CU.

Proposal 6: The target CU indicates the re-established/recovery beam, the TA value of RACH-based access and Source CU UE XnAP ID to source CU via XnAP: ACCESS AND MOBILITY INDICATION, which the source CU forwards to source DU via F1AP.

### 2.1.4 Successful HO

During intra-NR handover, if the target NR node fetches the SHR from the UE, it may forward the information to the source NR node using the Xn ACCESS AND MOBILITY INDICATION message. The same can be extended to Inter-CU Successful LTM switch.

Proposal 7: In Inter-CU LTM, the target CU sends the SHR using the existing XnAP: ACCESS AND MOBILITY INDICATION to the source CU for near failure cases detection.

### 2.1.5 Inter-CU LTM Ping-Pong issue

In intra-CU LTM, the source CU detects the ping-pong issue and indicates the target DU using CU-DU CELL SWITCH NOTIFICATION message. This message includes the Last Visited LTM Cells IE along with Mobility Issue IE indicating possible ping-pong issue.

The same can be extended for inter-CU LTM ping-pong, where source CU prepares the Last Visited LTM Cells IE along with Mobility Issue indicating ping-pong and sends to target CU using the XnAP CELL SWITCH NOTIFICATION message.

Proposal 8: The source CU indicates Inter-CU LTM ping-pong issue and the Last Visited LTM Cells information to the target CU via the XnAP: CELL SWITCH NOTIFICATION message.

## 2.2 New MRO scenarios

### 2.2.1 RACH-based LTM due to early UL sync failure

In RACH-less LTM, the source DU selects SSB for early UL synchronization based on measurements and signaling exchanged between the source and candidate CUs. However, if the chosen SSB is unsuitable, the early UL sync may fail. This prevents the target cell from providing the necessary TA, forcing the source cell to fall back to a RACH-based LTM procedure to a candidate cell, as shown in below figure. This fallback introduces delay and increases the risk of LTM failure and can happen in both Intra-CU LTM and Inter-CU LTM. Therefore, this scenario should be considered for MRO, as it highlights the need for improved SSB selection strategies and proactive detection of early UL sync issues.

Figure 3: RACH-based LTM due to early UL sync failure on an unsuitable SSB

Observation 5: The source cell may trigger RACH-based LTM to a candidate cell if early uplink sync fails or didn't happen. This can happen due to an unsuitable SSB indicated to the UE for early UL sync and thereby preventing the candidate cell from providing the TA to the source cell.

Proposal 9: RAN3 to consider detection of RACH-based LTM execution due to early UL sync failure for both Intra-CU LTM and Inter-CU LTM MRO enhancements.

### Beam switching after RACH-based LTM cell switch

In some scenarios, UE may successfully perform RACH-based LTM via SSB1 (e.g., based on the SSB index indicated in the LTM CSC) and subsequently switch to SSB2 for data Tx/Rx on the candidate cell (because the TCI state indicated to be used in the LTM candidate cell is different from SSB1). However, this transition may lead to beam failure on SSB2, necessitating recovery via SSB3, as shown in figure below.

Figure 4: Beam Failure Recovery on a different beam than LTM Initial access

Such dynamic beam switching and failure recovery highlight the issues in early UL sync beam selection and beam management in LTM. Detecting and analysing this beam switching is crucial for optimizing RACH-based LTM procedures, as it enables the network to adjust beam configurations and improve reliability of LTM.

Observation 6: The UE may perform RACH-based LTM cell switch via SSB1 (as indicated by the SSB/PBCH index in LTM CSC) and switch to SSB2 for data tx/rx based on TCI state indication provided in LTM Cell Switch Command. There could be beam failure shortly on SSB2 after successful LTM cell switch and beam recovery over SSB3.

Proposal 10: RAN3 should consider the MRO scenario where BFR happens shortly after successful LTM cell switch (Intra-CU/Inter-CU LTM) on a beam different from the one used for RACH-based LTM cell switch.

### 2.2.3 LTM Failure for L3-based vs L1-based LTM

In Rel-19, MRO enhancements for Intra-CU LTM considered L1 measurements for mobility decisions. However, both Rel-18 and Rel-19 have introduced support for L3 measurements-based LTM for Intra-CU and Inter-CU scenarios. This means LTM failure can occur due to issues in either L1-based or L3-based mobility decisions. Without distinguishing the source of failure, it becomes challenging to optimize mobility robustness and recovery strategies. Thus, we propose to introduce MRO support to distinguish whether an LTM failure occurred due to L1-based or L3-based mobility decisions.

Observation 7: Rel-19 MRO for Intra-CU LTM only considered L1 measurements for Intra-CU LTM.

Observation 8: In Rel-18 and Rel-19, L3 measurements-based LTM is also introduced for Intra-CU LTM, Inter-CU LTM. Hence, it is beneficial to distinguish for the LTM failure scenarios if the failure occurred for the case of L3 based LTM or L1 based LTM.

Proposal 11: Support distinguishing LTM failure is for the case of L3 based LTM or L1 based LTM for both Intra-CU LTM and Inter-CU LTM.

# 3 Summary

Below are the proposals discussed in this document.

Connection failure detection mechanism

Observation 1: In Intra-CU LTM, both connection failure reporting and failure detection are performed at the same NG-RAN node, as the source and target cells belong to the same node.

Observation 2: In Inter-CU LTM, connection failure detection and failure reporting occur at different NG-RAN nodes, since the recovery cell belongs to a different NG-RAN node than the source cell.

Observation 3: The intra-system connection failure detection mechanism, as defined in 3GPP TS 38.300, already supports scenarios where failure detection and reporting are performed at different NG-RAN nodes.

Proposal 1: Leverage the existing intra-system connection failure detection mechanism, including the Failure Indication and Handover Report procedures described in TS 38.300, to support connection failure detection for Inter-CU LTM cell switch scenarios.

Observation 4: Handover Report Type IE in XnAP Handover Report message only indicates HO failure types (HO too early, HO to wrong cell, Inter-system ping-pong, …, Too Late CHO with candidate SCG).

Proposal 2: To distinguish from L3 HO failure types, enhance XnAP Handover Report message to indicate LTM cell switch failure types (e.g., Too early LTM switch, LTM cell switch to wrong cell).

Beam Failure Recovery (BFR) shortly after a successful Inter-CU LTM cell switch

Proposal 3: The target DU shall identify that a Beam Failure Recovery (BFR) has happened in the UE “shortly” after a successful Inter-CU LTM cell switch and send “MRO information” to the source DU via target CU.

Proposal 4: The target CU shall indicate the recovery beam info and Source CU UE XnAP ID to source CU via XnAP: ACCESS AND MOBILITY INDICATION, which the source CU forwards to source DU via F1AP.

RACH-less Inter-CU LTM cell switch failure followed by successful re-establish/recovery

Proposal 5: The target DU shall identify that the UE successfully performs a Inter-CU LTM RACH-based access via re-establishment/recovery either to the same target cell but in a beam different from the target beam, or, to the same target cell and same target beam with different TA value, and send MRO information to the source DU via target CU.

Proposal 6: The target CU indicates the re-established/recovery beam, the TA value of RACH-based access and Source CU UE XnAP ID to source CU via XnAP: ACCESS AND MOBILITY INDICATION, which the source CU forwards to source DU via F1AP.

Successful HO

Proposal 7: In Inter-CU LTM, the target CU sends the SHR using the existing XnAP: ACCESS AND MOBILITY INDICATION to the source CU for near failure cases detection.

Inter-CU LTM Ping-Pong issue

Proposal 8: The source CU indicates Inter-CU LTM ping-pong issue and the Last Visited LTM Cells information to the target CU via the XnAP: CELL SWITCH NOTIFICATION message.

New MRO scenarios

Observation 5: The source cell may trigger RACH-based LTM to a candidate cell if early uplink sync fails or didn't happen. This can happen due to an unsuitable SSB indicated to the UE for early UL sync and thereby preventing the candidate cell from providing the TA to the source cell.

Proposal 9: RAN3 to consider detection of RACH-based LTM execution due to early UL sync failure for both Intra-CU LTM and Inter-CU LTM MRO enhancements.

Observation 6: The UE may perform RACH-based LTM cell switch via SSB1 (as indicated by the SSB/PBCH index in LTM CSC) and switch to SSB2 for data tx/rx based on TCI state indication provided in LTM Cell Switch Command. There could be beam failure shortly on SSB2 after successful LTM cell switch and beam recovery over SSB3.

Proposal 10: RAN3 should consider the MRO scenario where BFR happens shortly after successful LTM cell switch (Intra-CU/Inter-CU LTM) on a beam different from the one used for RACH-based LTM cell switch.

Observation 7: Rel-19 MRO for Intra-CU LTM only considered L1 measurements for Intra-CU LTM.

Observation 8: In Rel-18 and Rel-19, L3 measurements-based LTM is also introduced for Intra-CU LTM, Inter-CU LTM. Hence, it is beneficial to distinguish for the LTM failure scenarios if the failure occurred for the case of L3 based LTM or L1 based LTM.

Proposal 11: Support distinguishing LTM failure is for the case of L3 based LTM or L1 based LTM for both Intra-CU LTM and Inter-CU LTM.