3GPP TSG-RAN WG3 Meeting # 130	R3-258162

Dallas, US, 17 – 21 November, 2025

Agenda Item:	9.2.14

Source:	ZTE Corporation, Nokia, Ericsson, Huawei

Title:	Discussion on prioritized alternative QoS profile list

Document for:	Discussion and approval

1	Introduction

This contribution intends to disclosure the existing misalignment between SA2 and RAN3 specifications on the prioritized alternative QoS profile and discuss whether/how to fix this misalignment.

- 2	Discussion

- Alternative QoS profile is provided from CN to gNB which may be used by gNB to maximum maintain a specific QoS flow. CN transmits the AQP information by using the Alternative QoS Parameters Set List IE. The tabular and procedure text for this IE in NGAP is shown below as example:

- ==== content in TS 38.413-j00 ====

- For each PDU session in the PDU SESSION RESOURCE MODIFY REQUEST message, if the Alternative QoS Parameters Set List IE is included in the GBR QoS Flow Information IE in the PDU Session Resource Modify Request Transfer IE of the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node may accept the setup of the QoS flow when notification control has been enabled if the requested QoS parameters or at least one of the alternative QoS parameters sets can be fulfilled at the time of setup. In case the NG-RAN node accepts the setup fulfilling one of the alternative QoS parameters it shall indicate the alternative QoS parameters set which it currently fulfils in the Current QoS Parameters Set Index IE within the PDU Session Resource Modify Response Transfer IE of the PDU SESSION RESOURCE MODIFY RESPONSE message.

- <SKIP UNRELATED PART>

9.3.1.151	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  |  |  |
| >Alternative QoS Parameters Set Index | M |  | 9.3.1.152 |  | - |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Packet Delay Budget | O |  | 9.3.1.80 |  | - |  |
| >Packet Error Rate | O |  | 9.3.1.81 |  | - |  |
| >Maximum Data Burst Volume | O |  | 9.3.1.83 | Maximum Data Burst Volume is specified in TS 23.501 [9]. This IE may be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. | YES | ignore |
| >PDU Set Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.135 | This IE is included if the PDU Set Packet Delay Budget IE in the DL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >PDU Set Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.135 | This IE is included if the PDU Set Packet Delay Budget IE in the UL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >PDU Set Error Rate Downlink | O |  | Packet Error Rate 9.3.1.81 | This IE is included if the PDU Set Error Rate IE in the DL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >PDU Set Error Rate Uplink | O |  | Packet Error Rate 9.3.1.81 | This IE is included if the PDU Set Error Rate IE in the UL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSparaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS profile. Value is 8. |

9.3.1.152	Alternative QoS Parameters Set Index

This IE indicates the QoS parameters set which can currently be fulfilled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Index | M |  | INTEGER (1..8, ...) | Indicates the index of the item within the Alternative QoS Parameters Set List IE corresponding to the currently fulfilled alternative QoS parameters set. |

9.3.1.153	Alternative QoS Parameters Set Notify Index

This IE indicates the QoS parameters set which can currently be fulfilled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Notify Index | M |  | INTEGER (0..8, ...) | Indicates the index of the item within the Alternative QoS Parameters Set List IE corresponding to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameters set. |

- ==== content in TS 38.413-j00 ====

- Similar IE tabular can also be found in other RAN3 specifications, e.g. TS38.423, 38.473, 37.483. It is clear that, based on the RAN3 definition, the current Alternative QoS Parameters Set Index IE with the value range 1 to 8 which is contained in each provided Alternative QoS Parameters Set Item IE only indicates the index of an AQP set without further information(e.g. priority).

- Observation 1: There is no priority information contained in the Alternative QoS Parameters Set List IE defined by RAN3.

Meanwhile, the following description is shown in TS 23501:

- ==== content in TS 23.501-j50 ====

5.7.1.2a	Alternative QoS Profile

The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, a prioritized list of Alternative QoS Profile(s) to the NG-RAN. If the SMF provides a new prioritized list of Alternative QoS Profile(s) to the NG-RAN (if the corresponding PCC rule information changes), the NG-RAN shall replace any previously stored list with it.

- ==== content in TS 23.501-j50 ====

- It is observed that if the PCC rule contained the such information, the SMF shall provide a prioritized list of AQP to RAN side. In other word, based on the SA2 specification, the AQP list received by RAN shall contain the priority information.

- Observation 2: Based on SA2 understanding, a prioritized AQP list shall be provided from SMF to RAN.

- Based on above explanation, it is clear that the misalignment occurs between SA2 and RAN3 definition on the AQP.

- Proposal 1: There is misalignment between SA2 and RAN3 definition on the priority of AQP.

- In addition, we prefer to further discuss whether/how to fix this misalignment. Either SA2 or RAN3 can correct their specification to sync the AQP definition with the other side. We prefer to further dig into the SA2 specification to show why this priority information is needed from SA2 perspective. The following content can be found in TS 23.503-j50:

- ==== content in TS 23.503-j50 ====

6.1.3.22	AF session with required QoS

The AF may request that a data session to a UE is set up with a specific QoS (e.g. low latency or PDV) and priority handling. The AF can request the network to provide QoS for the AF session based on the service requirements with the help of a QoS Reference parameter that refers to pre-defined QoS information. Instead of the QoS Reference, the AF may provide individual QoS parameters associated to the Flow Description.

- <SKIP UNRELATED PART>

-	When the AF requests the network to provide QoS with individual QoS parameters, one or more Requested Alternative QoS Parameter Set(s) in a prioritized order. Each Requested Alternative QoS Parameter Set is comprised of the following individual parameters: Requested 5GS Delay, Requested Guaranteed Flow Bitrate, Requested Packet Error Rate and optionally, a Requested Averaging Window. Each requested Alternative QoS Parameter Set may also include a Maximum Burst Size parameter.

If the AF request is sent via the TSCTSF, the TSCTSF determines a Requested PDB considering the Requested 5GS Delay and the UE-DS-TT Residence Time.

An AF that provides Alternative Service Requirements shall also subscribe to receive notifications from the PCF for successful resource allocation and when the QoS targets can no longer (or can again) be fulfilled as described in clause 6.1.3.18.

When the PCF authorizes the service information from the AF and generates a PCC rule, it shall also derive Alternative QoS Parameter Sets for this PCC rule based on the QoS Reference parameters or the Requested Alternative QoS Parameter Sets in the Alternative Service Requirements. If the AF provided Requested Alternative QoS Parameter Sets in the request, the PCF may reject any of the Requested Alternative QoS Parameter Sets it has received based on operator policy or impossibility to support the requested values of the individual parameters. If this happens, the PCF may provide in the response to the AF one or more Requested Alternative QoS Parameters Sets that can be supported.

The PCF shall enable QoS Notification Control and include the derived Alternative QoS parameter sets (in the same prioritized order indicated by the AF) in the PCC rule sent to the SMF. When the PCF notifies the AF that QoS targets can no longer be fulfilled, the PCF shall include the QoS Reference parameter or the set of Requested Alternative QoS Parameters corresponding to the Alternative QoS parameter set referenced by the SMF, or an indication that the lowest priority QoS Reference or the lowest priority set of Requested Alternative QoS Parameters of the Alternative Service Requirements cannot be fulfilled (as described in clause 6.1.3.18).

- ==== content in TS 23.503-j50 ====

- Based on the description in TS 23.503 above and previous content shown in TS 23.501, the AQP priority information is defined by AF, PCF and SMF shall pass the same prioritized order AQP list to RAN. In other word, the priority for AQP list is defined by AF and directly passes from CN to RAN.

- Observation 3: The AQP priority information is defined by AF.

- Considering the AF preference may be various based on the different use cases. Without further information from CN, RAN can not decide which AQP set can provide better performance/QoE to app user all by itself. Hence, we believe it is RAN3 duty to fix this misalignment issue. One straight forward way to fix this is to add additional wording on the semantic description of the Alternative QoS Parameters Set Index IE to show the prioritized information. An example is shown below:

- ==== Example based on the content in TS 38.413-j00 ====

9.3.1.152	Alternative QoS Parameters Set Index

This IE indicates the index of alternative QoS parameters set.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Index | M |  | INTEGER (1..8, ...) | Values are ordered in decreasing order of priority, i.e., with 1 as the highest priority and 8 as the lowest priority. |

9.3.1.153	Alternative QoS Parameters Set Notify Index

This IE indicates the QoS parameters set which can currently be fulfilled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Notify Index | M |  | INTEGER (0..8, ...) | Indicates the index of the item within the Alternative QoS Parameters Set List IE corresponding to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest priority alternative parameters set. |

- ==== Example based on the content in TS 38.413-j00 ====

- It is proposed for RAN3 to discuss whether additional semantic description on the priority information for AQP is needed in RAN3 specifications including TS 38.413, TS 38.423, TS 38.473, and TS 37.483.

- Proposal 2: It is proposed for RAN3 to discuss whether additional semantic description on the priority information for AQP is needed in RAN3 specifications including TS 38.413, TS 38.423, TS 38.473, and TS 37.483.

- Besides, we also see the benefit to send LS to SA2 to inform RAN3 modification on prioritized& ordered AQP list in RAN3 specifications, which may assist CN to provide proper AQP list to RAN side. A draft LS is shown in annex and can be considered as baseline.

- Proposal 3: LS to SA2 on RAN3 understanding and modification on prioritized and ordered AQP list in RAN3 specifications.

- 3	Conclusion

- Observation 1: There is no priority information contained in the Alternative QoS Parameters Set List IE defined by RAN3.

- Observation 2: Based on SA2 understanding, a prioritized AQP list shall be provided from SMF to RAN.

- Proposal 1: There is misalignment between SA2 and RAN3 definition on the priority of AQP.

- Observation 3: The AQP priority information is defined by AF.

- Proposal 2: It is proposed for RAN3 to discuss whether additional semantic description on the priority information for AQP is needed in RAN3 specifications including TS 38.413, TS 38.423, TS 38.473, and TS 37.483.

- Proposal 3: LS to SA2 on RAN3 understanding and modification on prioritized and ordered AQP list in RAN3 specifications.

- Annex  LS to SA2 on prioritized AQP list

Title:	[Draft] LS on prioritized Alternative QoS Profile list

Response to:	-

Release:	Rel-16

Work Item:	NR_newRAT-Core, TEI16

Source:	ZTE Corporation [to be RAN3]

To:	SA2

Cc: 	-

# Contact Person:

# Name:	Liu Yansheng

# Email:		liu.yansheng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	[TBD]

# 1. Overall Description:

- RAN3 finds the misalignment on prioritized alternative QoS profile list definition between RAN3 and SA2 specifications. More specifically, based on RAN3 previous definition, there is no priority information in Alternative QoS Parameters Set List IE which is used to transmit the alternative QoS profile information in NGAP, XnAP, F1AP, and E1AP.

- RAN3 agreed the attached CRs to add the priority information for alternative QoS profile for one specific QoS flow.

# 2. Actions:

# To SA2:

ACTION: RAN3 kindly asks SA2 to take the above information into account and provide feedback if any.

# 3. Date of Next TSG RAN WG3 Meetings:

RAN3#131		09th Feb - 13th Feb 2026					Stor-Göteborg, SE

RAN3#131-bis		13th Nov - 17th Apr 2026					Malta, MT