3GPP TSG-RAN WG3 Meeting #130	R3-258168

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.2.2

Source:	Nokia, Jio Platforms, NTT DoCoMo

Title:	Resilient 6G RAN Architecture

Document for:	Discussion and Decision

# 1	Introduction

The new SID on 6G RAN was agreed by RAN#108 and further updated at RAN#109 [1].

The main objectives for RAN3 are listed below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

This paper addresses the 6G RAN Architecture aspects that can make it resilient and aims at proposing relevant requirements for the RAN logical architecture in 6G. It is an updated version of a paper originally submitted to RAN3 #129-bis [2].

# 2	Discussion

One of requirements from RAN plenary applicable to both the 6G RAN architecture and the 6G RAN as a whole relates to enhanced resiliency (TR 38.914 v0.2.0 section 5.2):

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

Before discussing details on the solution, it is important that RAN3 formulates the definition of resilience, especially from the service continuity point of view. Resiliency may have various dimensions, but it is inherently related to the failures: of nodes, of node modules or of interfaces. The more distributed architecture, the more opportunities to fail, but also large single nodes are subject to failures of their modules, e.g. computer units. Such failures are statistically inevitable, and always lead to some recovery action, which takes time. Therefore, resiliency may be directed toward minimizing probability of failure, or towards minimizing the recovery-related interruption, or both.

Addressing failure probability is always the core of any implementation: all vendors build their offered products so that they are as failure-proof as possible, while operators deploy these products so that they are isolated from external factors that may disturb operation. Thus, we believe, the minimalization of the failure probability has always been and will always be addressed without the need of standardization.

On the other hand, these failures still happen and recovery is needed. Depending on the scale of the failure, it may take milliseconds, causing degradation of QoE, or days causing severe service outages. Therefore, in our view, the resilience of 6G RAN should be manifested by minimization of interruption time of connectivity and increase of service continuity in case of RAN node outages and maintenance as well as UE radio link failures.

Observation 1: Failures of logical nodes, their modules or interfaces are inevitable and lead to interruptions, or full outages. The resilience of 6G RAN is manifested as minimization of interruption time of connectivity and increase of service continuity in case of RAN node outages and maintenance as well as UE radio link failures.

Proposal 1. RAN3 to study, how and if the 6G RAN can minimize the recovery interruption from partial or full gNB failure, or an interface failure, including RLF.

One of the key problem in recovering from a failure is the UE context recovery: after RLF (caused either by the wrong config, or a severe failure of the serving equipment, like 6G NodeB or its RAN interfaces), if the UE attempts to reconnect in another 6G NodeB, the new serving node must fetch the context of the UE from the last serving node, or build it from scratch, if that last serving node is not reachable. If the context can be fetched, additional problem is related to finding it: some identifier must be used, but the 5G UE IDs were short of very limited uniqueness and providing limited routing information concerning where to find the context. This is therefore the most prominent problem to solve in 6G.

Proposal 2: RAN3 to study, if UE and/or RAN can handle partial UE context so that it can be recovered as quickly as possible in case of a node or interface failure. Identification of the UE and its context, including needed routing information, shall be part of the study.

Therefore, possible enhancements that RAN3 should consider to enhance provisioning of resiliency for 6G RAN architecture is the introduction of a “RAN Set” concept inherited from the core network (i.e. Network Function Set). The Network Function (NF) set enables any NFs within the same set to access the same UE context. With the NF set, service interruption can be minimized in case NW performs maintenance for some of NFs in the set or NW experiences failure of some NFs in the set, as specified in [3]. The NF set has been a proven concept to offer 5GC resiliency from the commercial 5G SA network deployments over the world. Likewise, a RAN Set is composed of a group of 6G NodeBs as shown in Figure 2, that can cooperate with one another to increase the RAN resiliency and allow UE to maintain service continuity by sharing the UE context information within the RAN Set. For example, 6G NodeBs operating in an overlapping coverage area may be set up such that they can share the UE context and thus be able to overcome 6G NodeB failures by quickly switching the handling of a UE from the failed 6G NodeB to a cooperating one to reduce the interruption time perceived by the UE. Such mechanism avoids one 6G NodeB to appear as a single point of failure and if UE would need to re-establish a connection to any backup 6G NodeB after a failure of the serving 6G Node B, then the UE context is maintained and the connection can be resumed without the need of re-establishment.

Figure 2: 6G RAN set overview

In a similar fashion, 6G NodeBs operating in a non-overlapping coverage area may also be set up as RAN Set so that they can share the UE context. This is helpful in case of an RLF and/or if UE moves to the neighboring 6G NodeB where the interruption time is large, as the UE would need to send an RRC establishment request to the new/target 6G NodeB, whereas with the introduction of RAN Set, target 6G Node B would fetch the UE context from the previous serving 6G NodeB of the UE without the need of re-establishing the connection to the target 6G NodeB.

Observation 2: In 5G RAN Architecture, there is no specified mechanism to tackle the case of a 6G NodeB failure and efficient UE context fetching. This has to be addressed in 6G RAN.

Observation 3: SA2 has already specified mechanism of NF Set for resiliency and it is a proven mechanism to enable UE context sharing.

For example, if we look at Figure 3, we may have the following scenarios:

Case a) 6G NodeB 4 outside a RAN SET 1 (6G NodeB 1, 6G NodeB 2) may not have a direct Xn interface to the previous source 6G NodeB of the UE, but may have at least an Xn interface with a 6G NodeB within the RAN Set.

Case b) 6G NodeB 4 outside a RAN SET 1 (6G NodeB 1, 6G NodeB 2) may have Xn interface established with all 6G NodeBs of the RAN Set.

For case a) the RAN Set, where the UE context is shared among 6G NodeBs of the RAN Set, is beneficial to the 6G NodeB 4 outside the RAN Set, as it enables 6G NodeB 4 to be able to fetch the UE context from/via 6G NodeB2 which otherwise would not be possible with today’s mechanisms. That said, if a target 6G NodeB does not have an Xn interface with the previous source 6G NodeB of the UE, it cannot fetch the UE context.

Similarly, for case b) the RAN Set,where the UE context is shared among 6G NodeBs of the RAN Set, is beneficial to the 6G NodeB 4 outside the RAN Set if the Xn interface of 6G NodeB 4 with 6G NodeB 1 would not work due to an interface failure. In that case, 6G NodeB 4 may still be able to fetch the UE context from/via 6G NodeB 2 knowing that 6G NodeB 2 has a shared UE context with 6G NodeB 1 within the RAN Set 1.

Those two cases infers another benefit of the RAN set that 1) Xn connectivity among all nodes is not a prerequisite deployment condition and that 2) the RAN network is tolerant to Xn connection failure.

Observation 4: The RAN set offers resiliency even w/o Xn connectivity or in case of Xn connection failure.

Figure 3: RAN set under failure scenarios

The above considerations enable us to extend the basic 6G RAN architecture presented in Figure 1. The enhancement consists in defining SETs that may group 6G NodeBs, as presented in the Figure 4 below. As illustrated in the figure, resiliency by means of the RAN set should be supported by 6G Xn and 6G NG interfaces, thereby enabling inter-operability in 6G RAN deployments with RAN sets.

Figure 4: Resilient 6G RAN Architecture

Proposal 3: RAN3 to study the concept of RAN Set for 6G RAN based on the proposed resilient 6G RAN architecture and the guiding principle to enable inter-operability in resilient 6G RAN deployments.

# 4	Conclusion

This paper has discussed key principles for RAN logical architecture and we propose to consider resiliency to be built-in the 6G RAN architecture:

Observation 1: Failures of logical nodes, their modules or interfaces are inevitable and lead to interruptions, or full outages. The resilience of 6G RAN is manifested as minimization of interruption time of connectivity and increase of service continuity in case of RAN node outages and maintenance as well as UE radio link failures.

Proposal 1. RAN3 to study, how and if the 6G RAN can minimize the recovery interruption from partial or full gNB failure, or an interface failure, including RLF.

Proposal 2: RAN3 to study, if UE and/or RAN can handle partial UE context so that it can be recovered as quickly as possible in case of a node or interface failure. Identification of the UE and its context, including needed routing information, shall be part of the study.

Observation 2: In 5G RAN Architecture, there is no specified mechanism to tackle the case of a 6G NodeB failure and efficient UE context fetching.

Observation 3: SA2 has already specified mechanism of NF Set for resiliency and it is a proven mechanism to enable UE context sharing.

Observation 4: The RAN set offers resiliency even w/o Xn connectivity or in case of Xn connection failure.

Proposal 3: RAN3 to study the concept of RAN Set for 6G RAN based on the proposed resilient 6G RAN architecture and the guiding principle to enable inter-operability in resilient 6G RAN deployments.

The above considerations are reflected in the pCR to the TR 38.760 proposed in the Annex below.

# 5	References

- RP-252912, 6G RAN study, RANP #108

- R3-256557, RAN3 #129-bis

- 3GPP TS 23.501, “System architecture for the 5G System (5GS); Stage 2,”

# Annex:	Text proposal to TR 38.760-3

First change, omitted text not changed

## 5.3	Overall RAN Architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

### 5.3.1	RAN-CN Functional Split

Editor’s note: The aim of this section is to describe functions split between RAN and CN.

### 5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN.

### 5.3.Y	Resilient 6G RAN Architecture

The 6G RAN architecture will provide resiliency by design. The resiliency is defined as methods to enable enhanced preservation of the UE context in case of UE, RAN node or RAN interface failure (including RLF), and methods to enable its as fast as possible, full or partial, recovery.

The overall architecture of resilient 6G RAN is illustrated in Figure 5.3.2.X-1. The 6G RAN may include one or more 6G RAN set(s). A 6G RAN set consists of one or more 6G Node B(s) connected to the 6G Core Network through 6G NG interface. 6G Node Bs can be interconnected through 6G Xn interface. 6G Node Bs in the same 6G RAN set are interchangeable and have access to the same context data, e.g., UE context. One 6G Node B belongs to only one 6G RAN set. UE identification system enables finding the context during the recovery.

Editor’s note: Inter-gNB interface within 6G RAN set is FFS if the interface is the same as in 6G Xn or if it supports some additional functions.

Figure 5.3.2.X-1:	Overall architecture of resilient 6G RAN

Remaining text not changed