3GPP TSG-RAN WG3 Meeting #130	R3-258175

Dallas, USA, 17 – 21 October, 2025

Agenda item:	10.5.1

Source:	Nokia

Title:	AI/ML Use Cases in 6G

WID/SID:	FS_6G_Radio

Document for:	pseudo CR

# 1	Introduction

In the last meeting we agreed to study the following use cases in RAN3:

- AI/ML based Network energy saving

- AI/ML based mobility optimization

We also agreed to the following high-level principle:

The design of AI/ML algorithms and models for RAN3 led use cases are implementation specific and out of RAN3 scope.

In this paper, we provide our views with respect to the above use cases and highlight some problems that in our view would be useful to be optimized. We also provide our views on additional use cases to be considered in the first 6G release.

# Discussion

AI/ML discussions started in 3GPP in Rel-17 as a RAN3 led study item which standardized support for AI/ML in Rel-18 over three legacy SON use cases, namely AI/ML Mobility Optimization, AI/ML Load Balancing and AI/ML Energy Saving. In Rel-19, two additional use cases were standardized, namely AI/ML Slicing and AI/ML Coverage and Capacity Optimization (CCO). Two key enablers for those use cases have been UE Trajectory Prediction and UE Performance Feedback, as a feedback mechanism to enable the entity that makes AI/ML Inference determine whether the inference action was a good one or not. Analyzing and standardizing these AI/ML use cases took a lot of effort and was based on a common understanding of benefit of introducing AI/ML in those use cases. The study and work provided good insights on the potential of introducing AI/ML in NG-RAN. In the last meeting we agreed to study AI/ML based Mobility Optimization and AI/ML based Energy Saving. We think that we should also study AI/ML Load Balancing and AI/ML Slicing. Based on this, the 6G RAN may benefit from AI/ML-based optimisations from the first 6G release.

Proposal 1: Besides AI/ML Mobility Optimization and AI/ML Energy Saving, support also AI/ML Load Balancing and AI/ML Slicing from the first 6G release.

## 2.1.	AI/ML Energy Saving

In network energy saving scenarios the specification discriminates between two types of cells: coverage cells, used to provide or increase network coverage, and capacity cells, used to increase the capacity in a given area. In order to achieve network energy saving, the capacity cells can be switched off when the additional capacity they can offer is not needed. This topic was considered in Rel-18 in AI/ML for NG-RAN, where an Energy Cost index was introduced to measure the overall, node level, energy cost at a node to optimize cell switch off decisions. Reporting of an Energy Cost index, corresponding to a measured energy cost, from a target node of an offloading action enables the source node to determine the impact (a cost with respect to energy) of its cell switch-off decision and offloading of traffic to the target node.

The scenarios considered in 5G are simplified in that:

- AI/ML decisions for Energy Saving include only cell switch-off decisions

- Energy Cost is a node level metric of an Energy Consumption and finer granularity is not supported

- Energy is assumed to be provided from the electricity grid without consideration of renewable or non-renewable character of the electricity supply

Network energy saving is crucial for network operators since it impacts the network operational costs. By reducing the energy consumption consumed by the network nodes and network infrastructure, an operator can save a big part of its OPEX costs.  However, another important dimension is the environmental sustainability, which becomes more and more a necessity to reduce CO2 and other greenhouse gas emissions and to tackle climate changes. By lowering the greenhouse gas emissions, network operators reduce their carbon footprint and join the global effort to support sustainability goals. In particular, certain regions implement more strict regulations for energy efficiency, calling for operators in those regions to commit to further greener practices. Also, the operators’ clients may have sustainability targets, and include e.g. greenhouse emissions of their suppliers (e.g. the operator as provider of telecommunication services) in their overall emissions.

Given the above, the need for sustainability would call for a broader use of renewable energy sources for the network nodes, such as solar or wind power, which could be also combined with batteries to optimize power supply and reduce reliance on the electricity grid or fuel-powered electricity supplies. Some critical edge network infrastructure like RAN comprising of radio and baseband and even cloud-based deployments running in data centers could, therefore, be connected to alternative energy sources. This would give rise to energy that could be "green" or "non-green", depending on how it is generated. This could also affect the tariff of the energy during the time of day or time of week or year depending on the existence of necessary conditions (e.g., wind or sun) that support the production of green energy as opposed to non-green energy. In our view, this time-varying characteristics of the energy can no longer be simplified by a fixed “index”. Therefore, an “energy fingerprint” can be characterized by a number of attributes, including the following:

- type of energy i.e. green or non-green

- source, i.e., renewable or non-renewable

- cleanliness i.e. amount of green-house gases (GHG) & other pollutants per joule

- type wise % if multiple types

- source wise % if multiple sources

- cost per unit of energy (tariff)

- time pattern of energy usage / availability

New AI/ML Energy Saving decisions can emerge from this energy fingerprint which not only includes “cell switch-off” or “cell switch-on” decisions, but decisions that take into account the time-variability of the energy fingerprint. This would need to treat traffic differently depending on the type of traffic and based on the energy fingerprint of the network. For instance, certain best-effort traffic types without performance guarantees could be “postponed” in order to allow transmission when the network energy fingerprint improves. However, the situation becomes more complex in case a certain Quality of Service (QoS) needs to be guaranteed, especially when the time/frequency resources get fully loaded since a node may need to transmit at a higher power in order to accommodate the traffic. As a consequence, this will also impact differently certain network slices, depending on the QoS provided by the network slice and SLA applicable to the slice.

Similarly, in the current AI/ML Energy Saving actions, a node takes decisions without considering the energy fingerprint of its neighbours. For example, the Energy Cost measured after a cell-switch off AI/ML action does not take into account information related to how a node where the offloading takes place is powered. Also, when a node requests a neighbouring node to activate a capacity cell or beam, it doesn’t take such information into account. Given the aforementioned concerns, it seems beneficial that a node’s energy saving fingerprint is advertised to neighbours to help them make better energy saving decisions.

Proposal 2: Introduce the following objective in the TR 38.760-3: Study AI/ML Network Energy Saving that takes into account a time-varying energy fingerprint of the network that depends, among other things, on the sources producing the energy, on traffic types and network slice SLAs, to give a few examples.

## 2.2.	AI/ML Mobility Optimization

As mobility is a very important aspect of communication networks, it is crucial to identify means for optimizing mobility decisions at network nodes that influence user’s experience. AI/ML techniques are a promising method to optimize network functions, including mobility related decisions. AI/ML mobility optimization leverages artificial intelligence and machine learning techniques to enhance the performance, efficiency, and user experience of mobile networks by optimizing handovers and the corresponding required resources through predictions and measurements, as well as through a feedback loop to evaluate the handover decisions.

While AI/ML mobility optimizations were studied in 5G, there are still in our view remaining points that deserve attention and can be further enhanced. One aspect that can be optimized through AI/ML techniques are redundant handovers, which can be handovers to target cells where the UE stayed for a very short period of time (short stays, ping-pongs) or handovers that were prepared and never executed. Mobility robustness may be achieved by preparation of multiple target cells for a UE, however such additional HO preparation actions represent cost in terms of signalling and resource reservation for the network. AI/ML could therefore enable a per-UE optimization of the handover strategy, striking a balance between minimizing the risk for connection failure or handover failure for each of the served UEs and e.g. minimization of redundant handovers or handover preparations.

Proposal 3: Introduce the following objective in the TR 38.760-3: Study AI/ML Mobility Optimization with the objective of minimizing a handover cost e.g., related to redundant handovers or handover preparations.

Another area where AI/ML could prove beneficial with respect to mobility involves optimizing the required reporting during mobility. In the current 3GPP design, for a handover decision to be made at RAN, the gNB should configure the UE with required measurements to be done for the different frequencies and the events In case of inter-frequency related measurements, measurement gaps should be associated with the measurements. Usually, the A2 event measurement (serving cell becomes worse than threshold) is configured by default and based on the A2 event reported by UE, the gNB then takes decision to install A3/A5 measurements (neighbour becomes offset better than SpCell, SpCell becomes worse than threshold1 and neighbour becomes better than threshold2). The UE then starts to monitor the configured frequencies of neighboring cells and reports the events to the gNB. Based on the A3/A5 event gNB then triggers handover for the UE to the neighboring cell.

The aforementioned procedure creates considerable signalling overload in the cell and processing overload for the UE and the network, leading to, for example, reduced energy efficiency. Additionally, in case of inter-frequency measurements, the measurements can also lead to additional interruption time. Thus, we believe that measurement reduction could be an area where AI/ML techniques are beneficial to simplify the system design and eliminate unnecessary reporting and processing.

Proposal 4: Introduce the following objective in the TR 38.760-3: Study enhancements in Mobility Optimization reporting methods through AI/ML techniques e.g., by enabling a 6G Node B predict events related to UE e.g., HOF, RLF, A1-A6.

In 5G AI/ML for NG-RAN, an important enabler of the performance of AI/ML decisions has been UE Performance feedback from a target node to a source node. The selected metrics to measure UE Performance feedback have been related to the QoS of the UE soon after a handover execution, such as UL/DL Average Throughput, UL/DL Average Delay, UL Average Packet Loss and DL Average Packet Drop. In our view, the enhancement of these metrics with new ones would give an additional angle in measuring UE Performance, also possibly exploring measurements performed at the UE. Even though in 5G Average Throughput in the UL and DL was considered as a UE Performance metric, further metrics such as the measured traffic at the target node can be explored, which could be dependent on the exact service that a UE is using at the target node.

Proposal 5: Introduce the following objective in the TR 38.760-3: Study AI/ML Mobility Optimization through additional metrics of UE Performance feedback, that may possibly be measured by the UE.

## 2.3.	New Use Cases: QoE Optimization

In 5G discussions, a key mechanism to evaluate the performance of an AI/ML action has been the configuration of collection and reporting by a source node of UE Performance feedback from a target node. As discussed in the previous section, UE Performance comprises a number of UE Performance metrics that are based on the QoS of the UE at the target node. The source node, by taking into account UE Performance feedback, can determine the behavior of the QoS performance of the UE at the target node and through this determine if a handover decision was a good one (if the UE Performance improved or remained the same) or was a poor one (if the UE Performance reduced). Quality of experience (QoE) of a user is an important metric which is often replaced by QoS, simply because, for most practical scenarios, the latter is easier to measure.

In our view, the RAN visible QoE metrics can be exploited as a complement to (or instead of) the UE Performance feedback introduced in 5G to give a different dimension with respect to the experience of a user after a handover; this is particularly relevant since QoE relates to specific application measurements, compared to QoS which captures general user metrics. However, the available RAN visible QoE metrics are, at the moment, limited to application layer buffer level and playout delay for media startup. Given the importance of QoE and the benefits of RVQoE measurements in RAN, further RVQoE metrics may be required to capture performance of new services in 6G. Thus, we believe that it is beneficial to analyse in the present study item additional metrics, e.g. QoE related, that can be collected at the application layer and made visible at the RAN.

Figure 1 shows an example of UE Handover to a target node where feedback could be based on RVQoE metrics instead of (or together with) the QoS-level metrics introduced in 5G.

Figure 1 UE Performance feedback with RVQoE metrics.

As QoE related feedback has a role similar to UE Performance feedback, the impact of AI/ML-based network actions related to e.g., Energy Saving and Mobility Optimization, to name a few examples, on QoE can be evaluated. This could imply:

- Improving mobility decisions based on new metrics reflecting user’s experience in terms of the user's perception of service continuity and quality during mobility events, that may include application layer feedback.

- Combined performance optimization and energy saving actions in the 6G RAN.

- Combined energy saving actions in the UE and in the 6G RAN, based on multi-domain coordination potentially involving the UE, the RAN and the CN.

Proposal 6: Introduce the following objective in the TR 38.760-3: Study enhancements to QoE measurements, e.g., through additional metrics that can be collected at the application layer and made visible at the RAN, and evaluate the impact of already analysed AI/ML use cases e.g., Energy Saving, Mobility Optimization, on QoE.

# 3	Conclusion

In this paper, we make the following proposals:

Proposal 1: Besides AI/ML Mobility Optimization and AI/ML Energy Saving, support also AI/ML Load Balancing and AI/ML Slicing from the first 6G release.

Proposal 2: Introduce the following objective in the TR 38.760-3: Study AI/ML Network Energy Saving that takes into account a time-varying energy fingerprint of the network that depends, among other things, on the sources producing the energy, on traffic types and network slice SLAs, to give a few examples.

Proposal 3: Introduce the following objective in the TR 38.760-3: Study AI/ML Mobility Optimization with the objective of minimizing a handover cost e.g., related to redundant handovers or handover preparations.

Proposal 4: Introduce the following objective in the TR 38.760-3: Study enhancements in Mobility Optimization reporting methods through AI/ML techniques e.g., by enabling a 6G Node B predict events related to UE e.g., HOF, RLF, A1-A6.

Proposal 5: Introduce the following objective in the TR 38.760-3: Study AI/ML Mobility Optimization through additional metrics of UE Performance feedback, that may possibly be measured by the UE.

Proposal 6: Introduce the following objective in the TR 38.760-3: Study enhancements to QoE measurements, e.g., through additional metrics that can be collected at the application layer and made visible at the RAN, and evaluate the impact of already analysed AI/ML use cases e.g., Energy Saving, Mobility Optimization, on QoE.

# 4	References

- TR 38.760-3, Study on 6G Radio RAN3 aspects, v0.1.0

# Annex – pseudo CR to TR 38.760-3

This pseudo CR is based on [1].

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 7	AI/ML for RAN

## 7.1	AI/ML use cases

Editor’s note: Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

The following use cases are to be studied in 6G RAN:

- AI/ML based Network Energy Saving

- AI/ML based Mobility Optimization

- Application layer metrics based on RVQoE

### 7.1.1	AI/ML based Network Energy Saving

Network energy saving is crucial for network operators since it impacts the network operational costs. By reducing the energy consumption consumed by the network nodes and network infrastructure an operator can save a big part of its OPEX costs. However, another important dimension is the environmental sustainability, which becomes more and more a necessity to reduce CO2 and other greenhouse gas emissions and to tackle climate changes. By lowering the greenhouse gas emissions, network operators reduce their carbon footprint and join the global effort to support sustainability goals.

The need for sustainability would call for a broader use of renewable energy sources for the network nodes, such as solar or wind power, which could be also combined with batteries to optimize power supply and reduce reliance on the electricity grid or fuel-powered electricity supplies. Some critical edge network infrastructure like RAN comprising of radio and baseband and even cloud running into data centers could, therefore, be connected to alternative energy sources. This would give rise to energy that could be "green" or "non-green", depending on how it is generated. This could also affect the tariff of the energy during the time of day or time of week or year depending on the existence of necessary conditions (e.g., wind or sun) that support the production of green energy as opposed to non-green energy. This time-varying characteristics of the energy can no longer be simplified by a fixed “index”.

An “energy fingerprint” can be characterized by a number of attributes, including the following:

- type of energy, i.e., green or non-green

- source, i.e., renewable or non-renewable

- cleanliness i.e., amount of green-house gases (GHG) & other pollutants per joule

- type wise % if multiple types

- source wise % if multiple sources

- cost per unit (tariff)

- time pattern of energy usage/availability

Study the following enhancements in AI/ML Network Energy Saving:

- a time-varying energy fingerprint of the network that depends, among other things, on the sources producing the energy, on traffic types, and network slice SLAs.

- Combined performance optimization and energy saving actions in the 6G RAN

### 7.1.2	AI/ML based Mobility Optimization

AI/ML mobility optimization leverages artificial intelligence and machine learning techniques to enhance the performance, efficiency, and user experience provided by mobile networks by optimizing handovers and the corresponding required resources through predictions and measurements as well as through a feedback loop to evaluate the handover decisions. One aspect that can be optimized through AI/ML techniques are redundant handovers, which can be handovers to target cells where the UE stayed for a very short period of time (short-stays, ping-pongs) or handovers that were prepared and never executed.

Study the following enhancements in AI/ML Mobility Optimization:

- minimize a handover cost e.g., related to redundant handovers

- enable a 6G Node B predict events related to UE e.g., HOF, RLF, A1-A6

- introduce additional metrics of UE Performance feedback, that may be possibly measured by the UE and that may depend on application layer feedback

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>