3GPP TSG-RAN WG3 Meeting #130                                                             R3-258179

Dallas, USA, 17 – 21 November 2025

Agenda item: 	10.2.1

Source:  	Qualcomm Incorporated

Title:  	(pCR to TR 38.760-3) Support for protocol stack evolution throughout timeline of 6G

Document for:	Other

# 1	Introduction

In last meeting, RAN3 discussed the general principles for the 6G study. Based on this discussion, the following FFS was captured in the chairman notes:

Study whether the protocol stack of RAN3-defined interfaces can evolve over the lifespan of 6G.

This paper discusses benefits and issues related to protocols stack evolution for RAN3-defined interfaces throughout the lifespan of 6G.

# 2	Discussion

## 2.1 	Motivation

In past generations, RAN3 defined a single protocol stack for each of RAN-CN and RAN-internal interfaces in the first release of the G, and that protocol stack remained the only one supported for the corresponding interface throughout the timeframe of the G.

When considering protocol stack evolution for 6G, the following scenarios can be considered:

- Specification of multiple protocol stacks on Day 1

- Specification of additional protocol stacks throughout the lifespan of 6G.

Observation 1: 6G could support multiple protocol stacks for each RAN interface on Day1 and/or introduce additional protocol stacks in later releases.

Further, protocol stack evolution can be applied to any type of RAN3-defined interface, i.e., RAN-CN interface(s) and/or RAN-internal interface(s) as well as point-to-point interfaces and service-based interfaces.

Observation 2: Protocol stack evolution can be applied to point-to-point interfaces as well as to service-based interfaces.

Potential benefits of protocol stack evolution:

Allowing a more flexible protocol stack support in 6G has the following advantages:

- Optimal stack selection for deployment: The operator can select the optimal protocol for an interface specific to deployment scenario and network operation.

- Optimal interface functionality, performance, and/or operation over time: When a new protocol is introduced by a standard body (e.g., IETF) or another entity, the protocol(s) stack can be upgraded within the next 6G release so that the operator can take advantage of the improved functionality, performance, and/or operation associated with the new protocol.

Observation 3: Protocol stack evolution has the benefit that it allows the operator to select the best protocol stack specific for a deployment scenario, and to upgrade the stack over the time frame of 6G to improve interface functionality, performance and/or operation.

Issues of protocol stack evolution:

The following issues need to be addressed when allowing for multiple concurrent protocol stacks of an interface:

- Issue 1: Inter-vendor interworking: Interworking problems will occur if network nodes/functions of different vendors have to mutually communicate while using different protocols stacks.

- Issue 2: Protocol stack selection for communications: In case a network node/function support multiple protocol stacks and/or is upgraded to support additional protocol stacks, it needs to agree with each remote end point, which of the stacks should be used for communications.

Observation 4: Protocol stack evolution needs to allow for inter-vendor interworking and provide mechanisms for interface endpoints to perform protocol stack selection.

## 2.2 	Options for multi-stack support and protocol stack evolution

The following options can be considered when specifying multiple protocol stacks and/or protocol stack evolution for an interface:

Option A: Support of one mandatory “default” protocol stack. All other protocol stacks are optional.

Option B: Support for all protocols stacks is mandatory.

Option C: Support for all protocols stacks is optional.

Observation 5: The following options can be considered when specifying multiple protocol stacks of an interface:

- Option A: One mandatory default stack while all other stacks are optional,

- Option B: All stacks are mandatory,

- Option C: All stacks are optional.

These three options can address the above issues in the following manner:

Option A: Mandatory default stack and optional other stacks.

- Issue 1: Inter-vendor interworking: Can always be provided via the default protocol stack.

- Issue 2: Protocol stack selection for communications: In case multiple protocol stacks are supported by a network function/node, the following solutions could be considered for stack selection:

    - The interface endpoint is OAM-configured with the protocol stack to be used with each remote peer.

    - The interface end point uses the default stack to start communications and then negotiates the use of alternative stacks via this communications channel.

    - The interface endpoint selects a stack when initiating communications, and in case communications fails, it tries other stacks and/or falls back to the default stack.

Option B: All stacks are mandatory

- Issue 1: Inter-vendor interworking: Can always be provided via all protocol stacks defined on Day 1.

- Issue 2: Protocol stack selection for communications: The same solutions can be considered as in Option A.

Option C: All stacks are optional

- Issue 1: Inter-vendor interworking: Needs to be negotiated between operator and affected network vendors.

- Issue 2: Protocol stack selection for communications: The same solutions can be considered as in Option A.

Observation 5: All three options can support inter-vendor interworking and protocol stack selection for communications.

## 2.3 	Evaluation of options for multi-stack support and protocol stack evolution

While the above options can address issues 1 and 2 identified in section 2.1, they have different impact on implementation and operations, which affects the likelihood on whether multiple stacks and stack evolution will be specified or deployed. The following trade-offs can be identified:

Evaluation of Option A (one default stack, other stacks or optional):

- Implementation impact: Low impact since network vendors initially have to implement only the default stack, and the implementation of further protocol stacks only needs to be considered if requested by an operator.

- Operations impact: Low impact on operations since interconnectivity via default stack is possible without additional planning effort. In case multiple stacks are supported, stack selection can occur via autonomous mechanisms, i.e., avoiding the need for configuration-based match-up of stacks used between interface endpoints.

- Likelihood for specification of multiple stacks and/or stack upgrades: High since such specification does not result in any implication for network vendors to actually implement the additional stack(s).

- Likelihood for deployment of multiple stacks and/or stack upgrades: Small since network vendors have little inclination to support more that the default stack. In case a vendor supports a second stack, a peer vendor may not support it. It may be possible, however, that a network vendor uses an additional stack within its own deployment (e.g., for 6G intra-RAN interface(s)).

Evaluation of Option B (all stacks are mandatory):

- Implementation impact: High impact since network vendors need to support all stacks specified.

- Operations impact: Low impact on operations since interconnectivity via Day-1 protocol stacks is possible without additional planning effort. As for Option A, autonomous mechanisms for stack selection avoid need for configuration-based match-up of stacks used between interface endpoints.

- Likelihood for specification of multiple stacks and/or stack upgrades: Small since network vendors have little incentive to put up with the burden to implement a new stack without knowing whether it will ever be deployed.

- Likelihood for use of multiple stacks and/or stack upgrades: High since all stacks specified are available on network nodes/functions and autonomous selection mechanisms can ensure use of the optimal stack.

Evaluation of Option C (all stacks are optional):

- Implementation impact: Conditional on whether different operators request the vendor to support the same or different stacks.

- Operations impact: High since inter-vendor deployment requires explicit negotiations with vendors to converge on the protocol stack(s) to be supported.

- Likelihood for specification of multiple stacks and/or stack upgrades: High since such specification does not result in any implication for network vendors to actually implement the additional stack(s).

- Likelihood for use of multiple stacks and/or stack upgrades: Small since the industry will most likely converge on a de facto default stack to mitigate the impact on network vendors and operators. As for Option A, it is more likely that a network vendor uses an additional stack within its own deployment.

Observation 6: The three options have different trade-offs regarding implementation impact, operations impact, likelihood that specification of multiple stacks will occur, and likelihood that deployment of multiple stacks will occur.

## 2.4 	Summary on protocol stack evolution in 6G

RAN3 should include the above analysis on in the study and discuss whether protocol stack evolution is considered for the normative phase.

Proposal 1: RAN3 to include the above analysis in the study and discuss whether protocol stack evolution is considered for the normative phase.

The above discussion should be captured in the pCR to TR 38.760 in the Annex.

Proposal 2: Agree on pCR to TR 38.760 in the Annex.

# 3	Conclusion

This paper discussed benefits and issues related to protocols stack evolution for RAN3-defined interfaces throughout the lifespan of 6G. The following observations and proposals have been made:

Observation 1: 6G could support multiple protocol stacks for each RAN interface on Day1 and/or introduce additional protocol stacks in later releases.

Observation 2: Protocol stack evolution can be applied to point-to-point interfaces as well as to service-based interfaces.

Observation 3: Protocol stack evolution has the benefit that it allows the operator to select the best protocol stack specific for a deployment scenario, and to upgrade the stack over the time frame of 6G to improve interface functionality, performance and/or operation.

Observation 4: Protocol stack evolution needs to allow for inter-vendor interworking and provide mechanisms for interface endpoints to perform protocol stack selection.

Observation 5: The following options can be considered when specifying multiple protocol stacks of an interface:

- Option A: One mandatory default stack while all other stacks are optional,

- Option B: All stacks are mandatory,

- Option C: All stacks are optional.

Observation 6: The three options have different trade-offs regarding implementation impact, operations impact, likelihood that specification of multiple stacks will occur, and likelihood that deployment of multiple stacks will occur.

Proposal 1: RAN3 to include the above analysis in the study and discuss whether protocol stack evolution is considered for the normative phase.

Proposal 2: Agree on pCR to TR 38.760 in the Annex.

# 4	References

[1]  RP-251881, Study on 6G Radio, 3GPP TSG RAN #108, Prague, Czech Republic, June 2025. Further updates in RP-25912, 3GPP TSG RAN #109, Beijing, China, September 2025.

# Annex

## pCR to TR 38.760

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.1		RAN-CN Interface

>>>>>> Intermediate text omitted

### 6.1.x	Support for protocol stack evolutions throughout the timeline of 6G

In past generations, RAN3 defined a single protocol stack for each of RAN-CN and RAN-internal interfaces in the first release of the G, and that protocol stack remained the only one supported for the corresponding interface throughout the timeframe of the G.

When considering protocol stack evolution for 6G, the following scenarios can be considered:

- Specification of multiple protocol stacks on Day 1

- Specification of additional protocol stacks throughout the lifespan of 6G.

Further, protocol stack evolution can be applied to any type of RAN3-defined interface, i.e., RAN-CN interface(s) and/or RAN-internal interface(s) as well as point-to-point interfaces and service-based interfaces.

Potential benefits of protocol stack evolution:

Allowing a more flexible protocol stack support in 6G has the following advantages:

- Optimal stack selection for deployment: The operator can select the optimal protocol for an interface specific to deployment scenario and network operation.

- Optimal interface functionality, performance, and/or operation over time: When a new protocol is introduced by a standard body (e.g., IETF) or another entity, the protocol(s) stack can be upgraded within the next 6G release so that the operator can take advantage of the improved functionality, performance, and/or operation associated with the new protocol.

Issues of protocol stack evolution:

The following issues need to be addressed when allowing for multiple concurrent protocol stacks of an interface:

- Issue 1: Inter-vendor interworking: Interworking problems will occur if network nodes/functions of different vendors have to mutually communicate while using different protocols stacks.

- Issue 2: Protocol stack selection for communications: In case a network node/function support multiple protocol stacks and/or is upgraded to support additional protocol stacks, it needs to agree with each remote end point, which of the stacks should be used for communications.

Observation 4: Protocol stack evolution needs to allow for inter-vendor interworking and provide mechanisms for interface endpoints to perform protocol stack selection.

The following options can be considered when specifying multiple protocol stacks and/or protocol stack evolution for an interface:

Option A: Support of one mandatory “default” protocol stack. All other protocol stacks are optional.

Option B: Support for all protocols stacks is mandatory.

Option C: Support for all protocols stacks is optional.

These three options can address the above issues in the following manner:

Option A: Mandatory default stack and optional other stacks.

- Issue 1: Inter-vendor interworking: Can always be provided via the default protocol stack.

- Issue 2: Protocol stack selection for communications: In case multiple protocol stacks are supported by a network function/node, the following solutions could be considered for stack selection:

    - The interface endpoint is OAM-configured with the protocol stack to be used with each remote peer.

    - The interface end point uses the default stack to start communications and then negotiates the use of alternative stacks via this communications channel.

    - The interface endpoint selects a stack when initiating communications, and in case communications fails, it tries other stacks and/or falls back to the default stack.

Option B: All stacks are mandatory

- Issue 1: Inter-vendor interworking: Can always be provided via all protocol stacks defined on Day 1.

- Issue 2: Protocol stack selection for communications: The same solutions can be considered as in Option A.

Option C: All stacks are optional

- Issue 1: Inter-vendor interworking: Needs to be negotiated between operator and affected network vendors.

- Issue 2: Protocol stack selection for communications: The same solutions can be considered as in Option A.

While the above options can address issues 1 and 2 identified in section 2.1, they have different impact on implementation and operations, which affects the likelihood on whether multiple stacks and stack evolution will be specified or deployed. The following trade-offs can be identified:

Evaluation of Option A (one default stack, other stacks or optional):

- Implementation impact: Low impact since network vendors initially have to implement only the default stack, and the implementation of further protocol stacks only needs to be considered if requested by an operator.

- Operations impact: Low impact on operations since interconnectivity via default stack is possible without additional planning effort. In case multiple stacks are supported, stack selection can occur via autonomous mechanisms, i.e., avoiding the need for configuration-based match-up of stacks used between interface endpoints.

- Likelihood for specification of multiple stacks and/or stack upgrades: High since such specification does not result in any implication for network vendors to actually implement the additional stack(s).

- Likelihood for deployment of multiple stacks and/or stack upgrades: Small since network vendors have little inclination to support more that the default stack. In case a vendor supports a second stack, a peer vendor may not support it. It may be possible, however, that a network vendor uses an additional stack within its own deployment (e.g., for 6G intra-RAN interface(s)).

Evaluation of Option B (all stacks are mandatory):

- Implementation impact: High impact since network vendors need to support all stacks specified.

- Operations impact: Low impact on operations since interconnectivity via Day-1 protocol stacks is possible without additional planning effort. As for Option A, autonomous mechanisms for stack selection avoid need for configuration-based match-up of stacks used between interface endpoints.

- Likelihood for specification of multiple stacks and/or stack upgrades: Small since network vendors have little incentive to put up with the burden to implement a new stack without knowing whether it will ever be deployed.

- Likelihood for use of multiple stacks and/or stack upgrades: High since all stacks specified are available on network nodes/functions and autonomous selection mechanisms can ensure use of the optimal stack.

Evaluation of Option C (all stacks are optional):

- Implementation impact: Conditional on whether different operators request the vendor to support the same or different stacks.

- Operations impact: High since inter-vendor deployment requires explicit negotiations with vendors to converge on the protocol stack(s) to be supported.

- Likelihood for specification of multiple stacks and/or stack upgrades: High since such specification does not result in any implication for network vendors to actually implement the additional stack(s).

- Likelihood for use of multiple stacks and/or stack upgrades: Small since the industry will most likely converge on a de facto default stack to mitigate the impact on network vendors and operators. As for Option A, it is more likely that a network vendor uses an additional stack within its own deployment.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>