3GPP TSG-RAN WG3 Meeting #130                                                             R3-258182

Dallas, USA, 17 – 21 November 2025

Agenda item: 	10.3.2

Source:  	Qualcomm Inc, KT Corp, JIO Platforms, NTT Docomo, FiberCop, CEWiT, Reliance JIO, Charter Communications, Deutsche Telekom, Sony

Title:  	[pCR to TR 38.760-3] Characteristics of point-to-point and service-based interfaces

Document for:	Other

# 1	Introduction

TSG RAN#108 agreed on a new SI on 6G Radio. The SID contains a milestone for TSG#112 in June 2026 [1]:

TSG#112 (June/2026): RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI

Last RAN3 meeting, the following agreement was captured in R3-257324 as a pCR to TR 38.760-3 [2]:

The general principles for the specification of the 6G RAN-CN interface are as follows: … - the 6G RAN-CN interface supports all possible RAN deployment scenarios;

The following paper contains a description of point-to-point interface (PPI) characteristics and service-based interface characteristics (SBI). The description is based on 5G specification. The discussion further includes potential enhancements that can be considered for each interface type in 6G.

The paper includes a pCR to TR 38.760-3 in the Annex.

# Discussion

## 2.1	Point-to-point interfaces (PPI)

### 2.1.1 	5G PPI

Definition

The term point-to-point interface captures the essence of the specifications for NG, Xn, F1 and E1 interfaces. These specifications state that the respective interfaces are point-to-point. The term point-to-point is defined in 21.905, but it seems that there is no explicit definition of a point-to-point interface in 3GPP specifications.

NG, Xn and F1 support C-plane part and U-plane part. E1 only supports C-plane part. The analysis only focuses on the C-plane.

Principal 5G PPI characteristics

Typically, a point-to-point interface (PPI) applies to a specific pair of network-node/functions and it comprises all signaling procedures (i.e., Elementary Procedures), that can be conducted between this pair of network nodes/functions. The signaling procedures follow the client-server paradigm. There is at most one interface between each pair of network nodes. A schematic captures these principal 5G PPI characteristics in Fig.1-1.

There are exceptions to the above 5G PPI characteristics: The NG-C interface is not only used between gNB and AMF but also between gNB and N3IWF, gNB and TNGF, and between gNB and AIOTF.

Figure 1-1: Schematic of 5G PPI: The interface refers to a pair of network nodes/functions and contains all elementary procedures defined between these network nodes/functions.

Protocol stack of 5G PPI

5G PPIs use SCTP as transport protocol and ASN.1 for application layer encoding. Figure 1-2 shows the 5G PPI protocol stack.

Figure 1-2: C-plane protocol stack of 5G PPI

Security protection of 5G PPI

According to TS 33.501, 5G PPIs can be secured via IPsec or D-TLS.

Collocation of TNL endpoint and application layer endpoint of 5G PPI

The TNL endpoint and the application-layer endpoint of a 5G PPI at a network node/function are assumed to be collocated (Figure 1-3).

This collocation assumption allows the application-layer endpoint to derive information about TNL addresses from the collocated TNL endpoint. The application layer on an AMF, for instance, can derive the TNL address of a gNB from the IP header of the F1 Setup Request message received from this gNB.

The collocation of TNL endpoint and application-layer endpoint at a network node/function further allows using the TNL’s reliable delivery mechanism to ensure reliable message delivery of messages on application layer. Therefore, class-2 message can be reliably delivered without application-layer confirmation.

Figure 1-3: 5G PPI assumes that TNL- and app-layer endpoints are collocated at the network node/function.

Peer-node discovery for 5G PPI

5G PPIs support the following forms of peer-node/function discovery:

- Network node/function is OAM-configured with the peer’s TNL address.

- Network node/function is OAM-configured with the peer’s FQDN, which can be resolved to a TNL address via DNS.

- For Xn, a gNB resolves the peer gNB’s TNL address based on the peer gNB’s gNB-ID via a discovery service provided by the AMF.

Peer-node authorization for 5G PPI

5G PPIs can use OAM-based configuration of certificates for peer-node/function authorization.

5G PPI’s support of deployment scenarios

5G PPI can support traditional deployment scenarios that can be accommodated via customized implementations of network nodes/functions.

5G PPIs are not supported by commercial cloud solutions for containers, message brokers, and load balancers that expect TCP and/or QUIC/UDP on transport layer. Further, messages of 5G procedures cannot be handed by message brokers that rely on the HTTP layer. Such message brokers are typically supported in commercial cloud environments.

5G PPI’s support of new services

5G presently only supports AIOT as a new service with the IOTF in the CN. 5G AIOT supports a direct and an indirect interface between RAN-node ang AIOTF. The direct interface uses NG-C.

Support for ISAC in 5G is still under discussion.

### 2.1.2 	Potential enhancements of PPI for 6G

The following enhancements can be considered when using PPIs for 6G RAN:

Enhancement 6G PPI 1: TNL using QUIC

SCTP can be replaced with QUIC/UDP (Figure 1-4a).

- QUIC supports equivalent functionality as SCTP with respect to reliable stream and datagram transport, congestion control, mechanisms for head-of-line blocking, and IP multi-homing.

- QUIC includes TLS as security layer.

- QUIC is supported by commercial cloud solutions for containers, message brokers, etc.

Enhancement 6G PPI 2: Application-layer using HTTP

HTTP can be placed between TNL and application layer. HTTP logically belongs to the application layer (Figure 1-4b).

- HTTP provides separation between TNL and application layer.

- HTTP allows a PPI to operate without collocation of TNL endpoint and application-layer endpoint at the network node/function. In this case, the HTTP layer bridges between TNL- application-layer endpoints at a network node/function (as shown for SBI in Figure 2-3). This enables implementations where the network node/function supports multiple application-layer instances, e.g., for the purpose scalability and robustness.

- HTTP is supported by message brokers of typical cloud environments.

Figure 1-4: Enhancements of 6G PPI over 5G: a) TNL using QUIC/UDP, b) application layer including HTTP, c) combination of a) and b)

Enhancement PPI 3: Enhanced network-node/function discovery service

The AMF’s gNB-discovery service based on gNB-ID can be enhanced to a more general network-node/function service based on a larger number of identifiers.

- The network-node/function discovery service could be moved out of the AMF to reduce load on the AMF.

- The network-node/function discovery service could be used by other network nodes/functions than gNBs, e.g., core NFs supporting new services.

- Network-node/function discovery could be based on a more extended set of identifiers than gNB-IDs.

## 2.2	Service-based interfaces (SBI)

### 2.2.1 	5G SBI

Definition

The 5G SBA is defined in TS 29.500. The SBA defines service-based interfaces (SBI). TS 29.501 defines more details related to SBIs. 5G SBA is only defined for the C-plane.

Principal 5G SBI characteristics

The service-based interface represents how a set of services is provided by an individual network-node type, which is referred to as a service producer. These services can be used by multiple types of (authorized) network nodes, referred to as service consumers. It is possible to restrict the number of consumers to a single network node type by configuration. The signaling procedures are defined as API method calls which follow the client-server paradigm.

Figure 2-1 shows a schematic of the principal SBI characteristics.

Figure 2-1: Schematic of 5G SBI: The SBI comprises all API method calls for which the NF provides the server function. API method 2 of NF1’s SBI is used by both, NF2 and NF3.

Protocol stack of 5G SBI

5G SBIs uses TCP as transport protocol and HTTP on application layer. The application layer information is encoded via JSON. Figure 2-2 shows the 5G SBI protocol stack.

Figure 2-2: Protocol stack of 5G SBI.

Security protection of 5G SBI

According to TS 33.501, 5G SBIs can be secured via IPsec or TLS.

Collocation of TNL endpoint and application layer endpoint of 5G SBI

5G SBI does not require that TNL endpoint and application-layer endpoint at a network node/function are collocated. In case they are not collocated, the HTTP layer is used for bridging (Figure 2-3). This allows implementations where a logical network function uses multiple physical NF instances. The message broker at the HTTP bridge distributes the incoming messages over the various physical NF instances for load balancing purposes.

Since the application-layer endpoint may not be collocated with the TNL endpoint, it cannot derive TNL address information from this TNL endpoint. TNL-related information is therefore explicitly included in application layer signaling.

The application layer endpoint may also not be aware of the TNL address used by its own network node/function. 5G SBI therefore allows the application layer to send endpoint information in form of the network node’s/function’s FQDN instead of the network node’s/function’s TNL address. The peer can then resolve this FQDN to an TNL address via NRF or DNS.

Since TNL endpoint and application-layer endpoint may not be collocated, the application layer cannot rely on reliable data delivery provided by the TNL since messages may get lost between TNL endpoint and application layer endpoint.   Therefore, 5G SBI support message delivery confirmation on application layer (i.e., HTTP).

Figure 2-3: 5G SBI does not assumes that TNL- and app-layer endpoints are collocated at the network node/function. The red crosses indicates locations of potential packet loss.

Peer-node discovery for 5G SBI

5G SBIs support the following forms of peer-node/function discovery:

- Network node/function is OAM-configured with the peer’s TNL address.

- Network node/function is OAM-configured with the peer’s FQDN, which can be resolved to a TNL address via DNS.

- Network node/function discovery and TNL address discovery via NRF.

Peer-node authorization for 5G SBI

5G SBI can use OAM-based configuration of certificates for peer-node/function authorization. SBA also allows obtaining certificates from the NRF using the OAuth 2.0 framework.

5G SBI’s support of deployment scenarios

5G SBIs can support all traditional deployment scenarios that can be accommodated via customized implementations of network nodes/functions.

5G SBIs are supported by cloud solutions using containers, message brokers, and load balancers that expect TCP or QUIC/UDP on transport layer. Further, message brokers, that rely on the HTTP layer can handle 5G SBI messages.

5G SBI’s support of new services

5G presently only supports AIOT as a new service with the IOTF in the CN. 5G AIOT supports an SBI interface between AMF and AITOF.

Support for ISAC in 5G is still under discussion.

### 2.2.2 	Potential enhancements of SBI for 6G

The following enhancements can be considered when using SBIs for 6G RAN:

Enhancement SBI 1: TNL using QUIC

TCP can be replaced with QUIC/UDP (Figure 2-4a).

- QUIC supports the equivalent functionality as TCP such as reliable stream transport and congestion control. QUIC further supports datagram transport, mechanisms for head-of-line blocking, and IP multi-homing.

- QUIC includes TLS as security layer.

Enhancement SBI 2: Application-layer using ASN.1 or other encodings

The application layer can use ASN.1 or other encoding instead of JSON encoding (Figure 2-4b)

- ASN.1 encoding is natively supported by HTTP.

- The support of other encodings by HTTP needs to be considered in the study.

Figure 2-4: Enhancements of 6G SBI over 5G: a) TNL using QUIC/UDP, b) application layer using ASN.1 encoding, c) combination of a) and b)

## 2.3	Summary of discussion

The above discussion has been captured as a pCR to TR 38.760 in the Annex.

Proposal: Include the pCR in the Annex to TR 38.760-3.

# 3	Conclusion

This paper discussed point-to-point interface (PPI) characteristics and service-based interface characteristics (SBI) for 5G, and potential enhancement for 6G. The following proposal has been made:

Proposal: Include the pCR in the Annex to TR 38.760-3.

# 4	References

[1]  RP-251881, Study on 6G Radio, 3GPP TSG RAN #108, Prague, Czech Republic, June 2025. Further updates in RP-25912, 3GPP TSG RAN #109, Beijing, China, September 2025.

[2]  R3-257324, pCR to TR 38.760, 3GPP TSG RAN WG#129-bis, Prague, Czech Republic, October 2025.

# Annex

## pCR to TR 38.760-3

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

>>>>>> Intermediate text omitted

### 6.1.3.x	Characteristics of 5G point-to-point interfaces

Definition

The term point-to-point interface captures the essence of the specifications for NG, Xn, F1 and E1 interfaces. These specifications state that the respective interfaces are point-to-point. The term point-to-point is defined in 21.905, but it seems that there is no explicit definition of a point-to-point interface in 3GPP specifications.

NG, Xn and F1 support C-plane part and U-plane part. E1 only supports C-plane part. The analysis only focuses on the C-plane.

Principal 5G PPI characteristics

Typically, a point-to-point interface (PPI) applies to a specific pair of network-node/functions and it comprises all signaling procedures (i.e., Elementary Procedures), that can be conducted between this pair of network nodes/functions. The signaling procedures follow the client-server paradigm. There is at most one interface between each pair of network nodes. A schematic captures these principal 5G PPI characteristics in Fig. 6.1.3.x-1.

There are exceptions to the above 5G PPI characteristics: The NG-C interface is not only used between gNB and AMF but also between N3IWF and AMF, TNGF and AMF, and between gNB and AIOTF.

Figure 6.1.3.x-1: Schematic of 5G PPI: The interface refers to a pair of network nodes/functions and contains all elementary procedures defined between these network nodes/functions.

Protocol stack of 5G PPI

5G PPIs use SCTP as transport protocol and ASN.1 for application layer encoding. Figure 6.1.3.x-2 shows the 5G PPI protocol stack.

Figure 6.1.3.x-2: C-plane protocol stack of 5G PPI

Security protection of 5G PPI

According to TS 33.501, 5G PPIs can be secured via IPsec or D-TLS.

Collocation of TNL endpoint and application layer endpoint of 5G PPI

The TNL endpoint and the application-layer endpoint of a 5G PPI at a network node/function are assumed to be collocated (Figure 6.1.3.x-3).

This collocation assumption allows the application-layer endpoint to derive information about TNL addresses from the collocated TNL endpoint. The application layer on an AMF, for instance, can derive the TNL address of a gNB from the IP header of the F1 Setup Request message received from this gNB.

The collocation of TNL endpoint and application-layer endpoint at a network node/function further allows using the TNL’s reliable delivery mechanism to ensure reliable message delivery of messages on application layer. Therefore, class-2 message can be reliably delivered without application-layer confirmation.

Figure 6.1.3.x-3: 5G PPI assumes that TNL- and app-layer endpoints are collocated at the network node/function.

Peer-node discovery for 5G PPI

5G PPIs support the following forms of peer-node/function discovery:

- Network node/function is OAM-configured with the peer’s TNL address.

- Network node/function is OAM-configured with the peer’s FQDN, which can be resolved to a TNL address via DNS.

- For Xn, a gNB resolves the peer gNB’s TNL address based on the peer gNB’s gNB-ID via a discovery service provided by the AMF.

Peer-node authorization for 5G PPI

5G PPIs can use OAM-based configuration of certificates for peer-node/function authorization.

5G PPI’s support of deployment scenarios

5G PPI can support traditional deployment scenarios that can be accommodated via customized implementations of network nodes/functions.

5G PPIs are not supported by commercial cloud solutions for containers, message brokers, and load balancers that expect TCP and/or QUIC/UDP on transport layer. Further, messages of 5G procedures cannot be handed by message brokers that rely on the HTTP layer. Such message brokers are typically supported in commercial cloud environments.

5G PPI’s support of new services

5G presently only supports AIOT as a new service with the IOTF in the CN. 5G AIOT supports a direct and an indirect interface between RAN-node ang AIOTF. The direct interface uses NG-C.

Support for ISAC in 5G is still under discussion.

### 6.1.3.x+1 	Enhancements for point-to-point interfaces in 6G

The following enhancements can be considered when using PPIs for 6G RAN:

Enhancement 6G PPI 1: TNL using QUIC

SCTP can be replaced with QUIC/UDP (Figure 6.1.3.(x+1)-1a).

- QUIC supports equivalent functionality as SCTP with respect to reliable stream and datagram transport, congestion control, mechanisms for head-of-line blocking, and IP multi-homing.

- QUIC includes TLS as security layer.

- QUIC is supported by commercial cloud solutions for containers, message brokers, etc.

Enhancement 6G PPI 2: Application-layer using HTTP

HTTP can be placed between TNL and application layer. HTTP logically belongs to the application layer (Figure 6.1.3.(x+1)-1b).

- HTTP provides separation between TNL and application layer.

- HTTP allows a PPI to operate without collocation of TNL endpoint and application-layer endpoint at the network node/function. In this case, the HTTP layer bridges between TNL- application-layer endpoints at a network node/function (as shown for SBI in Figure 6.1.3.y-3). This enables implementations where the network node/function supports multiple application-layer instances, e.g., for the purpose scalability and robustness.

- HTTP is supported by message brokers of typical cloud environments.

Figure 6.1.3.(x+1)-1: Enhancements of 6G PPI over 5G: a) TNL using QUIC/UDP, b) application layer including HTTP, c) combination of a) and b)

Enhancement PPI 3: Enhanced network-node/function discovery service

The AMF’s gNB-discovery service based on gNB-ID can be enhanced to a more general network-node/function service based on a larger number of identifiers.

- The network-node/function discovery service could be moved out of the AMF to reduce load on the AMF.

- The network-node/function discovery service could be used by other network nodes/functions than gNBs, e.g., core NFs supporting new services.

- Network-node/function discovery could be based on a more extended set of identifiers than gNB-IDs.

### 6.1.3.y	Characteristics of 5G service-based interfaces

Definition

The 5G SBA is defined in TS 29.500. The SBA defines service-based interfaces (SBI). TS 29.501 defines more details related to SBIs. 5G SBA is only defined for the C-plane.

Principal 5G SBI characteristics

The service-based interface represents how a set of services is provided by an individual network-node type, which is referred to as a service producer. These services can be used by multiple types of (authorized) network nodes, referred to as service consumers. It is possible to restrict the number of consumers to a single network node type by configuration. The signaling procedures are defined as API method calls which follow the client-server paradigm.

Figure 6.1.3.y-1 shows a schematic of the principal SBI characteristics.

Figure 6.1.3.y-1: Schematic of 5G SBI: The SBI comprises all API method calls for which the NF provides the server function. API method 2 of NF1’s SBI is used by both, NF2 and NF3.

Protocol stack of 5G SBI

5G SBIs uses TCP as transport protocol and HTTP on application layer. The application layer information is encoded via JSON. Figure 6.1.3.y-2 shows the 5G SBI protocol stack.

Figure 6.1.3.y-2: Protocol stack of 5G SBI.

Security protection of 5G SBI

According to TS 33.501, 5G SBIs can be secured via IPsec or TLS.

Collocation of TNL endpoint and application layer endpoint of 5G SBI

5G SBI does not require that TNL endpoint and application-layer endpoint at a network node/function are collocated. In case they are not collocated, the HTTP layer is used for bridging (Figure 6.1.3.y-3). This allows implementations where a logical network function uses multiple physical NF instances. The message broker at the HTTP bridge distributes the incoming messages over the various physical NF instances for load balancing purposes.

Since the application-layer endpoint may not be collocated with the TNL endpoint, it cannot derive TNL address information from this TNL endpoint. TNL-related information is therefore explicitly included in application layer signaling.

The application layer endpoint may also not be aware of the TNL address used by its own network node/function. 5G SBI therefore allows the application layer to send endpoint information in form of the network node’s/function’s FQDN instead of the network node’s/function’s TNL address. The peer can then resolve this FQDN to an TNL address via NRF or DNS.

Since TNL endpoint and application-layer endpoint may not be collocated, the application layer cannot rely on reliable data delivery provided by the TNL since messages may get lost between TNL endpoint and application layer endpoint.   Therefore, 5G SBI support message delivery confirmation on application layer (i.e., HTTP).

Figure 6.1.3.y-3: 5G SBI does not assumes that TNL- and app-layer endpoints are collocated at the network node/function. The red crosses indicates locations of potential packet loss.

Peer-node discovery for 5G SBI

5G SBIs support the following forms of peer-node/function discovery:

- Network node/function is OAM-configured with the peer’s TNL address.

- Network node/function is OAM-configured with the peer’s FQDN, which can be resolved to a TNL address via DNS.

- Network node/function discovery and TNL address discovery via NRF.

Peer-node authorization for 5G SBI

5G SBI can use OAM-based configuration of certificates for peer-node/function authorization. SBA also allows obtaining certificates from the NRF using the OAuth 2.0 framework.

5G SBI’s support of deployment scenarios

5G SBIs can support all traditional deployment scenarios that can be accommodated via customized implementations of network nodes/functions.

5G SBIs are supported by cloud solutions using containers, message brokers, and load balancers that expect TCP or QUIC/UDP on transport layer. Further, message brokers, that rely on the HTTP layer can handle 5G SBI messages.

5G SBI’s support of new services

5G presently only supports AIOT as a new service with the IOTF in the CN. 5G AIOT supports an SBI interface between AMF and AITOF.

Support for ISAC in 5G is still under discussion.

### 6.1.3.y+1	Enhancements for service-based interfaces in 6G

The following enhancements can be considered when using SBIs for 6G RAN:

Enhancement SBI 1: TNL using QUIC

TCP can be replaced with QUIC/UDP (Figure 6.1.3.y+1)-4a).

- QUIC supports the equivalent functionality as TCP such as reliable stream transport and congestion control. QUIC further supports datagram transport, mechanisms for head-of-line blocking, and IP multi-homing.

- QUIC includes TLS as security layer.

Enhancement SBI 2: Application-layer using ASN.1 encoding

The application layer can use ASN.1 or other encodings instead of JSON encoding (Figure 6.1.3.y+1)-4b)

- ASN.1 encoding is natively supported by HTTP.

- The support of other encodings by HTTP needs to be considered in the study.

Figure 6.1.3.y+1)-4: Enhancements of 6G SBI over 5G: a) TNL using QUIC/UDP, b) application layer using ASN.1 encoding, c) combination of a) and b)

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>