3GPP TSG-RAN WG3 Meeting #130	R3-258189

Dallas, US, 17 - 21 November 2025

Agenda item	: 10.2

Source	: Tejas Networks

Title	: Overall RAN architecture

Document for	: Discussion & Decision

# Introduction

In this contribution, we provide Tejas Network’s view point on overall RAN architecture.

# RAN Architecture

## Option 6

Given the experiences learned from 5G deployment and the complexity of Non-Standalone (NSA) deployment, we prefer standalone deployment (SA) for 6GR, i.e., 6G radio will connect to 6G Core Network. However, within the scope of SA deployment, we are open to both monolithic RAN architecture as well as dis-aggregated architecture options. One of the disaggregated architecture options is Option 2 which introduces split between PDCP and RLC (CU and DU are separated by the network interface F1). Another dis-aggregated option is split between MAC and PHY (Option 6 in Figure Figure 1).

Figure 1: Various architecture split options considered for NR

For RAN logical architecture for 6GR, we propose that Option 6 be considered. Option 6 split can be considered as fronthaul. This we understand will be undertaken as a study by ORAN as per the decision in RAN plenary. However, our stand for co-existence of Option 2 (split between PDCP and RLC) with Option 6 is neutral. In what follows, we discuss Option 6 while assuming Option 2 is retained. Thus, PDCP will be hosted in the Central Unit (CU), RLC and MAC layers will be hosted in Distributed Unit (DU) and PHY and RF together will be hosted in a new network function (NF) ``DY-PHY”. In terms of interfaces, we need to introduce a new network interface ``G1” (for want of name) between DU and DU-PHY. An advantage with Option 6 is that PHY layer design and implementation can evolve independent of the hardware platforms considered for MAC and RLC. In addition to the interface between MAC and PHY (scheduling, resource allocation, DCI, etc.)  one should also consider messages that help RRC configuration towards the PHY layer.

In case of NTN, Option 6 allows CU and DU to be hosted in an earth station while DU-PHY will be moved into the satellite platform. The user data that gets exchanged over the interface corresponding to Option 6 will then be MAC PDUs which do not place heavy bit rate demand on the transport network implementing the interface.

Further, following the guideline provided in the SID (FS_6G_Radio, [2]) we stress that Option 6 paves the way for better TN and NTN integration as both networks follow the same architecture option.

Figure 2: Option 6 for RAN logical architecture for 6GR.

Thus, we summarize our preference for RAN overall architecture in Figure 3.

Figure 3: RAN architecture options

Proposal 1: The overall RAN architecture should follow Option 6, which introduces split between MAC and PHY and helps TN and NTN integration.

## Option 2

The use of option 2 split in actual deployment has been mostly restricted to small cell deployments in 5G networks. Further, Option 2 split was also preferred by some operators for FR2 band and may also be considered for Non-Public Network (NPN) deployment in 5G networks. However, the deployment of Option 2 split network has been relatively low.

Proposal 2: If option 2 is considered for the split, the choice of option 2 shall be based on the deployment scenario.

# RAN Internal Functional Split

RAN Functional split between various NFs CU, DU, and DU-PHY can be discussed after architecture option is frozen.

# Conclusion

Proposal 1: The overall RAN architecture should follow Option 6, which introduces split between MAC and PHY, which helps TN and NTN integration.

Proposal 2: If option 2 is considered for the split, the choice of option 2 shall be based on the deployment scenario.

# References

[1] 3GPP TR 38.801, `` Study on New Radio Access Technology; Radio Access Architecture and Interfaces”

# [2] RP-252912, `` Revised SID: Study on 6G Radio”