3GPP TSG-RAN WG3 Meeting #130 															 R3-258190

Dallas, US, 17 – 21 November 2025

Title:	AI/ML Use Cases for 6G RAN

Agenda Item:	10.5.1

Source:	Tejas Networks Limited

Document for:	Discussion and decision

# 1	Introduction

The following has been discussed in RAN3#129bis meeting.

Identify use case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc. from RAN3#129bis:

The design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope.

The following use cases will be studied in RAN3:

- AI/ML based Network energy saving

- AI/ML based mobility optimization

Other use cases can be discussed in future meetings based on contributions.

- This paper discusses further AI/ML use cases in 6G.

# 2	Discussion

- In this paper, we will discuss the possibility of exploring the existing use cases from 5G and potential new AI/ML use cases to be considered for 6G RAN.

## 2.1	AI/ML-assisted dynamic resource allocation for Network Slicing

AI/ML-assisted network slicing was already introduced as a use case in 5G Release 19, where mechanisms for exchanging predicted slice resources and available capacity between gNBs were agreed upon. Building on this foundation, dynamic slice resource management offers a more intelligent and adaptive solution for optimizing network slicing in 6G architecture. It employs time series forecasting models to predict user traffic per slice over a future time window. Based on these predictions, slice resources are dynamically allocated while strictly adhering to service-level agreements (SLAs), ensuring that each slice receives the appropriate resources to meet its performance targets, i.e., when a slice requires additional resources, the system intelligently reallocates resources from less critical or low-traffic slices, ensuring optimal utilization across the network.

Fig 1. Dynamic resource adjustment: Slice 1 consumes resources from Slices 2 and 3 during traffic peaks

This use case also supports inter-node collaboration: if a 6G RAN Node B cannot fulfil the predicted resource demands for a slice, it proactively requests slice-specific available capacity and predicted slice resources from neighbouring 6G RAN Node Bs. Users are then seamlessly transferred to these nodes based on their reports, maintaining SLA compliance and service continuity. Furthermore, when a slice requires additional resources, the system intelligently reallocates resources from less critical or low-traffic slices, ensuring optimal utilization across the network. This distributed and adaptive strategy enhances scalability, supports mobility, and aligns with the vision of autonomous and intelligent RAN systems.

Fig 2. Inter-node Collaboration for Slice Resource Prediction Sharing

Proposal 1: RAN3 to consider the 5G AI/ML-assisted network slicing features as the baseline for the 6G AI/ML use case for network slicing.

## 2.2	Agentic AI

Agentic AI introduces autonomous, goal-driven agents into the RAN, enabling networks to act proactively based on high-level intents rather than static configurations. These agents can perceive network conditions, reason about objectives such as improving mobility performance or reducing energy consumption and execute actions without continuous human intervention. In a 6G context, Agentic AI can dynamically interpret operator intents (e.g., “maximize energy efficiency while maintaining QoE”) and translate them into coordinated actions across multiple RAN entities, such as gNB-CU/DU and neighbouring nodes. This capability supports distributed decision-making and collaboration among agents to optimize handovers, adjust beamforming, manage compute resources, and recover from anomalies. To ensure safety and interoperability, agentic actions should be traceable through identifiers and confidence indicators, with fallback profiles for deterministic behaviour. Information exchange among agents may require new signalling constructs over NG-RAN interfaces (e.g., NGAP, XnAP, F1AP, E1AP) to carry intent identifiers, action outcome reports, and optional metadata for auditability. By enabling autonomous, intent-based control loops, Agentic AI can significantly enhance resilience, efficiency, and adaptability in future RAN deployments.

Fig 3. Overview of 6G RAN AI Agent(s)

Proposal 2: RAN3 to study agentic AI as a new use case for 6G RAN.

## 2.3	Network for AI

In 6G, the RAN is expected to play a critical role in supporting AI workloads as first-class traffic, enabling AI-as-a-service and distributed inference across the network. Network for AI refers to the capability of the RAN to provide optimized connectivity and resource management for AI applications, which may have stringent requirements on latency, reliability, and compute affinity. There may be need for optimizing the requirement of one or more of the KPI simultaneously like delay, throughput synchronization etc. For example, AI-driven services such as real-time inference for autonomous systems or immersive applications may require deterministic low-latency paths and dynamic bandwidth allocation. To achieve this, the RAN can expose AI-specific traffic attributes (e.g., latency sensitivity, compute location preference) and coordinate with underlying compute resources to ensure efficient placement of AI tasks. Information exchange between RAN nodes and the core network may be needed to convey AI slice attributes, inference placement hints, and compute load reports, enabling intelligent scheduling and resource orchestration. These capabilities should remain implementation-agnostic, focusing on signalling hooks and metadata rather than standardizing AI models or algorithms. By supporting AI workloads natively, Network for AI ensures that future RAN deployments can deliver predictable performance for AI-driven services while maintaining interoperability and scalability. There is need to identify use cases for the AI-driven services and identify the joint KPI requirements for each of this use case.

Fig 3. Overview of 6G Network for AI

Proposal 3: RAN3 to study network for AI as a new use case for 6G RAN.

Proposal 4: RAN3 to identify all the use cases which requires meeting multiple KPIs simultaneously. The joint KPIs per use case need to be specified.

# 3	Conclusion

This document proposes:

Proposal 1: RAN3 to consider the 5G AI/ML-assisted network slicing features as the baseline for the 6G AI/ML use case for network slicing.

Proposal 2: RAN3 to study agentic AI as a new use case for 6G RAN.

Proposal 3: RAN3 to study network for AI as a new use case for 6G RAN.

Proposal 4: RAN3 to identify all the use cases which requires meeting multiple KPIs simultaneously. The joint KPIs per use case need to be specified.

# 4	Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator), RAN#108

- 3GPP TS 38.300, NR; NR and NG-RAN Overall description; Stage-2, Rel-18, V18.7.0 (2025-09)

- R3-256501, RAN3 Chair Notes