3GPP TSG-RAN WG3 Meeting #130	R3-258191

Dallas, US, November 17-21, 2025

Agenda item:	12.2.2

Source:	Tejas Networks

Title:	Discussion on AI/ML for Intra CU LTM

Document for:	Discussion

# 1	Introduction

In this document, we will discuss open issues from the RAN3#129bis meeting.

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM.

Further discuss whether to study addition of beam prediction in UE Trajectory prediction.

RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

# 2	Discussion

2.1       L3 Measurement Report based Intra-CU LTM

The first step is to finalize the appropriate AI/ML use cases aimed to enhance Intra-CU LTM. Once these use cases are defined, we can determine the types of measurements required as inputs and performance feedback to optimize model performance and prediction accuracy. Incorporating both Layer 1 (L1) and Layer 3 (L3) measurement reports as model inputs can provide deeper insights, leading to more accurate predictions. Although feedback to the model typically comes from higher-layer reports making L3 measurements sufficient in many cases, but if we are proposing this as a foundational principle before finalizing the use cases, it is advisable to consider both L1 and L3 measurement reports as inputs and feedback sources for the models.

Proposal 1: Consider both L1 and L3 measurement reports as inputs and feedback sources for the models.

2.2   Beam prediction in UE Trajectory prediction

It was agreed in the previous meeting to study candidate beams along with candidate cells for AI/ML-based Intra-CU LTM handover (HO) preparation. It was also agreed to introduce AI/ML-assisted candidate cell selection for LTM handover preparation. The importance of predicted UE trajectory for AI/ML-assisted candidate cell selection is well understood, as it helps shortlist the most probable cells the UE may visit during its path.

Since we have agreed to consider candidate beams along with candidate cells, having beam-level UE trajectory information will be highly beneficial for selecting candidate beams in addition to candidate cells.

Proposal 2: Introduce beam-level UE trajectory prediction, which will significantly improve the accuracy of selecting candidate beams along with candidate cells

2.3 Inference in DU

Key Advantages of enabling model inferencing at the DU

A major benefit of introducing model inference capabilities at the Distributed Unit (DU) is the ability to deploy models such as UE trajectory prediction models, that are trained at centralized nodes like the gNB Central Unit (CU) or OAM systems. Once deployed, the DU can utilize the predicted UE trajectory to proactively identify and select the most suitable target cell for issuing cell switch commands. This enables faster and more intelligent handover decisions, significantly enhancing mobility management, reducing latency, and improving overall network performance.

An additional benefit of enabling model inference at the Distributed Unit (DU) is its ability to generate predictions based on data that is highly localized and specific to the DU such as radio resource usage or other DU-dependent metrics. These localized predictions can be shared with AI/ML models residing in the Central Unit (CU), such as those used for assisted cell selection during the LTM handover preparation stage. By incorporating DU-specific insights, these centralized models can make more accurate and context-aware decisions, ultimately improving the selection of the most suitable target cells and enhancing overall network performance.

Proposal 3: RAN3 should consider enabling model inference in gNB DU.

# 3	Conclusions

Proposal 1: Consider both L1 and L3 measurement reports as inputs and feedback sources for the models.

Proposal 2: Introduce beam-level UE trajectory prediction, which will significantly improve the accuracy of selecting candidate beams along with candidate cells.

Proposal 3: RAN3 should consider enabling model inference in gNB DU.

# 4	References

[1]  RAN3#129bis chair note

[2] R3-257239