3GPP TSG-RAN WG3 Meeting #130	R3-258208

# Dallas, US, 17 – 21 November 2025

Agenda item:	13.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to pCR) Discussion on RAN-CN procedures and signaling for ISAC

Document for:	Discussion and Decision

# 1	Introduction

The ISAC SID ([1]) agrees following RAN3 related objectives:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution discusses the RAN-CN procedures and signaling for ISAC.

# 2	Functions of the RAN-CN interface for ISAC

Last RAN3 meeting agreed the high level procedure for sensing.

## 8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1.	The SF sends sensing request to the gNB.

2.	The gNB sends sensing response to the SF.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

Current call flow does not have the detailed information for each step. This contribution provides the detail for required functionalities/procedures. The RAN-CN interface should support following functionalities for ISAC:

- Sensing Request

This function allows the CN to start a sensing operation in the RAN. This function is supported by a CN initiated Class-1 procedure.

- Sensing Modification

This function allows the CN to modify an ongoing sensing operation in the RAN.  This function is supported by a CN initiated Class-1 procedure.

- SF-initiated Stop Sensing

This function allows the CN to stop an ongoing sensing operation in the RAN. This function is supported by a CN initiated Class-2 procedure.

The Stop function is usually used to cancel/stop an ongoing sensing operation.

- gNB-initiated Stop Sensing

This function allows the gNB to stop an ongoing sensing operation in the RAN.  This function is supported by a gNB initiated Class-1 procedure.

In some scenarios, the gNB may not have resources to continue the sensing operation, so the gNB requests to stop an existing sensing.

Proposal 1: agree the following functionalities for RAN-CN interface.

- Start Sensing

- Modify Sensing

- SF-initiated Stop Sensing

- gNB-initiated Stop Sensing

# 3	RAN-CN procedures for ISAC

To support the above ISAC functionalities over NGAP, the following procedures are required.

## 3.1	Start Sensing procedure

The Start Sensing procedure is to start a sensing operation in the gNB. The SF initiates the procedure by sending the START SENSING REQUEST message to the gNB, including following parameters:

a) Sensing ID, which is used to uniquely identify the session in the gNB.

b) Sensing service type, e.g. detection of an object, tracking of an object, etc.

c)	Sensing target area; indicates the geographical area where the sensing service needs to be performed and may contain, TAI, cell ID, Universal Geographical Area Description (GAD) as specified in TS 23.032 [11], etc.

d)	Sensing time parameters; indicates the time period over which the sensing is to be performed e.g. start time and end time and/or total duration.

e)	Sensing contextual information:

-	target object type (e.g. drone, car, human, etc.): indicates the type of objects and its corresponding properties (e.g. object size, Radar Cross Section) that shall be detected in the specific sensing service request,

-	target object mobility profile (e.g. static, low mobility, etc.): indicates whether fixed objects or objects with a specific mobility profile shall be detected in the specific sensing service request,

f)	Requested Sensing QoS parameters (SQoS); The QoS parameters include positioning accuracy, velocity accuracy and confidence level for the detected object(s), sensing range resolution for object detection, maximum latency to obtain the sensing result, missed detection probability, false alarm probability etc (see also TS 22.137 [2]).

g)	Type of event reporting, e.g. one-time report, periodic report, or event-triggered report, etc.

## 3.2	Modify Sensing procedure

The Modify Sensing procedure is to modify an existing sensing operation in the gNB. The SF initiates the procedure by sending the MODIFY SENSING REQUEST message including the similar parameters as the START SENSING REQUEST message.

## 3.3	SF-initiated Stop Sensing procedure

The SF-initiated Stop Sensing procedure is to stop an existing sensing operation in the gNB. The SF initiates the procedure by sending the STOP SENSING REQUEST message including the Sensing ID.

## 3.4	gNB-initiated Stop Sensing procedure

The gNB-initiated Stop Sensing procedure is to request the stop an existing sensing operation in the SF. The gNB initiates the procedure by sending the STOP SENSING REQUIRED message including the Sensing ID.

Proposal 2: agree following RAN-CN procedures for ISAC.

- Start Sensing procedure

- Modify Sensing procedure

- SF-initiated Stop Sensing procedure

- gNB-initiated Stop Sensing procedure

# 4	Update to the sensing procedure

Based on the above sensing functionalities/procedures, it is proposed to update the call flow with more detailed information. An example is shown below:

## 8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1.	The SF sends sensing request to the gNB. The Sensing Request includes the sensing related configuration parameters, e.g. an ID to uniquely identify the session, sensing area, Quality of Sensing, etc. The Sensing Request also includes the related information for how the gNB sends the Sensing Report, e.g. the tunnel information in the SF to receive the Sensing Report from the gNB.

2.	If the gNB accepts the Sensing Request, the gNB sends sensing response to the SF. The gNB reserve the radio resource to perform the Sensing. The gNB also uses the received tunnel information to establish a tunnel with the SF.

In case the gNB cannot accept the Sensing Request, e.g. no enough resource to support the sensing, the gNB can reject the Sensing Request by sending a Sensing Failure, with appropriate cause value.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

## 8.x1	Modify Sensing Reporting

This call flow describes the SF modify an existing sensing operation.

Figure 8.x-1: Message flow for sensing reporting

- The gNB already have an ongoing sensing operation.

- The SF sends Sensing Modify Request to the gNB to modify the ongoing sensing operation, e.g. the sensing configuration. The Sensing Modify Request includes the sensing related configuration parameters, which can be similar to the parameters included in the Sensing Request.

2.	If the gNB accepts the Sensing Modify Request, the gNB sends Sensing Modify Response to the SF. The gNB modify the radio resource to perform the Sensing.

In case the gNB cannot accept the Sensing Modify Request, e.g. no enough resource to support the sensing, the gNB can reject the Sensing Modify Request by sending a Sensing Modify Failure, with appropriate cause value.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

## 8.x2	SF-initiated Stop Sensing

This call flow describes the SF request the gNB to stop an existing sensing operation.

Figure 8.x-1: Message flow for sensing reporting

- The gNB already have an ongoing sensing operation.

- The SF sends Sensing Stop Request to the gNB to stop an ongoing sensing operation. The Sensing Stop Request includes the sensing session ID to identify the sensing session in the gNB.

## 8.x3	gNB-initiated Stop Sensing

This call flow describes the gNB request the SF to stop an existing sensing operation.

Figure 8.x-1: Message flow for sensing reporting

- The gNB already have an ongoing sensing operation.

- The gNB sends Sensing Stop Required to the SF requesting to stop an ongoing sensing operation. The Sensing Stop Required includes the sensing session ID to identify the sensing session in the SF.

2.	The SF confirms the sensing stop by sending the Sensing Stop Confirm to the gNB. The gNB stops the ongoing sensing session and release the related resource.

Proposal 3: agree the above procedures in the Annex section.

# 5	Conclusion

In this contribution, we discussed the RAN-CN signaling for ISAC. Our proposals are:

Proposal 1: agree the following functionalities for RAN-CN interface.

- Start Sensing

- Modify Sensing

- SF-initiated Stop Sensing

- gNB-initiated Stop Sensing

Proposal 2: agree following RAN-CN procedures for ISAC.

- Start Sensing procedure

- Modify Sensing procedure

- SF-initiated Stop Sensing procedure

- gNB-initiated Stop Sensing procedure

Proposal 3: agree the above procedures in the Annex section.

# References

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- TR 23.700-14

- R3-257332, (pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR

# Annex – TP for pCR

# 8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

## 8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1.	The SF sends sensing request to the gNB. The Sensing Request includes the sensing related configuration parameters, e.g. an ID to uniquely identify the session, sensing area, Quality of Sensing, etc s. The Sensing Request also includes the related information for how the gNB sends the Sensing Report, e.g. the tunnel information in the SF to receive the Sensing Report from the gNB.

2.	If the gNB accepts the Sensing Request, the gNB sends sensing response to the SF. The gNB reserves the radio resource to perform the Sensing. The gNB also uses the received tunnel information to establish a tunnel with the SF.

In case the gNB cannot accept the Sensing Request, e.g. no enough resource to support the sensing, the gNB can reject the Sensing Request by sending a Sensing Failure, with appropriate cause value.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

## 8.x1	Modify Sensing Reporting

This call flow describes the SF modify an existing sensing operation.

Figure 8.x-1: Message flow for sensing reporting

The gNB already have an ongoing sensing operation.

1. 	The SF sends Sensing Modify Request to the gNB to modify the ongoing sensing operation, e.g. the sensing configuration. The Sensing Modify Request includes the sensing related configuration parameters, which can be similar to the parameters included in the Sensing Request.

2.	If the gNB accepts the Sensing Modify Request, the gNB sends Sensing Modify Response to the SF. The gNB modify the radio resource to perform the Sensing.

In case the gNB cannot accept the Sensing Modify Request, e.g. no enough resource to support the sensing, the gNB can reject the Sensing Modify Request by sending a Sensing Modify Failure, with appropriate cause value.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

## 8.x2	SF-initiated Stop Sensing

This call flow describes the SF request the gNB to stop an existing sensing operation.

Figure 8.x-1: Message flow for sensing reporting

1. The gNB already have an ongoing sensing operation.

2. The SF sends Sensing Stop Request to the gNB to stop an ongoing sensing operation. The Sensing Stop Request includes the sensing session ID to identify the sensing session in the gNB.

## 8.x3	gNB-initiated Stop Sensing

This call flow describes the gNB request the SF to stop an existing sensing operation.

Figure 8.x-1: Message flow for sensing reporting

The gNB already have an ongoing sensing operation.

1. The gNB sends Sensing Stop Required to the SF requesting to stop an ongoing sensing operation. The Sensing Stop Required includes the sensing session ID to identify the sensing session in the SF.

2. The SF confirms the sensing stop by sending the Sensing Stop Confirm to the gNB. The gNB stops the ongoing sensing session and releases the related resource.