3GPP TSG-RAN WG3 Meeting #130	R3-258232

Dallas, US, 17th – 21st November, 2025

Title:	Requirements for 6G RAN architecture design

Source:	FiberCop, CEWiT, Fujitsu, Jio Platform, KT Corp., Qualcomm Inc., Rakuten, Teja Networks, TIM-Telecom Italia, T-Mobile USA, Verizon Wireless

Agenda item:	10.2.1

Document Type:	Discussion and Decision

# 1	Introduction

In RAN#108 meeting [1], the 6G SID was approved addressing the following objectives on RAN architecture impacting the work of RAN3.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

TSG#112 (June/2026): …. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026.

In RAN#109 meeting [2], the following requirements have been agreed for the 6G Radio Access Technologies.

## 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

At the last RAN3#129 bis meeting, 6G RAN discussion started and only some very high-level principle and deployment scenarios were discussed, leading to the following agreement.

## 5.1 General Principles All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles. The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS] ## 5.2 Deployment Scenarios Editor’s note: This section may be used to describe the details/solutions related to deployment scenarios as per 38.914. The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914. - FFS on the implications of this requirement on 6G RAN architecture. - FFS whether all deployment scenarios of TR 38.914 can be supported.

During the discussion at last meeting, it was also pointed out that it is necessary, first and foremost, to identify the requirements that the 6G RAN architecture must support, based on concrete issues and use cases. In this context, the present contribution elaborates some use cases, taking into account the ongoing work by SA1 in its 6G Study Item [3], with the aim of deriving functional and architectural requirements to be considered in the RAN architecture design.

The analysis takes also into account the ongoing activities within the research project ORIGAMI [4]. The ORIGAMI project brings together leading industry and academic partners across the European mobile telecommunications ecosystem and is funded by the European Commission under the 6G Smart Networks and Services Joint Undertaking (SNS JU). Running from 2024 to 2026, ORIGAMI focuses on advancing architectural models for next-generation mobile networks and addressing key barriers to the deployment of successful 6G technologies. Specifically, ORIGAMI aims to study 6G architectural innovations that enables use cases of Network Intelligence (NI) designed to enhance network performance, security, and flexibility. Beyond technological advances, ORIGAMI is committed to establishing global unified standards, supporting the green transition, improving affordability and accessibility, and fostering the development of groundbreaking applications to create strong ecosystem, novel business models for operators to generate new revenue opportunities and allows operator networks to evolve into future generation with advanced capabilities.

# 2	Discussion

## General characteristics of 6G use cases

The evolution towards 6G is expected to enable a new generation of advanced services while ensuring more efficient, flexible, and intelligent network management. The 6G system shall support higher levels of adaptability, scalability, and dynamic resource allocation across diverse deployment scenarios, allowing networks to respond in real time to varying service and performance requirements.

At the same time, 6G must contribute to reducing both OPEX and CAPEX, improving energy efficiency, and promoting sustainable network evolution. Overall, 6G is envisioned to provide a unified, efficient, and intelligent framework capable of supporting future communication needs across industries and society.

As described in TR 22.870 by SA1 [3], the next generation system is expected to provide enhanced flexibility, scalability, and dynamicity to meet diverse service requirements and deployment scenarios.

The following definitions are proposed, highlighting their cross-relations as illustrated in Figure 1:

- Flexibility allows the system to dynamically adjust its configuration to meet diverse demands.

It refers to the ability of the 6G system to adapt its network behavior and resource allocation dynamically to the specific characteristics of each component within a service or application (e.g., video, voice, sensor data), enabling fine-grained optimization and policy enforcement.

- Scalability ensures efficient expansion or contraction of capabilities.

It refers to the ability of the 6G system to efficiently scale network resources and functionalities up or down in response to varying service demands, user densities, and application requirements, while maintaining performance and service quality.

- Dynamicity allows the system to adapt in time.

It refers to the capability of the 6G system to respond in near real-time to changes in service usage, traffic patterns, or network conditions by leveraging AI/ML for proactive service assurance, dynamic handover, and resource reallocation.

Figure 1: Semantic relationship between Flexibility, Scalability and Dynamicity

These principles collectively can be used as a guideline for the architectural and functional pillars expected from 6G RAN architecture, discussed in the following sections.

Observation 1: To support new 6G services, the design of RAN architecture and functions needs to ensure flexibility, scalability and dynamicity.

## Example of use cases and related RAN requirements

This section identifies some relevant use cases defined by SA1 [3] and derives corresponding RAN architectural requirements. The objective is to map service-level needs to RAN functionalities, highlighting architectural impacts and enablers such as flexibility, scalability, and dynamic behaviour. The analysis aims to ensure alignment between SA1 use cases and RAN3’s ongoing work on the evolution of the 6G RAN architecture design.

### Network Slicing

This use case is addressed in §5.7.5 of SA1 TR [3].

Use Case Description

The dynamic creation and management of network slices is essential to support diverse verticals and service demands. As outlined in relevant §5.7.5, the 6G system should enable automated, scalable, and flexible slice orchestration, with real-time responsiveness to traffic variations, QoS requirements, and service priorities. This includes support for multi-domain and multi-operator slicing, with fine-grained control at the RAN level.

Service Requirement

From [PR 5.7.5.2-1] – The 6G system should support potential enhancement of network slicing, including fast slice creation, efficient scaling, and improved slice access mechanisms.

Practical service example

Remote XR-assisted surgery – A hospital dynamically creates a dedicated RAN slice for a remote surgical procedure using XR headsets and haptic feedback. The slice guarantees ultra-low latency, synchronized multimodal streams, and prioritized traffic handling. Once the procedure is completed, the slice is automatically terminated.

RAN requirement

RAN-REQ-1 – The RAN shall ensure flexible and dynamic support for RAN Slice creation, configuration and management to meet diverse service requirements and vertical needs.

Drivers enabled

Flexibility and Dynamicity: the support of managed creation/configuration of slices requires flexible configurations (Flexibility) and dynamic management (Dynamicity).

Requirement Description

The 6G RAN must be capable of supporting the full lifecycle of RAN slices in a dynamic and service-aware manner. This means that slice creation, modification, and termination should be performed in real time, based on service-specific triggers or network conditions. The RAN should be able to allocate resources to each slice with fine granularity, ensuring that QoS parameters such as latency, throughput, and reliability are met according to the slice’s profile. Furthermore, the control and user plane functions should be slice-aware, allowing for differentiated mobility management, scheduling, and traffic routing. The RAN architecture should also support integration with slice orchestrators, enabling coordinated management across RAN, core, and edge domains. Finally, the radio interface should be capable of adapting its configuration, such as beamforming or TRP selection, based on the slice context, ensuring optimal performance for each service.

Reason Why

To support differentiated services and vertical-specific requirements, the RAN must instantiate and manage slices with distinct QoS profiles and resource isolation. Dynamic slice management allows for optimized resource utilization, reduced latency, and improved resilience. Integration with intent-based orchestration and AI-driven control further enhances adaptability and operational efficiency.

Current RAN architecture presents some limitations: lack of native slice-awareness and dynamic orchestration capabilities, making real-time slice lifecycle management complex and vendor-dependent. This limitation hinders scalable and automated slice deployment across multi-domain environments.

Architectural/Functional impact

- General: slice-aware scheduling and QoS enforcement

- Control Plane: slice-specific signaling and mobility control

- User Plane: traffic separation and routing per slice

- Radio interface: slice-based resource allocation and beamforming

- All entities: integration with slice orchestrators

### Use case on fast network provisioning to improve resilience

This use case is addressed in §5.6.2 of SA1 TR [3].

Use Case Description

Rapid provisioning of network services in emergency scenarios is a key capability of 6G to ensure operational continuity. As described in relevant §5.6.2, the system should support on-demand deployment of RAN and edge computing resources, with localized control and tailored functionalities, even in the absence of backhaul. This includes temporary slice activation and distributed CP/UP functions.

Service Requirement

From [PR 5.6.2.6-1] – The 6G system shall enable operators to provision network services on-demand in response to urgent events, with local control and specific functionalities.

Practical service example

Temporary emergency network in earthquake zone – Following an earthquake, an operator deploys mobile TRPs and edge cloud resources to provide voice and data connectivity to first responders and residents. The network operates autonomously, with dedicated slices for emergency services and configurable traffic priorities.

RAN requirement

RAN-REQ-2 – The RAN shall ensure automatic RAN resource scaling based on user density and traffic load, while considering capacity thresholds and constraints, e.g. on energy efficiency.

Drivers enabled

Dynamicity and Scalability: automatic response to load changes (Dynamicity) and ability to scale resources (Scalability).

Requirement Description

To ensure resilience in scenarios like an emergency, the 6G RAN must support the rapid and autonomous provisioning of network resources in response to sudden changes in user density, traffic load, or environmental conditions. This includes the ability to scale radio and compute resources elastically, without requiring manual intervention. The RAN should be able to deploy control and user plane functions locally, enabling autonomous operation even when connectivity to the central core is disrupted. Additionally, the system should support real-time activation and deactivation of TRPs and radio components, allowing the network footprint to be adapted to the needs of the moment. The orchestration of these resources should be intent-driven, meaning that high-level service goals (e.g., restore voice service in a disaster zone) can be translated into concrete provisioning actions. The RAN must also support fallback mechanisms and recovery procedures, ensuring that services can be maintained or restored quickly and efficiently.

Reason Why

In the event of natural disasters or infrastructure failures, the network must quickly restore essential services. Elastic and localized provisioning enables reduced downtime, enhanced resilience, and access to critical services (e.g., voice, data, alerts). AI-driven orchestration accelerates recovery and optimizes resource allocation.

Current RAN architecture presents some limitations: it relies heavily on static configurations and centralized control, limiting their ability to elastically scale resources in real time. This challenge highlights the need for distributed intent-based orchestration and on-demand (autonomous) CP/UP deployment.

Architectural/Functional impact

- General: dynamic carrier aggregation and MIMO adaptation

- Radio interface: real-time activation/deactivation of radio components

- Edge Cloud: elastic deployment of different Control Plane and User Plane functionalities

- All entities: intent-based orchestration and telemetry

### Use case on 6G localized network for vertical

This use case is addressed in §11.10 of SA1 TR [3].

Use Case Description

This use case addresses the deployment of localized 6G networks tailored to specific verticals, such as industrial automation, smart campuses, or healthcare facilities. These networks operate autonomously and handle traffic locally, enabling ultra-reliable, low-latency communication and enhanced privacy. As described in relevant §11.10, the 6G system should support flexible deployment models, including standalone non-public networks and hybrid configurations with centralized or distributed control.

Service Requirement

From [PR 11.10.6] – The 6G system shall support localized network deployment with autonomous control and traffic management.

Practical service example

Smart factory with autonomous robotics – A manufacturing plant deploys a localized 6G network to support autonomous mobile robots and real-time process control. User traffic is processed entirely within the factory premises, while control functions are distributed between local and central nodes. The network operates independently, ensuring uninterrupted service even during external connectivity disruptions.

RAN requirement

RAN-REQ-3 – The RAN shall ensure flexible and granular split between User Plane and Control Plane.

Drivers enabled

Flexibility: adaptive configuration with fine-grained control for specific optimization

Requirement Description

To support localized vertical deployments, the 6G RAN must enable a flexible and granular separation between control plane and user plane functionalities. This separation allows user traffic to be processed locally, minimizing latency and ensuring privacy, while control functions can be centralized or distributed depending on the deployment model. The architecture should support dynamic placement and scaling of CP/UP entities, with seamless integration into the overall network. Furthermore, the RAN must provide mechanisms for synchronization, policy consistency, and secure interfacing between local and central components, ensuring coherent operation across domains.

Reason Why

Vertical industries often require dedicated network environments with strict performance, security, and operational requirements. By enabling localized traffic handling and autonomous control, the RAN can meet these needs while reducing dependency on centralized infrastructure. This approach also supports regulatory compliance, data sovereignty, and service continuity in isolated or mission-critical environments.

Current RAN architecture presents some limitations: limited scalability and deployment flexibility. This is because instantiating CP or UP functions typically requires bringing up entire protocol stacks, which introduces complexity and overhead. Exposing specific CP/UP functionalities as modular services would enable differentiated placement and scaling based on deployment needs, hence improving operational agility to support localized or vertical scenarios.

Architectural/Functional impact

- User Plane: local breakout and edge routing

- Control Plane: centralized/distributed control logic

- Radio interface: flexible interfacing with CU-UP/CU-CP

- All entities: synchronization and policy consistency

### Use case on AI agent for network performance assurance

This use case is addressed in §6.43 of SA1 TR [3].

Use Case Description

This use case introduces AI agents embedded within the RAN to monitor, analyze, and optimize network performance in real time. As outlined in relevant §6.43, these agents leverage machine learning models to predict congestion, detect anomalies, and proactively adjust network parameters. The goal is to enhance service quality and operational efficiency through intelligent automation.

Service Requirement

From [PR 6.43.6] – The 6G system shall support AI agents for proactive performance assurance and dynamic network optimization.

Practical service example

AI-driven optimization in a dense urban area – In a busy city district, AI agents embedded in the RAN monitor traffic patterns and user mobility. When congestion is predicted, the agents proactively adjust scheduling, beam pattern of group of adjoining cells and handover parameters, ensuring smooth service delivery. The system continuously learns from real-time data, improving its performance over time.

RAN requirement

RAN-REQ-4 – The RAN shall ensure real-time traffic and resource management via AI/ML.

Drivers enabled

Dynamicity: AI/ML-based real-time management.

Requirement Description

The 6G RAN must support the integration of AI/ML mechanisms across its functional entities to enable real-time traffic and resource management. This includes the ability to collect and expose relevant KPIs, apply predictive models for congestion and mobility, and execute optimization actions such as dynamic scheduling, handover decisions, and beam adjustments. The architecture should allow for distributed AI processing, with coordination between edge and central agents. Additionally, the RAN must support feedback loops and continuous learning, enabling the system to evolve and improve over time.

Reason Why

Traditional network management relies on static configurations and reactive procedures, which are insufficient in dynamic environments with diverse service demands. AI agents enable proactive and adaptive control, improving responsiveness and reducing manual intervention. This is particularly valuable in scenarios with fluctuating traffic patterns, mobility challenges, or complex service requirements.

Current RAN architecture presents some limitations: lack standardized mechanisms to expose AI/ML functionalities as modular services and to orchestrate their execution across heterogeneous compute platforms. This limits the ability to deploy distributed intelligence and real-time feedback loops, as AI/ML components are tightly coupled to specific hardware and not abstracted for dynamic allocation with poor scalable and efficient integration of network intelligence.

Architectural/Functional impact

- General: exposure of KPIs and AI-driven scheduling

- Control Plane: AI-based mobility and resource control

- Radio interface: execution of AI-driven radio adjustments

### Use case on network sharing on radio access network with sensing capability

This use case is addressed in §5.7.10 of SA1 TR [3].

Use Case Description

This use case explores the possibility of sharing RAN infrastructure among multiple operators, with integrated sensing capabilities. As described in relevant §5.7.10, the 6G system should support federated access to sensing-enabled radio resources, allowing operators to extend coverage, reduce deployment costs, and offer differentiated services. The sensing functionality may include environmental awareness, object detection, or mobility tracking, and must be configurable per operator.

Service Requirement

From [PR 5.7.10.6-1] – Subject to regulatory requirements or operator policy, the 6G system shall support sharing of sensing-capable RAN among operators.

Practical service example

Shared industrial RAN with sensing for logistics – Two operators share a 6G RAN deployed in a large logistics hub. The infrastructure includes sensing modules that track vehicle movement and environmental conditions. Each operator configures its sensing functions independently to support different clients (one for fleet management, the other for warehouse automation) while maintaining strict data isolation.

RAN requirement

RAN-REQ-5 – The RAN shall enable secure and scalable sharing of sensing-capable RAN resources in multi-operator and multi-tenant environments, supporting federated control, independent configuration, and dynamic orchestration of shared resources.

Drivers enabled

Scalability and Flexibility: multi-tenant and multi-operator environment requires scalable RAN structure (Scalability) and ability to adapt to sharing/federation (Flexibility)

Requirement Description

To enable multi-operator sharing of sensing-capable RAN infrastructure, the 6G RAN must support federated access models with strong isolation and policy enforcement. Each operator should be able to configure sensing parameters independently, ensuring that data collection and interpretation align with their service goals and regulatory constraints. The RAN must provide mechanisms for secure orchestration, data segmentation, and traffic routing based on operator-specific policies. Additionally, sensing modules should be integrated into the RAN architecture in a modular and scalable way, allowing for dynamic activation and deactivation based on demand or context.

Reason Why

In rural, industrial, or high-density urban areas, deploying dedicated infrastructure for each operator is costly and inefficient. Sharing RAN resources with sensing capabilities allows operators to collaborate while maintaining service isolation and policy control. This approach supports cost-effective expansion, enables new services such as smart logistics or environmental monitoring, and promotes sustainability through reduced energy and hardware footprint.

Current RAN architecture presents some limitations: lack federated orchestration frameworks and secure multi-tenant interfaces, making sensing-enabled RAN sharing difficult to implement. These limitations highlight the need for dynamic trustless cooperation and policy-based resource segmentation.

Architectural/Functional impact

- General: multi-tenant access and data isolation

- Sensing Modules: operator-specific configuration

- User Plane: policy-based traffic routing

- All entities: secure orchestration and data sharing

### Use case on multiple application media synchronization

This use case is addressed in §9.7 of SA1 TR [3].

Use Case Description

This use case focuses on the synchronized delivery of multiple media streams, such as audio, video, and haptic feedback, across devices and modalities, particularly for immersive applications. As outlined in relevant §9.7, the 6G system should support precise timing alignment and coordinated transmission of independent traffic flows, enabling coherent user experiences in XR, gaming, or collaborative environments.

Service Requirement

From [PR 9.7.2] – Subject to operator policy, the 6G system including IMS shall support the synchronization of independent traffic flows of one or more applications, to be delivered to more than one device.

Practical service example

Immersive concert experience with synchronized media – A user participates in a live virtual concert using XR glasses and haptic gloves. The RAN ensures that audio, video, and tactile feedback are delivered in perfect sync, creating a realistic and immersive experience. The system dynamically adjusts transmission parameters to maintain synchronization despite network fluctuations.

RAN requirement

RAN-REQ-6 – The RAN shall ensure multi-modal traffic synchronization for immersive services.

Drivers enabled

Flexibility and Dynamicity: synchronization of multimodal flows requires network adaptation (Flexibility) and fast real-time response (Dynamicity).

Requirement Description

To support multi-modal synchronization, the 6G RAN must be capable of coordinating the transmission of heterogeneous media streams with precise timing constraints. This includes timestamping, buffering, and alignment mechanisms at the user plane level, as well as coordinated scheduling and prioritization at the radio interface. The RAN should also support integration with media-aware orchestration systems that can adapt transmission strategies based on application context and user feedback. Furthermore, the architecture must be able to handle synchronization across multiple devices, ensuring consistent experiences in collaborative or multi-user scenarios.

Reason Why

Immersive applications rely on the seamless integration of different sensory inputs. Any misalignment between audio, video, or tactile feedback can degrade the user experience, cause discomfort, or reduce the effectiveness of the service. The RAN must therefore ensure strict timing control and synchronization across modalities, even under varying network conditions, device capabilities, and service contexts.

Current RAN architecture presents some limitations: lack media-aware orchestration and synchronization mechanisms across heterogeneous flows and devices. This results in inconsistent user experiences and latency mismatches, whilst coordinated scheduling and stream alignment should be needed.

Architectural/Functional impact

- General: coordinated scheduling of media flows

- User Plane: timestamping, buffering, stream alignment

- Radio interface: synchronized transmission across modalities

- All entities: integration with media-aware orchestration

### Use case on Enhanced Network Service Awareness

This use case is addressed in §5.9.8 of SA1 TR [3].

Use Case Description

The 6G system shall support the ability to receive service characteristics information from authorized third-party service providers for each traffic flow component of an application. This enables the network to dynamically adapt its behaviour and resource allocation to meet specific service requirements, improving efficiency and user experience.

Service Requirement

From [PR-5.9.8.2-2] – Based on operator policy, the 6G network shall support mechanisms to dynamically adjust and optimize network resources based on the service characteristics, including their predicted changes, provided by the service or application.

Practical service example

Cloud gaming with real-time network adaptation – A multiplayer cloud gaming service provides the network with detailed information about video, audio, and user input flows, specifying ultra-low latency, high reliability, and synchronization requirements. The 6G network, leveraging this input, dynamically adjusts radio scheduling, traffic prioritization, and beamforming configuration to ensure a smooth and responsive gaming experience, even under varying load or user mobility.

RAN requirement

RAN-REQ-7 – The RAN shall support service characteristics awareness and dynamic adaptation of radio and network resources to meet specific application and service requirements.

Drivers enabled

Flexibility, Scalability and Dynamicity: the ability to adapt network behaviour based on service-specific input requires flexible configuration mechanisms (Flexibility), real-time responsiveness to changing service demands (Dynamicity), and scalable resource management to support multiple concurrent flows (Scalability).

Requirement Description

The 6G RAN shall be capable of receiving and interpreting service characteristics (e.g., latency, jitter, throughput, loss sensitivity) for each application flow. It shall adapt radio configuration (e.g., scheduling, beamforming, prioritization) and CP/UP functions to meet these requirements. Additionally, it shall support feedback mechanisms and continuous learning to improve operational efficiency.

Reason Why

The increasing diversity and complexity of services and applications in 6G (such as immersive media, cloud gaming, remote control, and AI-driven services) require a network that can intelligently and proactively adapt to varying performance demands. These services often consist of multiple traffic components (e.g., video, audio, control signals) with distinct QoS requirements. Without service-level awareness, the RAN may treat all flows uniformly, leading to inefficient resource allocation, degraded user experience, and inability to meet SLAs. By enabling the RAN to receive and act upon service characteristics provided by third-party applications, the network can optimize scheduling, prioritization, and resource distribution in real time. This allows for differentiated treatment of flows, ensuring that latency-sensitive or high-reliability components are handled appropriately. Service awareness enables optimized resource management, improved QoE, and SLA compliance.

Current RAN architecture presents some limitations: lack standardized mechanisms to ingest and process service-level input dynamically. This hinders the ability to deliver fine-grained optimization and responsiveness, especially in multi-service, multi-tenant environments, hence reducing efficiency and service customization.

Architectural/Functional impact

- General: intent-based orchestration and service-aware control

- Control Plane: adaptive signaling based on service characteristics

- User Plane: differentiated routing and prioritization per application flow

- Radio Interface: dynamic configuration based on service profiles

- All Entities: integration with application-level orchestration and analytics

## Use case on resiliency for 6G

This use case is addressed in §5.6.3 of SA1 TR [3].

Use Case Description

This use case addresses the need for enhanced resiliency in 6G networks, as emphasized by ITU-R M.2160 and the 6G Roadmap. Resiliency refers to the network’s ability to maintain continuous and reliable service despite disruptions such as physical damage, cyber-attacks, or natural disasters. The 6G system is expected to support autonomous detection, classification, and recovery from faults, leveraging cloud-native architecture and AI-based orchestration to ensure uninterrupted service delivery.

Service Requirement

From [PR 5.6.3.6-1] – Subject to operator’s policy, the 6G network (OAM) should support autonomous detection and recovery of failed service with minimum human intervention, to ensure continuous operation.

Practical Service Example

A major fiber cut caused by construction disrupts connectivity in a metropolitan area. The 6G network, built on cloud-native principles, detects the anomaly via AI-based monitoring and initiates autonomous recovery actions. These include rerouting traffic, activating redundant functions, and reallocating workloads to maintain service continuity without human intervention.

RAN Requirement

RAN-REQ-8 – The RAN shall support autonomous reconfiguration and recovery mechanisms to maintain service continuity under fault or disruption conditions.

Drivers Enabled

Flexibility and Dynamicity: the ability to detect, classify, and respond to disruptions in real time (Dynamicity) enables resilient and adaptive network behavior (Flexibility).

Requirement Description

The 6G RAN must be capable of autonomously detecting service disruptions and initiating recovery actions without manual intervention. This includes rerouting traffic, activating backup functions, and reallocating workloads across distributed infrastructure. The system should continuously monitor the effectiveness of recovery actions and adjust configurations dynamically to optimize performance. AI-based orchestration should be integrated to support intelligent decision-making and policy enforcement during fault conditions.

Reason Why

To ensure uninterrupted service delivery in the face of unpredictable disruptions, the RAN must be resilient and capable of autonomous recovery. This is critical for maintaining trust, security, and availability in 6G networks, especially for mission-critical and latency-sensitive applications.

Current RAN architecture presents some limitations: lack of autonomous fault classification, limited dynamic reconfiguration capabilities, and dependency on centralized recovery procedures. These gaps hinder rapid and efficient response to service disruptions

Architectural/Functional Impact

- General: autonomous fault detection and recovery

- Control Plane: AI-driven orchestration and policy enforcement

- User Plane: dynamic traffic rerouting and workload reallocation

- Radio Interface: redundant TRP activation and path reconfiguration

- All Entities: cloud-native deployment and distributed resilience management

## Summary

This contribution analyzes a selection of representative 6G use cases, as defined in SA1 TR 22.870 [3], and derives the corresponding RAN requirements, with particular attention to their architectural and functional implications, as summarized in the following Table 1. These requirements are intended to guide the evolution of 6G RAN functionalities toward greater flexibility, scalability, dynamic adaptability, and service awareness, key pillars for future 6G systems. The objective is to ensure a consistent alignment between service-level needs and the ongoing design of the 6G RAN architecture within RAN3.

| REQ ID | RAN REQUIREMENT | DRIVERS | SA1 USE CASE | IMPACTS |
| --- | --- | --- | --- | --- |
| RAN-REQ-1 | The RAN shall ensure flexible and dynamic support for RAN Slice creation, configuration and management to meet diverse service requirements and vertical needs | Flexibility, Dynamicity | Network Slicing | Slice-aware scheduling, CP/UP separation, integration with slice orchestrator |
| RAN-REQ-2 | The RAN shall ensure automatic RAN resource scaling based on user density and traffic load, while considering capacity thresholds and constraints, e.g. on energy efficiency. | Dynamicity, Scalability | Fast Network Provisioning (Resilience) | Elastic TRP activation, edge CP/UP deployment, intent-based orchestration |
| RAN-REQ-3 | The RAN shall ensure flexible and granular split between User Plane and Control Plane | Flexibility | Localized Network for Verticals | Local breakout, distributed control, flexible CU interfacing |
| RAN-REQ-4 | The RAN shall ensure real-time traffic and resource management via AI/ML | Dynamicity | AI Agent for Performance Assurance | AI-driven scheduling, mobility control, adaptive radio configuration |
| RAN-REQ-5 | The RAN shall enable secure and scalable sharing of sensing-capable RAN resources in multi-operator and multi-tenant environments, supporting federated control, independent configuration, and dynamic orchestration of shared resources | Scalability, Flexibility | Shared RAN with Sensing | Multi-tenant access, operator-specific sensing, secure orchestration |
| RAN-REQ-6 | The RAN shall ensure multi-modal traffic synchronization for immersive services | Flexibility, Dynamicity | Multi-modal Media Synchronization | Coordinated scheduling, stream alignment, synchronized transmission |
| RAN-REQ-7 | The RAN shall support service characteristics awareness and dynamic adaptation of radio and network resources to meet specific application and service requirements | Flexibility, Scalability, Dynamicity | Enhanced Network Service Awareness | Adaptive signalling, flow-aware UP routing, dynamic radio configuration |
| RAN-REQ-8 | The RAN shall support autonomous reconfiguration and recovery mechanisms to maintain service continuity under fault or disruption conditions | Flexibility, Dynamicity | Resiliency for 6G | Autonomous detection of service disruptions and initiation of recovery actions without manual intervention, intelligent decision-making and policy enforcement during fault conditions |

Table 1: Summary of RAN Requirements derived from some 6G Use Cases

Based on this analysis, the following proposal is made:

Proposal 1: RAN3 to agree on the introduction of the identified RAN requirements and to consider them in the design of the 6G RAN architecture.

- RAN-REQ-1 – The RAN shall ensure flexible and dynamic support for RAN Slice creation, configuration and management to meet diverse service requirements and vertical needs.

- RAN-REQ-2 – The RAN shall ensure automatic RAN resource scaling based on user density and traffic load, while considering capacity thresholds and constraints, e.g. on energy efficiency.

- RAN-REQ-3 – The RAN shall ensure flexible and granular split between User Plane and Control Plane.

- RAN-REQ-4 – The RAN shall ensure real-time traffic and resource management via AI/ML.

- RAN-REQ-5 – The RAN shall enable secure and scalable sharing of sensing-capable RAN resources in multi-operator and multi-tenant environments, supporting federated control, independent configuration, and dynamic orchestration of shared resources.

- RAN-REQ-6 – The RAN shall ensure multi-modal traffic synchronization for immersive services.

- RAN-REQ-7 – The RAN shall support service characteristics awareness and dynamic adaptation of radio and network resources to meet specific application and service requirements.

- RAN-REQ-8 – The RAN shall support autonomous reconfiguration and recovery mechanisms to maintain service continuity under fault or disruption conditions.

Given the variety of possible implementation models for the 6G RAN architecture, it is essential to evaluate all viable alternatives with respect to the identified RAN requirements. This evaluation should consider both what can already be addressed with existing architectural frameworks and what enhancements may be needed to fully meet the requirements. The goal is to ensure that the selected architectural solutions enable compliance with the functional needs derived from 6G use cases, while also achieving an optimal trade-off between implementation complexity and system performance. This includes assessing the feasibility, scalability, and integration effort of each option, as well as its alignment with long-term deployment scenarios and cost-efficiency targets.

Proposal 2: RAN3 to agree on evaluating candidate 6G RAN architecture options to meet requirements with the best trade-off between implementation complexity and performance.

A pCR for TR 38.760-3 is provided in the Annex of this paper.

# 3	Conclusion

In this paper we discuss some relevant use cases defined by SA1 [3] and derives corresponding 6G RAN architectural requirements, by leading to the following observations and proposals:

Observation 1: To support new 6G services, the design of RAN architecture and functions needs to ensure flexibility, scalability and dynamicity.

Proposal 1: RAN3 to agree on the introduction of the identified RAN requirements and to consider them in the design of the 6G RAN architecture.

- RAN-REQ-1 – The RAN shall ensure flexible and dynamic support for RAN Slice creation, configuration and management to meet diverse service requirements and vertical needs.

- RAN-REQ-2 – The RAN shall ensure automatic RAN resource scaling based on user density and traffic load, while considering capacity thresholds and constraints, e.g. on energy efficiency.

- RAN-REQ-3 – The RAN shall ensure flexible and granular split between User Plane and Control Plane.

- RAN-REQ-4 – The RAN shall ensure real-time traffic and resource management via AI/ML.

- RAN-REQ-5 – The RAN shall enable secure and scalable sharing of sensing-capable RAN resources in multi-operator and multi-tenant environments, supporting federated control, independent configuration, and dynamic orchestration of shared resources.

- RAN-REQ-6 – The RAN shall ensure multi-modal traffic synchronization for immersive services.

- RAN-REQ-7 – The RAN shall support service characteristics awareness and dynamic adaptation of radio and network resources to meet specific application and service requirements.

- RAN-REQ-8 – The RAN shall support autonomous reconfiguration and recovery mechanisms to maintain service continuity under fault or disruption conditions.

Proposal 2: RAN3 to agree on evaluating candidate 6G RAN architecture options to meet requirements with the best trade-off between implementation complexity and performance.

A pCR for TR 38.760-3 is provided in the Annex of this paper.

# 4	References

[1]	3GPP RP-252912, “Revised SID: Study on 6G Radio”, RAN#109

[2]	3GPP TR 38.914, “Study on 6G Scenarios and requirements” v0.2.0 (Rel-20)

[3]	3GPP TR 22.870, “Study on 6G Use Cases and Service Requirements” v0.4.1 (Rel-20)

[4]	https://sns-origami.eu/

# 5	Annex – pCR for TR 38.760-3

# 5	Objectives and requirements

Editor’s note: The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

## 5.1	General Principles

All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles.

The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS]

The RAN shall ensure flexible and dynamic support for RAN Slice creation, configuration and management to meet diverse service requirements and vertical needs.

The RAN shall ensure automatic RAN resource scaling based on user density and traffic load, while considering capacity thresholds and constraints, e.g. on energy efficiency.

The RAN shall ensure flexible and granular split between User Plane and Control Plane.

The RAN shall ensure real-time traffic and resource management via AI/ML.

The RAN shall enable secure and scalable sharing of sensing-capable RAN resources in multi-operator and multi-tenant environments, supporting federated control, independent configuration, and dynamic orchestration of shared resources.

The RAN shall ensure multi-modal traffic synchronization for immersive services.

The RAN shall support service characteristics awareness and dynamic adaptation of radio and network resources to meet specific application and service requirements.

The RAN shall support autonomous reconfiguration and recovery mechanisms to maintain service continuity under fault or disruption conditions.