3GPP TSG-RAN WG3 Meeting #130	R3-258234

Dallas, US, 17th – 21st November, 2025

Title:	Preliminary analysis on RAN-CN interface evolution towards Service Based Interface (SBI)

Source:	FiberCop, CEWiT, Deutsche Telekom, Qualcomm Inc.

Agenda item:	10.3.2

Document Type:	Discussion and Decision

# 1	Introduction

In RAN#108 meeting [1], the 6G SID was approved addressing the following objectives on RAN architecture impacting the work of RAN3.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

TSG#112 (June/2026): …. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026.

The RAN Plenary requested to study the RAN–CN interface also in its service-based (SBI) version, in order to evaluate it against the traditional protocol-based approach and determine which solution should be adopted for 6G.

The Service-Based Architecture (SBA) model, which has already demonstrated its effectiveness in the 5G Core Network (5CN) and in OAM domains since Rel-15, is widely recognized for enabling modularity, improved service flexibility, and streamlined integration processes. In contrast, the current interface towards the Core Network (CN) remains anchored to a Protocol-Based approach, characterized by tightly coupled interfaces and limited architectural agility.

This contribution builds on those foundations and focuses on exploring possible architectural directions to introduce SBA between the RAN and the CN.

Moreover, this contribution is also grounded on ongoing activities within the research project ORIGAMI [2]. The ORIGAMI project brings together leading industry and academic partners across the European mobile telecommunications ecosystem and is funded by the European Commission under the 6G Smart Networks and Services Joint Undertaking (SNS JU). Running from 2024 to 2026, ORIGAMI focuses on advancing architectural models for next-generation mobile networks and addressing key barriers to the deployment of successful 6G technologies. Specifically, ORIGAMI aims to study 6G architectural innovations, like the Global Service-Based Architecture (GSBA), that enables solutions of Network Intelligence (NI) designed to enhance network performance, security, and flexibility. Beyond technological advances, ORIGAMI is committed to establishing global unified standards, supporting the green transition, improving affordability and accessibility, and fostering the development of groundbreaking applications to create strong ecosystem and novel business models for operators to generate new revenue opportunities and allows operator networks to evolve into future generation with advanced capabilities.

# Discussion

## Service Based Architecture principles

SBA, also known as Service-Oriented Architectures (SOAs), is a distributed approach based on the service concept, which implements a specific functionality, and where each network function is treated as an independent and autonomous entity [3], as depicted in Figure 1.

Figure 1 – High level schema of SBA approach

In an SBA system, services are designed as “black boxes”, meaning their internal workings are irrelevant to other components of the system, performing tasks that satisfy a specific set of requirements. Each service is created by a Service Producer (SP), which is responsible for registering the service in one or more Service Directories (SD). Service Consumers (SC), other entities in the network, can discover available services and start using them without prior knowledge of the specific details or access points of those services.

This distributed approach increases the modularity of the system, allowing components to interact without needing direct knowledge of each other, enables producer and consumer to be deployed anywhere in cloud network, thus promoting agility, efficient resource utilization, scalability and efficient operation of networks. The SBA model aligns well with technologies like Network Function Virtualization (NFV) and Software Defined Networking (SDN), which are foundational to network cloudification and provide more flexible, dynamic network control. Additionally, the SBA design facilitates the integration of advanced AI/ML solutions in cloud native manner, which can be easily added and managed as services, with producer-consumer interactions based on service requests.

## General Context

The possibility of introducing SBA principles in the RAN-CN interface has already been considered in the past. During the work for 5G specification, a study on applying SBI to the interaction between RAN and CN for the control plane (i.e., N2 interface) was proposed within SA2 [4]. However, due to the lack of agreement during the discussion, the study item phase was not pursued. The main reasons raised were:

- the introduction of an architectural paradigm shift in the middle of a mature generation would have risked fragmentation of ongoing deployments. This point suggests that it could be fine to consider it for the next generation

- the proposed approach was limited to N2 interface only, without a comprehensive RAN-CN framework. This point suggests that a general framework also including UE-CN interaction (i.e., N1 interface) and RAN-CN for the user plane (i.e., N3 interface) could be considered for the next generation

- the introduction of the SBI at that stage (i.e., in the middle of a 5G generation) would have required maintaining both protocol-based and service-based solutions in parallel, adding complexity. This point suggests that addressing the SBA approach in a new generation will avoid the issue

As a result, the consensus was that such a change of paradigm could only be realistically addressed at the beginning of a new generation. Introducing SBI in RAN-CN as part of 6G system design ensures that the architecture can be conceived holistically, avoiding mid-generation disruptions and guaranteeing alignment with CN evolution. Hence, it follows that the advent of 6G represents the appropriate timing to study the feasibility of introducing SBI between RAN and CN. If not addressed from the outset, such a change would likely remain excluded, just as it was during the 5G cycle.

Observation 1: The introduction of the new generation (i.e., 6G) is the best timing for addressing a feasibility study on a RAN-CN interface paradigm change towards an SBI.

Of course, any work on SBI will require coordination with SA working groups (e.g., SA1 for use cases and requirements, SA2 for architecture, SA3 for security, SA5 for management) to ensure that RAN-CN SBI is designed consistently with the broader 6G system framework.

## Benefits of introducing an SBA approach

The 5G network design established a clear separation between the RAN and the CN, with only a few control-plane and user-plane interfaces crossing the two traditionally distinct domains. While this separation enabled standalone operation and decoupling of the two domains, the increasing trend of cloudification has led to limitations in addressing new use cases and requirements, particularly when it comes to tight integration of resources with radio access network functions. In contrast, SBA can offer several significant advantages, well aligned with the ongoing shift towards software-driven, modular network designs, but they also highlight some of the shortcomings of the traditional protocol-based approach.

A comparative analysis between the service-based approach and the traditional protocol-based approach, aimed at highlighting their respective benefits and constraints, has been conducted in [5] and is summarized in the Table 1 below.

| Aspect | Service-Based Architecture (SBA) | Protocol-Based Architecture (PBA) |
| --- | --- | --- |
| Modularity & Flexibility | ✓ Highly modular: functions exposed as independent, reusable services via APIs | ✗ Rigid layering with tightly coupled protocol stacks |
| Interoperability | ✓ Enables vendor-agnostic integration through open, standardized interfaces (e.g., REST) | ✗ Interfaces often proprietary with specific communication patterns |
| Scalability | ✓ Supports dynamic scaling of individual services based on load or demand | ✗ Scaling typically applies to entire network functions or elements |
| Upgradability | ✓ Individual services can be upgraded or replaced independently | ✗ Software updates often require revalidation of entire function stack |
| Observability & Monitoring | ✓ Fine-grained monitoring per service, enabling better analytics and closed-loop automation | ✗ Monitoring is often coarse-grained and tied to protocol endpoints |
| Automation & AI Integration | ✓ Natively supports closed-loop automation and AI/ML-based optimization | ✗ Automation is limited to predefined rule sets or external tools |
| Latency | ✗ May introduce some additional overhead/latency due to service granularity, API calls, and message translation | ✓ Efficient runtime performance with tightly integrated protocol layers |
| Legacy Compatibility | ✗ Requires adaptation layers to interoperate with protocol-based systems | ✓ Direct compatibility with existing legacy systems |

Table 1 – Comparison between architecture approaches

In addition to the benefits and constraints summarized in the previous table, further advantages can be observed at the operational level. The adoption of a Service-Based Architecture (SBA) can enhance:

- Network lifecycle cost efficiency, through modular deployment, upgrade, and decommissioning of individual services, contributing to reductions in both operational (OPEX) and capital (CAPEX) expenditures.

- Cloud-native alignment, by leveraging container-based deployments and microservice orchestration to support on-demand scalability and improved system resilience.

- Cross-domain coordination, by enabling orchestration across RAN, Core, and edge domains, thus supporting end-to-end network agility and optimization focused on user experience.

Observation 2: The introduction of the SBA approach can overcome the current limitations of protocol-based architectures, while enabling a more dynamic and future-proof design with improved operational efficiency.

Considering the RAN plenary indication, prior analysis conducted during the 5G standardization and the potential expected benefits over the current protocol-based architecture model, it is proposed RAN3 to initiate a study to evaluate the feasibility of introducing a SBI between the RAN and the CN.

Proposal 1: Based on RAN plenary indication, past studies and potential expected benefits, it is proposed that RAN3 study the feasibility of introducing an SBI between the RAN and the CN.

## Analysis of SBI for the RAN-CN interface

For the introduction of a SBI between RAN and CN, the current 5G System (5GS) architecture depicted in Figure 1 can be considered as a starting point, where there is the evidence of the difference between the service-based design of the 5GC and the protocol-based architecture of the RAN.

Figure 1 illustrates the existing 5GS architectural model, with the CN represented through a clear SBA approach. The RAN, shown in the red dashed ellipse, interfaces with the 5GC through standardized reference points (e.g., N2, N3), and internally does not yet adopt a service-based interaction model.

Figure 1 – Starting point: current 5GS architecture and the proposed SBA extension to RAN

From a technical standpoint, applying the SB paradigm to the RAN-CN interface implies a fundamental shift in how RAN NFs interact with 5GC domain.

To effectively evolve the current protocol-based RAN–CN interface towards a service-based model, a structured approach is required to identify, expose, and manage the interactions between network functions as services. The following aspects outline potential steps and considerations for enabling such an evolution.

- Service Identification from Signalling Procedures – The transition towards SBI requires an initial step of analysing existing signalling procedures to identify the underlying functionalities they implement. These functionalities can then be mapped into discrete services, allowing a clear definition of which modules act as service producers or consumers. This decomposition is essential for modularizing RAN behaviour and enabling service exposure, invocation, and reusability across functional boundaries

- Service Exposure and Discovery – RAN and CN NFs could expose their capabilities as discoverable services via standardized APIs, enabling other NFs (within RAN or external domains) to dynamically locate and invoke such services. This would require introducing mechanisms, similar to the Network Function Repository Function (NRF) used in the 5GC

- Consumer/Provider Roles – In the context of a service-based architecture, RAN functions, such as those hosted in the gNB Control Plane (CU-CP) in 5G, can act as service providers, exposing capabilities like resource management, UE context handling, or QoS provisioning to CN functions through service-based interactions. Instead of relying on tightly coupled protocol procedures, CN functions can consume these services via SBI, enabling more flexible and decoupled integration between RAN and CN domains. Conversely, CN functions (e.g., AMF, SMF, PCF), acting as producers, may expose services such as mobility management, session handling, or policy control that are consumed by RAN functions like the CU-CP in 5G

- Service Directory Repository – To enable dynamic service discovery and controlled access in a service-based approach, a centralized Service Directory Repository should be introduced. This repository acts as a registry of all services exposed by RAN and CN NFs, including metadata such as service type, producer identity, supported operations, and access policies. Similar to the NRF in 5GC, the Service Directory allows authorized consumer NFs to query available services and retrieve endpoint information for invocation. Access control mechanisms must be integrated to ensure that only authorized consumers (based on identity, role, or subscription context) can discover and consume specific services. This enables secure and scalable service interactions across RAN and CN domains, supporting modularity, reusability, and dynamic orchestration of network behaviour. The repository should support:

  - Service Registration: producers register their services with relevant descriptors and capabilities

  - Service Discovery: consumers query the repository to locate services matching functional needs

  - Access Control: policies enforce which consumers can access which services, possibly leveraging existing AAA frameworks

  - Service Lifecycle Management: including updates, deprecation, and versioning of service definitions

RAN-CN interworking based on SBI would allow more coherent and native integration between RAN and CN, possibly enabling RAN services to be easily invoked or orchestrated, especially in the context of AI/ML-driven network automation and intent-based management.

Observation 3: Transitioning from a protocol-based to a service-based approach requires studying a structured set of steps covering service identification, exposure, roles, and registry mechanisms.

In the 5G system, the N2 interface relies on the NGAP protocol [6], a tightly coupled signalling framework used between the gNB (or CU-CP) and the AMF to support mobility, session management, and UE context handling. To evolve toward a service-based paradigm, the first step involves a systematic analysis of existing NGAP procedures to identify and characterize the underlying functionalities they provide. These functionalities, or subsets of them, can then be restructured or redesigned into discrete, discoverable services with well-defined APIs. This decomposition enables a clearer separation of roles between RAN and CN functions, promotes modularization of network behaviour, and supports dynamic service invocation and reusability.

These considerations should be taken into account by RAN3 in the feasibility study, to guide the identification of the optimal solution for 6G RAN–CN interactions.

Proposal 2: In studying the 6G RAN-CN SBI, RAN3 to identify the required functionalities and architectural modules, and define their roles as service producers and/or consumers.

A pCR for TR 38.760-3 is provided in the Annex of this paper.

# 3	Conclusion

In this paper we discuss the scenario of introducing a service-based approach in the RAN-CN interface, by leading to the following observations and proposals:

Observation 1: The introduction of the new generation (i.e., 6G) is the best timing for addressing a feasibility study on a RAN-CN interface paradigm change towards an SBI.

Observation 2: The introduction of the SBA approach can overcome the current limitations of protocol-based architectures, while enabling a more dynamic and future-proof design with improved operational efficiency.

Proposal 1: Based on RAN plenary indication, past studies and potential expected benefits, it is proposed that RAN3 study the feasibility of introducing an SBI between the RAN and the CN.

Observation 3: Transitioning from a protocol-based to a service-based approach requires studying a structured set of steps covering service identification, exposure, roles, and registry mechanisms.

Proposal 2: In studying the 6G RAN-CN SBI, RAN3 to identify the required functionalities and architectural modules, and define their roles as service producers and/or consumers.

A pCR for TR 38.760-3 is provided in the Annex of this paper.

# 4	References

[1]	3GPP RP-252912, “Revised SID: Study on 6G Radio”, RAN#109

[2]	https://sns-origami.eu/

[3]	3GPP TS 28.533, “Management and orchestration; Architecture framework” (Rel.19)

[4]	3GPP S2-2105493, “Study on Service-based N2” (Rel.18)

[5]	R3-256544, Discussion on Feasibility of RAN Service Based Architecture (FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra)

[6]	3GPP TS 38.413, “NG-RAN; NG Application Protocol (NGAP)” (Rel-19)

# 5	Annex – pCR for TR 38.760-3

### 6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

#### 6.1.3.x	Analysis of Service Based Interface (SBI) option

A service-based model requires a systematic approach to identify, expose, and manage interactions between network functions as services. The following aspects need to be considered:

Service identification from signalling procedures

The transition towards an SBI requires an initial step of analysing existing signalling procedures to identify the underlying functionalities they realise. These functionalities can subsequently be mapped to distinct services, enabling a clear identification of which functional entities act as service producers, and which act as service consumers. Such decomposition is essential to enable service exposure, invocation, and reusability across functional boundaries.

Service exposure and discovery

RAN and CN Network Functions (NFs) may expose their capabilities as discoverable services via standardised APIs, enabling other NFs to dynamically discover and invoke such services.

Consumer and Producer roles

It is necessary to define the producer and consumer roles between RAN and CN functions for the identified services. RAN functions may act as service providers, exposing capabilities such as resource management, UE context handling, or QoS provisioning to CN functions, that may consume these services. Conversely, CN functions acting as service producers, may expose services such as mobility management, session handling, or policy control that can be consumed by RAN functions.

Service directory repository

To enable dynamic service discovery and controlled access a centralised Service Directory Repository needs to be defined. This repository acts as a registry of all services exposed by RAN and CN NFs, including associated metadata such as service type, producer identity, supported operations, and access policies. The Service Directory Repository allows authorised consumer NFs to query available services and obtain endpoint information required for service invocation. Access control mechanisms should be integrated to ensure that only authorised consumers (based on identity, role, or subscription context) can discover and consume specific services. The repository should support:

- Service Registration: producers register their services with relevant descriptors and capabilities

- Service Discovery: consumers query the repository to locate services matching functional needs

- Access Control: policies enforce which consumers can access which services, e.g. through AAA framework

#### Service Lifecycle Management: including updates, deprecation, and versioning of service definitions