3GPP TSG-RAN3 Meeting #130	R3-258238

# Dallas, USA, 17th Nov 2025 – 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0210 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on positioning data collection for assisting in the pairing with the TRP’s measurement. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For Case 3a, RAN1 concluded that Part A and Part B may be generated by different entities. To enable the correct pairing of these parts from separate sources, RAN1 agreed to use their timestamps as a key element within the training dataset. However, this agreement has not yet been captured in the relevant specifications. Since the pairing operation involves interaction between the gNB and the LMF (via NRPPa signaling), it is necessary for RAN3 to incorporate the corresponding procedures into TS 38.455. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the RAN1 agreement in TS 38.455 to use timestamps as the key for pairing Part A and Part B when they are generated by different entities, this is added in the semantic description of the Time Stamp IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this update is not approved, it would be ambiguous how the gNB should pair Part A and Part B when they originate from different entities, leading to potential interoperability issues. |  |  |  |  |  |  |  |  |
| , t |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.107 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

### 9.2.107	Positioning Data Information

This information element contains positioning information for data collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP Measurement List |  | 0..1 |  | Indicates the area of interest. |
| >TRP Measurement Item |  | 1..<maxnoofMeasTRPs> |  |  |
| >>TRP ID | M |  | 9.2.24 |  |
| >>UL RTOA | O |  | 9.2.39 |  |
| >>gNB Rx-Tx Time Difference | O |  | 9.2.40 |  |
| >>LoS/NLoS Information | O |  | 9.2.77 |  |
| >>Time Stamp | M |  | 9.2.42 | Used to assist in the pairing with the TRP's measurement. |
| >>Measurement Quality | O |  | 9.2.43 |  |

<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>