3GPP TSG-RAN WG3 Meeting #130	R3-258247

Dallas, Texas, USA, 17-21 November 2025

Agenda Item:		12.2.1

Source:		NEC

Title:			(pCR for TR38.745) Multi-hop UE trajectory across gNBs

Document for:		Discussion and decision

# Introduction

In RAN3#129bis meeting, the following agreements are achieved for multi-hop UE trajectory [1]:

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

The following solutions are considered for supporting multi-hop UE trajectory:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

Focus on cell-based granularity for measured and predicted multi-hop UE trajectory.

Study multi-hop UE trajectory for L3 HO.

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

Besides, a pCR to TR38.745 [2] is agreed to capture Use case description and Locations for AI/ML Model Training and AI/ML Model Inference.

In this contribution, we provide our further considerations on multi-hop UE trajectory across gNBs and TP to TR38.745.

# Discussion

2.1	UE trajectory prediction transfer

UE trajectory prediction is an output of AI/ML-based mobility optimization and can be used as an input for AI/ML based load balancing.

In legacy one-hop UE trajectory mechanism, Cell Based UE Trajectory Prediction IE, which contains a list of cells that a UE is predicted to be connected to and also the associated predicted time of UE expected to stay in each cells, is carried in HANDOVER REQUEST message from source gNB to target gNB during handover preparation procedure, and the target gNB may use it for e.g. mobility decisions.

As for multi-hop UE trajectory, during each handover preparation procedure for the subsequent handovers, the 1st target gNB removes the predicted cell with itself that are for the 1st hop, and adds a list of the remaining cells that the UE is predicted to be connected to after the 1st hop (and the associated predicted time of UE expected to stay in each remaining cells) in the Cell Based UE Trajectory Prediction IE in HANDOVER REQUEST message to the next target gNB. All the subsequent target gNBs apply the same procedure.

Proposal 1: Reuse the Cell Based UE Trajectory Prediction IE in HANDOVER REQUEST message for multi-hop UE Trajectory Prediction transfer.

Proposal 2: For subsequent handovers, the Cell Based UE Trajectory Prediction IE contains a list of the remaining cells that the UE is predicted to be connected to after the 1st hop and the associated predicted time of UE expected to stay in each remaining cells.

2.2	Measured UE trajectory collection

First, we recall the signalling procedure for one-hop UE trajectory:

Step 1. gNB1 triggers Data Collection Reporting Initiation procedure with gNB2, to request from gNB2 the reporting of UE trajectory. UE Trajectory Collection Configuration IE containing Collection Time Duration for UE Trajectory and Number of Visited Cells is present in the DATA COLLECTION REQUEST message

Step 2. During handover procedure (UE from gNB1 to gNB2), Data Collection ID IE is contained in the HANDOVER REQUEST message from gNB1 to gNB2.

Step 3. After UE handovers to gNB2, gNB2 performs UE trajectory collection and reporting via DATA COLLECTION UPDATE message. gNB1 still keeps UE context until Data collection completed.

In order to generate multi-hop UE trajectory, gNB needs to collect multi-hop UE trajectory. To achieve this, the following two solutions were proposed in Rel-19:

- Hop by hop based on current mechanism with the effort to keep UE context for more time in RAN node

- End to End solution with the risk that the connection between the source and target is not available

However, two issues were identified: 1) gNB needs to keep large number of UE context with it, which may bring large signalling cost among gNBs and increase network complexity; 2) multi-hop UE trajectory may not be accurate enough to help improve mobility performance.

Data Collection Reporting Initiation procedure and Data Collection Reporting procedure use non-UE-associated signalling and these procedures are not linked to a specific UE. After one-hop, both the Collection Time Duration for UE Trajectory and Number of Visited Cells need to be updated for the following handovers and the updates are UE specific, however, if legacy Data Collection Reporting Initiation procedure is triggered for each UEs, this will lead to large and unacceptable signalling cost. Therefore, we suggest to use UE-associate signalling for the UE trajectory collection afterwards. For example, source gNB in between updates the Collection Time Duration for UE Trajectory and Number of Visited Cells in UE Trajectory Collection Configuration, and sends it in the Handover Request message to target gNB for successive UE trajectory collection.

Besides, if all the involved gNBs need to be keep UE context after handover execution and UE context release needs per UE handling, this will increase network complexity. Therefore, we suggest to adopt the End to End solution that only requires the triggered gNB to keep UE context. To achieve this, gNB1 ID also needs to be sent together with UE Trajectory Collection Configuration.

The proposed procedures are showing as below:

Figure 1: Multi-hop UE trajectory collection

In addition, since continuous MDT has been specified in Rel-19 and therefore UE trajectory can be collected across RRC states, this can also facilitate multi-hop UE trajectory prediction.

Proposal 3: Adopt End to End solution for multi-hop UE trajectory collection, including:

- add Triggered gNB ID in the UE Trajectory Collection Configuration IE;

- add UE Trajectory Collection Configuration IE in Handover Request message.

2.3	Input, output and feedback

To predict the multi-hop UE trajectory, a gNB may need the following information as input data:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

From the neighbouring RAN nodes:

-	UE Mobility History Information

-	Position, QoS parameters and the performance information of historical HO-ed UE (e.g., loss rate, delay, etc.)

-	UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism

From the local node:

-	Multi-hop UE trajectory prediction

The output generated by AI/ML model inference is the multi-hop UE trajectory prediction, e.g., cell ID of UE over a future period of time and its associated predicted time of UE expected to stay in the cell.

Legacy UE performance feedback for those UEs handed over from the source gNB and SON reports (e.g., RLF, CEF, RA) can be used as feedback data.

2.4	Potential standard impacts

According to the analysis above, following standard impacts are foreseen for subsequent Rel-20 normative work:

Xn interface:

-	Enhanced existing Handover Request message with the following information:

-	Triggered gNB ID

-	UE Trajectory Collection Configuration

TP to TR38.745 which captures the above information is in Annex.

Proposal 3: Agree the TP to TR38.745 for multi-hop UE trajectory in Annex.

# Proposals

In this contribution, we provide our initial considerations on multi-hop UE trajectory across gNBs, including the input/output/feedback and the potential standard impact, and the following proposals were drawn from the analysis.

Proposal 1: Reuse the Cell Based UE Trajectory Prediction IE in HANDOVER REQUEST message for multi-hop UE Trajectory Prediction transfer.

Proposal 2: For subsequent handovers, the Cell Based UE Trajectory Prediction IE contains a list of the remaining cells that the UE is predicted to be connected to after the 1st hop and the associated predicted time of UE expected to stay in each remaining cells.

Proposal 3: Adopt End to End solution for multi-hop UE trajectory collection, including:

- add Triggered gNB ID in the UE Trajectory Collection Configuration IE;

- add UE Trajectory Collection Configuration IE in Handover Request message.

Proposal 4: Agree the TP to TR38.745 for multi-hop UE trajectory in Annex.

# Reference

[1] RAN3_129bis_agenda_20251017_EOM2

[2] R3-257329, (TP to TR 38.745) Consideration on Multiple-hop UE trajectory

# Annex	TP for TR38.745

<Start of change>

## 4.1	Multi-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

In order to generate multi-hop UE trajectory prediction, gNB needs to collect multi-hop UE trajectory. To achieve this, the following two solutions were proposed:

- Hop by hop based on current mechanism with the effort to keep UE context for more time in RAN node

- End to End solution with the risk that the connection between the source and target is not available

However, two issues were identified: 1) gNB needs to keep large number of UE context with it, which may bring large signalling cost among gNBs and increase network complexity; 2) multi-hop UE trajectory may not be accurate enough to help improve mobility performance.

The potential solutions and standard impact need to be studied with the aim of avoid increasing network complexity or signalling cost.

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Input data of multi-hop UE trajectory

To predict the multi-hop UE trajectory, a gNB may need the following information as input data:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

From the neighbouring RAN nodes:

-	UE Mobility History Information

-	Position, QoS parameters and the performance information of historical HO-ed UE (e.g., loss rate, delay, etc.)

-	UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism

From the local node:

-	Multi-hop UE trajectory prediction

#### 4.1.2.3	Output data of multi-hop UE trajectory

The following information can be generated as output:

-	Multi-hop UE trajectory prediction, e.g., cell ID of UE over a future period of time and its associated predicted time of UE expected to stay in the cell

#### 4.1.2.4	Feedback of multi-hop UE trajectory

To optimize the performance of multi-hop UE trajectory prediction, the following feedback can be considered to be collected from gNBs:

-	Legacy UE performance feedback for those UEs handed over from the source gNB

-	SON reports (e.g., RLF, CEF, RA)

#### 4.1.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-20 normative work compared with what was specified during Rel-18 and Rel-19.

Xn interface:

-	Enhanced existing Handover Request message with the following information:

-	Triggered gNB ID

-	UE Trajectory Collection Configuration

<End of change>