3GPP TSG-RAN WG3 Meeting #130	R3-258249

Dallas, Texas, USA, 17-21 November 2025

Agenda Item:		12.2.3

Source:		NEC, Qualcomm

Title:	(pCR for TR38.745) Other AI/ML based handover enhancements

Document for:		Discussion and decision

# Introduction

In RAN3#129bis meeting, we had some initial discussion on other AI/ML based handover in Rel-20, especially on inter-CU LTM, however no consensus can be achieved.

In this contribution, we provide our further considerations on other AI/ML based handover and TP to TR38.745.

# Discussion

Regarding other AI/ML based handover in Rel-20, we suggest to focus on enhancements which have limited additional standard impact compared with what has already agreed to study, i.e. multi-hop UE trajectory and intra-CU LTM.

Legacy LTM supports both intra-CU and inter-CU cases. In our understanding, a similar procedure, as described in [2], can be applied for both intra-CU LTM and inter-CU LTM, with only the following additional standard impacts on Xn interface:

- LTM predication needs to send from source gNB-CU to candidate gNB-CU(s) for LTM candidate cell preparation.

- UE performance feedback collected by target gNB-CU needs to send to source gNB-CU for model performance monitor.

Considering specifying inter-CU LTM case together with intra-CU LTM case can generate one complete solution for both LTM cases, with acceptable extra standard efforts. Therefore, we suggest to study AI/ML based inter-CU LTM in Rel-20.

Proposal 1: Study AI/ML based inter-CU LTM in Rel-20 and consider the following standard impact on Xn interface for normative work:

- Exchange LTM predication for LTM candidate cell preparation

- UE performance feedback, including beam level UE performance, after LTM execution for model performance monitor

For LTM, L1 measurements are collected and taken into account for LTM decision making. In order to assist mobility decision in finer granularity, especially for LTM, we suggest to expand on top of the cell level and study beam level UE trajectory UE trajectory.

Proposal 2: Study beam level UE trajectory prediction in Rel-20 based on cell level design.

TP to TR38.745 which captures the above information is in Annex.

Proposal 3: Agree the TP to TR38.745 for other AI/ML based handover enhancements in Annex.

# Proposals

In this contribution, we provide our initial considerations on other AI/ML based handover enhancements, and the following proposals were drawn from the analysis.

Proposal 1: Study AI/ML based inter-CU LTM in Rel-20 SI and consider the following standard impact on XN interface and F1 interface for normative work:

- Exchange LTM predication for LTM candidate cell preparation

- UE performance feedback, including beam level UE performance, after LTM execution for model performance monitor

Proposal 2: Study beam level UE trajectory prediction in Rel-20 based on cell level design.

Proposal 3: Agree the TP to TR38.745 for other AI/ML based handover enhancements in Annex.

# Annex	TP for TR38.745

<Start of change>

## 4.3	Other handover enhancements

Editor Note: Identify other handover enhancments via AI/ML, e.g., inter-CU LTM

### 4.3.1	Inter-CU LTM

Inter-CU LTM applies a similar procedure intra-CU LTM as described in section 4.2. Following additional standard impacts on Xn interface are listed for subsequent Rel-20 normative work.

- Exchange LTM predication for LTM candidate cell preparation

- UE performance feedback, including beam level UE performance, after LTM execution for model performance monitor

### 4.3.	Beam level UE trajectory

In order to assist mobility decision in finer granularity, especially for LTM, beam level UE trajectory prediction can be used as input data.

Beam level UE trajectory should be based on the cell level UE trajectory design.

<End of change>