3GPP TSG-RAN3 WG3 Meeting #130	R3-258264

Dallas, TX, USA, 13 – 17 October 2025

Agenda Item:	14.2

Source: 	Qualcomm Incorporated

Title: 	Architecture, functionalities and signaling to support Topology 2

Document for:	Discussion/Decision

# Background

For Rel-20 Ambient_IoT_Solutions_Ph2 WI, following are the RAN3 related objectives mentioned in the WID RP-252894:

- Specify signaling for Topology 2

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection.

  - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

In this paper, we provide our views for the above objectives and the corresponding proposals.

# Discussion

## Deployment scenarios for T2

In RAN3#129bis, there was a paper R3-257065 proposing to acknowledge two deployment scenarios for Topology 2 - (1) “fixed” UE reader assuming a well-planned network setup throughout the projected A-IoT service area and (2) mobile UE reader assuming hand-held devices.

While this is a good discussion on possible deployment scenarios, we think the distinction of UE readers based on their mobility is not necessarily the right and only way. In our view, UE readers can be either used for coverage extension (e.g. if BS readers in T1 are not able to reach certain regions within an interested area) or can be used on-demand (e.g., upon a worker pressing a button in a hand-held UE reader).

UE readers may be used as coverage-extension devices (e.g., where BS readers can’t reach) or may be used on-demand (e.g., workers using hand-held UE readers and pressing a button). Both coverage-extension UE readers and hand-held UE readers can be fixed or mobile.

WID already has an objective to support UE Reader mobility (and there is no objective to support fixed UE readers). Hence, there is no need to design for a special case where UE readers may be fixed.

RAN3 should discuss functionalities and procedures for Topology 2 assuming UE readers are mobile (as is in the scope of the WID). There is no need to discuss “fixed” UE reader scenario seperately.

## UE Reader selection

SA2 have captured the following as part of the agreed principles for Topology 2 (KI#1) in 23.700-30-020.

UE reader selection part:

Two scenarios will be supported for UE reader selection, AF providing UE reader ID case and AF providing Area case for all types of UE reader.

-	For AF providing UE reader ID case:

-   If UE Reader ID(s) is provided by the AF via the NEF for the operation, then that is used as the selected Reader(s) if authorized as UE reader. The AIOTF provides the selected UE Reader(s) to the NG-RAN.

Editor's note:	How to select UE reader(s) based on the AF provided area information is FFS.

SA2 has agreed to support two scenarios for UE reader selection – (1) AF providing UE Reader ID to AIOTF, (2) AF providing Area to AIOTF

Both the scenarios agreed by SA2 are valid scenarios. For example, AF providing a specific UE Reader ID can happen in a scenario where a human worker in the factory/supermarket presses a handheld UE reader and this handheld UE reader is to be used for performing inventory. There is no UE reader “selection” in this case. We call this UE-specific inventory request in this paper.

Also, in case (2), AIOTF can receive a “Target Area Information” indicating the area targeted for performing inventory and then the node selecting UE readers (AIOTF or gNB depending on the UE reader selection discussion) can select appropriate UE readers within the target area for performing inventory. We call this area-based inventory request in this paper.

- We therefore propose the following:

RAN3 confirms SA2 agreed principle to support both UE-specific inventory request (i.e., when AF provides a UE Reader ID to AIOTF) and area-specific inventory request (i.e., when AF provides an Area to AIOTF) and identify NGAP impacts.

As mentioned in the SA2 conclusions, in case a single UE reader ID is provided to the AIOTF from the AF (via the NEF), then that is used as the UE reader for performing A-IoT service operations (e.g., inventory). In this case, there is no UE reader “selection” procedure as such. Therefore, the following is proposed:

In case of UE-specific inventory request i.e., upon receiving a single UE reader ID from AF (via the NEF), AIOTF simply indicates that UE reader ID to gNB (via AMF) in NG: Inventory Request. There is no UE reader “selection” in this case.

However in case of area-based inventory request, upon receiving the Target Area Information from AF (via the NEF), AIOTF should be able to select UE readers for performing inventory, similar to how AIOTF can select RAN readers directly (e.g., if OAM has provisioned AIOTF with the NG-RAN selection information which includes A-IoT area list, RAN reader list, location of RAN readers).

In contrast to the selection of RAN readers (which is based on OAM provisioned information), AIOTF can obtain the UE reader’s “location” by interacting with the UE reader’s serving AMF to obtain the UE Reader’s serving gNB or even obtain granular UE location by interacting with LMF (either directly or indirectly).

In case of area-based inventory request i.e., upon receiving the Target Area Information from AF (via the NEF), AIOTF can select UE reader(s) for inventory (e.g., after interaction with AMF serving the target area to obtain the list of UE readers within the target area and/or after interaction with LMF to obtain UE reader’s location).

## Inventory for T2

It is assumed that the UE reader’s subscription/authorization information is stored at the UDM by the AMF and hence the UDM has knowledge of the UE reader’ serving AMF. AIOTF can obtain the UE reader’s serving AMF by interacting with UDM.

In case of area-based inventory request, after selecting UE reader(s), AIOTF should indicate the list of selected UE reader(s) to the each UE reader’s serving AMF(s)), which then forwards it to each UE reader’s serving gNB. Hence the following is proposed:

In case of area-based inventory request, after selecting UE reader(s), AIOTF can indicate the selected UE reader ID(s) to each UE reader’s serving gNB (via AMF) using NG: Inventory Request.

In case of UE-specific inventory request, even though there is no UE reader “selection” as mentioned in the previous section, the AIOTF can still indicate to AMF the UE reader ID to use, which then indicates it to the UE reader’s serving gNB.

In both cases (area-based and UE-specific inventory request), AMF receives the UE reader ID(s) (e.g., SUPI/GPSI) from AIOTF and should map it to the corresponding NGAP ID(s) (i.e., RAN UE NGAP ID, AMF UE NGAP ID) before sending the NG: Inventory Request. AMF should maintain the mapping between the UE Reader ID(s) received from AIOTF and the NGAP ID(s).

For each UE Reader selected to perform inventory, AMF maintains the mapping between UE Reader ID received from AIOTF (e.g., GPSI/SUPI) and the RAN UE NGAP ID/AMF UE NGAP ID.

In case multiple UE readers are selected by the AIOTF to perform inventory for a specific A-IoT session, RAN UE NGAP ID, AMF UE NGAP ID are included along with the existing Inventory Request Transfer container for each UE reader the AIOTF is interested in performing inventory. Hence the following is proposed:

Enhance NG: Inventory Request to include {RAN UE NGAP ID, AMF UE NGAP ID} outside the Inventory Request Transfer Container for each UE reader the AIOTF is interested in performing inventory.

Upon receiving the UE Reader’s NGAP ID(s) in NG: Inventory Request, gNB understands that this inventory session requires the use of the indicated UE reader, creates an RRC message for carrying Inventory Request to the UE Reader and sends it to the UE reader.

Similar to how the RAN Reader ID is indicated in the NG: Inventory Response/Report, the UE reader ID (the NGAP ID(s) in this case) can be indicated in the NG: Inventory Response/Report. Therefore, the following is proposed:

Enhance NG: Inventory Response/Inventory Report to include {RAN UE NGAP ID, AMF UE NGAP ID} for identifying the UE reader from which the inventory response/report is sent.

## Command for T2

Similar to the proposals for inventory procedure, AIOTF can indicate to AMF the UE reader ID(s) to be used for the command procedure. AMF can map these UE reader ID(s) received from AIOTF to the NGAP ID(s) and indicate it to the gNB in NG: Command Request.

Enhance NG: Command Request to include {RAN UE NGAP ID, AMF UE NGAP ID} along outside the Command Request Transfer Container to indicate the UE Reader via which the AIOTF is interested in sending the A-IoT Command.

Include RAN UE NGAP ID and AMF UE NGAP ID in NG: Command Response as well

## UE Reader mobility

In R20 A-IoT WID, there is an objective for RAN3 to specify necessary signaling support for inter-gNB RRC-connected UE Reader Mobility.

UE readers can be mobile and can be served by a new gNB upon mobility (e.g., upon moving from one corner of the factory to another corner).

An ongoing A-IoT session (e.g. inventory session) should not be terminated just because the UE reader connects to a new gNB due to mobility. Otherwise, the AF/AIOTF needs to start a new inventory session, perform reader selection yet again, configure the UE reader and target gNB with the A-IoT session parameters yet again every time there is UE reader mobility. A-IoT session parameters can include all things related to the A-IoT session and which are required at the gNB such as the A-IoT session ID, A-IoT Device Identification information, assistance information received from AIOTF e.g., expected number of devices, time interval for result aggregation, Correlation ID.

Also, in case the source gNB has stored some inventory reports received from UE reader over RRC and has not yet sent it to the AIOTF, there needs to be a mechanism to avoid the loss of stored inventory reports. In other words, the stored inventory reports should not be discarded every time there is a serving gNB change upon UE reader mobility. Otherwise, this would be a very inefficient system and lead to unnecessary and repeated inventory procedures. Therefore, the following is proposed:

RAN3 should discuss signaling enhancements over Xn to ensure A-IoT session continuity (i.e., to avoid terminating the ongoing A-IoT session and the AIOTF to configure target gNB with A-IoT session parameters after UE mobility).

RAN3 should discuss signaling enhancements over Xn to avoid loss of stored A-IoT data at source gNB upon UE reader mobility.

## UE Reader authorization

SA2 have captured the following as part of the agreed principles for Topology 2 (KI#1) in 23.700-30-020.

UE reader authorization and revocation part:

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor's note:	Whether and what additional subscription information for the UE Reader is needed is FFS.

-	UE Reader subscription information is available to AMF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Editor's note:	Whether to inform the UE about the authorization information is FFS.

-	If the subscription information has been revoked, the indication that the UE is not authorized to operate as a UE Reader from the UDM, then the AMF informs NG-RAN that the UE is not authorized to operate as a UE Reader.

Editor's note:	Whether and how UE Reader subscription information is provided to AIOTF is FFS.

Also, RAN3 agreed the following last meeting:

UE Reader Authorization is performed at 5GC​

UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB.​

FFS whether UE reader authorization status can be provided from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

RAN3 assumes that a UE’s A-IoT related subscription capabilities (e.g., an indication whether the UE is allowed to act as A-IoT Reader) is stored at the UDM and that the AIOTF/AMF interacts with the UDM to check UE’s A-IoT related subscription upon receiving an Inventory Request from AF. This UE reader authorization information should be passed to the gNB serving the UE reader, so that the serving gNB can provision for resources for this authorized UE reader. This is similar to how authorization information is passed from AMF to the gNB for other features such as V2X, ProSe, IAB etc. Therefore, the following is proposed.

UE reader authorization status can be provided from source gNB to the target gNB in Xn: HANDOVER REQUEST

## UE Reader capabilities

There is the following Editor’s note in SA2 TR:

Editor's note:	Whether UE reader capability is provided to AMF is FFS.

Just like any feature supported by UE, there would be a discussion needed on UE capabilities, both the AS capabilities and the NAS capabilities. Although the details are under remit of other WGs (e.g., RAN2 for AS capabilities and SA2/CT1 for NAS capabilities), RAN3 needs to discuss (in coordination with other WGs) on which node stores the A-IoT related capabilities as this also will have a say in the UE reader selection discussion

RAN3 can assume that UE Reader can provide its A-IoT related AS capabilities to gNB via RRC signaling e.g., as part of UE capability procedure. UE reader’s AS capabilities can include e.g., list of frequencies supported by UE Reader for AIoT communication, supported band combinations for Uu + AIoT, co-existence of A-IoT with other NR features etc. It is not clear whether these UE reader’s radio capabilities also need to be known/stored at the AMF/AIOTF.

UE reader’s capabilities may include e.g., list of frequencies supported by UE Reader for AIoT communication, supported band combinations for Uu + AIoT, co-existence of A-IoT with other NR features etc.

Also, it is not yet clear whether there would be any A-IoT related NAS capabilities and whether the UE Reader would provide such A-IoT related NAS capabilities to AMF via NAS signaling e.g., during registration procedure. A-IoT related NAS capabilities can include e.g., whether the UE Reader is capable of forwarding A-IoT NAS PDU(s) for DO-DTT operations such as Inventory/Command, whether UE Reader is capable of forwarding A-IoT NAS PDU(s) for DO-A operations).

Since this involves cross WG involvement, we propose the following:

RAN3 to LS SA2 to check whether AIOTF/AMF needs to be aware of UE Reader’s capabilities e.g., for selecting the appropriate UE Reader(s) upon receiving an Inventory/Command Request. Also check with SA2 whether there would be any NAS capabilities, in addition to radio capabilities.

## 2.7 Requested Reader Type

As R20 is now specifying support for UE readers in addition to RAN readers specified in R19, there needs to be a mechanism to indicate the desired type of reader when an AF sends an A-IoT related service request (e.g., for performing inventory/command). For instance, the AF might be interested in only using RAN readers or only using UE readers or it might want to use both. This can for example depend on the time of day e.g., only RAN readers are used in the night hours when there are not many workers in the factory, both UE readers and RAN readers can be used in work hours. Or this could be area-specific e.g., only UE readers are used in some restricted area of the factory. This can help AIOTF in eventually selecting the right type of readers. For this purpose, we propose the following:

Besides indicating the type of Reader (RAN reader or UE reader) via Requested Reader Type, AF might be interested in a specific reader type e.g., indoor/outdoor reader, DO-DTT/DO-A-capable reader or both, reader supporting certain device types (e.g., device 1 or 2a or 2b, passive or active devices or a combination of them), reader(s) supporting a certain coverage, reader(s) supporting a certain Tx/Rx power. Therefore, the following is proposed:

AIOTF can receive from AF (via the NEF) a “Requested Reader Type” to indicate the desired type of readers for performing an A-IoT service operation e.g., UE Reader(s) only, gNB Reader(s) only, Both UE Reader(s) and gNB Reader(s), readers supporting device 1/2a/2b, readers supporting DO-DTT/DO-A

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Deployment scenarios for T2

Observation 1:	UE readers may be used as coverage-extension devices (e.g., where BS readers can’t reach) or may be used on-demand (e.g., workers using hand-held UE readers and pressing a button). Both coverage-extension UE readers and hand-held UE readers can be fixed or mobile.

Observation 2:	WID already has an objective to support UE Reader mobility (and there is no objective to support fixed UE readers). Hence, there is no need to design for a special case where UE readers may be fixed.

Proposal 1:	RAN3 should discuss functionalities and procedures for Topology 2 assuming UE readers are mobile (as is in the scope of the WID). There is no need to discuss “fixed” UE reader scenario seperately.

2.2 UE Reader selection

Observation 3:	SA2 has agreed to support two scenarios for UE reader selection – (1) AF providing UE Reader ID to AIOTF, (2) AF providing Area to AIOTF

We therefore propose the following:

Proposal 2:	RAN3 confirms SA2 agreed principle to support both UE-specific inventory request (i.e., when AF provides a UE Reader ID to AIOTF) and area-specific inventory request (i.e., when AF provides an Area to AIOTF) and identify NGAP impacts.

Proposal 3:	In case of UE-specific inventory request i.e., upon receiving a single UE reader ID from AF (via the NEF), AIOTF simply indicates that UE reader ID to gNB (via the AMF) in NG: Inventory Request. There is no UE reader “selection” in this case.

Proposal 4:	In case of area-based inventory request i.e., upon receiving the Target Area Information from AF (via the NEF), AIOTF can select UE reader(s) for inventory (e.g., after interaction with AMF serving the target area to obtain the list of UE readers within the target area and/or after interaction with LMF to obtain UE reader’s location).

2.3 Inventory for T2

Proposal 5:	In case of area-based inventory request, after selecting UE reader(s), AIOTF can indicate the selected UE reader ID(s) to each UE reader’s serving gNB (via AMF) using NG: Inventory Request.

Proposal 6:	For each UE Reader selected to perform inventory, AMF maintains the mapping between UE Reader ID received from AIOTF (e.g., GPSI/SUPI) and the RAN UE NGAP ID/AMF UE NGAP ID.

Proposal 7:	Enhance NG: Inventory Request to include {RAN UE NGAP ID, AMF UE NGAP ID} outside the Inventory Request Transfer Container for each UE reader the AIOTF is interested in performing inventory.

Proposal 8:	Enhance NG: Inventory Response/Inventory Report to include {RAN UE NGAP ID, AMF UE NGAP ID} for identifying the UE reader from which the inventory response/report is sent.

2.4 Command for T2

Proposal 9:	Enhance NG: Command Request to include {RAN UE NGAP ID, AMF UE NGAP ID} along outside the Command Request Transfer Container to indicate the UE Reader via which the AIOTF is interested in sending the A-IoT Command.

Proposal 10:	Include RAN UE NGAP ID and AMF UE NGAP ID in NG: Command Response as well

2.5 UE Reader mobility

Proposal 11:	RAN3 should discuss signaling enhancements over Xn to ensure A-IoT session continuity (i.e., to avoid terminating the ongoing A-IoT session and the AIOTF to configure target gNB with A-IoT session parameters after UE mobility).

Proposal 12:	RAN3 should discuss signaling enhancements over Xn to avoid loss of stored A-IoT data at source gNB upon UE reader mobility.

2.6 UE Reader authorization

Proposal 13:	UE reader authorization status can be provided from source gNB to the target gNB in Xn: HANDOVER REQUEST

2.7 UE Reader capabilities

Observation 4:	UE reader’s capabilities may include e.g., list of frequencies supported by UE Reader for AIoT communication, supported band combinations for Uu + AIoT, co-existence of A-IoT with other NR features etc.

Proposal 14:	RAN3 to LS SA2 to check whether AIOTF/AMF needs to be aware of UE Reader’s capabilities e.g., for selecting the appropriate UE Reader(s) upon receiving an Inventory/Command Request. Also check with SA2 whether there would be any NAS capabilities, in addition to radio capabilities.

2.7 Requested Reader Type

Proposal 15:	AIOTF can receive from AF (via the NEF) a “Requested Reader Type” to indicate the desired type of readers for performing an A-IoT service operation e.g., UE Reader(s) only, gNB Reader(s) only, Both UE Reader(s) and gNB Reader(s), readers supporting device 1/2a/2b, readers supporting DO-DTT/DO-A