3GPP TSG-RAN WG3 Meeting #130	R3-258267

Dallas, TX, USA, November 17th – 21st 2025

Agenda Item:	10.5.1

Source:	Ericsson

Title:	(pCR for TR 38.760-3): Principles for 6G RAN AI/ML Use Case Study

Document for:	Agreement

# 1	Introduction

At the RAN3#129-bis meeting, the discussion on use cases for AI/ML data collection resulted in the following agreement:

The following use cases will be studied in RAN3:

- AI/ML based Network energy saving

- AI/ML based mobility optimization

Accordingly, in this paper we discuss the high-level principles to be followed in RAN3 discussions of 6G RAN AI/ML use cases.

A pCR for TR 38.760-3 is provided in the Annex.

# 2	Discussion

At the RAN3#129-bis meeting, RAN3 took a bold decision to study two AI/ML use cases without a data collection framework in place. To study and specify an “AI/ML for RAN” use case requires:

- Establishing the high-level principles for AI/ML data collection in the RAN.

- Verifying that the RAN architecture can support the use case.

- Specifying which data should be collected.

- Identifying the entities responsible for inference and model training.

- Specifying which logical entities and interfaces should be involved in the exchange of data, i.e., where the data needs to be available, and the feasibility thereof.

At the time when 5G AI/ML work was initiated in RAN3, the RAN architecture was fully specified. Furthermore, the features that were subject to AI/ML-based optimization use cases were specified as well. This is not the case today when we initiate the discussion on 6G AI/ML use cases - neither the 6G RAN architecture, nor the features subject to AI/ML-based optimization have been specified. For example, presently, it is neither clear which logical nodes constitute the 6G RAN nor is it clear how mobility or energy saving features will be designed.

On the other hand, we acknowledge RAN3’s decision to initiate the AI/ML use case discussion, and we need to consider which level of discussion is appropriate. For example, as of today, for the two AI/ML use cases to be studied, RAN3 can neither discuss which network entities can generate or collect data, nor can it discuss how and where the data can be transported. Therefore, for the time being, RAN3 should focus on establishing the high-level principles for 6G RAN AI/ML use cases.

The fundamental principle that should be followed is that the discussions should be based on certainties, rather than on assumptions. For example, this means that the placement of inference and model training for a use case can be discussed only after the 6G RAN architecture is agreed, and when the feature subject to the AI/ML-based optimization is specified.

Proposal 1: 6G AI/ML data collection use case descriptions in TR 38.760-3 should be based on certainties, i.e., based on the specification progress of 6G RAN architecture and on the progress of 6G RAN features subject to AI/ML use cases.

As said, given the need to reach better maturity on the 6G RAN architecture and on the features on which AI/ML should be applied, one possible focus area at this stage for RAN3 could be to outline general principles that would properly scope the AI/ML use case solution space.

With that in mind, we could look at the principles agreed during the RAN3 Rel-17 study on AI/ML reported in TR 37.817. While these principles were derived with the 5G network in mind, they are largely generic, and, as such, applicable for the design of AI/ML framework in 6G.

Such principles, taken from clause 4.1 of TR 37.817, are shown below (enumerated for easier referencing):

The following high-level principles should be applied for AI-enabled RAN intelligence: - The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope. - The study focuses on AI/ML functionality and corresponding types of inputs/outputs. - The input/output and the location of the Model Training and Model Inference function should be studied case by case. - The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope. - The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope. - Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases. - The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm. - The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference. - An AI/ML model used in a Model Inference function has to be initially trained, validated and tested by the Model Training function before deployment. - NG-RAN SA is prioritized; EN-DC and MR-DC are down-prioritized, but not precluded from Rel.18. - Functional framework and high-level procedures defined in this TR should not prevent from “thinking beyond” them during normative phase if a use case requires so. - User data privacy and anonymisation should be respected during AI/ML operation.

Accordingly, we propose to create a new section on “High-Level Principles” in clause 7 of TR 38.760-3, and to capture there the principles agreed in RAN3.

Proposal 2: Create a new section on “High-Level Principles” in clause 7 of TR 38.760-3, and capture there the AI/ML principles agreed in RAN3.

We can immediately note that principle 1) was already agreed at the RAN3#129-bis meeting in the following form:

The design of AI/ML algorithms and models for RAN3 led use cases are implementation specific and out of RAN3 scope.

Proposal 3: Capture in the new section on “High-Level Principles” in clause 7 of TR 38.760-3 the following principle:

- The design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope.

Regarding other principles from TR 37.817 reported above, we note that principle 10) would not be applicable to the work in 6G RAN. At the same time, principle 11) is not applicable at least for the time being, given that RAN3 is still not working on a framework for AI/ML.

The remaining principles are generic, useful to discuss and potentially agree in RAN3. in that respect, we have taken the principles from TR 37.817 and reworded them to adapt to the 6G AI/ML discussions in RAN3.

Proposal 4: Discuss the following list of high-level principles for 6G RAN AI/ML use case study:

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs/performance monitoring data.

- The input/output/performance monitoring data and the location of the Model Training and Model Inference function should be studied case-by-case.

- The study focuses on the analysis of data needed at the Model Training function from a data producer function, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope.

- The study focuses on the analysis of data needed at the Model Inference function from a data producer function, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope.

- Where AI/ML functionality resides within the 6G RAN architecture, depends on deployment and on the specific use cases.

- The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm.

- The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference.

- An AI/ML model used in a Model Inference function must be initially trained, validated and tested by the Model Training function before deployment.

- User data privacy and anonymization should be respected during AI/ML operation.

A pCR for TR 38.760-3 reflecting the proposals above is provided in the Annex.

- Conclusions

In this paper we discuss the high-level principles of 6G RAN AI/ML use cases. The following is proposed:

Proposal 1: 6G AI/ML data collection use case descriptions in TR 38.760-3 should be based on certainties, i.e., based on the specification progress of 6G RAN architecture and on the progress of 6G RAN features subject to AI/ML use cases.

Proposal 2: Create a new section on “High-Level Principles” in clause 7 of TR 38.760-3, and capture there the AI/ML principles agreed in RAN3.

Proposal 3: Capture in the new section on “High-Level Principles” in clause 7 of TR 38.760-3 the following principle:

- The design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope.

Proposal 4: Discuss the following list of high-level principles for 6G RAN AI/ML use case study:

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs/performance monitoring data.

- The input/output/performance monitoring data and the location of the Model Training and Model Inference function should be studied case-by-case.

- The study focuses on the analysis of data needed at the Model Training function from a data producer function, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope.

- The study focuses on the analysis of data needed at the Model Inference function from a data producer function, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope.

- Where AI/ML functionality resides within the 6G RAN architecture, depends on deployment and on the specific use cases.

- The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm.

- The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference.

- An AI/ML model used in a Model Inference function must be initially trained, validated and tested by the Model Training function before deployment.

- User data privacy and anonymization should be respected during AI/ML operation.

- Annex: pCR for TR 38.760-3

-------------------------------------------Start of changes-------------------------------------------

# 7	AI/ML for RAN

## 7.1	High-Level principles

The following high-level principles apply for the 6G RAN AI/ML study:

- The design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope.

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs/performance monitoring data.

- The input/output/performance monitoring data and the location of the Model Training and Model Inference function should be studied case-by-case.

- The study focuses on the analysis of data needed at the Model Training function from a data producer function, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope.

- The study focuses on the analysis of data needed at the Model Inference function from a data producer function, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope.

- Where AI/ML functionality resides within the 6G RAN architecture, depends on deployment and on the specific use cases.

- The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm.

- The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference.

- An AI/ML model used in a Model Inference function must be initially trained, validated and tested by the Model Training function before deployment.

- User data privacy and anonymization should be respected during AI/ML operation.

## 7.2	AI/ML use cases

Editor’s note: Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

## 7.3	AI/ML framework

-------------------------------------------End of changes-------------------------------------------