3GPP TSG-RAN WG3 Meeting #130	R3-258271

# Dallas, USA, 17 – 21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1645 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of AI/ML-based CCO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ofinno, Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CCO: - Need to simplify signalling of future coverage modification to neighbour DUs - Insufficient extendibility of the Future Coverage Modification Cause |  |  |  |  |  |  |  |  |
| Summary of change: |  | CCO: - Added Future Coverage Modification Cause to the Neighbour Future Coverage Modification Notification IE - Separate ASN.1 definition for the Future Coverage Modification Cause IE Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR is non backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and wrong specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.369, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.3.1.369	Neighbour Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by one or more neighbour node(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Future Coverage Modification Notification List |  | 1 |  |  |
| >Neighbour Future Coverage Modification Notification Item |  | 1..< maxNeighbourCellReport > |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Neighbour Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Neighbour Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Neighbour Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Neighbour Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Neighbour Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |
| Future Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, cancel, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxNeighbourCellReport | Maximum no. neighbour cells for which the future neighbour state can be reported. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<< next change >>>

### 9.4.5	Information Element Definitions

-- ASN1START

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

FlowsMappedToSLDRB-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Future-Coverage-Modification-Notification-Cause ::= ENUMERATED {coverage, cell-edge-capacity, cancel, ...}

Future-Coverage-Modification-Notification ::= SEQUENCE {

future-coverage-Modification-List			Future-Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Future-Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxCellingNBDU)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

futurecellCoverageState				FutureCellCoverageState,

futureSSBCoverageModificationList	FutureSSBCoverageModification-List	OPTIONAL,

timeforFutureCoverageModification	TimeforFutureCoverageModification	OPTIONAL,

futureCoverageModificationCause		Future-Coverage-Modification-Notification-Cause		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FutureCellCoverageState ::= INTEGER (0..63, ...)

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

Neighbour-Future-Coverage-Modification-Notification ::= SEQUENCE {

neighbour-future-coverage-Modification-List			Neighbour-Future-Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Neighbour-Future-Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Neighbour-Future-Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Neighbour-Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1.. maxnoofCellsinNG-RANnode)) OF Neighbour-Future-Coverage-Modification-Item

Neighbour-Future-Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

neighbourfuturecellCoverageState				NeighbourFutureCellCoverageState,

neighbourfutureSSBCoverageModificationList	NeighbourFutureSSBCoverageModification-List	OPTIONAL,

timeforneighbourFutureCoverageModification	TimeforNeighbourFutureCoverageModification	OPTIONAL,

futureCoverageModificationCause				Future-Coverage-Modification-Notification-Cause		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Neighbour-Future-Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Neighbour-Future-Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

maxnoofCellsinUEHistoryInfo				INTEGER ::= 16

maxnoofLTMCSI-RSResourceConfig			INTEGER ::= 112

maxnoofCSI-RSs							INTEGER ::= 192

maxnoofTAs								INTEGER ::= 2

maxnoofChannelRes						INTEGER ::= 24

maxNeighbourCellReport					INTEGER ::= 512

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

<<< end of changes >>>