3GPP TSG-RAN WG3 Meeting # 130	 R3-258312

# Dallas, USA,  17th - 21th November 2025

Agenda item:	9.2.5

Source:	Google

Title:	Discussion on supporting inter-CU SCG LTM and the L2 reset function in inter-CU LTM

Document for:	Discussion & Decision

- Introduction

To facilitate inter-CU MCG LTM, RAN3 has introduced several LTM-related signaling procedures within the Xn Application Protocol (XnAP) [1]. These procedures include:

- inter-CU LTM preparation (extend Handover Preparation procedure)

- inter-CU Cell Switch notification (new Cell Switch Notification procedure)

- subsequent LTM (new LTM Configuration Update procedure)

- inter-CU LTM with NR-DC (extend SN Addition and MN-initiated SN Modification procedures)

This paper aims to provide observations on the RRC specification and XnAP specification, identify potential issues, and provide proposals to address these issues.

- Discussion

## 2.1 	Inter-CU MCG LTM

RAN3 has extended the Handover Preparation procedure within the XnAP to support inter-CU MCG LTM, as specified in the TS 38.423v19.0.0 [1]. The following are the added signallings excerpted from the XnAP specification.

#### 9.1.1.1 HANDOVER REQUEST <unrelated part omitted> LTM Handover Information Request; O; YES; reject. >Proposed LTM No Security Change ID List; O; LTM No Security Change ID List 9.2.3.231; –. >Target NG-RAN node UE XnAP ID; O; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the target NG-RAN node.; –. >Reference Configuration; O; OCTET STRING; Includes the ltm-ReferenceConfiguration as defined in TS 38.331 [10].; –. >LTM Configuration ID Mapping List; O; 9.2.3.221; –. >CSI Resource Configuration; O; 9.2.3.223; –. >Request for CSI-RS Resource Configuration for L1 Measurements; O; ENUMERATED (true, …); –. >Request for CSI-RS Resource Configuration for Early CSI Acquisition; O; ENUMERATED (true, …); –. Early Sync Information Request; O; 9.2.3.217; YES; ignore. 9.1.1.2 HANDOVER REQUEST ACKNOWLEDGE <unrelated part omitted> LTM Handover Information Request Acknowledge; O; YES; ignore. >SSB Information; M; 9.2.3.228; –. >TCI States Configurations List; M; OCTET STRING; Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10].; –. >LTM Candidate Configuration; M; OCTET STRING; Includes the ltm-CandidateConfig as defined in TS 38.331 [10].; –. >LTM No Security Change ID; M; INTEGER(1..9,…); Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10].; –. >Complete Candidate Configuration Indicator; O; ENUMERATED (complete, ...); –. >CSI-RS Resource Configuration for L1 Measurements; O; 9.2.3.224; –. >CSI-RS Resource Configuration for Early CSI Acquisition; O; 9.2.3.224; –. >LTM CFRA Resource Information; O; 9.2.3.232; –. Early Sync Information Response; O; Early Sync Information 9.2.3.218; YES; ignore.

38.423 mandates the inclusion of the HandoverPreparationInformation inter-node message within the Handover Request during the Handover Preparation procedure. However, the current RRC specification [2] does not define any LTM-specific signaling elements within the HandoverPreparationInformation message. In fact, the RRC specification [2] introduces no new signaling for inter-CU MCG LTM within any inter-node messages. Consequently, we observe:

Observation 1: For inter-CU MCG LTM, LTM-related configurations, such as reference configuration, Channel State Information (CSI) resource configuration, and LTM candidate configuration, are exchanged via XnAP Information Elements (IEs). There is no signalling overlap (i.e., same IEs) in XnAP and HandoverPreparationInformation inter-node message.

## 2.2 	Inter-CU SCG LTM

The RAN3 baseline CR to 38.423 [1] adds LTM specific signalling elements in the SN Addition and SN Modification procedures for inter-CU SCG LTM in 38.423 as shown below.

#### 9.1.2.1 S-NODE ADDITION REQUEST <unrelated part omitted> LTM Candidate PSCell Addition Information Request; O; YES; reject. >LTM Request Indication; M; ENUMERATED (request, ...); –. >Maximum Number of PSCells To Prepare; M; INTEGER (1..8, …); Indicates the maximum number of PSCells that the candidate SN may prepare.; –. >Suggested LTM Candidate PSCell List; M; LTM Candidate PSCell Request List 9.2.3.242; –. >Proposed LTM No Security Change IDs; M; LTM No Security Change ID List 9.2.3.231; Indicates the LTM No Security Change IDs to be assigned during the preparation of candidate PSCells.; –. >CSI Resource Configuration; O; 9.2.3.223; –. >SCG Reference Configuration Request; O; ENUMERATED (request, …); Indicates that the reference configuration for LTM is requested.; –. >LTM Configuration ID Mapping List; O; 9.2.3.221; –. #### 9.1.2.2 S-NODE ADDITION REQUEST ACKNOWLEDGE <unrelated part omitted> LTM Candidate PSCell Addition Information Acknowledge; O; YES; reject. >LTM Candidate PSCell Prepared List; M; 9.2.3.243; –. #### 9.1.2.5 S-NODE MODIFICATION REQUEST <unrelated part omitted> LTM Candidate PSCell Information Update Request; O; YES; ignore. >Multiple S-NG-RAN Node List; 1; –. >>Multiple S-NG-RAN Node Item; 1 .. <maxnoofTargetSNs>; –. >>>S-NG-RAN node ID; M; Global NG-RAN Node ID 9.2.2.3; –. >>>LTM Candidate PSCell Prepared List; O; 9.2.3.243; –. >CSI Resource Configuration; O; 9.2.3.223; –. >LTM Configuration ID Mapping List; O; 9.2.3.221; –. >Proposed LTM No Security Change IDs; O; LTM No Security Change ID List 9.2.3.231; Indicates the LTM No Security Change IDs to be assigned during the preparation of candidate PSCells.; –. >LTM SCG Security Configuration; O; 9.2.3.241; –. >LTM Candidate PSCell To be Cancelled List; O; LTM Candidate PSCell Request List 9.2.3.242; –. #### 9.1.2.6 S-NODE MODIFICATION REQUEST ACKNOWLEDGE <unrelated part omitted> LTM Candidate PSCell Information Update Acknowledge; O; YES; ignore. >LTM Candidate PSCell Prepared List; O; 9.2.3.243; –. >CSI Resource Configuration; O; 9.2.3.223; –. >LTM Configuration ID Mapping List; O; 9.2.3.221; –. <unrelated part omitted> #### 9.2.3.242 LTM Candidate PSCell Request List This IE provides the information related to the requested LTM candidate PSCell(s). IE/Group Name; Presence; Range; IE type and reference; Semantics description. LTM Candidate PSCell Request List; 1. >LTM Candidate PSCell Request Item; 1 .. < maxnoofLTMCells>. >>PSCell ID; M; NR CGI 9.2.2.7. >>Early Sync Information Request; O; 9.2.3.xx1. >>Request for CSI-RS Resource Configuration for L1 Measurements; O; ENUMERATED (true, …). >>Request for CSI-RS Resource Configuration for Early CSI Acquisition; O; ENUMERATED (true, …). Range bound; Explanation. maxnoofLTMCells; Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8.. <<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>> #### 9.2.3.243 LTM Candidate PSCell Prepared List This IE provides the information related to the prepared LTM candidate PSCell(s). IE/Group Name; Presence; Range; IE type and reference; Semantics description. LTM Candidate PSCell Prepared List; 1. >LTM Candidate PSCell Prepared Item; 1 .. < maxnoofLTMCells>. >>PSCell ID; M; NR CGI 9.2.2.7. >>LTM No Security Change ID; M; INTEGER (0.. 9, …); Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10]. >>TCI States Configurations List; O; OCTET STRING; Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10].. >>Early Sync Information; O; 9.2.1.xx2. >>LTM CFRA Resource Information; O; 9.2.3.xx16. >>SSB Information; O; 9.2.3.xx13. >>CSI-RS Resource Configuration for Layer 1 Measurements; O; CSI-RS Resource Configuration 9.2.3.xx8. >>CSI-RS Resource Configuration for Early CSI Acquisition; O; CSI-RS Resource Configuration 9.2.3.xx8. >>Complete Candidate Configuration Indicator; O; ENUMERATED (complete, ...). Range bound; Explanation. maxnoofLTMCells; Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8..

Distinct from the inter-CU MCG LTM handling, where no new RRC signaling is introduced in inter-node messages, the Rel-19 RRC [2] incorporates new signaling elements for inter-CU SCG LTM within the CG-Config and CG-ConfigInfo inter-node messages as shown below.

#### – CG-Config <unrelated part omitted> CG-Config-v19xy-IEs ::= SEQUENCE { ltm-ReferenceConfigurationSCG-r19 ReferenceConfiguration-r18 OPTIONAL, ltm-Config-r19 LTM-Config-r18 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } <unrelated part omitted> requestedInterSN-LTM-r19 ENUMERATED{true} OPTIONAL <unrelated part omitted> ltm-ReferenceConfigurationSCG The field contains the LTM reference configuration to be used at the SCG. This field is only used in NR-DC.. ltm-Config The field contains the LTM candidate configuration(s) to be used at the SCG. This field is only used in NR-DC.. <unrelated part omitted>. requestedInterSN-LTM Used to request to configure inter-SN LTM candidate configuration(s) at the SCG. This field is only used in NR-DC.. #### – CG-ConfigInfo <unrelated part omitted> CG-ConfigInfo-v19xy-IEs ::= SEQUENCE { ltm-ReferenceConfigurationMCG-r19 ReferenceConfiguration-r18 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } <unrelated part omitted> [[ allowedInterSN-LTM-r19 ENUMERATED{allowed} OPTIONAL ]] <unrelated part omitted> allowedInterSN-LTM Used to indicate whether the SCG can configure inter-SN LTM candidate configuration. If the field is absent the SN is not allowed to configure inter-SN LTM candidate configurations. This field is only used in NR-DC.. <unrelated part omitted>. ltm-ReferenceConfigurationMCG The field contains the LTM reference configuration to be used at the MCG. This field is only used in NR-DC..

As specified in 38.423, the CG-ConfigInfo is included in the SN Addition Request message to the candidate SN and SN Modification Request message to the candidate SN or to the source SN. As shown above, there is no signalling overlap in the CG-ConfigInfo, the SN Addition Request message and SN Modification Request message.

Observation 2: For inter-CU SCG LTM, there is no signaling overlap in the CG-ConfigInfo, the SN Addition Request message and SN Modification Request message.

As specified in 38.423, the CG-Config is included in the SN Addition Request Acknowledge message and SN Modification Request Acknowledge message. As shown above, the LTM-Config is added in the CG-Config in theRel-19 RRC specification [2]. The following table shows signalling overlaps in the LTM-Config in CG-Config, SN Addition Request Acknowledge message, and SN Modification Request Acknowledge message.

| LTM-Config in CG-Config in SN Addition Request Acknowledge message or SN Modification Request Acknowledge message | SN Addition Request Acknowledge message | SN Modification Request Acknowledge message |
| --- | --- | --- |
| The following configurations are or may be included in each LTM-Candidate for a corresponding LTM candidate PSCell: - ltm-CandidateId - ltm-CandidatePCI - ltm-SSB-Config - ltm-CandidateConfig - ltm-ConfigComplete - ltm-EarlyUL-SyncConfig (for UL) - ltm-EarlyUL-SyncConfigSUL - ltm-TCI-Info - ltm-NoResetID-r18 - ltm-UE-MeasuredTA-ID - ltm-NoSecurityChangeID-r19 - ltm-ExecutionCondition - ltm-NZP-CSI-RS-ResourceToAddModList - ltm-NZP-CSI-RS-ResourceToReleaseList - ltm-NZP-CSI-RS-ResourceSetToAddModList - ltm-NZP-CSI-RS-ResourceSetToReleaseList - ltm-CSI-ReportConfig-r19 - ltm-CSI-IM-ResourceToAddModList - ltm-CSI-IM-ResourceToReleaseList - ltm-CSI-IM-ResourceSetToAddModList - ltm-CSI-IM-ResourceSetToReleaseList | The following configurations are included as XnAP IEs for each LTM candidate PSCell: - Assuming that the scg-CellGroupConfig in the CG-Config or CG-CandidateList in the mandatory S-NG-RAN node to M-NG-RAN node Container to carry the SCG candidate configuration for LTM can be reused - PSCell ID - LTM No Security Change ID - TCI States Configurations List - Early Sync Information - LTM CFRA Resource Information - SSB Information - CSI-RS Resource Configuration for Layer 1 Measurements - CSI-RS Resource Configuration for Early CSI Acquisition - Complete Candidate Configuration Indicator | The following configurations are included as XnAP IEs for each LTM candidate PSCell: - Assuming that the scg-CellGroupConfig in the CG-Config or CG-CandidateList in the mandatory S-NG-RAN node to M-NG-RAN node Container to carry the SCG candidate configuration for LTM can be reused - PSCell ID - LTM No Security Change ID - TCI States Configurations List - Early Sync Information - LTM CFRA Resource Information - SSB Information - CSI-RS Resource Configuration for Layer 1 Measurements - CSI-RS Resource Configuration for Early CSI Acquisition - Complete Candidate Configuration Indicator |
| The following configurations are included as the first level IEs of LTM-Config: - ltm-ReferenceConfiguration - ltm-CandidateToReleaseList - ltm-CandidateToAddModList - ltm-ServingCellNoResetID - ltm-CSI-ResourceConfigToAddModList - ltm-CSI-ResourceConfigToReleaseList - attemptLTM-Switch - ltm-ServingCellUE-MeasuredTA-ID - ltm-ServingCellNoSecurityChangeID - ltm-ServingCellExecutionCondition |  | The following configurations shall be signalled as XnAP IEs from the source SN to the MN after the intial LTM preparation(s) at the candidate SN(s): - CSI Resource Configuration - LTM Configuration ID Mapping List |
| The ltm-ReferenceConfigurationSCG-r19 is included as a CG-Config v19 IE. |  |  |

As shown in table above, we can oberserve:

Observation 3: For inter-CU SCG LTM, there are signalling overlaps in the CG-Config (specifically, the LTM-Config), the SN Addition Request Acknowledge message and SN Modification Request Acknowledge message.

The signalling overlaps lead to the following issues, for example.

- In the SN Addition Request Acknowledge message, should the candidate SN use the LTM-Config or XnAP IEs to provide LTM-TCI-Info, Early UL synchronziatio configurations for UL and/or SUL, and the CSI-RS resources, … etc.?

- In the SN Modification Request Acknowledge message, should the candidate SN use the LTM-Config or XnAP IEs to provide a SCG Configuration for a LTM candidate PSCell?

Additionally, the ltm-CandidateId is a mandatory field in the ltm-Candidate IE in the LTM-Config and should be assigned by the source SN in the DC scenario; therefore, the candidate SN may not be able to fill it properly in the ltm-Candidate and the MN needs to handle it properly (e.g., ignore the ltm-CandidateId). There are also other optional fields (e.g., the CSI Resource Configurations) that should only be signalled from the MN to the UE or from the source SN to the MN and not suitable to be signalled from the candidate SN to the MN.

From the standardization perspective, we should avoid specifying mulitple options for the same purpose. The signaling overlaps just create complexity in the gNB implementation, so we don’t prefer the signalling overlaps for the same purpose as indicated above. Therefore, we propose:

Proposal 1: 	RAN3 should discuss and decide on the specific configuration IEs to be included within XnAP IEs or the inter-node CG-Config message.

To maintain consistent design logic, we propose utilizing the same design approach for exchanging LTM-related information in inter-CU MCG LTM and inter-CU SCG LTM preparation. Using the exchange of LTM-related information via the XnAP IEs is more preferrable from our point of view. A draft LS reflecting such viewpoint is attached in the appendix of this discussion paper.

Proposal 2: 	If RAN3 decides to keep the XnAP IEs, RAN3 should send a LS to RAN2 to inform the RAN3’s decision.

## 2.3 	L2 Reset Issues in inter-CU LTM

In LTM preparations and the RRC reconfiguration to the UE, the netwrok can also configure the ltm-NoResetID associated with the candidate cell in the ltm-Candidate IE. In Rel-18, RAN3 had introduced the LTM Reset Information IE in the UE Context Modification procedure in the F1AP.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID.

If the LTM Reset Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch as specified in TS38.331 [8].

However, it has not been discussed in RAN3 how to support this mechanism in Rel-19 inter-CU LTM. For inter-CU MCG LTM, the source gNB also needs to configure the ltm-NoResetID to the UE; however, as the real deployment or architecture at the candidate gNB may be hidden from the source gNB, the candidate gNB has better knowledge about whether a L2 reset would be required between the candidate cells served by the candidate gNB than the source gNB. Although it is likely this LTM reset information is not needed for the network side intially (i.e., before the cell switch to the candidate gNB), especially for the cells served by the candidate gNB(s), as any inter-CU cell switch shall incur a L2 reset. As the UE still relies on this information to decide whether to perform L2 reset and assuming that follow-up RRC reconfigurations should be avoided for subsequent LTM, this information shall be ready during the LTM reconfiguration.

Observation 4: For inter-CU LTM, the source gNB or MN should also configure the L2 reset IDs associated with candidate cells in the candidate gNB or SN to the UE.

###### Option 1: The candidate gNB or SN provides a LTM No Reset ID associated with the candidate cell or candidate PSCell to the source gNB or MN, respectively; The source gNB or MN resolves the shared LTM No Reset ID issues between the candidate gNBs or SNs and provides an update to the candidate gNBs or SNs.

In the preparation stage, for example, the candidate gNB-CU shall provide a ltm-NoResetID1 for the cell 1 and a ltm-NoResetID2 for the cell2. The candidate gNB can set the values of ltm-NoResetID1 and ltm-NoResetID2 to be the same or different and that is actually what needs to be informed to the source gNB. With the knowledge of whether a value of the ltm-NoResetID1 for a cell1 and a value of the ltm-NoResetID2 for a cell2 from this candidate gNB and similarly from other candidate gNBs during the inter-CU LTM preparation phase, the source gNB is able to correctly configure the UE with a suitable value for the ltm-NoResetID associated with each of the candidate cell. For the network side LTM updating phase for subsequent LTM, the source gNB can either provide a full picture of LTM Reset Information including the ltm-ServingCellNoResetID or simply provide a final value of the ltm-NoResetID associated with the candidate cells to the candidate gNB so that the candidate gNB and the UE are on the same page about whether to perform a L2 Reset during a cell switch.

In a post-meeting offline discussion regarding this option, there was a comment that the source gNB or the MN may not need to request the LTM No Reset ID. From RRC point of view this field is optional, however, it is also noted that if one candidate cell is configured every other candidate cells needs to be configured as well. From network point of view, since the source or the MN needs to otain the information to properly set the LTM No Reset ID. Therefore, we are also open that the LTM No Reset ID for each candidate cell is proactively provided from the candidate gNB to the source gNB or from the candidate SN to the MN.

Therefore, we propose the following for the option 1:

Proposal 3:	RAN3 to discuss the associated CR in R3-258313 for option 1 to resolve the L2 reset issue.

Proposal 3-1: 	In inter-CU MCG LTM, the candidate gNB shall provide a LTM No Reset ID associated with the candidate cell.

Proposal 3-2: 	In inter-CU SCG LTM, the candidate SN shall provide a LTM No Reset ID associated with each prepared candidate PSCell.

###### Option 2: The source gNB or MN provides a Proposed LTM No Reset ID list associated with the candidate cells or candidate PSCells to the candidate gNB or SN, respectively.

In the post-meeting offline discussion, there were also comments on whether to follow a similar method as the Rel-19 ID (i.e., the LTM No Security Change ID) to resolve this issue. Although this could work and beneficial in having the source or the MN to make sure the uniqueness of LTM Reset ID, we observe some drawbacks with this method against the above candidate proposed method (i.e., Option 1). In the inter-CU MCG LTM, the current specifcation requires the Option 2 (i.e., the Rel-19 Set ID method) to have the source to provide a Proposed LTM No Security Change ID List IE which is a list with at most 8 IDs to the target.

For inter-CU MCG LTM, as only one target cell can be prepared by a Handover Preparation procedure, it is unclear in the specification whether the source gives one LTM No Security Change ID or more if there are also other target cell(s) to be prepared at the target. Although it mentioned “act as specified in TS 38.300 [9]”, there is no clear guidance captured in the TS 38.300 actually.

For inter-CU SCG LTM, as the MN only suggests a list of PSCells and sets the maximum number of PSCells the candidate SN can prepare, the exact PSCells shall be determined by the candidate SN. Therefore, the MN may inevitably give an execess number of distinct LTM No Security Change IDs to accommodate the case that every suggested PSCell belongs to a different DUs. The MN can only utilize the unused ones after a SN Addition preparation procedure with a candidate SN.

The relavant pargaphs are recapped below for easier reference.

Recapped from TS 38.423v19.0.0 #### 9.2.3.231 LTM No Security Change ID List This IE contains the UE security related information for LTM. IE/Group Name; Presence; Range; IE type and reference; Semantics description. LTM No Security Change ID List; 1.. <maxnoofLTMCells>. >LTM No Security Change ID; M; INTEGER(1..9,…); Corresponds to information provided in the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10]. Range bound; Explanation. maxnoofLTMCells; Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8..

Recapped from TS 38.423v19.0.0 If the Proposed LTM No Security Change ID List IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM No Security Change ID IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9]. …

Recapped from TS 38.300v19.0.0 3. The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-gNB LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and may contain the CSI resource configuration for subsequent LTM. For both intra and inter-gNB LTM, the source gNB may request the candidate cell(s)/gNB(s) to provide the CSI-RS resource configuration for L1 RSRP measurement and/or for early CSI acquisition. The source gNB may include the LTM security information. For inter-gNB LTM, the source gNB includes the same source NG-RAN node UE XnAP ID for all HANDOVER REQUEST messages to a candidate gNB. 4. Admission Control may be performed by the candidate cells(s)/gNB(s). 5. The candidate prepares and provides the LTM configuration(s) to the source gNB. For inter-gNB LTM, the candidate gNB(s) respond(s) with HANDOVER REQUEST ACKNOWLEDGE message to the source gNB including the generated RRC configurations for the accepted candidate cell. For both intra and inter-gNB LTM, the candidate may also include additional information related to the CSI-RS resource configuration and early sync information upon request. The candidate gNB also responds the selected LTM security information. For inter-gNB LTM, each candidate gNB includes the same target NG-RAN node UE XnAP ID for all HANDOVER REQUEST ACKNOWLEDGE messages it responds.

Recapped from TS 37.340v19.0.0 2. The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN provides the list of LTM No Security Change ID which can be used in each candidate SN, and forwards the received measurement results to each candidate SN(s). The MN may transfer L1 RS configuration request and/or early sync information for suggested candidate PSCell(s) to SN. The MN also provides upper limit for the number of PSCells to each candidate SN. The MN may select one of the candidate SN(s) and request providing the SCG reference configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s). NOTE 7: The subsequent SCG LTM configurations can be included within an MN or an SN RRC message. For intra-SN SCG LTM in MN RRC message case, the source SN and the target SN shown in Figure 10.5.2-5 are the same node, and step 2 can be skipped. 3. The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by each candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement or CSI acquisition, early UL/DL sync configuration, by sending SN Addition Request Acknowledge message to the MN. The candidate SN provides the assigned LTM No Security Change ID for each candidate PSCell.

To help RAN3 discuss and decide the options to resolve the L2 reset issue, we also prepare another CR covering the option 2. Instead of providing a list of proposed L2 Reset IDs and candidate cell(s) in the request message, we made some changes to the Rel-19 ID solution so that the number of suggested candidate cell and the number of proposed LTM No Reset ID is matched. The CR is provided in R3-258317 for option 2.

Proposal 4:	RAN3 to discuss the associated CR in R3-258317 for option 2 to resolve the L2 reset issue.

# Conclusion

We provide the following observations in this paper.

Observation 1: For inter-CU MCG LTM, LTM-related configurations, such as reference configuration, Channel State Information (CSI) resource configuration, and LTM candidate configuration, are exchanged via XnAP Information Elements (IEs). There is no signaling overlap (i.e., same IEs) in XnAP and HandoverPreparationInformation inter-node message.

Observation 2: For inter-CU SCG LTM, there is no signalling overlap in the CG-ConfigInfo, the SN Addition Request message and SN Modification Request message.

Observation 3: For inter-CU SCG LTM, there are signalling overlaps in the CG-Config, the SN Addition Request Acknowledge message and SN Modification Request Acknowledge message.

Observation 4: For inter-CU LTM, the source gNB or MN should also configure the L2 reset IDs associated with candidate cells in the candidate gNB or SN to the UE.

Based on the observations, we identifies the issues and provides the following proposals. RAN3 is kindly to discuss and agree the following proposals and the corresponding CR.

Proposal 1: 	RAN3 should discuss and decide on the specific configuration IEs to be included within XnAP IEs or the inter-node CG-Config message.

Proposal 2: 	If RAN3 decides to keep the XnAP IEs, RAN3 should send a LS to RAN2 to inform the RAN3’s decision.

Proposal 3:	RAN3 to discuss the associated CR in R3-258313 for option 1 to resolve the L2 reset issue.

Proposal 3-1: 	In inter-CU MCG LTM, the candidate gNB shall provide a LTM No Reset ID associated with the candidate cell.

Proposal 3-2: 	In inter-CU SCG LTM, the candidate SN shall provide a LTM No Reset ID associated with each prepared candidate PSCell.

Proposal 4:	RAN3 to discuss the associated CR in R3-258317 for option 2 to resolve the L2 reset issue.

# Reference

[1] 3GPP TS 38.423v19.0.0

[2] 3GPP TS 38.331v19.0.0

[3] R2-2501977, Discussion on inter-CU LTM, Google

[4] R3-258313, Correction to L2 reset in inter-CU LTM, Google, Nokia, CATT, Ericsson

[5] R3-258317, Correction to L2 reset in inter-CU LTM (option 2), Google, Huawei

# Appendix – draft LS to RAN2 on inter-node message

3GPP TSG-RAN WG3 Meeting # 130	 R3-25XXXX

# Dallas, USA,  17th - 21th November 2025

Title:	LS on inter-node signaling for the inter-CU SCG LTM

Response to:

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	Google (to be RAN3)

To:	RAN2

Cc:

Contact person:	Jing-Rong Hsieh

jinghsieh@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 had discussed the necessary parameters required for the inter-CU SCG LTM and designed the related XnAP IEs in the LTM Candidate PSCell Prepared List. However, some signaling duplications regarding the inter-CU SCG LTM are also detected in the inter-node RRC message (i.e., the CG-Config) in the RRC specification.

As the RAN3 would like to follow the same signaling design for both the inter-CU MCG LTM and the inter-CU SCG LTM, while the duplications only appear in the inter-CU SCG LTM, RAN3 seeks to resolve this issue with RAN2. From RAN3 point of view, it would be sufficient to use the XnAP IEs to signal the necessary parameters.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to take the above RAN3 feedback into account.

# 3	Dates of next RAN WG 3 meetings

RAN3#131	09 – 13 February 2026	Stor-Göteborg, SE

RAN3#131-bis	13 – 17 April 2026	Malta, MT