3GPP TSG-RAN WG3 Meeting #130	R3-258314

Dallas, US, 17-21 Nov, 2025

Title: 	Discussion on allocation of CN assigned subgroup ID for LP-WUS

Source: 	Huawei, Qualcomm, Vivo

Agenda item:	9.2.9

Document Type:	Discussion

# 1. Introduction

As the reply LS to [1], SA2#171 meeting agreed the LS on the allocation of CN assigned subgroup ID for LP-WUS, with the following contents [2].

1. Overall Description: SA2 thanks RAN3 for the LS (S2-2508191/R3-255941) on allocation of CN assigned subgroup ID for LP-WUS. The LS highlights the following: # Overall description Regarding the range of CN assigned subgroup ID for LP-WUS, RAN3 made the following agreements: - The range of CN assigned subgroup ID in NG/Xn/F1 message is 0.. 30. This agreement means that when the AMF allocates CN assigned subgroup ID for the UE, it should consider the least range of subgroup ID available over NG-RAN cells corresponding to the UE registration area. RAN3 would like to provide following background information: according to the TS 38.213, for CN assigned subgroup ID range, dependent on the per-cell PO (Paging Occasion) to LO (LP-WUS Occasion) association: - in case PO-to-LO association is 4, subgroup ID #0 to #6 can be used;. - in case PO-to-LO association is 2, subgroup ID #0 to #14 can be used; - in case PO-to-LO association is 1, subgroup ID #0 to #30 can be used; RAN3 would like to ask SA2 to take the above into consideration, and discuss the potential specification impact, if any. # Actions To SA2: ACTION: RAN3 would like to ask SA2 to take the above into consideration, and discuss potential specification impact, if any. SA2 discussed the issue and would like to receive some further information from RAN3 and RAN2: a) What is the typical area that a PO-to-LO association configuration will apply to? (e.g., is the configuration uniform in a PLMN, different in every cell, etc, and is it mandatory for an LP-WUS capable gNB to support all 3 PO-to-LO association configurations?) This information will help SA2 to understand what kind of solution to propose. b) Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less? c) Will RAN3 specify the relation between subgroup ID value range and the PO-to-LO association (e.g. based on RAN1 specification)? This could avoid SA2 having to refer to a complex part of TS 38.213. SA2 would appreciate replies to the above questions. 2. Actions: To RAN3, RAN2 ACTION: SA2 kindly asks RAN3 to provide answers for a, c) and RAN2 to provide answers for b).

This contribution provides our views to answer those questions contained in the incoming LS.

# 2. Discussion

Given the reply LS is talking about the per-cell PO (Paging Occasion) to LO (LP-WUS Occasion) association, below provides the specification details of TS 38.331 and TS 38.213 for reference.

- In TS 38.331 DownlinkConfigCommonSIB ::= SEQUENCE { <<<<<<<<<<<<<<<<<<<< Irrelevant Text Omitted >>>>>>>>>>>>>>>>>>>> lpwus-MoNumPerLo-r19 ENUMERATED {n1, n2, n3, n4}, lpwus-PoNumPerLo-r19 ENUMERATED {po1, po2, po4} OPTIONAL, -- Need R LP-SubgroupConfig-r19 ::= SEQUENCE { lp-SubgroupsNumPerPO-r19 INTEGER (1.. maxNrofPagingSubgroupsLP-r19), lp-SubgroupsNumForUEID-r19 INTEGER (1.. maxNrofPagingSubgroupsLP-r19) OPTIONAL, -- Need S ... } lpwus-PoNumPerLo Indicates the number of PO(s) associated with one LO for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C). Value po1 means 1 PO associated with one LO, value po2 means 2 POs associated with one LO, and so on.. lp-SubgroupsNumPerPO Total number of subgroups per Paging Occasion (PO) for UE to monitor codepoint from LP-WUS signaling. The field represents the sum of CN-assigned and UEID-based subgroups supported for LP-WUS by the network. When lowPowerConfig is configured, there is always at least one subgroup (UEID-based subgroup or CN-assigned subgroup) configured. The maximum number of subgroups per PO is 31, 15, and 7 when the number of PO(s) associated with one LO for LP-WUS, i.e., lpwus-PoNumPerLo, is configured as 1, 2, and 4, respectively.. - In TS 38.213: ## 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE <<<<<<<<<<<<<<<<<<<< Irrelevant Text Omitted >>>>>>>>>>>>>>>>>>>> - A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on PO-to-LO association.

It can be observed that the PO to LO association could take 1, 2 or 4 broadcasted in SIB1 as a per Cell information, and, the maximum number of subgroups per PO is 31, 15, and 7 when the number of PO(s) associated with one LO for LP-WUS, i.e., lpwus-PoNumPerLo, is configured as 1, 2, and 4, respectively. This means that the cell can broadcast its own association, while UEs monitoring its own PO could monitor the associated LO accordingly.

For CN assigned subgrouping, it is expected that AMF is configured with a ‘CN subgroup ID range’, which is used for assigning LP-WUS CN subgroup IDs for the UEs, satisfies the range for all these cells, i.e. within the total number of subgroups for the cells. Also as indicated in the [1], the CN subgroup ID range can take the following values dependent on the PO-to-LO association.

- in case PO-to-LO association is 4, subgroup ID #0 to #6 can be used;

- in case PO-to-LO association is 2, subgroup ID #0 to #14 can be used;

- in case PO-to-LO association is 1, subgroup ID #0 to #30 can be used;

- Observation 1: The PO-to-LO association as enumerated 1, 2, 4 value is broadcasted as per cell information. This would lead to different the CN subgroup ID value range for different PO-to-LO association.

- Below provides our thinking towards those questions in [2].

- For question a) on the typical area that a PO-to-LO association configuration, as discussed above, this is per cell broadcast information. However, this association cannot be configured on a cell level basis. One example is that if the UE moves from a cell with PO-to-LO association as one to a cell where the association is four, then the CN subgroup ID larger than 6 cannot be used, which leads to paging loss, and results at the defective LP-WUS assisted paging.

- Hence a reasonable assumption is that the configuration should be pre-configured in a typical relatively large area. This configuration could be up to operator's policy or up to implementation, e.g., at least at the tracking area list level.

RAN3 to reply a) that the typical area of the PO-to-LO association configuration should be pre-configured (e.g., up to operator's policy) or up to implementation, e.g., at the tracking area list level. Whether an LP-WUS capable gNB supports all 3 PO-to-LO association configurations is also pre-configured (e.g., up to operator’s policy).

- For the question b) above, as discussed above, it is not expected that the paging PO-to-LO association configuration will be changed often. Even if it happens, the PO-to-LO association configuration should be changed for all cells within the preconfigured area. The final reply could be up to RAN2.

RAN3 to reply b) that from RAN3 perspective, the PO-to-LO association configuration should be changed for all cells within the preconfigured area if any change is needed. But final reply is up to RAN2.

- For the question c) above, we think RAN3 could specify the stage2 texts on the relation between CN subgroup ID value range and the PO-to-LO association in TS 38.300. Note that as quoted above, there are no exact descriptions on the relation neither in the TS 38.213, nor in the TS 38.331.

RAN3 to agree the stage 2 CR to specify the relation between CN subgroup ID value range and the PO-to-LO association, as well as the typical area of the PO-to-LO association configuration given in Proposal 1.

- Finally, it is proposed for RAN3 to agree the reply LS to answer those questions to SA2 as discussed above.

RAN3 to agree the reply LS containing the above thinking.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Observation 1: The PO-to-LO association taking 1, 2, 4 is broadcasted as per cell information, which will lead to different the CN subgroup ID value range.

- Observation 1: The PO-to-LO association as enumerated 1, 2, 4 value is broadcasted as per cell information. This would lead to different the CN subgroup ID value range for different PO-to-LO association.

- RAN3 to reply a) that the typical area of the PO-to-LO association configuration should be pre-configured (e.g., up to operator's policy) or up to implementation, e.g., at the tracking area list level. Whether an LP-WUS capable gNB supports all 3 PO-to-LO association configurations is also pre-configured (e.g., up to operator’s policy).

- RAN3 to reply b) that from RAN3 perspective, the PO-to-LO association configuration should be changed for all cells within the preconfigured area if any change is needed. But final reply is up to RAN2.

- RAN3 to agree the stage 2 CR to specify the relation between CN subgroup ID value range and the PO-to-LO association, as well as the typical area of the PO-to-LO association configuration given in Proposal 1.

- RAN3 to agree the reply LS containing the above thinking.

- The stage 2 CR and the draft reply LS can be found in [4, 5] respectively.

# 4. Reference

- R3-255941 LS on allocation of CN assigned subgroup ID for LP-WUS, RAN3

- S2-2509834 	Reply LS on allocation of CN assigned subgroup ID for LP-WUS, SA2

- R3-25xxxx	Allocation of CN assigned subgroup ID for LP-WUS, Huawei

- R3-25xxxx	[Draft] Reply LS on allocation of CN assigned subgroup ID for LP-WUS, Huawei