3GPP TSG-RAN WG3 Meeting # 130	 R3-258317

# Dallas, USA,  17th - 21th November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1624 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to L2 reset in inter-CU LTM (option 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For inter-CU LTM, the source gNB or the MN also needs to configure the L2 No Reset ID associated to the UE so that the UE is able to correctly perform L2 reset during a LTM cell switch. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the proposed L2 Reset ID and L2 Reset ID for each candidate PCell or PSCell for the inter-CU signaling: - Handover Preparation procedure - SN Addition procedure - Add IEs in the corresponding messages - Add corresponding ASN.1 changes Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the L2 reset and the related procedure text for the LTM operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The source gNB or the MN cannot properly configure the L2 No Reset ID and the candidate gNB or the candidate SN may be unsynchronized with the UE for the L2 reset. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1, 8.3.1.1, 9.1.1.1, 9.1.1.2, 9.1.5.1, 9.2.3.242, 9.2.3.243, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------First Change -------------------------------------------

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

If the procedure concerns an LTM, parallel transactions are allowed only when using the same source UE AP ID. Possible parallel requests are identified by the target cell ID.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

-------------------------------------------Skip unchanged -------------------------------------------

If the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the LTM information for the UE in the target NG-RAN node and include the LTM Handover Information Request Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Reference Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for generating the LTM configuration. If the Request for CSI-RS Resource Configuration for L1 Measurements IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for L1 Measurements IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for Early CSI Acquisition IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM Candidate Configuration IE for the requested candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Proposed LTM L2 Reset Configuration IE is included in the LTM Handover Information Request IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, decide the L2 Reset Configuration for the accepted candidate cell by taking the L2 Reset Configurations of other candidate cells of the UE into account and include it in the LTM L2 Reset Configuration IE in the LTM Handover Information Request Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell and act as specified in TS 38.300 [9].

If the Proposed LTM No Security Change ID List IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM No Security Change ID IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the Target NG-RAN node UE XnAP ID IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, consider that the source NG-RAN node requests LTM preparation for an already established UE association.

If the Early Sync Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the early synchronization information for the UE in the target NG-RAN node.

If the Early Sync Information Response IE is contained in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, store it and use it as defined in TS 38.300 [9].

If the LTM CFRA Resource Information IE is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [35].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the target NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, and the Geographical Area IE contains the MDT PLMN List IE, the target NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover via the Data Collection Reporting procedure and according to clause 8.4.13.2, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node,.

-------------------------------------------Next Change, Skip Unchanged ---------------------------------------

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

-------------------------------------------Next Change, Skip Unchanged ---------------------------------------

If the LTM Candidate PSCell Addition Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use the information to prepare for LTM candidate PSCell(s) configuration, and include the LTM Candidate PSCell Addition Information Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the Proposed LTM L2 Reset Configuration IE is included in the Suggested LTM Candidate PSCell List IE in the LTM Candidate PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, decide the L2 Reset Configuration(s) for the accepted candidate PSCell(s) for the UE and include the LTM L2 Reset Configuration IE in the LTM Candidate PSCell Prepared List IE in the LTM Candidate PSCell Addition Information Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message for the prepared LTM candidate PSCell(s).

If the CSI Resource Configuration IE is contained in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration.

If the SCG Reference Configuration Request IE set to "request" is contained in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, provide the SCG reference configuration for LTM.

If the LTM Configuration ID Mapping List IE is contained in in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider this as the mapping information for the suggested LTM candidate PSCell(s).

If the LTM Information SN Addition IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the S-NG-RAN node Addition Preparation procedure has been triggered as part of an MCG LTM.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

Interaction with the Data Collection Reporting Initiation and the Data Collection Reporting procedures:

If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, report to the M-NG-RAN node after successful S-NG-RAN node addition, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the S-NG-RAN node.

-------------------------------------------Next Change, Skip Unchanged ---------------------------------------

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. NOTE: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| >NR A2X UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| >LTE A2X UE PC5 Aggregate Maximum Bit Rate | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.2.3.108 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Conditional Handover Time Based Information | O |  |  | This IE only applies to NTN. | YES | reject |
| >>Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to information provided in the t1-Threshold contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >>Handover Window Duration | M |  | INTEGER (1..6000) | Corresponds to information provided in the duration contained in the condEventT1 contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >Maximum Number of Conditional Reconfigurations to Prepare | O |  | INTEGER (1..8 , …) | Indicates that the target NG-RAN node may prepare for CHO with candidate PSCell(s) configuration and indicates the maximum number of conditional reconfigurations that the target NG-RAN node may prepare. | YES | reject |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED(authorized, not authorized, ...) | This IE indicates the authorization status of the IAB node. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) | This IE indicates that information on DL LBT Failures occurring at the target gNB that results in mobility failure is requested | YES | ignore |
| Aerial UE Subscription Information | O |  | 9.2.3.175 |  | YES | ignore |
| NR A2X Services Authorized | O |  | 9.2.3.176 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.2.3.177 |  | YES | ignore |
| A2X PC5 QoS Parameters | O |  | 9.2.3.178 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.180 | This IE contains information about cells that a UE is predicted to be connected to. | YES | ignore |
| Data Collection ID | O |  | 9.2.3.184 |  | YES | ignore |
| Candidate Relay UE Info List | O |  | 9.2.3.188 |  | YES | reject |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC configuration information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Mobile IAB Authorization Status | O |  | 9.2.2.105 |  | YES | reject |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.208 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| LTM Handover Information Request | O |  |  |  | YES | reject |
| >Proposed LTM No Security Change ID List | O |  | LTM No Security Change ID List 9.2.3.231 |  | – |  |
| >Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | – |  |
| >Reference Configuration | O |  | OCTET STRING | Includes the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >Request for CSI-RS Resource Configuration for L1 Measurements | O |  | ENUMERATED (true, …) |  | – |  |
| >Request for CSI-RS Resource Configuration for Early CSI Acquisition | O |  | ENUMERATED (true, …) |  | – |  |
| >Proposed LTM L2 Reset Configuration | O |  | INTEGER(1..9,…) | Corresponds to the ltm-NoResetID as defined in TS38.331 [10], for the LTM candidate cell identified by the Target Cell Global ID IE. | – |  |
| Early Sync Information Request | O |  | 9.2.3.217 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. Ignored if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >CHO-CPAC Information | O |  | 9.2.3.202 |  | YES | reject |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | ignore |
| PDU Set based Handling Indicator | O |  | 9.2.3.206 |  | YES | ignore |
| LTM Handover Information Request Acknowledge | O |  |  |  | YES | ignore |
| >SSB Information | M |  | 9.2.3.228 |  | – |  |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. | – |  |
| >LTM Candidate Configuration | M |  | OCTET STRING | Includes the ltm-CandidateConfig as defined in TS 38.331 [10]. | – |  |
| >LTM No Security Change ID | M |  | INTEGER(1..9,…) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10]. | – |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | – |  |
| >CSI-RS Resource Configuration for L1 Measurements | O |  | 9.2.3.224 |  | – |  |
| >CSI-RS Resource Configuration for Early CSI Acquisition | O |  | 9.2.3.224 |  | – |  |
| >LTM CFRA Resource Information | O |  | 9.2.3.232 |  | – |  |
| >LTM L2 Reset Configuration | O |  | INTEGER(1..9,…) | Corresponds to the ltm-NoResetID as defined in TS38.331 [10], for the LTM candidate cell identified by the Target Cell Global ID IE. | – |  |
| Early Sync Information Response | O |  | Early Sync Information 9.2.3.218 |  | YES | ignore |

-------------------------------------------Next Change, Skip Unchanged ---------------------------------------

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 | This IE is ignored if the S-CPAC Request Information IE is present in the Conditional PSCell Addition Information Request IE or the LTM Candidate PSCell Addition Information Request IE is present. | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Source NG-RAN Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source NG-RAN node, or the source SN in e.g. NR-DC to NR-DC (conditional) handover. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >S-CPAC Request Information | O |  | 9.2.3.192 | Indicates that SN addition is for S-CPAC preparation. | YES | reject |
| >S-CPAC Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for S-CPAC is requested. | YES | ignore |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| Selected NID | O |  | NID 9.2.2.65 | This IE, together with the Selected PLMN IE, indicates the SNPN proposed for the SCG to the S-NG-RAN node. | YES | ignore |
| QMC Coordination Request | O |  | 9.2.3.197 | This IE contains information for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements at the S-NG-RAN node subject to addition. | YES | ignore |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Source M-NG-RAN node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source M-NG-RAN node in e.g. NR-DC to NR-DC handover. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | ignore |
| Data Collection ID | O |  | 9.2.3.184 |  | YES | ignore |
| LTM Candidate PSCell Addition Information Request | O |  |  |  | YES | reject |
| >LTM Request Indication | M |  | ENUMERATED (request, ...) |  | – |  |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the candidate SN may prepare. | – |  |
| >Suggested LTM Candidate PSCell List | M |  | LTM Candidate PSCell Request List 9.2.3.242 |  | – |  |
| >Proposed LTM No Security Change IDs | M |  | LTM No Security Change ID List 9.2.3.231 | Indicates the LTM No Security Change IDs to be assigned during the preparation of candidate PSCells. | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >SCG Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for LTM is requested. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| LTM Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

-------------------------------------------Next Change, Skip Unchanged ---------------------------------------

#### 9.1.5.1	LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| LTM Updates to Candidate Node Information |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration | O |  | OCTET STRING | Includes the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| LTM Updates to Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >LTM Updates to Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Early Sync Information | O |  | 9.2.3.218 | Indicates the consolidated early synchronization information to be used for subsequent LTM. | – |  |
| >>LTM CFRA Resource Information | O |  | 9.2.3.232 |  | – |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate cell identified by the Candidate Cell ID IE. | – |  |
| >>AS Security Information | O |  | 9.2.3.50 |  | – |  |
| >>LTM No Security Change ID | O |  | INTEGER(1..9,…) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10] | – |  |
| >> LTM L2 Reset Configuration | O |  | INTEGER(1..9,…) | Corresponds to the ltm-NoResetID as defined in TS38.331 [10]. |  |  |
| >>Data Forwarding Info for LTM List |  | 0..1 |  |  | – |  |
| >>>Data Forwarding Info for LTM Item |  | 1..<maxnoofPDUSessions> |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>>Data Forwarding Info from target NG-RAN node | M |  | 9.2.1.16 |  | – |  |
| >>Last Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| LTM UE Security Information | O |  | 9.2.3.230 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

-------------------------------------------Next Change, Skip Unchanged ---------------------------------------

-------------------------------------------Next Change, Skip Unchanged ---------------------------------------

#### 9.2.3.242	LTM Candidate PSCell Request List

This IE provides the information related to the requested LTM candidate PSCell(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM Candidate PSCell Request List |  | 1 |  |  |
| >LTM Candidate PSCell Request Item |  | 1 .. < maxnoofLTMCells> |  |  |
| >>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| >>Early Sync Information Request | O |  | 9.2.3.217 |  |
| >>Request for CSI-RS Resource Configuration for L1 Measurements | O |  | ENUMERATED (true, …) |  |
| >>Request for CSI-RS Resource Configuration for Early CSI Acquisition | O |  | ENUMERATED (true, …) |  |
| >>Proposed LTM L2 Reset Configuration | O |  | INTEGER(1..9,…) | Corresponds to the ltm-NoResetID as defined in TS38.331 [10], for the LTM candidate cell identified by the PSCell ID IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

#### 9.2.3.243	LTM Candidate PSCell Prepared List

This IE provides the information related to the prepared LTM candidate PSCell(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM Candidate PSCell Prepared List |  | 1 |  |  |
| >LTM Candidate PSCell Prepared Item |  | 1 .. < maxnoofLTMCells> |  |  |
| >>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| >>LTM No Security Change ID | M |  | INTEGER (0.. 9, …) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10] |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. |
| >>Early Sync Information | O |  | 9.2.3.218 |  |
| >>LTM CFRA Resource Information | O |  | 9.2.3.232 |  |
| >>SSB Information | O |  | 9.2.3.228 |  |
| >>CSI-RS Resource Configuration for Layer 1 Measurements | O |  | CSI-RS Resource Configuration 9.2.3.224 |  |
| >>CSI-RS Resource Configuration for Early CSI Acquisition | O |  | CSI-RS Resource Configuration 9.2.3.224 |  |
| >>Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  |
| >>LTM L2 Reset Configuration | O |  | INTEGER(1..9,…) | Corresponds to the ltm-NoResetID as defined in TS38.331 [10], for the LTM candidate cell identified by the PSCell ID IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

-------------------------------------------Next Change, Skip Unchanged ---------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

-------------------------------------------Next Change, Skip Unchanged ---------------------------------------

LTMHandoverInformationRequest ::= SEQUENCE {

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List		OPTIONAL,				-- This IE may need to be refined.

targetNG-RANnodeUEXnAPID				NG-RANnodeUEXnAPID				OPTIONAL,

referenceConfiguration					ReferenceConfiguration			OPTIONAL,

lTMConfigurationIDMappingList			LTMConfigurationIDMappingList	OPTIONAL,

cSIResourceConfiguration				CSIResourceConfiguration		OPTIONAL,

requestForCSI-RSResourceConfigForLayer1Measurements		RequestForCSI-RSResourceConfigForLayer1Measurements		OPTIONAL,

requestForCSI-RSResourceConfigForEarlyCSIAcquisition	RequestForCSI-RSResourceConfigForEarlyCSIAcquisition		OPTIONAL,

proposedLTML2ResetConfig				LTML2ResetConfig				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMHandoverInformationRequest-ExtIEs} } OPTIONAL,

...

}

LTMHandoverInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMConfigurationIDMappingList ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMConfigurationIDMapping-Item

LTMConfigurationIDMapping-Item::= SEQUENCE{

lTMCellID			NR-CGI,

lTMConfigurationID	LTMConfigurationID,

iE-Extensions		ProtocolExtensionContainer {{ LTMConfigurationIDMapping-Item-ExtIEs}}		OPTIONAL

}

LTMConfigurationIDMapping-Item-ExtIEs 	XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMConfigurationID ::=  INTEGER (1..8)

LTM-NoSecurityChangeID-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTM-NoSecurityChangeID

LTM-NoSecurityChangeID ::= INTEGER (1..9,...)

LTML2ResetConfig	::= INTEGER (1..9,...)

LTMHandoverInformationRequestAcknowledge ::= SEQUENCE {

sSBInformation								SSBInformation,

tCIStateConfigurationList					OCTET STRING,

lTMCandidateConfiguration					OCTET STRING,

lTM-NoSecurityChangeID						LTM-NoSecurityChangeID,

completeCandidateConfigurationIndicator		CompleteCandidateConfigurationIndicator 	OPTIONAL,

cSI-RSResourceConfigurationForLayer1Measurements		CSI-RSResourceConfiguration		OPTIONAL,

cSI-RSResourceConfigurationForEarlyCSIAcquisition		CSI-RSResourceConfiguration		OPTIONAL,

lTMCFRAResourceInformation								LTMCFRAResourceInformation 		OPTIONAL,

ltmL2ResetConfig							LTML2ResetConfig							OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMHandoverInformationAcknowledge-ExtIEs} } OPTIONAL,

...

}

LTMHandoverInformationAcknowledge-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMCellSwitchInformation	::= SEQUENCE {

jointorDLTCIStateID				JointorDLTCIStateID,

uLTCIStateID					ULTCIStateID 				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { LTMCellSwitchInformation-ExtIEs } }	OPTIONAL,

...

}

LTMCellSwitchInformation-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUEAssociationInformation-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTMUEAssociationInformation-Item

LTMUEAssociationInformation-Item ::= SEQUENCE {

candidateCellID									NR-CGI,    -- This IE needs to be refined

lastTarget-NG-RANnodeUEXnAPID					NG-RANnodeUEXnAPID,

iE-Extensions		ProtocolExtensionContainer { { LTMUEAssociationInformation-Item-ExtIEs} } OPTIONAL,

...

}

LTMUEAssociationInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUpdatesToCandidateNodeInformation ::= SEQUENCE {

referenceConfiguration					ReferenceConfiguration			OPTIONAL,

lTMConfigurationIDMappingList			LTMConfigurationIDMappingList	OPTIONAL,

cSIResourceConfiguration				CSIResourceConfiguration		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMUpdatesToCandidateNodeInformation-ExtIEs} } OPTIONAL,

...

}

LTMUpdatesToCandidateNodeInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUpdatesToCandidateCellInformation-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMUpdatesToCandidateCellInformation-Item

LTMUpdatesToCandidateCellInformation-Item ::= SEQUENCE {

candidateCellID							NR-CGI,

earlySyncInformation					EarlySyncInformation				OPTIONAL,

lTMCFRAResourceInformation				LTMCFRAResourceInformation 			OPTIONAL,

ueBasedTAMeasurementConfiguration		OCTET STRING 						OPTIONAL,

aSSecurityInformation					AS-SecurityInformation				OPTIONAL,

lTM-NoSecurityChangeID					LTM-NoSecurityChangeID				OPTIONAL,

ltmL2ResetConfig						LTML2ResetConfig					OPTIONAL,

lTMUEAssociation						LTMUEAssociationInformation-List	OPTIONAL,

dataForwardingInfoForLTM-List			DataForwardingInfoForLTM-List		OPTIONAL,

lastTarget-NG-RANnodeUEXnAPID			NG-RANnodeUEXnAPID					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMUpdatesToCandidateCellInformation-Item-ExtIEs} } OPTIONAL,

...

}

LTMUpdatesToCandidateCellInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMCFRAResourceInformation ::= SEQUENCE {

lTMCFRAResourceConfiguration			OCTET STRING						OPTIONAL,

lTMCFRAResourceConfigurationSUL			OCTET STRING						OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMCFRAResourceInformation-ExtIEs} } OPTIONAL,

...

}

LTMCFRAResourceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUpdatesFromCandidateCellInformation-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMUpdatesFromCandidateCellInformation-Item

LTMUpdatesFromCandidateCellInformation-Item ::= SEQUENCE {

candidateCellID										NR-CGI,

lTMCandidateConfiguration							OCTET STRING,

completeCandidateConfigurationIndicator				CompleteCandidateConfigurationIndicator 	OPTIONAL,

cSI-RSReportConfigurationForEarlyCSIAcquisition		OCTET STRING								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMUpdatesFromCandidateCellInformation-Item-ExtIEs} } OPTIONAL,

...

}

LTMUpdatesFromCandidateCellInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUESecurityInformation ::= SEQUENCE {

lTMUESecurityCapabilities						LTMUESecurityCapabilities,

lTMUserPlaneSecurityInformation-List			LTMUserPlaneSecurityInformation-List,

iE-Extension		ProtocolExtensionContainer { { LTMUESecurityInformation-ExtIEs} }	OPTIONAL,

...

}

LTMUESecurityInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUESecurityCapabilities ::= SEQUENCE {

nr-EncyptionAlgorithms					BIT STRING {nea1-128(1),

nea2-128(2),

nea3-128(3)} (SIZE(16, ...)),

nr-IntegrityProtectionAlgorithms			BIT STRING {nia1-128(1),

nia2-128(2),

nia3-128(3)} (SIZE(16, ...)),

iE-Extension			ProtocolExtensionContainer { {LTMUESecurityCapabilities-ExtIEs} } OPTIONAL,

...

}

LTMUESecurityCapabilities-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMUserPlaneSecurityInformation-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF LTMUserPlaneSecurityInformation-Item

LTMUserPlaneSecurityInformation-Item ::= SEQUENCE {

pDUSessionID				PDUSession-ID,

securityIndication			SecurityIndication		OPTIONAL,

iE-Extension				ProtocolExtensionContainer { { LTMUserPlaneSecurityInformation-Item-ExtIEs} } OPTIONAL,

...

}

LTMUserPlaneSecurityInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMCandidateCellsToBeCancelled-List ::= SEQUENCE (SIZE(0..maxnoofLTMCells)) OF LTMCandidateCellsToBeCancelled-Item

LTMCandidateCellsToBeCancelled-Item ::= SEQUENCE {

targetCellID				NR-CGI,

iE-Extensions				ProtocolExtensionContainer { { LTMCandidateCellsToBeCancelled-Item-ExtIEs } }	OPTIONAL,

...

}

LTMCandidateCellsToBeCancelled-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

Layer1MeasurementRequest-List ::= SEQUENCE (SIZE(1..maxnoofCSIResourceConfigurationsPlus1)) OF Layer1MeasurementRequest-Item

Layer1MeasurementRequest-Item ::= SEQUENCE {

requestForSemiPersistentCSI-RSResources			ENUMERATED {activate, deactivate,...},

cSIResourceConfigurationID						CSIResourceConfigurationID,

iE-Extensions									ProtocolExtensionContainer { { Layer1MeasurementRequest-Item-ExtIEs} }	OPTIONAL,

...

}

Layer1MeasurementRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Layer1MeasurementResponse-List ::= SEQUENCE (SIZE(1..maxnoofCSIResourceConfigurationsPlus1)) OF Layer1MeasurementResponse-Item

Layer1MeasurementResponse-Item ::= SEQUENCE {

semiPersistentCSI-RSResourceResponse			ENUMERATED {activated, deactivated,...},

cSIResourceConfigurationID						CSIResourceConfigurationID,

iE-Extensions									ProtocolExtensionContainer { { Layer1MeasurementResponse-Item-ExtIEs} }	OPTIONAL,

...

}

Layer1MeasurementResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMInformationSCG-AddReq ::= SEQUENCE {

source-M-NGRAN-node-ID					GlobalNG-RANNode-ID,

source-M-NGRAN-node-UE-XnAP-ID			NG-RANnodeUEXnAPID,

iE-Extensions 							ProtocolExtensionContainer { { LTMInformationSCG-AddReq-ExtIEs} } OPTIONAL,
	...

}

LTMInformationSCG-AddReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMInformationSCG-AddReqAck ::= SEQUENCE {

pCell-ID					GlobalNG-RANCell-ID,

iE-Extensions 				ProtocolExtensionContainer { { LTMInformationSCG-AddReqAck-ExtIEs} } OPTIONAL,
	...

}

LTMInformationSCG-AddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMInformationSCG-ModReq ::= SEQUENCE {

lTMReconfig					ENUMERATED {intra-mn-ltm, ...},

iE-Extensions 				ProtocolExtensionContainer { { LTMInformationSCG-ModReq-ExtIEs } } OPTIONAL,
	...

}

LTMInformationSCG-ModReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMInterSNExecutionNotification	::= ENUMERATED {executed, ...}

LTM-DC-DataForwarding-Indicator ::= ENUMERATED {triggered, ...}

LTMPSCellInformation-AddReq ::= SEQUENCE {

lTM-RequestIndication					ENUMERATED {request, ...},

maxNrofPSCellsToPrepare					MaxNrofPSCellsToPrepare,

suggestedLTMCandidatePSCellList			LTM-PSCell-Request-List,

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List,

cSI-ResourceConfiguration				CSIResourceConfiguration													OPTIONAL,

sCG-ReferenceConfigRequest				ENUMERATED {request, ...} 													OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList												OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-AddReq-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-AddReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-AddReqAck ::= SEQUENCE {

lTM-CandidatePSCellPreparedList		LTM-PSCell-Prepared-List												OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { LTMPSCellInformation-AddReqAck-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-AddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-UpdateReq ::= SEQUENCE {

multipleSN-List							MultipleCandidateSN-List,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List											OPTIONAL,

lTM-SCG-SecurityConfiguration			LTM-SCG-SecurityConfiguration-List									OPTIONAL,

lTM-CandidatePSCellToBeCancelled		LTM-PSCell-Request-List												OPTIONAL,

-- reference to the LTM-PSCell-Request-List to be refined

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReq-ExtIEs} }				OPTIONAL,

...

}

LTMPSCellInformation-UpdateReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-UpdateReqAck ::= SEQUENCE {

lTM-CandidatePSCellPreparedList			LTM-PSCell-Prepared-List											OPTIONAL,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReqAck-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-UpdateReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-UpdateReqired ::= SEQUENCE {

lTM-CandidatePSCellToBeCancelled		LTM-PSCell-Request-List												OPTIONAL,

-- reference to the LTM-PSCell-Request-List to be refined

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReqired-ExtIEs} }		OPTIONAL,

...

}

LTMPSCellInformation-UpdateReqired-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-UpdateConfirm ::= SEQUENCE {

lTM-CandidatePSCellPreparedList			LTM-PSCell-Prepared-List											OPTIONAL,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-UpdateConfirm-ExtIEs} }		OPTIONAL,

...

}

LTMPSCellInformation-UpdateConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-ChangeRequired ::= SEQUENCE {

lTM-RequestIndication					LTM-RequestIndication,

multipleSNChangeRequired-List			MultipleCandidateSNChangeRequired-List,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-ChangeRequired-ExtIEs } }		OPTIONAL,

...

}

LTMPSCellInformation-ChangeRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-ChangeConfirm ::= SEQUENCE {

lTM-SCG-SecurityConfiguration			LTM-SCG-SecurityConfiguration-List									OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-ChangeConfirm-ExtIEs} }		OPTIONAL,

...

}

LTMPSCellInformation-ChangeConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTM-PSCell-Request-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTM-PSCell-Request-Item

LTM-PSCell-Request-Item ::= SEQUENCE {

pscell-id												NR-CGI,

earlySyncInformationRequest								EarlySyncInformationRequest										OPTIONAL,

requestForCSI-RSResourceConfigForLayer1Measurements		RequestForCSI-RSResourceConfigForLayer1Measurements		OPTIONAL,

requestForCSI-RSResourceConfigForEarlyCSIAcquisition	RequestForCSI-RSResourceConfigForEarlyCSIAcquisition		OPTIONAL,

proposedLTML2ResetConfig								LTML2ResetConfig												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTM-PSCell-Request-Item-ExtIEs} } 									OPTIONAL,

...

}

LTM-PSCell-Request-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTM-PSCell-Prepared-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTM-PSCell-Prepared-Item

LTM-PSCell-Prepared-Item ::= SEQUENCE {

pscell-id											NR-CGI,

lTM-NoSecurityChangeID								LTM-NoSecurityChangeID,

tCI-StatesConfigurationsList						OCTET STRING											OPTIONAL,

earlySyncInformation								EarlySyncInformation									OPTIONAL,

lTMCFRAResourceInformation							LTMCFRAResourceInformation 								OPTIONAL,

sSBInformation										SSBInformation											OPTIONAL,

cSI-RSResourceConfigurationForLayer1Measurements	CSI-RSResourceConfiguration								OPTIONAL,

cSI-RSResourceConfigurationForEarlyCSIAcquisition	CSI-RSResourceConfiguration								OPTIONAL,

complete-CandidateConfigurationIndicator			ENUMERATED {complete, ...}								OPTIONAL,

ltmL2ResetConfig									LTML2ResetConfig										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTM-PSCell-Prepared-Item-ExtIEs} } 						OPTIONAL,

...

}

LTM-PSCell-Prepared-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTM-RequestIndication ::= ENUMERATED {request, ...}

LTM-SCG-SecurityConfiguration-List ::= SEQUENCE (SIZE(1..maxnoofLTMCellsPlusOne)) OF LTM-SCG-SecurityConfiguration-Item

LTM-SCG-SecurityConfiguration-Item ::= SEQUENCE {

lTM-NoSecurityChangeID				LTM-NoSecurityChangeID,

lTM-SCG-SecurityConfig-Info			LTM-SCG-SecurityConfig-Info-List,

iE-Extensions						ProtocolExtensionContainer { { LTM-SCG-SecurityConfiguration-Item-ExtIEs} } OPTIONAL,

...

}

LTM-SCG-SecurityConfiguration-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTM-SCG-SecurityConfig-Info-List ::= SEQUENCE (SIZE(1..maxnoofSCGSecurityConfigurations)) OF LTM-SCG-SecurityConfig-Info-Item

LTM-SCG-SecurityConfig-Info-Item ::= SEQUENCE {

s-ng-RANnode-SecurityKey			S-NG-RANnode-SecurityKey,

sk-counter							SK-COUNTER,

iE-Extensions		ProtocolExtensionContainer { { LTM-SCG-SecurityConfig-Info-Item-ExtIEs} } OPTIONAL,

...

}

LTM-SCG-SecurityConfig-Info-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-------------------------------------------End of Change ---------------------------------------