3GPP TSG-RAN WG3 Meeting #130       	   				    R3-258351

Dallas, US, 17 – 21 November 2025

Agenda Item:		14.2

Source:			China Telecom

Title:	Support of Ambient IoT Topology 2

Document for:	 	Discussion & Decision

- Introduction

The following agreements were made in previous RAN3 meeting:

For Rel-20 T2 discussion, based on the RRC-based solution, and RAN3 focus is indirect connectivity is applied.

NGAP procedures specified for Topology 1 are base line for Topology 2.

The AIOT procedures (Topology 2, RRC based solution) in TR38.769 can be as base line.

UE Reader Authorization is performed at 5GC

The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB.

FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

In this contribution, we discuss the signalling design and open issues for Topology 2 based on the progress achieved in last RAN3 meeting.

- Discussion

2.1 Signalling Procedures in Split Architecture

In Rel-19, T1 deployment does not support the CU/DU split architecture. Based on progress of last meeting, it was agreed that the support of Topology 2 in Rel-20 will be based on the RRC-based option with indirect connectivity. As existing NG-RAN nodes can be upgraded to support Topology 2, the CU/DU split architecture will thereby be enabled in Release 20. The procedures for A-IoT Command and Inventory in a CU/DU scenario can take the procedures in TR 38.769 [2] as baseline, with further taking into account the impact of the split architecture, aligned with the objectives of the Rel-20 A-IoT Study Item.

Figure 1 depicts the basic communication among AIoT devices, UE Reader, gNB-CU, gNB-DU and A-IoT CN node for the Inventory procedure.

Figure 1: Inventory procedure in Topology 2

1.	The AMF initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB-CU. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

2.	Allocates and co-ordinates the usage of A-IoT radio resources among UE Reader, gNB-DU and gNB-CU. The resource allocation can be triggered by CN request. The gNB-CU provides AIoT resource information to be used for AIoT interface transmissions to the UE Reader via RRC dedicated signaling. UE reader may provide assistance information related to AIoT transmissions.

3.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the UE Reader to gNB-DU by encapsulating the RRC Inventory Request message. RRC message details subject to RAN2.

4.	The gNB-DU sends RRC Inventory Request message to the UE Reader.

Editor’s Note:	The gNB will provide AIoT information to the UE Reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information.

5.	The UE Reader sends RRC Inventory Response message to the gNB-DU.

6.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

7.	The gNB confirms the request from the AMF by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

8.	The Inventory procedure over the A-IoT radio interface is performed, i.e., A-IoT paging, A-IoT Access Procedure and D2R data transmission.

9./10.	Upon receiving the RRC Inventory Report message(s) from the UE Reader, the gNB-CU sends the Inventory Report message to the AMF. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the AMF.

Upon completion of the inventory procedure the gNB sends an Inventory Complete Indication in the Inventory Report message.

11.	If there is no follow-on command, the AMF triggers the A-IoT Session Release procedure.

Figure 2 depicts the basic communication among AIoT devices, UE Reader, gNB-CU, gNB-DU and A-IoT CN node for the Command procedure.

Figure 2: Command procedure in Topology 2

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

2.	The AMF initiates the Command procedure over NG-C by sending the Command Request message to the gNB-CU in order to send a command to a single A-IoT device. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

3.	Allocates and co-ordinates the usage of A-IoT radio resources among UE Reader, gNB-DU and gNB-CU. The resource allocation can be triggered by CN request. The gNB-CU provides AIoT resource information to be used for AIoT interface transmissions to the UE Reader via RRC dedicated signaling. UE reader may provide assistance information related to AIoT transmissions.

4.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the UE Reader to gNB-DU by encapsulating the RRC Command Request message.

5.	The gNB-DU sends RRC Command Request message to the UE Reader.

Editor’s Note:	The gNB will provide AIoT information to the UE Reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information.

6.	The Command procedure over the A-IoT radio interface is performed, i.e., R2D and D2R data transmission.

7.	The UE Reader sends RRC Command Response message include the command result to the gNB-DU.

8.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

9.	The gNB-CU replies to the Command Request with the Command Response message including the AIOTF Identifier the Correlation ID and the RAN A-IoT device NGAP ID received in the Command Request, and the A-IoT NAS PDU received from the device.

NOTE:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the AMF.

10.	Upon completion of all the command transmission for all related devices within the session, the AMF triggers A-IoT Session Release procedure.

Proposal 1: Capture the above A-IoT Command and Inventory signalling procedures for Topology 2.

2.2 inter-gNB RRC-connected UE Reader Mobility

It is need to specify the necessary signalling support for inter-gNB RRC-connected UE Reader Mobility in Rel-20. During the discussion, the following issue still needs further discussed.

FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

Authorization for the UE Reader is performed in the 5GC and provided to the NG-RAN node over the NG interface. Since this authorization is tied to a geographic area, it may not change during a handover to a target gNB. As such, the source node shall include the current authorization information in both the Handover Request and Path Switch Request messages. In the event that the CN detects a change in the authorization details, it shall provide the updated information to the target node.

Proposal 2: the source node shall include the current authorization information in both the Handover Request and Path Switch Request messages. In the event that the CN detects a change in the authorization details, it shall provide the updated information to the target node.

2.3 Resource Allocation

The following agreements were achieved in last RAN2 meeting.

1.  gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

Resource allocation information can be delivered from the CN to the NG-RAN node. However, the specific details await further RAN2 progress. Subsequently, the A-IoT resource allocation is sent from the gNB-CU to the gNB-DU. Leveraging this information, the gNB-DU determines the appropriate resource configuration for the UE Reader.

Proposal 3:  Resource allocation information specified in RAN2 can be delivered from the CN to the NG-RAN node. And the A-IoT resource allocation is sent from the gNB-CU to the gNB-DU

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Capture the above A-IoT Command and Inventory signalling procedures for Topology 2.

Proposal 2: the source node shall include the current authorization information in both the Handover Request and Path Switch Request messages. In the event that the CN detects a change in the authorization details, it shall provide the updated information to the target node.

Proposal 3:  Resource allocation information specified in RAN2 can be delivered from the CN to the NG-RAN node. And the A-IoT resource allocation is sent from the gNB-CU to the gNB-DU

- References

[1] 	RP-252894, Solutions for Ambient IoT (Internet of Things) in NR Phase 2.

[2] 	3GPP TR38.769, Study on solutions for ambient IoT (Internet of Things).