3GPP TSG-RAN WG3 Meeting #130	R3-258363

Dallas, US, 17 – 21 November 2025

Agenda item:	10.3.2

Source: 	Lenovo

Title: 	6G RAN-CN Interface Options

Document for:	pCR

1. Introduction

The general principles and requirements of 6G RAN-CN functional split and interface were discussed at the RAN3#129bis meeting with the following agreements [1].

The general principles for the specification of the 6G RAN-CN interface are as follows: - the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN; - the 6G RAN-CN interface supports control plane and user plane separation; - the 6G RAN-CN interface supports future enhancements; - the 6G RAN-CN interface supports all possible RAN deployment scenarios; - the 6G RAN-CN interface supports RAN sharing between multiple operators; - the 6G RAN-CN interface supports the operation of network slicing; - the 6G RAN-CN interface supports enhanced service awareness in RAN; - the 6G RAN-CN control plane interface supports reliable signalling transmission; - the 6G RAN-CN interface is designed with a clear functional split between RAN and CN. RAN-CN control plane interface supports following functions: - UE context management : The functionality to manage UE context between the RAN and CN; - Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress; - PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress.

In this contribution, we discuss the 6G RAN-CN interface options.

2. Discussion

The 5G RAN-CN interface is designed as a point-to-point interface, where all UE related signalling, e.g., registration, connection, mobility, etc., are handled in a single entity (AMF in 5GC) between the RAN and CN. This approach guarantees a clear separation of concerns between the RAN and CN and reduces the security complexity.

The point-to-point interface between the NR and 5GC showed its flexibility and multi-vendor support. Motivated by the relatively recent uptake by many operators of 5GC, it is reasonable to assume that the 6G CN is an evolution of the 5GC to reduce the investments. In line with the assumption, for fundamental functions that is relevant to at least eMBB services, e.g., interface management, UE context management, UE mobility management, and data transmission, the 6G RAN-CN interface can be the evolution of the interfaces related to the 5G RAN, and there is no strong requirement to change the functionality assignment between RAN and CN. Therefore, for the control plane, the 6GR communicates directly with the 6G CN (e.g., AMF like function in the 6G CN) over a point-to-point interface.

In legacy system, the SCTP is the transport layer protocol used in the control plane, which provides high reliability and performance. For 6G, other transport layer protocols can also be considered. For example, the QUIC provides integrated security (TLS 1.3), native multi-homing and resiliency to path failure, and scalability without the association with the specific IP/port pairs.

With respect to the user plane, GTP/UDP has provided to be sufficient and efficient in previous generations. Therefore, it should be adopted in the user plane interface between the 6G RAN and 6G CN.

In summary, for fundamental functions that is relevant to at least eMBB services, e.g., interface management, UE context management, UE mobility management, and PDU packet transmission,

-	The 6G RAN is connected to the 6G CN via an NG-C like interface for control plane, e.g., by means of the evolution of NG-C interface.

-	The 6G RAN is connected to the 6G CN via an NG-U like interface for user plane, e.g., by means of the evolution of GTP-U based NG-U interface.

Proposal 1: To support the fundamental eMBB services, point-to-point interface is considered as the option for signaling/messaging between 6G RAN and 6G CN.

-	The 6G RAN is connected to the 6G CN via an NG-C like interface for control plane, e.g., by means of the evolution of NG-C interface.

-	The 6G RAN is connected to the 6G CN via an NG-U like interface for user plane, e.g., by means of the evolution of GTP-U based NG-U interface.

The 6G network are expected to include new service types beyond traditional communication, such as data collection, sensing and AI. The characteristics of those new services are quite different from legacy functions considering:

- Less time/robustness critical compared to regular UL/DL UE data traffic, e.g., the data collection for AI model training can be conducted in a best effort manner.

- Decoupled with the regular UL/DL UE data traffic (e.g., audio/video), e.g., support of the new services shall not impact the legacy UL/DL communication.

- The consumer of the service could be variant, i.e., more than 6G AMF, e.g., NWDAF and SMF could benefit from the RAN side performance status.

- Extensible and cloud-native, e.g., the implementation of those services has to deal with various of sub use cases and transfer/handling of large amount of data

As a result, more demands on network capability exposure will be required, while the complexity for configuration and internal function interaction should be reduced. A potential approach is to adopt a HTTPs-based service-based interface (SBI) between the RAN and CN for the new functions supporting the new service types, which can avoid too many P2P interface definitions and unnecessary transparent forwarding among network entities, thus enable efficient coordination between the RAN and CN. In addition, the RAN can more directly expose information to the CN.

Therefore, we propose to consider the service-based interface between the RAN and CN to facilitate the exchange of RAN related information for 6G new services, e.g., AI, sensing.

Proposal 2: To support new services (e.g., data collection, AI and sensing) with the following characteristics, RAN3 considers HTPPs-based SBI as a feasible option for signalling/messaging between 6G RAN and 6G CN:

- Less time/robustness critical compared to regular UL/DL UE data traffic

- Decoupled from the regular UL/DL UE data traffic (e.g., audio/video)

- The consumer of the service could be variant

- Extensible and cloud-native

An example of the RAN-CN interface is shown as below, where the new function communicates with the 6G CN via the service-based interface while the 6G BS communicates with the 6G CN via the point-to-point interface for legacy service.

NOTE: the new function may be an internal function provided by the 6G BS.

NOTE: the new function may be provided by a RAN node different from the 6G BS.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: To support the fundamental eMBB services, point-to-point interface is considered as the option for signaling/messaging between 6G RAN and 6G CN.

-	The 6G RAN is connected to the 6G CN via an NG-C like interface for control plane, e.g., by means of the evolution of NG-C interface.

-	The 6G RAN is connected to the 6G CN via an NG-U like interface for user plane, e.g., by means of the evolution of GTP-U based NG-U interface.

Proposal 2: To support new services (e.g., data collection, AI and sensing) with the following characteristics, RAN3 considers HTPPs-based SBI as a feasible option for signalling/messaging between 6G RAN and 6G CN:

- Less time/robustness critical compared to regular UL/DL UE data traffic

- Decoupled from the regular UL/DL UE data traffic (e.g., audio/video)

- The consumer of the service could be variant

- Extensible and cloud-native

4. Reference

- TR 38.760-3 v0.1.0: Study on 6G radio RAN3 aspects.

- TR 38.801: Study on new radio access technology: radio access architecture and interfaces.

# 5. Text Proposal

pCR for TR 38.760-3.

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

The RAN-CN interface is a point-to-point interface to support the fundamental eMBB service.

-	the RAN node is connected to the CN via an NG-U like interface for user plane, which is an evolution of GTP-U based NG-U interface;

-	the RAN node is connected to the CN via an NG-C like interface for control plane, which is an evolution of NG-C interface.

The RAN-CN interface is a service-based interface to support the new service such as data collection, AI and sensing.

The RAN-CN interface is illustrated in Figure 6.1.3-1.

Figure 6.1.3-1: 6G RAN-CN interface

NOTE: the new RAN function may be an internal function provided by the 6G BS, or it may be provided by a RAN node different from the 6G BS.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>