3GPP TSG-RAN WG3 Meeting #130	R3-258367

Dallas, US,17 - 21 November 2025

Agenda item:	 11.2.1

Source:		Lenovo

Title:		Discussion on MRO for inter-CU LTM

Document for:		Discussion and Decision

1	Introduction

It was agreed in RAN3#129bis meeting that:

Use cases

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.:

BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA;

LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell;

Near failure case: SHR successful HO

Successful case: ping/pong

Potential new cases will not be precluded.

Solution options

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as base line.

For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

For the connection failure cases:

-	After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis.

L3 based inter-CU LTM is within the scope of the WID

In this paper, we would further discuss MRO for inter-CU LTM.

2	Discussion

Last RAN3#129bis agreed to support Too Late LTM, Too Early LTM and LTM to Wrong Cell for inter-CU LTM, and the definition of connection failure cases and the detection mechanism of connection failure cases in 38.300 are taken as base line. Additionally in stage 2, the existing connection failure definition and failure detection mechanism of Too Late LTM, Too Early LTM and LTM to Wrong Cell captured in TS38.300 can be reused for inter-CU LTM.

Similar as intra-CU LTM, for Too Late LTM, Too Early LTM or LTM to Wrong Cell in inter-CU LTM, the failure may occur due to wrong selection of candidate LTM cell, inappropriate cell switch triggering or wrong target cell selection for LTM cell switch.

Proposal 1: The connection failure due to wrong selection of candidate LTM cell, inappropriate cell switch triggering or wrong target cell selection for LTM cell switch should be considered for inter-CU LTM.

The mechanism of failure detection and optimization for L1 based intra-CU LTM as below can be taken as baseline for L1 based inter-CU LTM, e.g. CU performs initial MRO analysis,

- for failures due to wrong selection of candidate LTM cell, CU performs MRO optimization;

- for failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, CU forwards the RLF report to the DU responsible for the failure (e.g. the last serving DU in case of too late LTM cell switch, or to the source DU in case of too early LTM cell switch or LTM cell switch to wrong cell), then the DU performs MRO optimization.

However, the key point for failure detection and optimization for inter-CU LTM is which CU/DU (e.g. source CU/DU or target CU/DU) performs MRO analysis or MRO optimization.

For Too Late LTM, a CU receives a RLF report associated to the inter-CU LTM from the UE forwards the RLF report to the source CU via the Failure Indication message over Xn. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the last serving DU, then, the last serving DU performs MRO optimization.

For HOF in Too Early LTM, the source CU receives the RLF report associated to the inter-CU LTM from the UE, or the source CU receives the RLF report via the Failure Indication message over Xn from a CU which receives the RLF report from the UE. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the source DU, then, the source DU performs MRO optimization.

For RLF in Too Early LTM, the source CU receives the RLF report associated to the inter-CU LTM from the UE, or the source CU receives the RLF report via the Handover Report message over Xn from the target CU. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the source DU, then, the source DU performs MRO optimization.

For HOF in LTM to Wrong Cell, the source CU receives the RLF report associated to the inter-CU LTM via the Failure Indication message over Xn from a CU which receives the RLF report from the UE. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the source DU, then, the source DU performs MRO optimization.

For RLF in LTM to Wrong Cell, the source CU receives the RLF report associated to the inter-CU LTM via the Handover Report message over Xn from the target CU. The source CU performs analysis,

- in case of failures due to wrong selection of candidate LTM cell, the source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the source CU forwards the RLF report to the source DU, then, the source DU performs MRO optimization.

Proposal 2: For L1 based inter-CU LTM, souce CU performs MRO analysis,

- in case of failures due to wrong selection of candidate LTM cell, source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, source CU forwards the RLF report to the DU responsible for the failure (e.g. the last serving DU in case of too late LTM cell switch, or to the source DU in case of too early LTM cell switch or LTM cell switch to wrong cell), then the DU performs MRO optimization.

Last RAN3#129bis meeting had common understanding that L3 based inter-CU LTM is also within the scope of the WID.

For L3 based LTM, source CU may send one of below IE to source DU in CU-DU MOBILITY INITIATION REQUEST message:

- Mobility Triggering Indication (e.g. including Triggering Indication (e.g. which indicates early UL sync, early DL sync, or cell switch), candidate cell and associated SSB index for cell switch, candidate cells and associated SSB indexes information for early UL sync, and/or, candidate cells and associated SSB indexes information for early DL sync)

- Mobility Assistance Information including L3 measurement information (e.g. SSB measurement results of serving cell and neighbour cell)

Similar as above analysis for L1 based inter-CU LTM, for L3 based inter-CU LTM, it is souce CU performs MRO analysis, for the case that source CU sends Mobility Assistance Information including L3 measurement information to source DU, since it is still source DU decides the trigger and target cell for LTM cell switch, the failure detection and optimization for L3 based inter-CU LTM is same as Proposal 2 shown for L1 based inter-CU LTM.

However, for the case that source CU sends Mobility Triggering Indication (e.g. including Triggering Indication for cell switch), candidate cell and associated SSB index for cell switch to source DU, since it is source CU proposes the trigger and target cell for LTM cell switch, in any case of failures due to wrong selection of candidate LTM cell, failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, source CU performs MRO optimization, source CU does not forward the RLF report to any DU.

Proposal 3: For L3 based inter-CU LTM, souce CU performs MRO analysis,

- for the case that source CU sends Mobility Assistance Information including L3 measurement information to source DU,

  - in case of failures due to wrong selection of candidate LTM cell, source CU performs MRO optimization;

  - in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, source CU forwards the RLF report to the DU responsible for the failure (e.g. the last serving DU in case of too late LTM cell switch, or to the source DU in case of too early LTM cell switch or LTM cell switch to wrong cell), then the DU performs MRO optimization.

- for the case that source CU sends Mobility Triggering Indication (e.g. including Triggering Indication for cell switch), candidate cell and associated SSB index for cell switch to source DU, source CU performs MRO optimization.

Last RAN3#129bis meeting also agreed to consider the case of BFR shortly after successful LTM cell switch to wrong beam, LTM cell switch failure due to wrong beam and LTM cell switch failure due to outdated TA for MRO for inter-CU LTM.

The following mechanism of identification and optimization for connection failure due to BFR shortly after successful LTM cell switch to wrong beam in L1 based intra-CU LTM can be taken as baseline for L1 based inter-CU LTM, e.g.

- the target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

- the target DU performs initial analysis, in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, the target DU sends the recovery beam information to the source DU via CU;

- the source DU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

Also, the following mechanism of identification and optimization for LTM cell switch failure due to wrong beam and/or LTM cell switch failure due to outdated TA in L1 based intra-CU LTM as below can be taken as baseline for L1 based inter-CU LTM, e.g.

- the target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- the target DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source DU via CU;

- the source DU may take it into account for optimisation of target beam selection and/or optimisation of Timing Advance related parameters.

Compared with L1 based intra-CU LTM, for L1 based inter-CU LTM, both the source CU and target CU should be involved for relevant information (e.g. the BFR beam information, the re-established or recovery beam information, and/or the TA value used at successful RACH-based access) transfer, which means that Xn interface should be enhanced to transfer the relevant information.

Proposal 4: For connection failure due to BFR shortly after successful LTM cell switch to wrong beam in L1 based inter-CU LTM,

- target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

- target DU performs initial analysis: in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, target DU sends the recovery beam information to source DU via target CU and source CU;

- source DU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

Proposal 5: For LTM cell switch failure due to wrong beam and/or LTM cell switch failure due to outdated TA in L1 based inter-CU LTM,

- target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- target DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to source DU via target CU and source CU;

- source DU may take it into account for optimisation of target beam selection and/or optimisation of Timing Advance related parameters.

Since last RAN3#129bis meeting had common understanding that L3 based inter-CU LTM is also within the scope of the WID, similar as above analysis for L1 based inter-CU LTM, for L3 based inter-CU LTM, if source CU sends Mobility Assistance Information including L3 measurement information to source DU, since it is still source DU decides the target beam for LTM cell switch, failure detection and optimization mechanism for the connection failure due to BFR shortly after successful LTM cell switch to wrong beam or the LTM cell switch failure due to wrong beam in L3 based inter-CU LTM is same as Proposal 4 and Proposal 5 shown for L1 based inter-CU LTM.

However, for the case that source CU sends Mobility Triggering Indication (e.g. including Triggering Indication for cell switch), candidate cell and associated SSB index for cell switch to source DU, since it is source CU proposes the trigger and target beam for LTM cell switch, in any case of the failure due to BFR shortly after successful LTM cell switch to wrong beam or the failure due to wrong beam, source CU performs MRO optimization, source CU does not forward the RLF report to source DU.

Proposal 6: For connection failure due to BFR shortly after successful LTM cell switch to wrong beam in L3 based inter-CU LTM,

- for the case that source CU sends Mobility Assistance Information including L3 measurement information to source DU,

  - target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

  - target DU performs initial analysis: in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, target DU sends the recovery beam information to source DU via target CU and source CU;

  - source DU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

- for the case that source CU sends Mobility Triggering Indication (e.g. including Triggering Indication for cell switch), candidate cell and associated SSB index for cell switch to source DU,

  - target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

  - target DU performs initial analysis: in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, target DU sends the recovery beam information to source CU via target CU;

  - source CU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

Proposal 7: For LTM cell switch failure due to wrong beam in L3 based inter-CU LTM,

- for the case that source CU sends Mobility Assistance Information including L3 measurement information to source DU,

  - target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

  - target DU sends the re-established or recovery beam information to source DU via target CU and source CU;

  - source DU may take it into account for optimisation of target beam selection.

- for the case that source CU sends Mobility Triggering Indication (e.g. including Triggering Indication for cell switch), candidate cell and associated SSB index for cell switch to source DU,

  - target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

  - target DU sends the re-established or recovery beam information to source CU via target CU;

  - source CU may take it into account for optimisation of target beam selection.

Proposal 8: For LTM cell switch failure due to outdated TA in L3 based inter-CU LTM,

- target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- target DU sends the TA value used at successful RACH-based access to source DU via target CU and source CU;

- source DU may take it into account for optimisation of Timing Advance related parameters.

3	Conclusion

In this paper, MRO for inter-CU LTM is discussed. We have the following proposals:

Proposal 1: The connection failure due to wrong selection of candidate LTM cell, inappropriate cell switch triggering or wrong target cell selection for LTM cell switch should be considered for inter-CU LTM.

Proposal 2: For L1 based inter-CU LTM, souce CU performs MRO analysis,

- in case of failures due to wrong selection of candidate LTM cell, source CU performs MRO optimization;

- in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, source CU forwards the RLF report to the DU responsible for the failure (e.g. the last serving DU in case of too late LTM cell switch, or to the source DU in case of too early LTM cell switch or LTM cell switch to wrong cell), then the DU performs MRO optimization.

Proposal 3: For L3 based inter-CU LTM, souce CU performs MRO analysis,

- for the case that source CU sends Mobility Assistance Information including L3 measurement information to source DU,

  - in case of failures due to wrong selection of candidate LTM cell, source CU performs MRO optimization;

  - in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, source CU forwards the RLF report to the DU responsible for the failure (e.g. the last serving DU in case of too late LTM cell switch, or to the source DU in case of too early LTM cell switch or LTM cell switch to wrong cell), then the DU performs MRO optimization.

- for the case that source CU sends Mobility Triggering Indication (e.g. including Triggering Indication for cell switch), candidate cell and associated SSB index for cell switch to source DU, source CU performs MRO optimization.

Proposal 4: For connection failure due to BFR shortly after successful LTM cell switch to wrong beam in L1 based inter-CU LTM,

- target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

- target DU performs initial analysis: in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, target DU sends the recovery beam information to source DU via target CU and source CU;

- source DU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

Proposal 5: For LTM cell switch failure due to wrong beam and/or LTM cell switch failure due to outdated TA in L1 based inter-CU LTM,

- target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- target DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to source DU via target CU and source CU;

- source DU may take it into account for optimisation of target beam selection and/or optimisation of Timing Advance related parameters.

Proposal 6: For connection failure due to BFR shortly after successful LTM cell switch to wrong beam in L3 based inter-CU LTM,

- for the case that source CU sends Mobility Assistance Information including L3 measurement information to source DU,

  - target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

  - target DU performs initial analysis: in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, target DU sends the recovery beam information to source DU via target CU and source CU;

  - source DU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

- for the case that source CU sends Mobility Triggering Indication (e.g. including Triggering Indication for cell switch), candidate cell and associated SSB index for cell switch to source DU,

  - target DU identifies that a Beam Failure Recovery has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt);

  - target DU performs initial analysis: in case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization; otherwise, target DU sends the recovery beam information to source CU via target CU;

  - source CU considers that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU, and may take it into account for optimisation of target beam selection.

Proposal 7: For LTM cell switch failure due to wrong beam in L3 based inter-CU LTM,

- for the case that source CU sends Mobility Assistance Information including L3 measurement information to source DU,

  - target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

  - target DU sends the re-established or recovery beam information to source DU via target CU and source CU;

  - source DU may take it into account for optimisation of target beam selection.

- for the case that source CU sends Mobility Triggering Indication (e.g. including Triggering Indication for cell switch), candidate cell and associated SSB index for cell switch to source DU,

  - target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

  - target DU sends the re-established or recovery beam information to source CU via target CU;

  - source CU may take it into account for optimisation of target beam selection.

Proposal 8: For LTM cell switch failure due to outdated TA in L3 based inter-CU LTM,

- target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- target DU sends the TA value used at successful RACH-based access to source DU via target CU and source CU;

- source DU may take it into account for optimisation of Timing Advance related parameters.