3GPP TSG-RAN WG3 Meeting #130	R3-258368

Dallas, US,17 - 21 November 2025

Agenda item:	 11.2.2

Source:		Lenovo

Title:		(TP for TS38.300) Discussion on MRO for intra-CU CLTM

Document for:		Discussion and Decision

1	Introduction

It was agreed in RAN3#129bis meeting that:

Use cases:

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU conditional LTM, i.e.:

BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA;

FFS whether both L1 and L3 apply to beam related selection optimization.

LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell;

Near failure case: SHR successful HO

Successful case: ping/pong

L3 based intra-CU conditional LTM is within the scope of the WID

Solution options:

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO for intra-CU C-LTM, if applicable.

For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as baseline.

For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as baseline.

For the connection failure cases, the last serving gNB-CU performs initial analysis.

Additional information required from UE

In this paper, we would discuss MRO for intra-CU CLTM.

2	Discussion

Intra-CU CLTM meachnism is supported in R19, wherenin source gNB sends an RRCReconfiguration message to the UE including the CLTM configurations of candidate cells as well as the execution condition for the CLTM, and the UE starts evaluating the execution conditions based on the provided configuration, when the CLTM execution condition for the candidate LTM cell is satisfied, the UE performs the CLTM switch by applying the configuration of the satisfied LTM candidate cell.

Similar as intra-CU/inter-CU LTM, early UL synchronization is also supported in CLTM. Since there is no LTM cell switch command MAC CE for CLTM, the source gNB can provide the TA value for each candidate cells the UE has performed UL synchronization with via an LTM Candidate Timing Advance Command MAC CE. CLTM execution can be RACH-based or RACH-less, e.g. if the UE has valid TA when CLTM execution is triggered (e.g. CLTM execution condition is satisfied), the UE skips RACH, otherwise, RACH is performed as part of the CLTM switch.

- For L1 based intra-CU CLTM, events e.g. CondEvent LTM2/LTM3/LTM4/LTM5 based on beam specific quality of serving cell and/or candidate cells are supported, DU generates the L1 based intra-CU CLTM execution conditions based on the request from CU;

- For L3 based intra-CU CLTM, events e.g. CondEvent A3/A5 based on cell specific quality of serving cell and/or candidate cells are supported similar as legacy CHO, CU generates the L3 based intra-CU CLTM execution conditions.

  - 2.1 MRO for connection failure case in intra-CU CLTM

In a CLTM procedure, a CLTM execution failure (i.e. HOF) or RLF may happen. For CLTM connection failure, last RAN3#129bis meeting agreed to support too late C-LTM cell switch execution, too early C-LTM cell switch execution and C-LTM cell switch execution to wrong cell. Also, for intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 and the detection mechanism of connection failure cases in 38.300 are agreed to be taken as baseline. However, as shown in Annex, some necessary updates are needed in TS 38.300 for intra-CU CLTM.

Proposal 1: Agree the TP for TS38.300 for intra-CU CLTM.

Last RAN3#129bis meeting agreed to consider the use cases of BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA for intra-CU CLTM, and it is FFS on whether both L1 and L3 apply to beam related selection optimization.

For L1 based CLTM, CLTM events e.g. CondEvent LTM2/3/4/5 which is based on beam specific quality of serving cell and candidate cells are supported. For example, if CondEventLTM3 is configured for one CLTM candidate cell, CLTM cell switch may be executed when beam of the CLTM candidate cell becomes amount of offset better than beam of serving cell. As we can see for L1 based CLTM, it is UE evaluating the CLTM execution condition and deciding the target cell/beam for cell switch execution, that is totally different from LTM cell switch procedure in which target beam is decided by source DU and indicated to the UE by LTM cell switch command MAC CE. In R19 MRO for intra-CU LTM, the use cases of BFR shortly after successful C-LTM cell switch execution and failure due to wrong beam are supported, because the failure is caused due to inappripriate/wrong beam selection in network side. However, for intra-CU CLTM, following MRO principle, UE behavior optimization is out of MRO scope, BFR shortly after successful C-LTM cell switch execution and failure due to wrong beam should not be considered for L1 based CLTM.

For L3 based CLTM, CLTM events e.g. CondEvent A3/5 which is based on cell specific quality of serving cell and candidate cells are supported. For example, if CondEventA3 is configured for one CLTM candidate cell, CLTM cell switch may be executed when the CLTM candidate cell becomes amount of offset better than serving cell. After deciding the target cell, UE further selects the target beam based on a beam quality threshold if network configured, e.g. UE selects a beam whose quality is higher than the quality threshold to access to the target cell. Also for L3 based CLTM, it is UE deciding the target cell/beam for cell switch execution, similarly, BFR shortly after successful C-LTM cell switch execution and failure due to wrong beam should not be considered for L3 based CLTM.

Proposal 2: BFR shortly after successful C-LTM cell switch execution and failure due to wrong beam should not be considered for L1 or L3 based CLTM.

For last RAN3#129bis meeting agreed failure case due to outdated TA for intra-CU CLTM, the relevant case is that UE performs RACH-less CLTM cell switch execution with the target cell but fails when T304 is expired, then UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell with a TA different from the one included in LTM Candidate Timing Advance Command MAC CE. The following mechanism of identification and optimization for LTM cell switch failure due to outdated TA in intra-CU LTM as below can be taken as baseline for intra-CU CLTM, e.g.

- the target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- the target DU sends the TA value used at successful RACH-based access to the source DU via CU;

- the source DU may take it into account for optimisation of Timing Advance related parameters.

Proposal 3: For CLTM cell switch failure due to outdated TA:

- the target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- the target DU sends the TA value used at successful RACH-based access to the source DU via CU;

- the source DU may take it into account for optimisation of Timing Advance related parameters.

On the other hand, similar as MRO for intra-CU LTM agreed in R19, the failure occuring in intra-CU CLTM may be due to wrong selection of CLTM candidate cell, e.g. the cell for re-establishment is not one of at least one CLTM candidate cell selected by CU for CLTM preparation. For failures due to wrong selection of CLTM candidate cell, CU performs analysis and MRO optimization.

Proposal 4: The connection failure due to wrong selection of CLTM candidate cell should be considered for MRO for intra-CU CLTM, e.g. CU performs analysis and MRO optimization.

Additionaly, the failure occuring in intra-CU CLTM may be due to inappropriate CLTM execution condition, e.g. the cell for re-establishment or CLTM failure recovery is one of the at least one CLTM candidate cell selected by CU. Concerning failure detection and optimization,

- for failures due to inappropriate CLTM execution condition in L3 based intra-CU CLTM, CU performs analysis, since it is CU generates the L3 based CLTM execution conditions, CU proforms MRO optimization;

- for failures due to inappropriate CLTM execution condition in L1 based intra-CU CLTM, CU initiates analysis, since it is DU generates the L1 based CLTM execution conditions based on the request from CU, CU forwards the RLF report to the DU responsible for the failure (e.g. source DU, target DU or candidate DU which generates the inappropriate CLTM execution condition), then, the relevant DU performs MRO optimization.

Proposal 5: The connection failure due to inappropriate CLTM execution condition should be considered for MRO for intra-CU CLTM, e.g.

- for failures due to inappropriate CLTM execution condition in L3 based intra-CU CLTM, CU performs analysis and MRO optimization;

- for failures due to inappropriate CLTM execution condition in L1 based intra-CU CLTM, CU initiates analysis and forwards the RLF report to the DU responsible for the failure (e.g. source DU, target DU or candidate DU which generates the inappropriate CLTM execution condition), and the relevant DU then performs MRO optimization.

Specifically, for intra-CU CLTM, since the TA value for early UL synchronization is provided to the UE via an LTM Candidate Timing Advance Command MAC CE, it is possible that CLTM TAT timer starts before CLTM cell switch execution is triggered. Even early TA for RACH-less CLTM is configured to the UE, RACH-less CLTM cell switch execution may not be triggered as shown in following, this failure case should also be considered for MRO for intra-CU CLTM:

- Case I: CLTM TAT associated with target cell expires before/at the time of CLTM cell switch execution (e.g. before/at the time when CLTM execution condition of target cell is fulfilled), UE performs RACH based CLTM cell switch execution but fails, then, UE performs CLTM fast failure recovery or RRC re-establishment.

Moreover, even if RACH-less CLTM cell switch execution is triggered, RACH-less CLTM cell switch execution may fail due to wrong CLTM TAT as shown in following, this failure case should also be considered for MRO for intra-CU CLTM:

- Case II: UE performs RACH-less CLTM cell switch execution with the target cell, but CLTM TAT associated with target cell expires during RACH-less CLTM cell switch execution is on-going, while T304 is still running, then UE fallbacks to RACH based CLTM cell switch execution towards the target cell with a same TA as indicated in LTM Candidate Timing Advance Command MAC CE.

Proposal 6: The following cases should be considered for MRO for intra-CU CLTM,

- Case I: CLTM TAT associated with target cell expires before/at the time of CLTM cell switch execution (e.g. before/at the time when CLTM execution condition of target cell is fulfilled), UE performs RACH based CLTM cell switch execution but fails, then, UE performs CLTM fast failure recovery or RRC re-establishment.

- Case II: UE performs RACH-less CLTM cell switch execution with the target cell, but CLTM TAT associated with target cell expires during RACH-less CLTM cell switch execution is on-going, while T304 is still running, then UE fallbacks to RACH based CLTM cell switch execution towards the target cell with a same TA as indicated in LTM Candidate Timing Advance Command MAC CE.

  - 2.2 MRO for near failure successful case in intra-CU CLTM

Last RAN3#129bis meeting agreed to consider near failure successful case in intra-CU CLTM.

In R18 intra-CU LTM, fallback from RACH-less to RACH based LTM is not supported. For example, when receiving the LTM cell switch command MAC CE which includes TA value for early UL synchronization, UE starts the LTM TA validity timer and T304 timer, and performs RACH-less LTM as the TA value is valid.  If RACH-less LTM execution fails (e.g. LTM TA validity timer expires) but T304 is still running, the UE will wait T304 to expiry and then perform RRC re-establishment or RACH-based LTM failure recovery procedure.

However, in intra-CU CLTM, since the TA value for early UL synchronization is provided to the UE via an LTM Candidate Timing Advance Command MAC CE, it is possible that CLTM TAT timer starts before T304 timer starts. Fallback from RACH-less to RACH based CLTM is supported, e.g. UE has valid TA value of target cell at CLTM execution, UE performs RACH-less CLTM execution with target cell, but CLTM TAT timer associated with target cell expires during RACH-less CLTM execution, while T304 is still running, then UE will fallback to RACH based CLTM execution. This fallback happens due to the early TA value is outdated. Since CLTM aims at reducing mobility interruption time, fallback from RACH-less to RACH based CLTM is against this purpose. Even fallback from RACH-less to RACH based CLTM is successful, this may be considered as a near failure successful case.

Proposal 7: Consider successful fallback from RACH-less to RACH based CLTM as a near failure successful case for MRO.

3	Conclusion

In this paper, MRO for intra-CU CLTM is discussed. We have the following proposals:

Proposal 1: Agree the TP for TS38.300 for intra-CU CLTM.

Proposal 2: BFR shortly after successful C-LTM cell switch execution and failure due to wrong beam should not be considered for L1 or L3 based CLTM.

Proposal 3: For CLTM cell switch failure due to outdated TA:

- the target DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell;

- the target DU sends the TA value used at successful RACH-based access to the source DU via CU;

- the source DU may take it into account for optimisation of Timing Advance related parameters.

Proposal 4: The connection failure due to wrong selection of CLTM candidate cell should be considered for MRO for intra-CU CLTM, e.g. CU performs analysis and MRO optimization.

Proposal 5: The connection failure due to inappropriate CLTM execution condition should be considered for MRO for intra-CU CLTM, e.g.

- for failures due to inappropriate CLTM execution condition in L3 based intra-CU CLTM, CU performs analysis and MRO optimization;

- for failures due to inappropriate CLTM execution condition in L1 based intra-CU CLTM, CU initiates analysis and forwards the RLF report to the DU responsible for the failure (e.g. source DU, target DU or candidate DU which generates the inappropriate CLTM execution condition), and the relevant DU then performs MRO optimization.

Proposal 6: The following cases should be considered for MRO for intra-CU CLTM,

- Case I: CLTM TAT associated with target cell expires before/at the time of CLTM cell switch execution (e.g. before/at the time when CLTM execution condition of target cell is fulfilled), UE performs RACH based CLTM cell switch execution but fails, then, UE performs CLTM fast failure recovery or RRC re-establishment.

- Case II: UE performs RACH-less CLTM cell switch execution with the target cell, but CLTM TAT associated with target cell expires during RACH-less CLTM cell switch execution is on-going, while T304 is still running, then UE fallbacks to RACH based CLTM cell switch execution towards the target cell with a same TA as indicated in LTM Candidate Timing Advance Command MAC CE.

Proposal 7: Consider successful fallback from RACH-less to RACH based CLTM as a near failure successful case for MRO.

Annex: TP for TS38.300

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CLTM, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based CLTM or the successful reception of RRCReconfigurationComplete in case of RACH-less CLTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

In case of CHO with candidate SCG(s), the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO with candidate SCG(s), Too Early CHO with candidate SCG and CHO Execution to Wrong Cell.

In case of CLTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CLTM, Too Early CLTM and CLTM to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM/CLTM or CHO with candidate SCG(s) is configured but the CHO/LTM/CLTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover/LTM cell switch/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM/CLTM recovery is the cell that served the UE at the last handover/LTM/CLTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM/CLTM recovery is neither the cell that served the UE at the last handover/LTM/CLTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM/CLTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM/CLTM execution until connection failure.