3GPP TSG-RAN WG3 Meeting #130	R3-258372

Dallas, US, 17 – 21 November 2025

Agenda Item:	13.3

Source: 	Lenovo

Title: 	(TP to TR 38.765) Discussion on general procedures for gNB-based sensing

Document for:	Other

- Introduction

A new SID for Integrated Sensing and Communication [1] has been approved to be discussed in Rel-20 and following detail objectives are identified for RAN3.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution is to discuss general procedure for gNB-based mono-static sensing, including the procedures, signaling impacts in NG and F1.

- Discussion

Sensing entity selection

Based on the discussion in SA2 [2], sensing entity selection may be performed by the sensing related CN, e.g., AMF or SF. For example, in the indirect connectivity, the AMF can select the gNB for sensing based on sensing service parameters received from the SF and the sensing capabilities supported by the candidate sensing entities. Alternatively, the SF may perform discovery and selection of sensing entity. Anyway, the sensing related CN needs to be aware of the sensing capabilities supported by the candidate sensing entities before the sensing entity selection. Therefore, the candidate sensing gNBs need to report sensing capability to the sensing related CN for sensing entity selection, and for the contents of the sensing capabilities, it may at least include supported sensing geographic area, supported sensing service type, supported sensing functionality (Tx-only, Rx-only or Tx&Rx), supported sensing frequency band, sensing resolution…

Proposal 1: Candidate sensing gNBs report sensing capability to the sensing related CN for sensing entity selection.

Proposal 2: The sensing capability at least includes supported sensing geographic area, supported sensing service type, supported sensing functionality (Tx-only, Rx-only or Tx&Rx), supported sensing frequency band, sensing resolution.

Sensing request with configuration

Once the sensing related CN has determined the gNB for sensing, the CN node will initiate a sensing procedure and send a sensing request message to the selected sensing gNB. In the sensing request message, it may indicate the sensing functionality of the sensing entity, but in case of gNB-based mono-static sensing, the selected sensing gNB implicitly supports working as both sensing transmitter and sensing receiver. The sensing request message may further indicate sensing service ID, sensing area, object description, sensing KPIs, sensing Tx configuration, sensing Rx configuration and sensing result reporting configuration after measurement of the received sensing signal.

Proposal 3: The sensing related CN initiates a sensing procedure and sends a sensing request message to the selected sensing gNB.

Proposal 4: The sensing request message may indicate sensing functionality of the sensing entity, sensing service ID, sensing area, object description, sensing KPIs, the sensing Tx configuration, the sensing Rx configuration and the sensing result reporting configuration.

During the sensing request procedure between the CN and the sensing gNB, the CN (e.g., SF) may trigger data connection establishment towards the Sensing gNB. For example, the sensing gNB establishes direct data connection with the SF based on the address or end point information of the SF. In addition, the following sensing data, e.g., sensing report, can be transported via the data connection and encapsulated by GTP-U. As for the granularity of the data connection, it can be per node or per sensing service.

Proposal 5: During the sensing request procedure, the SF triggers data connection establishment towards the Sensing gNB with the address or end point information of the SF.

In case of CU-DU split architecture, the sensing gNB-CU will initiate one or more sensing request procedure in F1AP to trigger sensing at sensing gNB-DU(s). Wherein the F1AP sensing request message may also indicate at least sensing functionality of each gNB-DU, sensing Tx configuration, sensing Rx configuration and the sensing result reporting configuration to the corresponding gNB-DU.

Proposal 6: In case CU-DU split architecture, the sensing gNB-CU will initiate one or more sensing request procedure in F1AP to trigger sensing at sensing gNB-DU(s).

Sensing procedure update/cancellation

Based on the updated sensing requirement from the sensing consumer, or based on the condition of the of the sensing object/area, e.g., sensing UAV mobility, the sensing related CN may update or cancel the ongoing sensing procedure.

Because no inter-gNB coordination will be studied at this release, when considering sensing object mobility, the sensing related CN needs to cancel the ongoing sensing procedure at the original sensing gNB, and then the sensing related CN triggers a new sensing procedure at another sensing gNB based on the updated location of the sensing object to continuously perform sensing for the mobility object.

Proposal 7: Based on the sensing requirement or based on the condition of the sensing object/area, the sensing related CN may update or cancel the sensing procedure.

General sensing procedure

Based on the discussion and proposals above, following is a general sensing procedure for gNB-based mono-static sensing.

Figure 1: General procedure of gNB-based mono-static sensing

Step 1: The candidate sensing gNB reports its sensing capability to the sensing related CN (e.g., AMF for indirect connectivity or SF for direct connectivity).

Step 2: The sensing related CN selects the gNB as the sensing entity based on the sensing capability of the gNB and the sensing requirement.

Step 3: The sensing related CN determines to activate a sensing procedure at the sensing gNB and sends a sensing request message to the sensing gNB. The sensing request message at least includes the function of the sensing gNB (e.g., sensing transmitter and/or sensing receiver) and the configuration parameters (e.g., sensing signal configuration and sensing report configuration) for the sensing procedure.

Step 4: The SF in the CN triggers data connection establishment towards the sensing gNB with the address or end point information of the SF.

Step 5: The sensing gNB sends a sensing response message to the sensing related CN.

Step 6: The sensing gNB performs the sensing procedure by sending the sensing signal and measuring the received sensing signal to obtain sensing data.

Step 7: The sensing gNB sends one or more sensing report messages to the sensing related CN via the data connection established at Step 4.

Step 8: Based on the sensing requirement or based on the condition of the sensing object/area, the sensing related CN may update or cancel the sensing procedure.

In case CU-DU split architecture of the Sensing gNB, F1 interface between CU and DU needs to be further studied for sensing. Figure 2 is a general sensing procedure for gNB-based mono-static sensing when considering split RAN architecture.

Figure 2: General procedure of gNB-based mono-static sensing for split architecture

Step 1: The candidate sensing gNB-CU reports its sensing capability to the sensing related CN (e.g., AMF for indirect connectivity or SF for direct connectivity).

Step 2: The sensing related CN selects the gNB as the sensing entity based on the sensing capability of the gNB and the sensing requirement.

Step 3: The sensing related CN determines to activate a sensing procedure at the sensing gNB and sends a sensing request message to the sensing gNB-CU. The sensing request message at least includes the function of the sensing gNB (e.g., sensing transmitter and/or sensing receiver) and the configuration parameters (e.g., sensing signal configuration and sensing report configuration) for the sensing procedure.

Step 4: The gNB-CU sends a sensing request message via F1AP to the gNB-DU to trigger the sensing procedure.

Step 5: The gNB-DU sends a sensing response message to the gNB-CU via F1AP.

Step 6: The SF in the CN triggers data connection establishment towards the sensing gNB-CU with the address or end point information of the SF.

Step 7: The gNB-CU sends a sensing response message to the sensing related CN.

Step 8: The gNB-DU performs the sensing procedure by sending the sensing signal and measuring the received sensing signal to obtain sensing data.

Step 9: The gNB-DU sends one or more sensing report messages to the gNB-CU via F1AP.

Step 10: When receiving the sensing report from the gNB-DU, the gNB-CU sends one or more sensing report messages to the sensing related CN via the data connection established at Step 6.

Step 11: Based on the sensing requirement or based on the condition of the sensing object/area, the sensing related CN may update or cancel the sensing procedure.

Proposal 8: Agree the general procedure of gNB-based mono-static sensing for both aggregate RAN and split RAN architectures.

# Conclusion

This contribution aims to discuss general procedure for gNB-based mono-static sensing. And following observations and proposals are concluded.

Proposal 1: Candidate sensing gNBs report sensing capability to the sensing related CN for sensing entity selection.

Proposal 2: The sensing capability at least includes supported sensing geographic area, supported sensing service type, supported sensing functionality (Tx-only, Rx-only or Tx&Rx), supported sensing frequency band, sensing resolution.

Proposal 3: The sensing related CN initiates a sensing procedure and sends a sensing request message to the selected sensing gNB.

Proposal 4: The sensing request message may indicate sensing functionality of the sensing entity, sensing service ID, sensing area, object description, sensing KPIs, the sensing Tx configuration, the sensing Rx configuration and the sensing result reporting configuration.

Proposal 5: During the sensing request procedure, the SF triggers data connection establishment towards the Sensing gNB with the address or end point information of the SF.

Proposal 6: In case CU-DU split architecture, the sensing gNB-CU will initiate one or more sensing request procedure in F1AP to trigger sensing at sensing gNB-DU(s).

Proposal 7: Based on the sensing requirement or based on the condition of the sensing object/area, the sensing related CN may update or cancel the sensing procedure.

Proposal 8: Agree the general procedure of gNB-based mono-static sensing for both aggregate RAN and split RAN architectures.

# Reference

- RP-252819. Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR.

- TR 23.700-14. Study on Integrated Sensing and Communication; Stage 2 (Release 20).

# TP to TR 38.765

# 8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

## 8.a	General sensing procedure for aggregate RAN

Figure 8.a-1: General sensing procedure for aggregate RAN

Step 1: The candidate sensing gNB reports its sensing capability to the sensing related CN (e.g., AMF for indirect connectivity or SF for direct connectivity).

Step 2: The sensing related CN selects the gNB as the sensing entity based on the sensing capability of the gNB and the sensing requirement.

Step 3: The sensing related CN determines to activate a sensing procedure at the sensing gNB and sends a sensing request message to the sensing gNB. The sensing request message at least includes the function of the sensing gNB (e.g., sensing transmitter and/or sensing receiver) and the configuration parameters (e.g., sensing signal configuration and sensing report configuration) for the sensing procedure.

Step 4: The SF in the CN triggers data connection establishment towards the sensing gNB with the address or end point information of the SF.

Step 5: The sensing gNB sends a sensing response message to the sensing related CN.

Step 6: The sensing gNB performs the sensing procedure by sending the sensing signal and measuring the received sensing signal to obtain sensing data.

Step 7: The sensing gNB sends one or more sensing report messages to the sensing related CN via the data connection established at Step 4.

Step 8: Based on the sensing requirement or based on the condition of the sensing object/area, the sensing related CN may update or cancel the sensing procedure.

## 8.b	General sensing procedure for split RAN

Figure 8.b-1: General sensing procedure for split RAN

Step 1: The candidate sensing gNB-CU reports its sensing capability to the sensing related CN (e.g., AMF for indirect connectivity or SF for direct connectivity).

Step 2: The sensing related CN selects the gNB as the sensing entity based on the sensing capability of the gNB and the sensing requirement.

Step 3: The sensing related CN determines to activate a sensing procedure at the sensing gNB and sends a sensing request message to the sensing gNB-CU. The sensing request message at least includes the function of the sensing gNB (e.g., sensing transmitter and/or sensing receiver) and the configuration parameters (e.g., sensing signal configuration and sensing report configuration) for the sensing procedure.

Step 4: The gNB-CU sends a sensing request message via F1AP to the gNB-DU to trigger the sensing procedure.

Step 5: The gNB-DU sends a sensing response message to the gNB-CU via F1AP.

Step 6: The SF in the CN triggers data connection establishment towards the sensing gNB-CU with the address or end point information of the SF.

Step 7: The gNB-CU sends a sensing response message to the sensing related CN.

Step 8: The gNB-DU performs the sensing procedure by sending the sensing signal and measuring the received sensing signal to obtain sensing data.

Step 9: The gNB-DU sends one or more sensing report messages to the gNB-CU via F1AP.

Step 10: When receiving the sensing report from the gNB-DU, the gNB-CU sends one or more sensing report messages to the sensing related CN via the data connection established at Step 6.

Step 11: Based on the sensing requirement or based on the condition of the sensing object/area, the sensing related CN may update or cancel the sensing procedure.