3GPP TSG-RAN WG3 Meeting #130	R3-258425

Dallas, US, 17 – 21 November 2025

Agenda item:	10.2.2

Source:	Nokia, T-Mobile USA, DT

Title:	Service awareness in RAN

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we follow up on the discussion initiated at RAN3#129bis relative to the study of impact of enhanced service awareness on the 6G RAN architecture as captured in chair’s notes:

Study the impact of enhanced RAN-based service awareness on 6G RAN architecture.

We further provide clarifications to the service awareness in RAN and discuss existing 5G mechanisms and potential enhancements required to help the progress of the topic in RAN3.

# 2	Discussion on service awareness in RAN

## 2.1	Motivation

As we embark into the 6G era, new services and requirements from new applications need to be accommodated into 6G networks. For example, new applications and services such as XR and AI exhibit different traffic patterns compared to eMBB and other traditional traffic types.

Taking AI as a first example:

AI services are expected to become more relevant for 6G, with an explosive growth of AI applications and its integration in different types of devices. Currently in SA1 TR 22.870, different use cases and service requirements for 6G are studied, where AI is included as one of the key technologies.

RAN has already taken an assumption in [1] for network for AI according:

Note: NW for AI is assumed to be covered by new services

According to this assumption, network for AI could from the perspective of the 6G RAN represent feature(s) enabling the UE to transfer data over the user plane in a more efficient way such that user experience can be enhanced.

On a similar note, a new angle of dynamicity for 6G networks lies on new UE behaviour related to AI traffic (e.g., AI prompts) difficult to predict in the RAN. Therefore, collecting enhanced information from the UE to help the RAN handle such dynamic UE behaviour becomes crucial.

Another example of such new services could be evolution of XR:

In recent 5G-A releases, XR has been explored in 3GPP resulting in the introduction of PDU Sets and additional traffic assistance information. XR services are a clear example of how application awareness can help optimizing QoS flow provisioning and other RAN operations like UE power saving configurations. Still, for 6G, XR use cases continue to evolve further, bringing more challenges to RAN.

Considering these two examples, and the fact that RAN needs to optimize scarce radio resources, it is important that RAN obtains the proper means to adequately manage its operations to provide the best possible user experience. It has already been acknowledged by RAN Plenary that the awareness of the application requirements and characteristics in the 5G RAN is currently insufficient and needs further enhancements, as captured by the following requirement in TR 38.914 [3]:

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

Observation 1: Traffic generated by new applications such as XR and AI are of dynamic nature, thus they exhibit certain traffic characteristics which should be leveraged in RAN to provide better user experience as well as scheduling efficiency.

## 2.2 	Terminology and definition

While the need for enhanced service awareness in 6G RAN is acknowledged, we think that it is important in a first step, before discussing further enhancements, that RAN3 agrees on a common understanding of the terms “service awareness in RAN”,  “RAN service awareness” or “RAN-based service awareness” and establishes a clear definition of service awareness in RAN.

Proposal 1: RAN3 to agree on a common definition and converge on the usage of a single term for “service awareness in RAN”, “RAN service awareness” or “RAN-based service awareness” and its definition.

In our view, we believe that service awareness in RAN refers to methods for RAN obtaining relevant information to handle an application or service.

Proposal 2: Add the following definition to the TR: Service awareness in RAN refers to the methods for RAN obtaining relevant information to handle an application or a service.

## 2.3 	Building on 5G existing mechanisms

Although not explicitly handled under the umbrella topic of service awareness in RAN, there are already existing mechanisms in 5G standards that specify means for providing service awareness in RAN. Examples of such mechanisms are provided below and further illustrated in the Figure 1:

- QoS framework (e.g. traditional QoS flow-based framework, PDU Set QoS handling for XR traffic).

- QoE Measurement framework to make QoE metrics visible at RAN (i.e., RAN-visible QoE) which are provided by UE.

- UE Assistance Information (UAI) with UL traffic information which is provided by UE with respect to an application.

- Assistance information provided by AF via CN for use in RAN (e.g., PDU Set relevant information via GTP-u header, TSC traffic characteristics (TSCAI) describes the TSC traffic characteristics etc.)

Figure 1: Existing 5G framework and information that can be considered for Service Awareness in RAN

Based on the definition of service awareness in RAN provided in Proposal 2 and the efforts put into specifying existing 5G mechanisms, RAN3 should first analyse new 6G services and their needs in consideration of existing 5G mechanisms which should form the basis for further needed enhancements for service awareness in the 6G RAN architecture. Also, proposals for enhancements on existing mechanisms as well as new proposals need to be discussed and evaluated.

Proposal 3: Service awareness in 6G RAN is based on existing 5G mechanisms. FFS on how to enhance 5G mechanisms considering new 6G services and their needs.

It is observed that several mechanisms have impact on multiple 3GPP working groups, for example UAI and RVQoE on RAN2, QoS framework and information provided by AF via CN on SA2. Therefore, it is important that coordination with the WGs is maintained to ensure the right progress of the topic in RAN3.

Observation 2: RAN3 will need to coordinate with RAN2 and SA2 on aspects that impact service awareness in RAN.

## 2.4	Impact of enhanced service awareness on 6G RAN architecture

Based on the requirement from RAN plenary [2] and the clear need for enhanced visibility of RAN for new applications and services, a study is needed in RAN3 to understand the requirements of new services, define the information availability which can assist RAN to enhance the user experience and define the related framework that is used to provide such information to the RAN (from UE as well as from AF via CN).

Below are a few examples of service awareness information that can be leveraged in RAN:

- QoS requirements and related performance metrics preferences.

- Traffic characteristics e.g., direction, periodicity, burst size, burst arrival time, burst ending time, etc.

- Relationship/relevance of packets within the traffic flow e.g., burst, PDU Set information, etc.

- Relationship between traffic flows.

- Application-level QoE metrics.

Proposal 4: RAN3 to focus the study on the information/metrics needed in RAN to provide better user experience considering new services and the mechanisms used to provide such information/metrics to RAN from UE as well as from AF via CN.

Similarly, monitoring connectivity and QoE metrics are equally important for enabling services with advanced requirements such as immersive communications and AI traffic. Therefore, 3GPP specifications should enable mechanisms to allow better awareness in terms of tracking the fulfillment of service requirements in RAN and enabling enhanced reporting with different frequency as well as granularity.

Proposal 5: RAN3 to study mechanisms for 6G RAN to allow enhanced user experience and QoS monitoring to ensure service assurance.

# 3	Conclusion

This paper has discussed key aspects of 6G RAN-based service awareness and has made the following proposals:

Proposal 1: RAN3 to agree on a common definition and converge on the usage of a single term for “Service awareness in RAN”, “RAN service awareness” or “RAN-based service awareness” and its definition.

Proposal 2: Add the following definition to the TR: Service awareness in RAN refers to the methods for RAN obtaining relevant information to handle an application or a service.

Proposal 3: Service awareness in 6G RAN is based on existing 5G mechanisms. FFS on how to enhance 5G mechanisms considering new 6G services and their needs.

Proposal 4: RAN3 to focus the study on the information/metrics needed in RAN to provide better user experience considering new services and the mechanisms used to provide such information/metrics to RAN from UE as well as from AF via CN.

Proposal 5: RAN3 to study mechanisms for 6G RAN to allow enhanced user experience and QoS monitoring to ensure service assurance.

A corresponding pCR to the TR is provided in annex of this paper.

# 4	References

[1] RP-252912, 6G RAN study

[2] RP-251866, New WID on XR (eXtended Reality) for NR Phase 4, RAN#108

[3] TR 38.760-3, Study on 6G Radio RAN3 aspects, v0.1.0

# Annex – pCR to TR 38.760-3

This pCR is based on [3].

# 4	Definitions of terms, symbols and abbreviations

## 4.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Service awareness in RAN: Refers to the methods for RAN obtaining relevant information to handle an application or a service.

## 4.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 4.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 5	Objectives and requirements

Editor’s note: The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

## 5.1	General Principles

All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles.

The following requirement from TR 38.914 includes terms defined in clause 4.1 for the purpose of the present study:

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

## 5.2	Deployment Scenarios

Editor’s note: This section may be used to describe the details/solutions related to deployment scenarios as per 38.914.

The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914.

-  FFS on the implications of this requirement on 6G RAN architecture.

-  FFS whether all deployment scenarios of TR 38.914 can be supported.

## 5.3	Overall RAN Architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

### 5.3.1	RAN-CN Functional Split

Editor’s note: The aim of this section is to describe functions split between RAN and CN.

### 5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN.

### 5.3.2.X Service awareness in RAN

6G radio access network (6G RAN) shall have means to address evolving and new services like immersive communication or AI traffic effectively and adapt to changing conditions.

6G RAN needs to obtain more visibility to application-level attributes (e.g., performance, requirements, etc.) to effectively allocate radio resources and utilize the different RAN resources to meet the applications minimum requirements while maintaining healthy overall system spectrum efficiency and utilization of RAN resources.

6G RAN shall therefore fulfil the following requirements:

- 6G RAN shall be able to understand and adapt to the traffic demands of applications while maintaining overall system efficiency.

- 6G RAN shall be able to receive and exploit traffic assistance information regarding applications from the UE.

- 6G RAN shall be able to receive and exploit further application information from AF via the 6G CN

- 6G RAN shall be able to perform enhanced service monitoring to ensure service assurance.

- 6G RAN shall be able to receive enhanced reporting from the UE to improve service awareness.