3GPP TSG-RAN WG3 Meeting #130	R3-258426

Dallas, US, 17 – 21 November 2025

Agenda item:	10.3.1

Source:	Nokia, Orange

Title:	6G RAN-CN interface Requirements

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

# 1	Introduction

At RANP#108 the new SID on 6G RAN was agreed in [1] and was further revised at RAN#109 in [2].

The main architectural impacts for RAN3 are listed below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

As part of point b/, RAN3 was tasked with a first milestone to achieve:

#### 4.1.1	Interim Milestone

Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810].

TSG#112 (June/2026):

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

This paper addresses specifically the first item of RAN-CN interface.

At last RAN3#129bis an initial list of “principles” were agreed to be fulfilled by RAN-CN interface solution/framework. This paper aims at proposing additional requirements or principles for the proposed solutions.

# 2	Discussion

RAN3 must also take into account the following requirements expressed in the justification that led to the objectives of the 6G RAN study item as described in [2]:

Avoiding excessive configurations and minimizing multiple options for the same functionality directly addresses real-world concerns about implementation complexity, energy consumption, and UE burden. This is especially relevant given the expected wide adoption of 6G across both consumer and industrial verticals, where cost-efficiency and performance trade-offs are critical.

Performance

Performance is listed as key justification item for the study.

The RAN-CN control plane interface is involved in several types of CP traffic. Taking 5G as a reference, the CP traffic over NG interface can be classified into three categories: UE to NFx (NAS and NRPPa), RAN to RAN (SON configuration transfer, remote interference management), RAN and NFx (MBS, PWS, etc.). The NAS transfer is the one probably the most critical for the performance and the below figure shows that service request is the prominently used procedure currently:

We think that it would be hardly acceptable to introduce 6G with KPI for service request worse than 5G. Performance for service request is therefore a key requirement. Talking about performance includes latency but also signaling overhead and processing time.

Requirement 1: the RAN-CN CP interface shall not degrade the latency, signaling overhead and processing time for service requests compared to 5G.

Cost efficiency

Cost efficiency is also listed above as a key justification item for the study. Moreover, last RAN#109 agreed the following requirement in clause 5.2 of TR 38.914 [4]:

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

Cost efficiency seems emphasized for the purpose of “avoiding multiple options for the same functionality”. For example, SBI is currently specified in core network with four models A, B,C, and D, and if SBI is proposed for RAN-CN interface it is assumed that down-selection would first be done among these options.

In general, solutions which minimize implementation complexity should be targeted to reduce CAPEX. For example, it should preferably be built on off-the-shelf software widely available to minimize cost.

Requirement 2: the RAN-CN CP interface shall minimize complexity and avoid multiple options.

Interoperability and Standardization/testing Effort

It has always been a key requirement from operators to keep RAN-CN interface fully open in order to enable multi-vendor system deployment. Therefore, the solution should be standardized in order to be fully interoperable in multi-vendor context. This principle was again stated at last RAN#109 in clause 5.2 of the latest TR 38.914 [4]:

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

At the same time, standardization alone is not enough for successful inter-operation between different vendors. Testing must be done between any two vendors and this is an additional burden and cost which operators have always tried to minimize.

Requirement 3: The RAN-CN interface shall minimize standardization and testing efforts to reduce OPEX.

Deployment ease and flexibility

Deployment aspects, such as scalability, should also be considered to adapt to the variations of traffic, support of cloud-friendly deployments, etc..

For example, current 5G solution inherits some limitations related to SCTP. In order to flexibly adjust the load, 3GPP has designed the TNLA solution for load balancing but this is not very efficient. It is expected that 6G brings easier scalability.

Requirement 4: The RAN-CN interface shall be scalable.

Smooth Migration

The migration from 5G RAN-CN CP interface to 6G RAN-CN CP interface should preferably not entail dramatic and disruptive changes in order to ease migration. For example, coordination between nodes and failure handling logic between RAN and CN should not drastically change to enable reuse of nodal behaviors and interoperability solutions. This will minimize both implementation and deployment costs.

For example, some NAS messages (e.g.  authentication, GUTI re-allocation), but not all, may have higher priority than e.g. SM messages. In 5G this collision handling is managed by the AMF and therefore not visible on the RAN-CN interface. To allow a smooth migration, 6G RAN-CN interface should remain transparent and shall not result in managing additional collision handling scenarios.

Requirement 5: The RAN-CN interface shall not result in additional collision handling scenarios.

# 3	Conclusion

This paper has elaborated additional key requirements for the control plane solution of RAN-CN interface.

It is proposed to agree and capture the following requirements:

Requirement 1: the RAN-CN CP interface shall not degrade the latency, signaling overhead and processing time for service requests compared to 5G.

Requirement 2: the RAN-CN CP interface shall minimize complexity and avoid multiple options.

Requirement 3: The RAN-CN interface shall minimize standardization and testing efforts to reduce OPEX.

Requirement 4: The RAN-CN interface shall be scalable.

Requirement 5: The RAN-CN interface shall not result in additional collision handling scenarios.

# 4	References

- RP-251881, 6G RAN study

- RP-252912, 6G RAN Study (revision of RP-251881)

- SP-250806, Study on Architecture for 6G System

- RP-252882, TR 38.914, Study on 6G Scenarios and Requirements (release 20)

## pCR for TR 38.760-3

## 6.1		RAN-CN Interface

### 6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

### 6.1.2	Requirements

Editor’s note: The aim of this section is to describe requirements helping to define RAN-CN Interface solution/framework.

The requirements for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN control plane interface shall not degrade the latency, the signalling overhead and the processing time of service requests compared to 5G;

-	the 6G RAN-CN interface shall minimize complexity and avoid multiple options;

-	the 6G RAN-CN interface shall minimize the standardization and testing efforts to reduce OPEX;

-	the 6G RAN-CN interface shall be scalable;

-	the 6G RAN-CN interface shall not result in additional collision handling scenarios.