3GPP TSG-RAN WG3 #130	R3-258435

Dallas, U.S.A., 17-21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1641 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Abnormal conditions for gNB-CU Configuration Update with updated Presence IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio Platforms, InterDigital, Fibercop |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When a predicted CCO issue is being signalled or cancelled, some IEs need to always be present, while other IEs need to have matching information. In the Predicted CCO Assistance Information IE, the Predicted CCO Issue IE and the Predicted Affected Cells and Beams IE are always needed, and their presence should be mandatory. Furthermore, the presence of the Neighbour Future Coverage Modification Notification IE in the GNB-CU CONFIGURATION UPDATE message is dependent on the presence of the Predicted CCO Assistance Information IE. Also, the list of cells and beams in the Neighbour Future Coverage Modification Notification IE needs to match the list of cells and beams not served by the gNB-DU in the Predicted Affected Cells and Beams IE. Finally, whenever a predicted CCO issue is cancelled, the indicated cells and beams need to match the cells and beams of a previously signalled predicted CCO issue. Therefore, the following cases of abnormal conditions in the gNB-CU Configuration Update procedure may arise: - the gNB-DU receives a GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE with the Predicted CCO issue IE set to “cancel” but the listed cells and beams do not match the list of cells and beams contained in a previously received Predicted CCO Assistance Information IE. - the gNB-DU not serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE, but the Neighbour Future Coverage Modification Notification IE is not present. - the gNB-DU receives a GNB-CU CONFIGURATION UPDATE message which includes the Neighbour Future Coverage Modification Notification IE, but the Predicted CCO Assistance Information IE is not present. - the gNB-DU not controlling the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE. The list of cells and beams in the Predicted Affected Cells and Beams IE not served by the gNB-DU is not the same as the list of cells and beams contained in the Neighbour Future Coverage Modification Notification IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the presence of the Predicted CCO Issue IE and the Predicted Affected Cells and Beams IE to mandatory. Add abnormal conditions in the gNB-CU Configuration Update procedure for the cases above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relevant information for the gNB-DU to understand predicted CCO issues might not be present. In the abnormal conditions described for cases above, the handling for predicted CCO issues is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.4, 9.3.1.367, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

** skip unchanged **

#### 8.2.5.4	Abnormal Conditions

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted CCO Issue IE is set to “cancel”, and

- the list of cells and beams contained in the Predicted Affected Cells and Beams IE is not the same as the list of cells and beams contained in a previously received Predicted CCO Assistance Information IE,

then the gNB-DU shall discard the received Predicted CCO Assistance Information IE and assume that the predicted CCO issue is not cancelled.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the list of cells and beams contained in the Predicted Affected Cells and Beams IE includes cells and beams that are not served by the gNB-DU, and

- the Neighbour Future Coverage Modification Notification IE is not present,

then the gNB-DU shall discard the received Predicted CCO Assistance Information IE.

If the Neighbour Future Coverage Modification Notification IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted CCO Assistance Information IE is not present,

then the gNB-DU shall discard the received Neighbour Future Coverage Modification Notification IE.

If the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE are contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the list of cells and beams contained in the Predicted Affected Cells and Beams IE includes cells and beams that are not served by the gNB-DU, and

- the list of cells and beams contained in the Neighbour Future Coverage Modification Notification IE is not the same as the list of cells and beams not served by the gNB-DU contained in the Predicted Affected Cells and Beams IE,

then the gNB-DU shall discard the received Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.367	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO Issue | M |  | ENUMERATED (coverage, cell edge capacity, cancel, ...) | Indicates the type of predicted CCO issue or that the previously sent predicted CCO issue is cancelled. |
| Predicted Affected Cells and Beams | M |  | Affected Cells and Beams 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

<<<<<<<<<<<<<<<<<<<< Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### -- P

<<<<<<<<<<<<<<<<<<<< Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

Predicted-CCO-Assistance-Information ::= SEQUENCE {

predicted-CCO-issue-detection			Predicted-CCO-issue-detection	,

predictedAffectedCellsAndBeams-List		AffectedCellsAndBeams-List		,

timeforPredictedCCOIssue				TimeforPredictedCCOIssue		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {Predicted-CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

Predicted-CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Predicted-CCO-issue-detection	 ::=	 ENUMERATED {coverage, cell-edge-capacity, cancel, ...}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>