3GPP TSG-RAN WG3 Meeting #130	R3-258447

Dallas, US, 17th – 21st November, 2025

Agenda Item:	10.3.1

Source:	CATT

Title:	Considerations on 6G RAN-CN interface principles and functions

Document for:	Decision

1	Introduction

In the last RAN3 meeting, we initially discussed the principles and functions for 6G RAN CN interface. Some general principles and interface functions have been captured in the TR38.760-3 [6], as highlighted below:

## 6.1		RAN-CN Interface

### 6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

### 6.1.2	RAN-CN Interface Functions

RAN-CN control plane interface supports following functions:

-	UE context management	: The functionality to manage UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress;

-	PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress.

However, there’re still a few legacy few principles and functions not agreed yet, neither for the principles and functions for the new services.

In this contribution, we will further discuss the principles and functions for the 6G RAN CN interface, considering both legacy communication based functions and the new services based functions. Base on the discussion, we will provide the observations and proposals accordingly.

2	Discussion

2.1 6G RAN-CN interface general principles

As discussed and proposed in the [6], the legacy NG-like way could be considered for the legacy functions (communication related functions as defined for NG). But for the new services (e.g. AI/ML, Sensing), we could consider direct connection and more efficient data transmission between 6G RAN node and corresponding function(s) in the 6GC.

- Consider legacy services/functions and new services separately when we define the 6G RAN-CN interface(s).

In the last RAN3 meetings, the following general principles for 6G RAN CN interface have been captured to the TR:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

Besides the principles above, the following principles for 5G NG interface as specified in TS 38.410 have not been captured, as below:

-	the NG interface is open;

-	from a logical standpoint, the NG is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC;

-	the NG interface separates Radio Network Layer and Transport Network Layer;

-	the NG Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency.

We understand there’s no doubt that “6G RAN-CN interface is open” to allow inter-vendor operation between RAN and CN, this principle should also be applied to the other interfaces.

- The 6G RAN-CN interface should be open to allow inter-vendor operations.

Similar to the S1/NG interface designed for LTE/NR, the 6G RAN CN interface also separates Radio Network Layer and Transport Network Layer. We do not see any necessary to change the principle. For the detail protocol stack, the detail Transport Layer to be adopted could be further discussed.

- The 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer.

As majority of companies prefer to reuse the legacy 5G NG interface as much as possible. For the remaining principles as above, they could be adopted for 6G RAN CN interface, at least for the legacy communication based functionalities. But for the new services, e.g. AI/ML, Sensing, whether adopt P2P vs SBI interface to be further decided. To make more straightforward and efficient signalling and data transmission between 6G RAN node and corresponding service function(s) in the CN, direct connection between 6G RAN and new service function(s) is preferred, especially for the non-UE associated procedures.

- The 6G RAN-CN interface is a point-to-point interface for legacy functions, FFS for the new service functions.

For the Application Protocol procedures design, the NG Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency. It’s assumed that the same principle could be adopted for 6G, at least for legacy functions. For the new services, more study is needed.

- The 6G RAN-CN interface Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding/decoding efficiency, at least for legacy functions. FFS for the new service functions.

In the last RAN3 meeting, we also discussed whether use different procedures for different functionalities in the 6G RAN-CN interface. We understand that, normally different procedures are defined for different functionalities, but we do not see any necessary to preclude some optimized design, e.g. UE Context Retrieval procedure over the Xn could be applied to several cases, inactive operations, connection reestablishment, etc. With the understanding above, the 6G RAN-CN interface uses same or different procedures for different functionalities, no need to make any restriction on this.

- The 6G RAN-CN interface uses same or different procedures for different functionalities, no need to capture it as a principle.

As above, we propose to capture the following bullets as the general principles on 6G RAN-CN interface:

-	the 6G RAN-CN interface is open;

-	the 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer;

-	from a logical standpoint, the 6G RAN-CN interface is a point-to-point interface between an 6G RAN node and a Core Network node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the 6G RAN and CN; [Applied to legacy functions, FFS for the new services]

-	the 6G RAN CN interface Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency. [Applied to legacy functions, FFS for the new services]

2.2 6G RAN-CN Interface Functions

In the last RAN3 meeting, we initially discussed the 6G RAN-CN interface functions, and agreed that:

RAN-CN control plane interface supports following functions:

-	UE context management	: The functionality to manage UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress;

-	PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress.

Besides these functions, there are still some legacy NG interface functions not captured. Here we will further check one by one.

-	interface management;

This is a very basic function for an interface, which includes the functionality to setup/update/reset/remove of the interface, we understand it can still be applied to the 6G RAN-CN interface.

-	UE mobility management;

The mobility function for UEs in CM-CONNECTED includes which includes intra/inter system handover. It comprises the preparation, execution and completion of handover via the 6G RAN-CN interface. [FFS on details]

-	Paging;

The paging function supports the sending of paging requests to the RAN nodes involved in the paging area e.g. the TA(s) the UE is registered, subject to SA2 progress.

-	Configuration Transfer;

The Configuration Transfer function is a generic mechanism that allows the request and transfer of RAN configuration information (e.g. SON information) between two RAN nodes via the core network. As we could not assume the direct interface between two RAN nodes is always available, it's beneficial to support this function in 6G.

-	Warning Message Transmission.

The warning message transmission function provides means to transfer warning messages or cancel ongoing broadcast of warning messages. To satisfy the requirements of public safety services, this function is also essential for the 6G system. This is also subject to SA2 progress.

We understand the above RAN-CN functions are most basic and essential, which should also be applied to 6G for legacy communication based functions, even though some details are pending to SA2 progress.

- Further capture the following RAN-CN interface functions for legacy communication based functions:

-	interface management;

-	UE mobility management;

-	Paging;

-	Configuration Transfer;

-	Warning Message Transmission.

In TS38.410 section 5, there’re still many other NG functions, e.g.:

- Trace function;

- AMF Management function;

- Multiple TNL Associations Support Function;

- Location Reporting function

- Overload Control function

- …

RAN3 should discuss and decide whether and how to take the other NG functions to 6G RAN-CN interface.

- RAN3 is requested to discuss and decide whether and how to take the other NG functions as specified in section 5 of TS38.410 to 6G RAN-CN interface.

On support of the new services, it’s assumed that a few new 6G RAN-CN interface functions will be defined, e.g. sensing related function, they could be further added base on the progress of the discussion on new services.

- 6G RAN-CN interface functions for the new services to be added later base on the progress of the discussion on the new services.

As above, a pCR to TR 38.760-3 is provided in section 5, target to add some general principles and some functions for 6G RAN-CN interface, RAN3 is requested to discuss and agree the pCR.

- Discuss and agree the pCR to TR 38.760-3 on general principles and functions for 6G RAN-CN interface.

3	Conclusion

Base on the discussion above, we provide the following observations and proposal:

- Consider legacy services/functions and new services separately when we define the 6G RAN-CN interface(s).

- The 6G RAN-CN interface should be open to allow inter-vendor operations.

- The 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer.

- The 6G RAN-CN interface is a point-to-point interface for legacy functions, FFS for the new service functions.

- The 6G RAN-CN interface Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding/decoding efficiency, at least for legacy functions. FFS for the new service functions.

- The 6G RAN-CN interface uses same or different procedures for different functionalities, no need to capture it as a principle.

- Further capture the following RAN-CN interface functions for legacy communication based functions:

- interface management;

- UE mobility management;

- Paging;

- Configuration Transfer;

- Warning Message Transmission.

- RAN3 is requested to discuss and decide whether and how to take the other NG functions as specified in section 5 of TS38.410 to 6G RAN-CN interface.

- 6G RAN-CN interface functions for the new services to be added later base on the progress of the discussion on the new services.

- Discuss and agree the pCR to TR 38.760-3 on general principles and functions for 6G RAN-CN interface.

4	Reference

- RP-251881 New SID: Study on 6G Radio

- 3GPP TR38.801 v14.0 Study on new radio access technology: Radio access architecture and interfaces;

- 3GPP TS38.410 v18.6.0 NG general aspects and principles

- 3GPP TS38.413 v18.6.0 NG Application Protocol (NGAP)

- 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

- TR 38.760-3 Study on 6G Radio RAN3 aspects, v0.1.0

5	pCR to TR 38.760-3

<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

6.1		RAN-CN Interface

6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

-	the 6G RAN-CN interface is open;

-	the 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer;

-	from a logical standpoint, the 6G RAN-CN interface is a point-to-point interface between an 6G RAN node and a Core Network node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the 6G RAN and CN; [Applied to legacy functions, FFS for new services]

-	the 6G RAN CN interface Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency. [Applied to legacy functions, FFS for new services]

NOTE: The principles above may need to be updated according to the progress of new services discussion.

<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>

6.1.2	RAN-CN Interface Functions

RAN-CN control plane interface supports following functions:

-	UE context management	: The functionality to manage UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress;

-	PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress.

-	interface management: The functionality to maintain  the interface between 6G RAN-CN, including Interface Setup, Configuration Update, Interface Remove, Reset, Error Indication;

-	UE mobility management: The functionality to support intra/inter system handover; [FFS on the details]

-	Paging: The functionality to page a UE in the paging area, e.g. the TA(s) UE is registered, subject to SA2 progress;

-	Configuration Transfer: The functionality to allow request and transfer of RAN configuration information between two RAN nodes via the core network;

-	Warning Message Transmission: The functionality to transfer warning messages or cancel ongoing broadcast of warning messages, subject to SA2 progress.

NOTE: 	the above RAN-CN interface functions are for legacy communication based functions, FFS on whether and how to take the other legacy NG interface functions as specified in TS38.410, FFS on the functions for the new services.

<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>