3GPP TSG-RAN WG3 #130	R3-258454

Dallas USA, 17-21 November 2025

Agenda Item:	13.2

Source:	Ericsson

Title:	ISAC Logical Architecture and Protocol Stack

Document for:	Agreement

# Introduction

Current status for ISAC architecture after RAN3 #129bis is as follows: [1][2]

- Agreements:

  - Logical architecture, captured in [3]; details for Nx interface between gNB and SF to be further discussed

  - All terms, symbols and abbreviations in TR 38.765 refer to TR 23.700-14 (to be further checked with TR Rapporteur; no TP agreed at this point)

  - RAN3 focuses on sensing protocol in coordination with SA2

- For further discussion:

  - Network architecture: Protocol stacks, in particular the protocol stack for sensing reporting (to be aligned with SA2)

  - RAN-CN procedures and signaling: Necessary interface functions and detailed signaling procedures [covered in another contribution]

This contribution provides further discussion on the Network architecture and protocol stack aspects.

# Discussion

## General Considerations

It seems beneficial to start with general considerations and principles that can guide us in the discussion of architecture options. In this section we continue to elaborate on general considerations from discussion at RAN3 #129bis and from our previous contribution [4].

According to the ISAC SID [5], no inter-gNB coordination will be studied; from this it descends that there will be no Xn impact.

Observation 1: ISAC does not impact Xn.

Only gNB-based sensing is in the scope of the ISAC SID [5], so there are no UEs involved. From this it descends that there is no impact on UE-associated signaling due to ISAC. Furthermore, as UP is terminated in the UEs, it also descends that ISAC does not impact UP.

Observation 2: ISAC impacts neither UE-associated signaling nor UP.

The logical consequence is that Nx, the logical interface between gNB and SF, only carries non-UE-associated CP signaling.

Observation 3: Nx only carries non-UE-associated CP signaling.

Ongoing discussions and simulations in RAN1 refer to the TRP (Transmission and Reception Point), which is responsible for transmitting and/or receiving sensing signals. From the point of view of logical architecture, however, we can consider the TRP as a logical function of the gNB (consistently with other legacy NG-RAN features that involve the TRP).

Observation 4: The TRP, responsible for transmitting and/or receiving sensing signals, is a logical function of the gNB.

Monostatic sensing is in the scope of the ISAC SID, and architecture choices that enable e.g. bistatic sensing are not precluded [5]. For this reason, it does not seem necessary to restrict a gNB to only have a single TRP for sensing.

Observation 5: A gNB may have one or more TRPs.

Even though a number of TRPs may be involved in the same sensing session, the filtering, processing and fusion of the results (depending on RAN deployment) should take place in the RAN (See Sec. 2.2 for details). For this reason, it does not seem needed to expose TRPs and their parameters outside of the RAN.

Observation 6: The TRPs are not visible outside of the RAN.

Proposal 1: Capture the above observations as general architecture principles of ISAC in the TR.

## Reporting Results to the SF

The issue of how large the sensing reports from RAN to SF is likely to depend on the ongoing RAN1 discussions on Sensing measurements and quantization. As we previously noted in [4], sensing results to be reported from RAN may include the (x,y,z) object estimated location, 3D estimated velocity, and possibly others. For tracking purposes, sensing results produced by the RAN may need to be reported periodically. Sensing results will be generated by appropriately processing one or more sensing measurements, as collected by TRP(s) within the RAN.

To limit as much as possible the processing load on CN and the backhaul traffic load, it seems critical that measurement processing, to generate the sensing reports, is done in the RAN. This also reduces the need, and the amount of side information that RAN would have to additionally report to SF to assist in processing any raw or intermediate results.

Proposal 2: As much processing as possible should be done within the RAN to generate the sensing results to be reported to the SF.

Depending on the amount of data to be reported (especially considering the case of periodic reporting), it seems beneficial to optionally allow reporting sensing results via a direct data connection from NG-RAN to SF, similarly to existing MDT functionality (i.e. trace). An example of direct data connection can be based on RFC 6455 WebSocket. Such a direct data connection should not be intended as the termination of an additional interface: the SF may include the needed information (e.g., a Uniform Resource Identifier (URI) indicating the streaming channel over which the sensing reports can be streamed) in the SENSING REQUEST message, and if the gNB accepts the request, it uses this information for reporting. This additional, optional, functionality is fully accommodated by the logical architecture agreed by RAN3.

Proposal 3: A direct data connection for result reporting, to be optionally set up based on information signaled by the SF in the SENSING REQUEST (e.g., similar to MDT trace function), should be supported; it can be fully accommodated by the architecture agreed by RAN3.

## Protocol Stack

Given the agreed logical architecture, it seems logical to adopt a protocol that supports this dedicated CP interface, Nx (see Figure 1).

Given that all existing CP interfaces in NG-RAN use SCTP as transport protocol, we propose that also Nx uses SCTP as transport protocol.

Figure 1 Protocol stack for the sensing interface Nx.

Proposal 4: The protocol stack for the direct, dedicated sensing interface Nx shall reuse SCTP.

## Considerations on Split gNB for ISAC

We should first note that F1 impacts are not mentioned in the ISAC SID [5]. None the less, some considerations on ISAC with respect to split gNB can be made.

Rel-20 is fairly late in the NG-RAN lifecycle, and ISAC is a new technology. Furthermore, ISAC is also expected to be adopted as an early 6G feature, whereas 6G RAN architecture discussions are conducted independently of ISAC discussions. Hence, we should avoid the risk of pre-empting 6G ISAC in any way due to architecture considerations which at this point are only pertinent to legacy NG-RAN, like CU-DU split.

For this reason, at this point it seems beneficial to de-prioritize discussion on ISAC for split gNBs. This seems to have the following benefits:

- It avoids impact to already deployed CUs and DUs,

- It avoids F1AP impacts due to ISAC;

Proposal 5: To limit impacts on NG-RAN and any possible additional impacts on 6G ISAC, split gNB architecture should not be considered (or at least de-prioritized) in NG-RAN ISAC.

# Observations and Proposals

Our observations and proposals are summarized below.

Observation 1: ISAC does not impact Xn.

Observation 2: ISAC impacts neither UE-associated signaling nor UP.

Observation 3: Nx only carries non-UE-associated CP signaling.

Observation 4: The TRP, responsible for transmitting and/or receiving sensing signals, is a logical function of the gNB.

Observation 5: A gNB may have one or more TRPs.

Observation 6: The TRPs are not visible outside of the RAN.

Proposal 1: Capture the above observations as general architecture principles of ISAC in the TR.

Proposal 2: As much processing as possible should be done within the RAN to generate the sensing results to be reported to the SF.

Proposal 3: A direct data connection for result reporting, to be optionally set up based on information signaled by the SF in the SENSING REQUEST (e.g., similar to MDT trace function), should be supported; it can be fully accommodated by the architecture agreed by RAN3.

Proposal 4: The protocol stack for the direct, dedicated sensing interface Nx shall reuse SCTP.

Proposal 5: To limit impacts on NG-RAN and any possible additional impacts on 6G ISAC, split gNB architecture should not be considered (or at least de-prioritized) in NG-RAN ISAC.

# References

RAN3 #129bis Chair’s Notes.

R3-257299	Summary of Offline Discussion for CB#21 ISAC, China Telecom (moderator).

R3-257325	Discussion on Network Architecture for ISAC, ZTE Corporation.

R3-256922	Discussion on Network Architecture for ISAC, Ericsson, Jio Platforms.

RP-252819	Study on Integrated Sensing and Communication (ISAC) for NR.

S2-25xxxx	Adding direct P2P interface between NG-RAN and SF as principles for system architecture (KI#1), Ericsson.

# Text Proposal for TR 38.765

START OF CHANGES

# 7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

## 7.x	ISAC Architecture Principles

- Inter-gNB coordination is out of scope of the current study, so for the purpose of this TR ISAC has no Xn impact.

- Only gNB-based sensing is in the scope of the current study, so there are no UEs involved.

- As a consequence, there is no impact on UE-associated signaling due to ISAC.

- UP is terminated in the UEs, so ISAC does not impact UP. Hence, Nx only carries non-UE-associated CP signaling.

- From the point of view of logical architecture, the TRP (Transmission and Reception Point, responsible for transmitting and/or receiving the sensing signal) is part of the gNB.

- A gNB may have one or more TRPs.

- The TRPs are not visible outside of the RAN.

- Split gNB architecture is not considered in this study.

## 7.y	Logical Architecture and Protocol Stack

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.y-1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.y-1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:	The details of "Nx" interface need further discussion and decision.

The protocol stack for Nx is based on a direct, dedicated sensing CP interface, as shown in Figure 7.y-2 below. Nx uses SCTP as transport protocol.

Figure 7.y-2 Protocol stack for the Nx interface

It seems beneficial to also (optionally) support sensing result reporting via other means, i.e., a direct data connection to be set up based on information exchanged over CP signaling between gNB and SF (e.g., a Uniform Resource Identifier (URI) indicating the streaming channel over which the sensing reports can be streamed). Such a direct data connection can be based on RFC 6455 WebSocket and should not be considered a termination of an additional interface. This provides maximum flexibility and scalability and allows to optimize the load on transport networks.

END OF CHANGES

Additional details are currently being discussed in SA2. [6]

With the agreed logical architecture, the only thing the AMF might do is route Nx packets (in which case it is not part of the logical architecture).