3GPP TSG-RAN WG3 Meeting #130                          R3-258457

Dallas, US, 17 – 21 November 2025

Title:	Discussion on support of AI/ML enabled inter-CU LTM

Agenda Item:	12.2.3

Source:	CATT

Document for:	Discussion

# 1	Introduction

During the last meeting, several companies proposed postponing discussions on AI for inter-CU LTM. However, taking into account the SID’s intent, associated workload, and the commercial deployment of AI/ML, we maintain that inter-CU LTM should be supported in R20, with details outlined in R3-258192. There is no doubt most of the AI/ML based optimization discussed and studied for Intra-CU LTM over F1 interface will be applicable for Inter-CU LTM over Xn interface. And in this contribution, we will focus on the specific part of AI for inter-CU LTM.

- Discussion

2.1 Latency Reduce in Inter-CU LTM

During execution phase, it is up to the gNB-DU implementation when will the gNB-DU signal to the CU. This does not mean that the gNB-DU is “allowed” to signal to the gNB-CU before LTM command is sent to the UE.

Follow LTM procedure, during execution phase, the source DU shall first send the cell switch command MAC CE to the UE, and then inform source CU via DU-CU CELL SWICH NOTIFICATION message. The candidate DU will start to detect UE access after it received the CU-DU CELL SWITCH NOTIFICATION message.

Compared with intra-CU LTM, the interface delay in inter-CU case is much longer. Source DU shall send the DU-CU CELL SWITCH NOTIFICATION to the source CU, source CU transfer it to candidate CU via Xn, and then to the candidate DU. The long latency may decrease LTM performance, i.e., UE finish processing, but the candidate DU still not received the CU-DU CELL SWITCH NOFTIVATION, and cannot detect UE access.

Observation 1: In inter-CU LTM, the latency among interface is longer than intra-CU LTM, which may limit UE performance.

It is considering to use AI minimize the drawback bring by longer interface latency, and try to obtain similar UE performance as intra-CU LTM. Possible solutions state as below:

- Source node send the predicted cell/beam in advance, then the candidate cell can start UE detect early

- Candidate node predicted UE access, and start UE detect before receive CU-DU CELL SWITCH NOTIFICATION message.

Proposal 1: For inter CU LTM, RAN3 support using AI to mitigate the drawback bring by longer interface latency.

- Source node send the predicted cell/beam in advance, then the candidate cell can start UE detect early

- Candidate node predicted UE access, and start UE detect before receiving CU-DU CELL SWITCH NOTIFICATION message.

2.2 UE performance feedback in inter-CU LTM

One of the core features of LTM is supporting subsequent handovers (HO), i.e., with a single RRCReconfiguration, the UE can perform multiple cell switches among candidate cells. Specifically, the UE may first be configured by the initial source gNB1, which decides to trigger LTM to the candidate gNB2. Based on the configuration provided by gNB1, gNB2 will continuously evaluate other candidate cells and may decide to trigger another LTM Cell Switch. Subsequently, the UE may further execute LTM to candidate gNB3, gNB4, and so on.

There are some differences from what we discussed regarding Rel-18/19 AI mobility. In Rel-18/19 AI mobility, we only addressed one-time handover, with feedback limited to a single hop. For LTM, candidate cells/nodes are selected by the source node. If we retain this single-hop feedback mechanism, the initial source node might select 5 candidate gNBs but only receive feedback from 1 candidate gNB. The initial source node has no chance to know whether the candidate gNBs/cells selection is suitable.

Proposal 2: RAN3 to discuss in inter-CU LTM, each time after UE execute LTM, whether the feedback needs to be sent to the initial source node.

From another perspective, in Rel-18/19 AI mobility, our common understanding is that feedback should be sent to the node responsible for initial data collection. For the LTM scenario, this node should be the initial source gNB. In subsequent LTM processes, the target cell is determined by the current serving cell. Therefore, RAN3 needs to discuss whether feedback should also be sent to the current serving cell to assess the validity of its cell switch decision.

Proposal 3: RAN3 to discuss in inter-CU LTM, each time after UE execute LTM, whether the feedback needs to be sent to the last serving node.

3	Conclusion

Observation 1: In inter-CU LTM, the latency among interface is longer than intra-CU LTM, which may limit UE performance.

Proposal 1: For inter CU LTM, RAN3 support using AI to mitigate the drawback bring by longer interface latency.

- Source node send the predicted cell/beam in advance, then the candidate cell can start UE detect early

- Candidate node predicted UE access, and start UE detect before receiving CU-DU CELL SWITCH NOTIFICATION message.

Proposal 2: RAN3 to discuss in inter-CU LTM, each time after UE execute LTM, whether the feedback needs to be sent to the initial source node.

Proposal 3: RAN3 to discuss in inter-CU LTM, each time after UE execute LTM, whether the feedback needs to be sent to the last serving node.