3GPP TSG-RAN WG3 Meeting #130	R3-258460

Dallas, US, 17-21 Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0512 | Rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Failure report without RLF report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, JIO plattforms, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was an agreement to enable failure indication without RLF report over F1AP, and a solution in stage3 was introduced. But there is no description in stage2. Pre-Rel19 UEs or UEs not capable of providing the LTM enhanced RLF report will send the legacy RLF report and should therefore be covered by the existing text. The last serving CU may use stored context to understand why a UE performing LTM cell switch still report a legacy RLF report but the current text would still apply for this case. The remaining case seems to be for cases where an RLF report is not provided from another gNB or in cases where the network chooses not to download the RLF report from the UE. Hence the triggering mechanism is not related to the UE ability but rather if the network has decided to retrieve the report. This means there is no point in restricting the usage on e.g. UE support, but we should assume that the network will be configured whether to use this signalling or not. This may lead to duplicated error reports, if the network will provide the failure indication with and without RLF report, but we assume this resolution can be handled in the involved nodes and there is no need to capture this resolution in the spec. Therefore, it is proposed to capture a new, simple statement that the forwarding to the gNB-DU may be triggered by UE re-establishing or reconnecting after failure or by receiving a Failure Indication without any RLF report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the description of the behaviour in the intermediate node (gNB-CU) in case of failure report without RLF report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No stage2 text describing the behaviour in the intermediate node in case of failure report without RLF report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.12	MRO support for LTM

The gNB-CU receives an RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis. In case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late LTM cell switch, or to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell.

If a UE re-establishes or re-connects after failure, or if the gNB-CU receives a Failure Indication without an RLF report from another gNB, the gNB-CU may send a failure report (without an RLF report) to the last serving gNB-DU in case of too late LTM cell switch, or to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt). The target gNB-DU performs initial analysis and sends the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. The target gNB-DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU.