3GPP TSG-RAN WG3 Meeting #130	R3-258462

Dallas, US, 17-21 Nov, 2025

Title:	(TP for BLCR for 38.413) Inter-CU LTM

Agenda Item:	11.2.1

Source:	Huawei

Document for:	other

# 1	Introduction

This paper takes the agreements from last meeting as a baseline, outline possible solutions and outline the potential impact on the specification.

Use cases:

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.:

BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA;

LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell;

Near failure case: SHR successful HO

Successful case: ping/pong

Solution options:

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as baseline.

For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as baseline.

For the connection failure cases:

-	After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis.

- L3 based inter-CU LTM is within the scope of the WID

# 2	Discussion

## 2.1	Enhanced RLF reporting, forwarded to the source DU

In Rel19, it was agreed that the CU that received the RLF report could perform a failure analysis and send the RLF report to the source DU together with a failure indication. Note that the receiving node may in some be the last serving DU or the DU that was serving the UE before a successful handover. In the latter case, the CU would also include the C-RNTI that was used in the source cell, since this C-RNTI is not included in the RLF report.

In general, we assume that the legacy mechanism for forwarding the RLF report shall be re-used as much as possible for the inter -CU LTM case. There are two scenarios where forwarding over Xn may be needed:

- Forwarding of RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to CU managing the last serving cell. This could be solved by re-using the Failure Indication message.

- Forwarding of the RLF report from the CU managing the re-establishment cell to the CU managing the source cell in case of failure shortly after successful HO. This could be solved by re-using the Handover Report message.

As a first step, we propose to agree on the following:

Re-use the Failure Indication message over Xn to forward the RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to CU managing the last serving cell.

Re-use the Handover Report message over Xn to forward the RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to the CU managing the source cell.

## 2.2	Recovery/re-establishment/BFR information sent from target DU to source DU

In the Rel-19 functionality, the information is forwarded from target DU to source DU via the CU. With the inter-LTM scenario, one impact is that we need additional forwarding between the two CUs. The assumption in the Rel-19 solution is that the UE is identified by the F1 APID of the receiver node. The same principle should apply also for the forwarding over Xn.

Forwarding over Xn seems quite straightforward. The only potential issue to be discussed is how to manage the UE identity. Over F1AP, the F1AP IDs are used. Therefore, it seems most straightforward to use XnAP IDs over Xn, more specifically the XnAP ID of the receiving node (the source CU). It would be possible to re-use Access and Mobility indication to transfer the information.

In the Rel19 solution, information from source and target are combined in the source. Therefore, the target need to provide the APID to enable the source to combine the received information with information stored in the source. One possible system implementation is that the CU does not release the context in the source until the information is sent from the target. It is however also possible for the source-DU to store information relevant for MRO analysis even if the context is released. Since the message is not UE-associated, we think the APID can be used even if the context would have been released in the receiving node. We think this principle can be extended also over Xn, in case we use a non-UE associated message. Therefore, we propose:

Re-use the Access and Mobility Indication message over Xn to forward MRO information from target CU to source DU.

## 2.3	Information of last visited LTM cells sent from CU to the serving DU

It was agreed that the CN can provide the list of consecutive visited cells with LTM cell switch. In order to support this in the Inter-CU scenario, the source CU need to include information on which of the visited cells the mobility was based on LTM cell switch.

Agree to include an indicator for LTM cell switch in the Legacy IE for Visited Cells.

Agree the TP included in the annex.

In addition to this, we need to also enable the transfer of last visited cells over Xn for the subsequent case. We would like to differentiate between two cases

-	Information to target cell: This is to enable candidate cells to adjust the thresholds for cell switch on order to avoid e.g. ping pong.

-	Information to the initiating node: This is to enable the initiating node to optimise the candidate cell selection, and also to determine whether it was beneficial to use subsequent or if normal LTM cell switch could have been enough.

For the information to the target cell, we believe that a straightforward solution is to use the CELL SWITCH NOTIFICATION message:

Re-use the Cell Switch Notification message over Xn to forward UHI information from source CU to target CU.

In Rel-19 functionally, it was agreed that the CU may perform detection of potential LTM ping-pong issues and provides the related UHI information together with a ping-pong indicator towards target DU. In inter-CU LTM, the ping-pong detection can still be performed by the target CU based on the received UHI from the source CU and the local UHI. Therefore,

No need to introduce the ping-pong indicator in the Cell Switch Notification message over Xn.

For initiating node, in general, there is no strong motivation to obtain the updated UHI for each subsequent LTM execution, since the target CU is responsible for considering the candidate cell configuration update for subsequent switch. Therefore we propose to:

Agree that the serving CU shall transfer the UHI information to the initiating node only upon release in the UE CONTEXT RELEASE message.

# 3	Conclusion

This document proposes:

Proposal 1:	Re-use the Failure Indication message over Xn to forward the RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to CU managing the last serving cell.

Proposal 2:	Re-use the Handover Report message over Xn to forward the RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to the CU managing the source cell.

Proposal 3:	Re-use the Access and Mobility Indication message over Xn to forward MRO information from target CU to source DU.

Proposal 4:	Agree to include an indicator for LTM cell switch in the Legacy IE for Visited Cells.

Proposal 5:	Agree the TP included in the annex.

Proposal 6:	Re-use the Cell Switch Notification message over Xn to forward UHI information from source CU to target CU.

Proposal 7:	No need to introduce the ping-pong indicator in the Cell Switch Notification message over Xn.

Proposal 8:	Agree that the serving CU shall transfer the UHI information to the initiating node only upon release in the UE CONTEXT RELEASE message.

# Annex1	TP to 38.413

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| LTM cell switch indicator | O |  | ENUMERATED (true,...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8. |

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

[snip]

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-ExtendedBackupAMFName,

id-LTMCellSwitchIndicator,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

[snip]

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastVisitedPSCellList	CRITICALITY ignore	EXTENSION LastVisitedPSCellList	PRESENCE optional }|

{ ID id-LTMCellSwitchIndicator	CRITICALITY ignore	EXTENSION LTMCellSwitchIndicator	PRESENCE optional },

...

}

LTMCellSwitchIndicator ::= ENUMERATED {

true,

...

}

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

[snip]

id-ExtendedBackupAMFName    							ProtocolIE-ID ::= 442

id-ExtendedOldAMF			    						ProtocolIE-ID ::= 443

id-LTMCellSwitchIndicator	    						ProtocolIE-ID ::= 999 – to be assigned by MCC