3GPP TSG-RAN WG3 Meeting #130	R3-258471

Dallas, USA, 17-21 Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1645 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTM Cell Swicth Type Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, LGE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During LTM execution and upon cell switch decision, the gNB-DU sends a MAC CE command to the UE which includes the TCI state indicating the UE which beams it should use to access the target cell (applicable for both RACH-less and RACH-based mobility), as well as a Timing Advance Command (TAC). Likewise, the value in the TAC indicates to the UE whether it should perform a RACH-less switch. For RACH-less case, the TCI state information is used by the UE to determine the configuration of UL transmission and DL reception in the target cell. In contrast, for RACH-based scenarios, the indicated TCI state is used by the UE after the completion of the RACH procedure. However, even though the Cell Switch Notification messages over XnAP and F1AP include the TCI State ID, they lack support for indicating whether RACH-less mobility was triggered to the UE. Therefore, the Candidate gNB-DU/Candidate gNB is unaware whether the upcoming UE access is RACH-less or RACH-based. Therefore, the target cell cannot determine whether to issue a Dynamic Grant (DG) or a Configured Grant (CG), leading to PDCCH and PUSCH resource waste. For these reasons, an IE from Source gNB-DU/gNB is required in the Cell Switch Notification messages to indicate whether the cell switch triggered was RACH-less based correspondingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a Cell Switch Type IE in XnAP CELL SWITCH NOTIFICATION message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only impacts the Cell Switch Notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Candidate gNB is unaware whether the upcoming UE access is RACH-less or RACH-based. Thus, unable to determine whether to issue a Dynamic Grant (DG) or a Configured Grant (CG), leading to PDCCH and PUSCH resource waste. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.5, 9.4 (ASN.1) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

#### 9.1.5.5	CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

This message is also sent by the source S-NG-RAN node to the M-NG-RAN node or from the M-NG-RAN node to the target S-NG-RAN node to inform the target S-NG-RAN node about the initiation of the cell switch command to the UE.Direction: source NG-RAN node  target NG-RAN node.

Direction: M-NG-RAN node  target S-NG-RAN node or source S-NG-RAN node  M-NG-RAN node (Dual Connectivity).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Include the TCI-StateId IE as defined in TS 38.331 [10]. | – |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE as defined in TS 38.331 [10]. | – |  |
| >Cell Switch Type | O |  | ENUMERATED (RACH-less, …) | Indicates the cell switch type triggered. | - |  |
| LTM UE Association Information List |  | 0..1 |  |  | YES | ignore |
| >LTM UE Association Item |  | 1.. <maxnoofLTMCells> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Last Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Cell Switch TA Information List |  | 0..1 |  |  | YES | ignore |
| >Cell Switch TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | – |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [10]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<< Next Change >>

#### -- C

<< skip unchanged parts >>

CellSwitchTAInformation-List ::= SEQUENCE (SIZE (1.. maxnoofTAList)) OF CellSwitchTAInformation-Item

CellSwitchTAInformation-Item ::= SEQUENCE {

candidateCellID					NR-CGI,

tAValue							TAValue,

tagIDPointer					TagIDPointer OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CellSwitchTAInformation-List-ExtIEs } }	OPTIONAL,

...

}

CellSwitchTAInformation-List-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellSwitchType	::= ENUMERATED {RACH-less, ...}

<< skip unchanged parts >>

<< Next Change >>

#### -- L

<< skip unchanged parts >>

LTMCellSwitchInformation	::= SEQUENCE {

jointorDLTCIStateID				JointorDLTCIStateID,

uLTCIStateID					ULTCIStateID 				OPTIONAL,

cellSwitchType					CellSwitchType 				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { LTMCellSwitchInformation-ExtIEs } }	OPTIONAL,

...

}

LTMCellSwitchInformation-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

CHANGES END