3GPP TSG-RAN WG3 Meeting #130	R3-258477

Dallas, US, 17th – 21st November, 2025

Title: 	Discussion on XnAP reporting of predicted radio resource status per slice

Source: 	Huawei

Agenda item:	9.2.2

Document Type:	Discussion and Decision

# 1. Introduction

This contribution provides some observations and proposals for discussion pertaining the reporting of the predicted radio resource status per slice over Xn based on the agreements reached in RAN3 during the Rel-19 WI Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN.

# 2. Discussion

During the Rel-19 SI Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN RAN3 discussed the AI/ML-based Network Slicing use case, and agreed to introduce new predictions for both the radio resource status per slice and slice available capacity that could be used by an AI/ML model in the gNB as captured in TR 38.743 [1]:

### 4.1.2 Solutions and standard impacts ### *** skip non-relevant text *** #### 4.1.2.2 Input data of AI/ML based Network Slicing To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing: From local node: - Measured/Predicted radio resource status per slice - Measured/Predicted slice available capacity - Legacy predicted UE trajectory From neighbouring gNBs: - Measured/Predicted radio resource status per slice - Measured/Predicted slice available capacity *** skip non-relevant text *** #### 4.1.2.3 Output data of AI/ML based Network Slicing AI/ML-based network slicing model in a gNB can generate the following information as output: - Predicted radio resource status per slice - Predicted slice available capacity - Resource management decisions for resources within RRM policies (used by gNB internally)- Slice aware mobility decisions (used by gNB internally) *** skip non-relevant text *** #### 4.1.2.5 Potential standard impacts Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18. Xn interface: - Enhanced existing procedure to collect predicted information between gNBs: ■ Predicted radio resource status per slice ■ Predicted slice available capacity

From the above excerpt, it can be noted that the legacy Rel-18 predicted radio resource status seems not needed to actually run an AI/ML model used for AI/ML-based Network Slicing use case, since only the per-slice .

Observation 1: The legacy Rel-18 predicted radio resource status seems not needed to run an AI/ML model used for AI/ML-based Network Slicing use case.

Then, during the normative phase, RAN3 discussed the stage 3 aspects related to the signalling and encoding of these new predictions and agreed the following enhancements for the Data Collection Reporting (Initiation) procedures [2][3]:

--- From RAN3#125bis --- Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface. Introduce Slice To Report List for Data Collection IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message. --- From RAN3#126 --- First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message. Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message. Introduce a Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message to report the requested predicted slice available capacity to the requesting node. Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE.

The above agreements have been reflected in the first version of Rel-19 TS 38.423 (V19.0.0 – September 2025) [4]:

### 8.4.13 Data Collection Reporting Initiation #### 8.4.13.1 General This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN. The procedure uses non UE-associated signalling. *** skip non-relevant text *** Interaction with the Data Collection Reporting procedure When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message: - the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the Slice Radio Resource Status Item IE. - the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. *** skip non-relevant text *** - the Predicted Slice Available Capacity Group IE, if the tenth bit, "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1"and if - the Slice To Report List for Data Collection IE is included for the cell, and - the measurement object is admitted by NG-RAN node2. *** skip non-relevant text *** #### 9.1.3.26 DATA COLLECTION REQUEST This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message. Direction: NG-RAN node1 NG-RAN node2. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. *** skip non-relevant text ***. Report Characteristics for Data Collection; C-ifRegistrationRequestForDataCollectionStart; BITSTRING (SIZE(32)); Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections, Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL, Eighth Bit = Energy Cost, Ninth Bit = Measured UE Trajectory, Tenth Bit = Predicted Slice Available Capacity, Eleventh Bit = Slice Average UE Throughput DL, Twelfth Bit = Slice Average UE Throughput UL, Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL, Fifteenth Bit = Slice Average Packet Loss UL. Other bits are ignored by the NG-RAN node2.; YES; reject. Cell To Report List for Data Collection; 0..1; Cell ID list to which the request applies.; YES; ignore. >Cell To Report Item for Data Collection; 1 .. <maxnoofCellsinNG-RANnode>; –. >>Cell ID; M; Global NG-RAN Cell Identity 9.2.2.27; Indicates an NR Cell Identity.; –. >>Slice To Report List for Data Collection; 0..1; S-NSSAI list to which the request applies.; YES; ignore. >>>Slice To Report Item for Data Collection; 1 .. < maxnoofBPLMNs >; –. >>>>PLMN Identity; M; 9.2.2.4; Broadcast PLMN; –. >>>>S-NSSAI List; 1; –. >>>>>S-NSSAI Item; 1 .. < maxnoofSliceItems>; –. >>>>>>S-NSSAI; M; 9.2.3.21; –. *** skip non-relevant text ***. *** skip non-relevant text *** #### 9.1.3.29 DATA COLLECTION UPDATE This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information. Direction: NG-RAN node2 NG-RAN node1. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; ignore. Cell Measurement Result for Data Collection List; 0..1; YES; ignore. >Cell Info Result for Data Collection Item; 1 .. < maxnoofCellsinNG-RANnode >; –. >>Cell ID; M; Global NG-RAN Cell Identity 9.2.2.27; Indicates an NR Cell Identity.; –. >>Predicted Radio Resource Status; O; Radio Resource Status 9.2.2.50; The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and optionally includes the Slice Radio Resource Status List IE.; –. >>Predicted Number of Active UEs; O; Number of Active UEs 9.2.2.62; –. >>Predicted RRC Connections; O; RRC Connections 9.2.2.56; –. >>Predicted Slice Available Capacity Group; O; 9.2.3.237; YES; ignore. *** skip non-relevant text ***. a

From the procedural text specified in the “Interaction with the Data Collection Reporting procedure” section of clause 8.4.13.2 of TS 38.423 pertaining the reporting of slice-related information (i.e., predicted radio resource status per slice and slice available capacity) from the gNB receiving a DATA COLLECTION REQUEST message (i.e., NG-RAN node2), the following can be observed:

- If bit 10 "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is the only bit set to "1", meaning that the sending node (NG-RAN node1) requests the reporting of the predicted slice available capacity, this also mandates the presence of the Slice To Report List for Data Collection IE in the DATA COLLECTION REQUEST message, otherwise the reporting cannot be done by NG-RAN node2;

- If bit 1 "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is the only bit set to "1", there could be the possible scenarios:

  - if the Slice To Report List for Data Collection IE is not included in the DATA COLLECTION REQUEST message, this means that the sending node (NG-RAN node1) requests the reporting of the predicted radio resource status only, i.e., the “legacy” Rel-18 predicted information;

  - if the Slice To Report List for Data Collection IE is included in the DATA COLLECTION REQUEST message, according to the current procedural text for determining the content of the Predicted Radio Resource Status IE, such procedural text may be interpreted in such a way that the sending node (NG-RAN node1) has requested the reporting of the predicted radio resource status per slice only, i.e., the “new” Rel-19 predicted information; but this is not the current behaviour because the current signalling design only allows the reporting of the Rel-19 predicted radio resource status per slice together with the Rel-18 predicted radio resource status.

This is also confirmed by checking the semantics description of the Predicted Radio Resource Status IE within the DATA COLLECTION UPDATE message, where it is stated that the IE includes the SSB Area Radio Resource Status List IE (excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE), and optionally the Slice Radio Resource Status List IE, where the “and optionally” is pending on whether the Slice To Report List for Data Collection IE is included in the DATA COLLECTION REQUEST message.

Observation 2: If a node sends a DATA COLLECTION REQUEST message to request the reporting of the predicted slice available capacity from the receiving node, this mandates the presence of the Slice To Report List for Data Collection IE in the same DATA COLLECTION REQUEST message.

Observation 3: Even if the Slice To Report List for Data Collection IE is included in the DATA COLLECTION REQUEST message, the node receving such message will provide the predicted radio resource status per slice together with the legacy predicted radio resource status.

Based on Observation 3, it should be discussed in RAN3 whether there are cases where the sending node (NG-RAN node1) requests the reporting node receiving the DATA COLLECTION REQUEST message (i.e., NG-RAN node2) to provide only the predicted radio resource status per slice when the Slice To Report List for Data Collection IE is included in the DATA COLLECTION REQUEST message, i.e., without also reporting the legacy Rel-18 predicted radio resource status.

Proposal 1: RAN3 to discuss whether there are cases where the sending node requests the reporting node to provide only the predicted radio resource status per slice when the Slice To Report List for Data Collection IE is included in the DATA COLLECTION REQUEST message.

In our view, from the XnAP signalling perspective it is not harmful for the reporting node to report both the Rel-19 predicted radio resource status per slice together with the Rel-18 predicted radio resource status. If not needed, the requesting node may eventually discard the Rel-18 predicted radio resource status, and even if this actually represents a waste in signalling resources, it may still allow some implementations of the AI/ML model hosted in the requesting node to have more predicted information pertaining the same metric, i.e., the radio resource status information, but at different granularities.

Observation 4: It is not harmful for the reporting node to report both the Rel-19 predicted radio resource status per slice together with the Rel-18 predicted radio resource status. If not needed, the requesting node may eventually discard the Rel-18 predicted radio resource status, while still allowing some implementations of the AI/ML model in the requesting node to have more predicted information pertaining the radio resource status information at different granularities.

Based on the above we think that the current signalling design in XnAP should be essentially kept as it is, but a correction should be agreed to clarify, in the procedural text describing the reporting of the predicted radio resource status per slice, that when the Slice To Report List for Data Collection IE is present in the DATA COLLECTION REQUEST message, the Rel-19 ‘predicted radio resource status per slice’ is also reported along with the Rel-18 ‘predicted radio resource status’.

Proposal 2: RAN3 to agree that, when the Slice To Report List for Data Collection IE is present in the DATA COLLECTION REQUEST message, the Rel-19 ‘predicted radio resource status per slice’ is also reported along with the Rel-18 ‘predicted radio resource status’.

The above Proposal 2 can be implemented as follows, as per the proposed Rel-19 CR to XnAP in [5]:

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>> ### 8.4.13 Data Collection Reporting Initiation #### 8.4.13.1 General This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN. The procedure uses non UE-associated signalling. #### 8.4.13.2 Successful Operation *** unmodified text omitted *** The result of the UE performance collection may be reported periodically during the Collection Time Duration for UE Performance via DATA COLLECTION UPDATE messages and, after collection is terminated, it shall be reported at the next available DATA COLLECTION UPDATE message. Interaction with the Data Collection Reporting procedure When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message: - the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, also include the Slice Radio Resource Status Item IE. *** unmodified text omitted *** <<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

Proposal 3: RAN3 to agree the Rel-19 XnAP CR in R3-258476.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Observation 1: The legacy Rel-18 predicted radio resource status seems not needed to run an AI/ML model used for AI/ML-based Network Slicing use case.

Observation 2: If a node sends a DATA COLLECTION REQUEST message to request the reporting of the predicted slice available capacity from the receiving node, this mandates the presence of the Slice To Report List for Data Collection IE in the same DATA COLLECTION REQUEST message.

Observation 3: Even if the Slice To Report List for Data Collection IE is included in the DATA COLLECTION REQUEST message, the node receving such message will provide the predicted radio resource status per slice together with the legacy predicted radio resource status.

Proposal 1: RAN3 to discuss whether there are cases where the sending node requests the reporting node to provide only the predicted radio resource status per slice when the Slice To Report List for Data Collection IE is included in the DATA COLLECTION REQUEST message.

Observation 4: It is not harmful for the reporting node to report both the Rel-19 predicted radio resource status per slice together with the Rel-18 predicted radio resource status. If not needed, the requesting node may eventually discard the Rel-18 predicted radio resource status, while still allowing some implementations of the AI/ML model in the requesting node to have more predicted information pertaining the radio resource status information at different granularities.

Proposal 2: RAN3 to agree that, when the Slice To Report List for Data Collection IE is present in the DATA COLLECTION REQUEST message, the Rel-19 ‘predicted radio resource status per slice’ is also reported along with the Rel-18 ‘predicted radio resource status’.

Proposal 3: RAN3 to agree the Rel-19 XnAP CR in R3-258476.

# 4. Reference

- TR 38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

- 3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- TS 38.423, NG-RAN; Xn Application Protocol (XnAP)

- R3-258476, Correction on procedural text for reporting of predicted radio resource status per slice, Rel-19 Cat. F CR to TS 38.423, Huawei